Color is tri-barriers

Color has 3 dimensions.

In-order to nail down a specific color – numerically,  it requires 3 numbers.

Color can be thought of as 3D object

Colours to use for Lab:

- Grey is very versatile color. Shades of grey.

- Use colors found in nation

Image Maps

- remote sending: collecting images fo the eearth from a distance

- Observing feature in the environment using films or instruments (sensors) that are sensitive to the energy emitted or reflected from objects

- Aerial Photography

- Orthophotos and Orthophotomaps

Image Maps: Aerial Photography

- Aerial photography: the image which is taken from above he ground with a camera

- Near-ultraviolet to near-infrared portion of the electromagnetic spectrum

- Platform: the vessels, crafts or instruments from which the images are taken

- For mapping two kinds of camera are in use:

  - Film camera (aerial mapping cameras)

  - Digital mapping camera

- Aerial Mapping cameras:

  - Produce individual pictures called frames (9-inch by 9-nich films)

  - Uses complex lenses to minimize geographical distortaion

  - Mapping cameras are mounted on a gyrostabilizer

- Digital mapping cameras:

  - Record picture electronically

  - Instead of photographic Film, they use CCD

- To obstain large-Scale areal photos

  - The aircraft at around10,000ft above the ground along flight line. (highly detailed)

  - The path that aircraft follows is called flight lines

- Sidelap is only 20-30% is for stereo-viewing

- Types of aerial photography:

  - Black-and-white photography

    - Use a film that records electromagnetic radiation in the visible spectrum

    - Need UV haze filter to be placed over the camera lens to increase the clarity of the photo

  - True-color photography

    - Color films cost more and its more epxensive to develop

    - More accurate to identify objects

  - Color infrared photography

    - WWII: films that were sensitive to near-infrared  (near IR) wavelengths (0.7-0.9) as well as visible light, color infrared (CIR) film

    - The leaves of plants are among the highest reflecting material in the near IR wavelength

    - The healthier the vegetation, the higher the reflection