Chapter 13

Position Finding and Navigation

Orienting the Map

- Orient a map means to determine how directions on the map align to directions on the ground.

- Before maps were drawn with China on Top, but now made using True North at the top

- Inspection Method

  - Easiest ways to orient map – inspection method

  - With this technique, do not need to know which way north is.

  - No need of tools. Only 2 conditions need to be met:

    - Must be able to see one or more linear features or prominent objects in your vicinity

    - Must be able to identify the same features on the map

  - One Linea Feature

    - Position yourself on a linear feature shown on the map, such as a road

    - Then turn the map until mapped feature lines up with the real on in front of you.

    - The line you see between you and the features is called sight line.

    - 2nd method is to assume a position that lies on a straight-line extension of the ground feature.

    - 3rd option is to take up a position to either side of the linear ground feature and then to align the feature shown on the map so that it is parallel with the ground feature.

    - 1st and 3rd have problems.

  - Two linear features (better option than one linear feature)

    - 3 options:

    - 1st – more to position on either of the two linear features,

    - 2nd – move to a point which lies on a straight line extension of either feature

    - 3rd – or assume a position off to one side of both features.

    - Turn the map until 2 features align with those on ground

    - 3rd is the least accurate

  - Prominent Features

    - Reversal in map orientation can be avoided by using  2 features to align yourself.

    - Move to one feature, or to a position on a line extended through the two features.

    - If you are located off to the side of the features, estimate your position relative to the straightedge line between the prominent features.

    - Identify ground features that can be found on the map. = success!

  - Compass Method

    - Firstly, need compass, a map showing magnetic north with declination diagram.

    - Then do this:

      - Find magnetic north indicator on map

      - Hold map under compass, turn map until compass needle lines up with magnetic north on the map

    - This will orient with true north as well.

    - Essential to take magnetic declination because true north is a bit off.

    - Magnetic north isn’t always shown on maps

    - Commit the basic declination pattern to memory.

Finding Your Ground Position

- Distance Estimation

  - Most common technique

  - In which you use your distance and direction to other objects to location your position on the map.

  - Two useful methods: by inspection and using a compass

  - Inspection Method

    - Looking at ground features without any special aids. Find ground features you find on map with ground and estimate their distance from you

    - First, orient your map and select a feature on the ground that you can also identify on the map

    - Second, estimate the distance from your ground position to the distant feature, and convert this figure to map distance unites by using the map scale.

    - Mark out the computed map distance along the proper direction line and you have established your position

    - A further complication is that farther distances are easier to estimate than nearer distances.

    - As you approach vanishing point (point on horizon where parallel lines appear to converge), it becomes harder to judge distance.

  - Compass Method

    - Distance estimation using map and a magnetic compass, which indicates directions magnetically buy the alignment of a magnetic needle or floating disk to the earth’s magnetic field.

    - Compass method of distances estimation you plot lines that interest at your position. Advantage = don’t need to orient your map before finding your position

    - Azimuth – angle measured clockwise in degrees between the direction line and the reference point. (Back azimuth = add 180 if less than 180, Subtract 180 if more than 180)

- Resection method

  - Resection method – technique for locating position that involves plotting lines that cross, or resect” at your position.

  - Position found using this method is called cross fix

  - Most accurate to use three lines.

  - Inspection Method

    - 3 features on ground n’ map

    - Backsight – draw from the known position of a distant feature back to your position. (draw these from the 3 points = your location)

    - Fix – position determined using a navigational method < find this when 2 sight lines cross at 90 degree angles

    - Error triangle – caused through intersection of the 3 points

    - Your location = assume at center of triangle

    - Reasons line don’t intersect at a point due to error in:

      - Positioning the known locations correctly on the map or in plotting the sight lines

      - Didn’t sigh to the correct feature

      - Mapmaker misplaced one or more of the features on the map

  - Compass method

    - Compass method of resection – makes use of back azimuths as resection lines

- Measurement instrument

  - Stadia

    - Stadia principle – using a constant angle to determine a distance between a marker and a sighting device. These devices have built in pair of horizontal wires, called stadia, that are spaced to subtend a small angular distance

  - Laser rangefinder

    - “Point and Read” instrument.

    - Very accurate.

    - Infrared laser beans built in binoculars and shit

    - Known also as electronic rulers

  - Altimeter

    - Used to measure the height above fixed level, usually sea level

    - Mountain regions, cloudy, forested, used there.

    - Kinda shitty cause of calibration problems due to being like a barometer using air pressure.

    - Better one = temperature compensated altimeter – compensates for the effect of temperature changes.

    - To establish position altimeter

    - Linear Feature Method:

- Read elevation from altimeter

- Follow along linear feature on map until you come to elevation indicated by altimeter

- This will be approx. ground position

- Method works if the linear feature has a steadily changing elevation.

  - Backsight line method:

    - plot backsight line to visible feature

    - read elevation from altimeter

    - follow along sight line until reach this elevation value

Locating a Distant Point

- Intersection method

  - Made by foresights – lines from your position to a distant feature

  - Using 3 lines increases accuracy

  - Two main techniques:

    - Inspection method – orient map > straightedge from map to distant ground feature > plot. > do same from 2nd position. > found feature location

      - Two fixed positions can also be used to determine location of a distant point

    - Compass Method – Sighting on a the distant feature, and recording the foresight azimth as noted on the compass.  Do from 2nd position.

      - Place of intersection is the map location of distant feature.

- Trilateration Method

  - Method for finding location of a distant feature using only the distances from known locations to the feature.

  - Drawing intersecting arcs of circles from two points on a sheet of paper.

Maps and Navigation

- Navigation is a two part activity – planning your route then following route

- Planning = roads, places stuff maps and shit

- Land Navigation

  - Land navigation restricted to specific routes such as trails, subways or railways.

  - Simple maps cause don’t care for environment.

  - Maps and Atlases

    - Peutinger table – romans road maps. Showing road network. Ignored distances and exact direction

    - Railroad maps – arranged to show favorable impression

    - Standard Road Map – general picture for route finding. Defined by state, region, or country. Variety of info.

    - TripTik – contains a planned route between your departure point and arrival.

    - Strip Map – same as triptik, but on multiple paper in order

  - Route Planning

    - Involves determining where you will go and how you will go there

  - Personal Navigation Systems

    - Combine cellphone and GPS tech into navigation system.

  - Vehicle Navigation System

    - Build into car or extensions.

- Marine Navigation

  - Nautical Charts

    - Map specific for sea navigators

    - Designed to be drawn on. Courses (routes)

    - Show soundings, isobaths etc.

    - Hydrographic charts – inland waters.

  - Marine navigation methods

    - Piloting – plan intended route and then find your position and direction of movement by paying attention to landmarks.

    - Dead reckoning – estimating your present position relative to your last accurately determined location by using direction, speed, distance, and time information.

      - Dead reckoning plot

      - Intended dead reckoning tracks

      - Course heading

    - Azimuth is called Bearing.

- Air Navigation

  - A lot of maps for different types of settings.