Map Projection

- a map projection is a geometrical transformation of the earth’s spherical or ellipsoidal surface onto a flat map surface

Map Projection Properties

- Scale

- Completeness

- Correspondence relation

- Continuity

Map Projection Properties: Scale

- Scale

  - The stated scale is true only at selected points or along particular lines

- There are two map scales:

  - Principal scale: the scale of generating globe (no variation)

  - Actual Scale: the scale we measure at any point on the map. Scale variations here is the result of geometric distortion

  - SF = Actual Scale / Principal scale

Map Projections Classification Categories

- Projectin families based on map projection surface

  - Projection surface

  - Developble Surface: the flat surface that the earths features are projected onto

  - Three projections families based on the developable surface:

    - Planar

    - Conic

    - Cylindrical