Monday, October 22nd

3 sections

A: 4 short answers worth 5 marks

- What are the different product environments and the difference between them?

  - ETO: products require a unique engineering design based on customer specifications

  - MTO: manufacturing does not start until an order is received. The product is usually is made from standard raw material and possibly some custome-designed components, but does not require as much design lead-time as ETO

  - ATO: the product is made from standard components or sub-assemblies helf in inventory awaiting a ustomer order for final assembly

  - MTS: the manufacturer prodcues products and sells them from finished goods inventory. The supply is available before receipt of a customer order

  - Mass customization: the product is made in many different configuarations mainly from stock components, buy is quickly assembled and in high volume.

- Characteristics of intermittent manufacturing processes

  - Intermittent lot or batch production

  - Work moves only to require stations

  - Many different parts processed at work stations

  - General-purpose machinery

  - Relatively east ot change product or volume

  - Complex and expensive production and inventory control

  - High WIP inventory levels; long lead time

- Why do organizations create forecasts for demand and what are the sources of demand

  - Forecasts: predictions of future  demand based on quantitative or qualitative methods, or a combo of both

  - Customer orders: these orders are from external customers for particular prodcts or number of products, often reffered to as actual demand as opposed to a forecasted demand, including service parts and spare parts

  - Replenish orders from distribution centers: based on customer order places on distribution centers and on distribution center forecasts

  - Interplant transfer: orders for manufactured parts or components from toher divisions in the company

  - Other sources of demand: examples include orders for products needed for marketing and product demonstrations

- What do we use forecast measurements for?

- What are the functions and characteristics of sales and operation planning?

  - Translates the strategic business plan into production rates that meet business goals

  - Continually updates the production, financial, and sales plan

  - Seeks input from various functions

  - Requires regular meetings with senior executives to resolve tradeoffs

  - Heck availability of resources to validate the production plan

- Describe chase production strategy

  - The goal is to produce the quantity demanded at any time. Because inventory does not build up, the cost of inventory is kept at a minimal. Production is adjusted to match sales requirements.

  - Stable inventory

  - Varied production to meet sales requirements

- What are the inputs to MRP

  - (determine requirements and keep priorities)

  - MPS (planned and scheduled orders for end items. End item quantities and due dates)

  - Product structure file (BOMs. Components required, quantity of each component per parent)

  - Inventory record file (inventory status and lead times)

  - Item master file (planning factors. Lot size, lead time, yield and scrap factors, safety stock)

- What are the uses of Bills of Materials?

  - A listing of all the subassemblies, intermediates, parts, and raw materials that go into a parent assembly showing the quantity of each required to make an assembly.

- Identify and describe components of lead time

  - Queue: time spent waiting before an operation begins

  - Setup: time spent getting ready for an operation

  - Run: time spent performing an operation

  - Wait: time spent waiting after operation ends

  - Move: time spent physically moving materials between operations

- If a work center is over/under loaded, what can be done to solve the problem?

  - Changing capacity

    - Use overtime or undertime

    - Hire or lay off

    - Add shifts

    - Use alternative routings

    - Subcontract

    - Reduce setup

    - Improve processes

  - Altering load

    - Alter lot size

    - Reschedule

B: 6 calculations worth 35 marks

- PAB and MPS

- ATP

- BOM: how many of certain components are required

- Utilization efficiency and available time to calculate capacity

- Calculate cumulative variance planned and actual backlog

C: 20 multiple choice worth 1 mark each

Season 1:

- why is manufacturing important to the economy?

  - It adds value to products to generate wealth

- In a traditional manufacturing environment, which of the following objectivs are in conflict?

  - Minimizing poruction costs and decreasing inventory investment

- Which of the following statements are true?

  - The objectives of marketing and production can be met with higher inventories

- Whih of the following manufacturing environments, or manufacturing strategies produces the longest delivery lead time?

  - Make to order (MTO)

- Which of the following is a major objective of materials management?

  - Provide the required level of customer service

- Which of th following are primary activities of  MPC?

  - Production planning and inventory management

- Which of the following is the performance measure that relates to daily work routines, such as cycle time and utilization?

  - Operational

- Wich of the following systems is unique in its focus on the reduction of defects by reducing variation?

  - Six sigma

- Which of the following statements is true about the product layout, or the continuous process?

  - There is little build up of work in process (WIP) inventory

- Which of the following statements is true regarding process layout or intermittent manufacturing

  - Each work station must be flexible

Session 2:

- which of the following is a characteristic of demand?

  - Seasonal variation

- Product A is made from components B and C. Component B is made from parts D and E. which items should be forecasted?

  - Only A

- Which if the following is the best statement about the general principles of forecasting?

  - Every forecast should include an estimate of error

- What importat assumption is made about quantitative forecasing methods?

  - The past is a valid indicator of the future

- Which forecasting technique takes the average demand for some past number of periods?

  - Moving average

- Why is it important to monitor the forecasts?

  - To improve out forecasting methods

- Which of the following statements is most accurate?

  - The seasonal index is an estimate of how much demand furing the season will be above or below the average demand.

- Which of the following causes forecast error?

  - Random variation from the average demand

- Which of the following statements is more accurate?

  - Independent demand items should be forecasted

- Which of the following statements is most accurate?

  - The circumstances relating to demand data should be recorded

Season 3:

- Which basic production planning strategy will build inventory and avoid the costs of excess capacity?

  - Level production strategy

- For the purpose of production planning, product families should be established on the basis of:

  - Similarity of manufacturing process

- If the opening inventory is 100 units, sales are 500 units, and the ending inventory is 200 units, what will the manufacturing produce?

  - Sales + ending – beginning = 600 units

- Under which of the following circumstances will firms generally make-to-stock?

  - Required delivery times are shorter than the time needed to make the product

- The information needed to develop a master schedule will come from which of the following?

  - The forecast of individual end items

- Which of the following is a characteristic of the master production schedule (MPS)?

  - It is an agreed-upon plan between production and marketing and sales

- Which of the following  is an objective of an MPS?

  - Maintain the desired level of customer service

- Which of the following statements is most accurate about the MPS?

  - If it is poorly done, we can expect past-due schedules and unreliable delivery promises

- What is the name for the portion of inventory or production not committed to customer orders?

  - ATP

- Which of the following is true about time fences in a master schedule system?

  - Changes far out in the planning horizon can be made with little or no cost manufacturing

Session 4:

- which is the independent demand item?

  - X

- Which of the following is an objective of MRP?

  - Keep priorities current

- What is the name of the set of priority planning techniques for planning componenet items below the end-item level?

  - MRP

- Which of the following is a major input to an MRP system?

  - The production plan

- What is the name of the document that showsthe componenet parts and the number of parts needed to make one of an assembly or subassembly?

  - BOM

- Which of the following statements is true?

  - A part has one and only one part number

- which is accurate?

  - Item A is the parent of item B

- What is the term for the ability to relate the demand for an item shown on an MRP record back to the parent causing the demand?

  - Pegging

Session 5:

- A work center consists of 2 machines working 8 hours ad and 5 days a week. Utilization has been 80 percent and efficiency has been 90 percent. What is the rated capacity?

  - Available time x utilization x efficiency

- What term indicates the percentage of time the work center is active compared to the available time?

  - Utilization

- What term is used for the ratio of standard hours of work produced to actual hours worked?

  - Efficiency

- Which of the following shows the capacity required at each work center based on planned and released for each time period of the plan?

  - Load profile

- Which of the following PAC activities must be done when planning the flow of work?

  - Be sure the material and tooling needed are available

- Which of the following is concerened with very short-term detailed planning of the flow of orders through manufacturing?

  - PAC

- Which of the following is an element of manufacturing lead time?

  - Wait time

- What is the name given to a scheduling system in which the last operation on a routing is scheduled first for completion on the due date?

  - Backward scheduling

- Which of the following statements is most accurate?

  - A bottle neck will control the throughput of all products processed by it.