Unit 2A: Chapter 10 – Mechanical Hazards and Machine Safeguarding plus Lecture 2A

Ontario Legislation for Guarding in General can be found in:

  - Construction Projects

  - Health care & Residential Facilities

  - Industrial Establishments

Machine Guarding=found in Industrial Establishments and only tells u what performance measure you have to meet, but doesn’t tell you how to do it specifically.

Mechanical Hazards = associated with power driven machines, whether automated or manually operated

-even with all the advances in safeguarding these hazards still huge concern

Ie an unguarded industrial mixer is an entanglement hazard

Common Mechanical Injuries =injury that have occurred due to misuse of a power driven machine

-these types of injuries include:

Cutting and Tearing = occurs when body part comes in contact with sharp edge, seriousness depends on how much damage is done to the skin, veins muscles and bones

Shearing = results from forces being applied to a body by 2 contacting parts moving parallel to the plane  of contact ….like paper cutter ….amputate fingers and hands

Crushing =wen part of the body is caught between 2 hard surfaces that progressively move 	 together, thereby crushing anything between them…there are 2 Types of these hazards:

-Squeeze point: when there’s 2 hard surfaces with one being in motion, come together

-Run in point: 2 objects, at least one is rotating, come progressively closer …ne gap between 	them doesn’t have to b completely closed, only needs to b smaller than the body part lodged in ie belt pully or pizza dough machine……nip points = spot where finger, clothes hair can get caught in

Breaking = caused by machines used to deform engineering materials …break in bone which is called a fracture

Simple fracture = break in bone that does not pierce to the skin

Compound Fracture= bone pierces through surrounding tissue and skin

Incomplete Fracture= leaves bone in one piece but it’s cracked

Transverse Fracture = break straight across bone

Oblique Fracture = diagonal

Comminuted Fracture = bone breaks into # of small pieces at point of fracture

Straining and Spraining

Strain=overstretched muscle and Sprain = result of torn ligament …both cause swelling and pain

Puncturing

-object penetrates straight into body and pulls back out, creating wound (like getting stabbed with knife)

RISK ASSESSMENT IN MACHINE OPERATION (process of quantifying the level of risk)

Answers the Q’s: How severe r potential injuries? How frequently are employees exposed to these potential hazards? Whats possibility of avoiding hazard if it occurs?  Whats the likelihood of injury if the safety control system fails?

REQUIREMENTS FOR ALL SAFEGAURDS

1)Prevent Contact =with potentially harmful machine parts

2)Be secure and Durable = so that they cant be removed to increase production…also shouldn’t wear out easily

3)Protect Against falling Objects = shield the moving parts of machines from falling objects

4)Create no new hazard = thus should not have sharp edges

5)Create no interference = so that they don’t fuck with production deadlines

6)Allow Safe maintenance = allow it to be done easily, w/o having it to be taken off every time

POINT OF OPERATION GAURDS

-these are machine guards that provide protection right at the point where the user operates the machine …there are 3 types:

Fixed Guards: provide permanent barrier b/w worker and point of operation …these are limited to specific operation, hard to clean and maintain, advantage is they r very specifically applied and are used on high production/repetitive machines

Interlocked Guards: shut down machine when guard not in place properly …therefore very safe wen machine jammed but disadvantage is hard to maintain

Adjustable Guards: provide barrier against a variety of dif hazards associated with dif production operations …advantage being that they r flexible and disadvantage being there not dependable

POINT OF OPERATION DEVICES

Include…

Photoelectric Devices = optical devices that shut down machine wen light field is broken , but u can still get threw if u do it quickly without the machine getting shut down, this is an engineer control on the hierarchy list

Pressure Mats= if ur on it, then machine will shut off

Radio-frequency Devices = capacitance devices that brake(stop) the machine if the capacitance field is interrupted by a workers body

Electrochemical Devices = contact bars that allow only a specified amount of movement b.w the worker and the hazards …if worker moves contact bar beyond specified point, machine wont cycle

Restraints= restrict persons hands from reaching ne further then they need to

Pullback Devices = pull the operators hands out of the danger zone when the machine starts to cycle

Restraint Devices = hold the operator back from the danger zone …limits operators movement

Safety Trip Devices = tripe wire, rods and body bars …all of these stop the machine wen tripped

2 hand Controls = requires operator to use both hands concurrently to activate the machine…ensures hands can’t stray into the danger zone

Gates: provide barrier between danger zone and the workers

MACHINE GUARDING SELF ASSESSMENT

-done to see if safeguards are proper, done by supervisor or employees and should answer the question located on PAGE 173

ROBOT SAFEGAURDS

-main hazards associated with them include: entrapment of worker b/w robot and solid surface…impact with moving robot arm and ….impact with object ejected or dropped by robot

-best way to guard against this is to provide a barrier around robots work envelope = total area within which the moving parts of a robot actually move …barrier should withstand heaviest object robot ejects

-could barriers include a guard containing a sensing device that shuts down robot if person comes too close …this Is important cuz robots are unpredictable and them being motionless could just mean they r changing between cycles

CONTROL OF HAZARDOUS ENERGY

Lockout/tagout systems = system for incapacitating a machine until it can b made safe to operate

Lockout= physically locking machine with lock

Tagout = applying tag that warns employee not to operate machine

Some terms involved with this system:

Affected employee=workers who perform their job in areas in which this system is implemented

Authorized employee = perform service or maintenance on a machine and use lockout/tagout for their own protection

Energized = a plugged in machine, or an unplugged machine that still has E stored

Capable of being locked out =when ne one of the following criterions are met…

i)has hasp to which lock can b attached

ii)has integral part through which a lock can b attached

3)has built in locking mechanism

4)can b locked w/o permanently dismantling, or replacing E isolating device

Energy isolating device = prevents release or transmission of E

Energy Source = something that can activate machine/piece of equipment

Energy Control procedure = written document, with info on how to properly control hazardous energy when shutting down machine

Energy control program = prevents accidental energizing of machines during maintenance

Lockout = helps with energy control program by placing lockout device like a padlock on energy isolatingdevice

Lockout Device = ne device that uses positive means to keen an E isolation device in safe position so that it doesn’t accidentally energize machine during maintenance

Tagout= placing tag on an energy isolation device to warn ppl not to accidentally energize machine

Tagout Device = a prominent warning device such as a tag that’s to be stuck on an energy isolation device

Control – Lockout (Lecture)

Protect from unexpected release of kinetic or potential energy

  - Electrical Contact from terminals of…...

    - Equipment, panels, motors

    - Storage devices (capacitors, batteries etc.)

  - Mechanical hazards = hazard even wen turned off like lifting devices, pressure in pipes, high temp. in vessels

    - Belts, conveyors, springs

    - Pneumatic / hydraulic actuators & controls

    - Movable parts stopped at other than their “zero energy” position

    - Objects or materials in motion

- Electrical Energy: low & high voltage equipment & conductors

- Kinetic Energy: moving equipment or materials

- Potential Energy: pressurized, suspended, elevated or coiled materials

- Radiation: Energy of ionizing or non-ionizing radiation

- Chemical Energy: chemical reactions or flammable, combustible or corrosive substances.

- Thermal Energy: Energy through hot or cold materials: Steam, hot water, gases, liquid nitrogen

Lock-out Tag-out - Basic Procedures

Prepare for shut down=Identify all types and sources of energy

Shut Down =Perform normal stopping procedures and Visually check that all moving parts have stopped

Isolate=Operate all isolation devices & dissipate or block stored energy

Apply lock and tag=Each worker applies lock & tag to each isolation device

Verify Isolation=Operate all activating controls/devices to ensure isolation and Return all activating controls/devices to OFF

Evaluating Lockout/Tagout programs

-lockout/tagout violations are frequently cited during on site inspections

QUESTIONS TO ANSWER ARE ON PAGE 179

GENERAL PRECAUTIONS

-operators should b trained in safe operation/maintenance plus in emergency procedures during accident.

-employees should kno how to activate emergency shutdown

-maintenance of safe gaurds should b done on the regs, supervisors should ensure they r in place prprly

-operators should dress properly to operate machine,no long hair etc..

BASIC PROGRAM CONTENT

-machine safeguarding should b organized systematic and comprehensive ….should have the following elements:

-safeguard policy

-machine hazard analysis

-lockout/tagout materials and procedures along with employee training

-comprehensive documentation

-periodic safeguarding audits

TAKING CORRECTIVE ACTION

-should be done wen u observe a mechanical hazard …with a key being immediacy

Hazard Recognition Barrier Guard

=gotcha stick used to see if it would hurt ur finger, at the top of the ruler, it gets narrower and narrower, top reps pinky area widest is ur wrist and mid is ur hand