Types of DC Motors

- Permanent magnet motors

  - Uses a magnetic to supply field flux

  - Has excellent starting torque

  - Good speed regulation

  - Limited to the amount of load they can drive

  - Found on low Hp application

  - Limited to 150% of rated torque to prevent demagnetization of the magnet

>>Used in:

      - Pumps

      - Fans

      - Compressors

      - Small hoists

      - Centrifuges

      - Power tools

- Series Motor

  - The field is connected in series with the armature

  - The field is wound with a few turns of large wire because it must carry the full armature current

  - Large starting torque

  - Speed varies widely from full load to no load

  - The speed with no load increases to the point where the motor can be damaged

    - A load must always be connected

  - Not suitable for variable speed applications

>>Used in:

      - Elevators

      - Small electrical appliances

      - Winches

      - Trains

- Shunt Motor

  - The field is connected in parallel (Shunt) with the armature windings

  - Good speed regulation

  - An advantage of separately exciting the shunt field is the ability of a variable speed drive to provide independent control of the armature and field

  - Simplified control for reversing

>>Used In:

      - Machine tools

      - Winding/unwinding machines

      - Lathes

      - Drills

- Compound Motor

  - Series field provides better starting torque

  - Shunt field provides better speed regulation

  - Series field can cause control problems in variable frequency drive applications

>>Used in:

      - Agitators

      - Mixers

      - Presses

      - Reciprocating machines

Speed Torque Curve

      - At the point of equilibrium, the torque produced by the motor is equal to the amount of torque required to turn the load at constant speed.