Torque= Force x radius (n x m)
                     (lb x ft)

S= distance/time

W= F x d (Joules)

Power (hp) = T x N/5252

Power (watts) = T x N/ 9.55

Ex. A motor develops a starting torque of 150 NxM if a pulley on the shaft has a diameter of 1m calculate the breaking force needed to stop the motor.

T= 150NxM

d= 1m  r= 0.5m

T= F x R
F= T/R
=150 NxM/0.5
=300N

Ex. In tightening a bolt a mechanic exerts a force of 200N at the end of a wrench having a length of 0.3m. What is the torque?

T= FxR

=200N x 0.3m
=60NxM

Ex. If the max torque is 50 NxM how long a bar is needed if the applied for is 35N?

T= FxR
r=t/f

= 50 NxM/36N
=1.39m

Ex. If a max torque required is 100lbxFt how long a bar is needed if the applied force is 13.25lbs?

T= FxR

R= T/F
= 100lbxFt/13.25lbs
=7.5ft

Ex. A fan requires 50lb/ft of torque to turn at its rated speed of 1725RPM what is the min hp rating for the motor to drive the fan?

Php= TxN/5252
= 50lbft x 1725RPM/5252
= 16.4Hp

How much torque does a 50Hp motor produce if it has a rated speed of 1200RPM?

Hp= TxN/5252

T= Hp(5252)/N
=50Hp(5252)/1200RPM
=218.8lbft

Efficiency of a Motor

- Ratio of useful power output to the power input

%E=Pout/Pin x100
- efficiency is low when thermal energy is converted mechanical energy

- Steam turbines 25-30%

- Combustion engines 15-30%

- Electric motors 75-98%

Ex. A 150kW electric motor has an efficiency of 92% when it operates at full load. Calculate the losses.

150kW=Pout

Pin= Pout/0.92 = 163kW
Losses= Pin-Pout
=163k-150k
=13kW

Ex. If a motor is connected to a 500VDC source and is drawing 5.5A what is the efficiency of the motor if it produces 2.38Hp.

%E=Pout/Pin x 100

1hp=736W

=2.38Hp(746)/500(5.5A)

=64.6%

Ex. A 13hp motor is connected to a 500V, 28A supply. What is the efficiency of the motor?

%E=Pout/Pin x 100
=13Hp(746) / 500(28)
= 69.3%

Speed regulation

- Classification of motors on the basis of how the speed changes with load

- If the speed of a motor is relatively constant over a normal range

Speed Regulation(SR)= Snl – Sfl/ Sfl x 100

Ex. A shunt motor turns at 1550RPM with no load, at a full load it slows to 13.25RPM. What is the %SR?

%SR= Snl – Sfl/Sfl x100
1550RPM – 13.25RPM/ 13.25RPM x 100

= 16.98%

Ex. %SR=?? Sfl=615RPM Snl=700RPM
%SR=Snl – Sfl/Sfl x100
700 – 615/ 615 x 100
13.8%

Ex. A hoist is lifting a 2000lb iron ball, the drum of the hoist has a diameter of 8”. The drum turns at a steady speed of 180RPM. The motor is a DC motor with a full load amperage(FLA) of 90A at 250VDC. With no load it turns at 220RPM.

- How much torque is the motor providing?

- How much Hp is the motor providing?

- What is the input power?

- %E=??    %SR=??

- T=FxR
=2000lbsx0.33ft
=667lb/ft

- TxN/5252
=667x180/5252
=22.8Hp

- 250V(90A)
=22.5kW

- =75.6%
=22.2%