Chapter 14 Lecture 20: Mass Wasting/Slope Failure

The Hope Slide 1965-Largest slope slide in Canada

Mass wasting-the downslope movement of rock and soil under influence of gravity

The role of wasting:

Geological Process follows weathering

Combined effects of mass wasting and running water (erosion) produce stream valleys

Controls and Triggers of Mass Wasting

Gravity is the controlling force

Other Contributing Factors

Saturation of materials with water

Over steepened slopes-unstable

Loss of anchoring vegetation

Earthquake ground vibration

The role of water

Saturation of the materials with water

Reduces particle cohesion

Water adds weight

Oversteepened slopes

Stable slope angle (angle of repose) is different for various materials (25-40 degrees)

Removal of vegetation

Root systems bind soil and regolith;leaves protect slope surface from raindrops

Earthquakes as triggers

Can dislodge large volumes of rock and unconsolidated materials

Liquefaction

Classification of Mass Wasting Process

Type of motion

Fall (detached)

Slide (materials move along a surface of coherent mass)

Flow (material viscously deforms)

Type of materials

Debris,mud,earth,rock

Rate of movement (fast to slow)

Falls

Rockfalls are common type of extremely rapid mass movement in which rocks of any size fall through the air

Trigger: undercutting, earthquakes, frost wedging

Rockslides: slide of material on some surface

Blocks of bedrock slide down a slope

Generally very fast and destructive

Common on inclined slope slopes

Triggers include earthquakes…etc

--------------------------------------------------------------------------------

Structural/ Slope Engineering

Simple slope stability calculation

Factor of safety= resisting forces /driving forces

Cohesion and Friction: Engineering Material Properties

Slope force required to Slip

Applied Normal Force

Normal Force

Shear Force -                                                        Sliding Surface Area A

Water Pressure on Fractures and Pores

Circular Surface Failure

Risk

Likelihood

Extent of Hazard

Unstable Geology

-Rock type

-Slope

-Weathering

-Structure

Trigger Potential

Rain Frost earthquakes

Rockfall

-Slide Runout

X Consequnce

Vehicle Frequency

Occupancy

Avoidance

Speed

Type of Vehicle

Critical Path