Ch. 2

- Total energy is the sum of all forms of energy; in the absence of magnetic, electrical and surface tension effects it consists of potential, kinetic and internal energies.

- Thermal energy is the latent form of internal energy and is referred as heat.

- Mechanical energy creates mechanical work, thermal energy cannot create work, and the forms of mechanical energy found in a fluid stream are kinetic, potential and flow energies.

- Natural Gas (CH4) is a fuel and a major energy source. Hydrogen is not an energy source because there are not hydrogen reserves in the world, but it can be converted into a fuel through special systems, therefore we can call it an energy carrier.

- Energy can cross the boundaries of a closed system in the forms if heat and work.

- When energy cross the boundaries of a closed system because of a temperature change it is called heat, when it crosses in all other forms is called work.

- The adiabatic process is a process in which there’s no heat transfer, an adiabatic system is a system in which there’s no heat interaction with its boundaries.

- Point functions depend on the state while path functions depend on the process’s path. All properties are point functions, while heat and work are path functions.

- Mechanical efficiency is the ratio between mechanical energy output to mechanical energy input.

- A mechanical efficiency of 100% for a hydraulic turbine means that 100% of the mechanical energy received is completely transformed into mechanical work.

- The combined pump-motor efficiency of a pump and motor is defined as the ratio of increase in mechanical energy of the fluid to the electrical power consumed by the motor. The combined motor-pump efficiency cant be greater than each efficiency separately because both are less than one, and the product of two numbers less than one is less than each number.

- Turbine efficiency= mechanical energy output/mechanical energy from fluid

- Generator efficiency= electrical power output/mechanical power input

- Combined efficiency= electrical power out/mechanical energy from fluid

- Thermodynamics is the study of the interaction between a system and its environment.

- A thermodynamics system consists in a collection of mass inside a boundary.

- A system boundary that separates a system from its surroundings may be:

  - Real or imaginary

  - At rest or in motion

  - Change its size and/or shape

- Work and heat can be transferred across system boundaries.

- Environment is everything around a system boundary.

- A control volume is a system with an identifiable volume with flow in and out.

- Types of thermodynamic systems:

  - Closed: No mass transfer through boundaries. Energy exchange with surroundings possible.

  - Open: Energy and heat transfer possible through boundaries. Control volume.

  - Isolated: No interaction at all with surroundings.

- Control surface: Boundaries of a control volume, real or imaginary.

- The thermodynamics state of a system is defined by the specifying values of a set of measurable properties enough to determine all other properties.

- A system state is a condition of the thermodynamics system described by its properties. If none of the properties change with time, is a steady state.

- Types of properties:

  - Intensive: No dependence on size.

  - Extensive: Dependence on size

- Formulas:

- Equilibrium is a state in which properties have definite unchanged values.

- For a system to be in thermodynamic equilibrium it must satisfy:

  - Thermal equilibrium (same temperature)

  - Mechanical equilibrium (same pressure)

  - Chemical equilibrium (constant chemical composition)

  - Phase equilibrium (mass equilibrium between phases)

- The zeroth law of thermodynamics states that if two systems are independently in thermal equilibrium with a third body,  they are also in thermal equilibrium with each other.

- Freezing point is a mixture of ice and water in equilibrium with air saturated with vapor at 1 atm.

- Steam point is a mixture of liquid and water vapor in equilibrium at 1 atm.

- Thermodynamic scales:

  - Farenheit

  - Kelvin

  - Rankine

- Pressure is a normal force excerted by a fluid per unit area.

  - Formulas

- A simple compressible system does not involves electrical, magnetic, gravitational, motion and surface tension effects.

- The state postulate states that to determine the state of a simple compressible system we only need to know two independent intensive properties

  - Examples: Temperature and Pressure

  - Exclusions: Pressure only in single phase state

- Process is a change from one equilibrium state to another.

- Path is a series of states that a system passes through a process.

- A thermodynamic cycle is a sequence of processes in which the final state is the same as the initial state of the process.

- Quasi-equilibrium is a specific process in which a system remains infinitesimally close to equilibrium state at all times. It is easy to analyze and very convenient in engineering for its versatility in calculations of properties.

- In a steady state or steady-flow process, properties do not change with time.

- Types of processes:

  - Isothermal: constant temperature

  - Isobaric: constant pressure

  - Isochoric: constant specific volume

  - Steady-flow: long periods of time under same conditions

- Formulas: