Chemistry Module 4

Safety Hazards

- Based on the nature of the work, safety hazards are due to material handling, machines, energy, work practices, and confined spaces.

- Material handling hazards: These can lead to muscle strains due to lifting and carrying materials. The use of devices such as forklifts, cranes, etc. exposes the worker to accidental contact with the moving equipment.

- Machine hazards: Rotating shafts, moving belts, presses, etc. can cause very serious injuries.

- Energy hazards: All energy sources such as electricity, steam, hydraulic pressure, etc. can lead to serious injury.

- Work practice hazards: Failure to follow safe operating procedures may lead to serious injuries.

- Confined space hazards: In such spaces (for example, silos, storage tanks, pipelines etc.) hazards arise due to difficulty of entry and exit, build-up of hazardous materials and oxygen deficiency.

Health Hazards

- The extent of occupational health hazards varies with the type of activity. However, such hazards may have the potential to cause severe discomfort, illness, and lack of efficiency among workers. Based on the nature of the causative agent (or factor), occupational health hazards can be classified as physical, chemical, biological and ergonomic.

- Physical hazards: due to the presence of physical agents such as pressure and temperature extremes, excessive noise and vibration, and exposure to radiation.

- Chemical hazards: due to the presence of chemical agents such as dust, fume, gas, mist, smoke and vapor.

- Biological hazards: due to the presence of biological agents such as bacteria, moulds and viruses.

- Ergonomic hazards: due to ergonomic stresses such as fatigue and repetitious work.

Physical and Chemical

- Causative agent: classified as hazardous in certain circumstances such as high concentration or intensity and a prolonged exposure

Workers have the right to:

- Work in a safe and healthy environment

- Know the processes and substances they are working with

- Know potential hazards from these processes and substances

Hazard Assessment

- In order to recognize and assess the potential impact of occupational hazards, walk-through surveys are performed. Such surveys generally include a study of the following parameters in terms of their impact on the surrounding environment and thus the workers.

- Processes, operations and related activities: generally various emissions are evaluated because the emission of any physical, chemical or biological agents has the potential to be a health hazard.

- Equipment: generally assessed in terms of mechanical and electrical safety and the potential to create excessive noise and vibration.

- Properties of substances used and produced: an evaluation of raw materials and finished products in terms of their physical, chemical and toxic properties and effects.

- Control measures: all engineering controls in place for proper materials handling, storage, etc. are evaluated. Also considered are the ventilation system and the availability of personal protective equipment.

Physical Agents

- Although we are constantly exposed to such agents, their intensity and duration of exposure may be more significant in the workplace. Such agents may cause immediate or cumulative adverse health effects.

- Commonly encountered physical conditions with a potential to cause adverse health effects include: pressure and temperature extremes, excessive noise and vibration and radiation

- The action of physical agents consists of transfer of residual energy through the surrounding air or the equipment the worker is in contact with. Except for radiation, our senses can detect all other physical agents.

Pressure Extremes

- Our bodies are conditioned to work at normal atmospheric pressure. Any extreme can be detrimental to one's health. Two types of abnormal pressure-related conditions are hyperbaric and hypobaric.

- Hyperbaric: a pressure higher than normal atmospheric pressure. Mining and underwater workers may be exposed to hyperbaric conditions.

- Hypobaric: a pressure lower than atmospheric pressure. Hypobaric conditions are encountered by those working at high elevations such as ski instructors and airline pilots.

Temperature Extremes

- The biochemical processes in one’s body take place within a very narrow temperature range and hence the regulation of body temperature is an important function. Temperature extremes affect the working efficiency as well as the health of a worker.

- Hot or cold depends on various factors

- It is important to realize that apart from the surrounding temperature, sensation of hot or cold depends on some other factors such as: air movement, hot or cold objects in the vicinity and relative humidity

- The terms heat stress and cold stress refer to excessive exposure to very hot or very cold work environments. Both such conditions may interfere with worker’s performance and may even be fatal.

Heat

- Some of the adverse effects in very hot work environments (depending upon the individual worker and the heat intensity) include:

- Heat exhaustion

- Fainting

- Heat stroke

- Heat-related problems may arise for:

- Outdoor workers (working in construction)

- Industrial workers (working near furnaces)

Cold

- In very cold work environments (depending upon the individual worker and intensity of cold), the adverse effects include:

- Frostbite

- Hypothermia

- Workers at risk include:

- Outdoor workers (divers)

- Meat packers and handlers (working in refrigerated warehouses)

Noise

- Noise is a form of irregular vibration.

- It may be conducted through gases (or vapors), liquids, or solids.

- Above a certain level, the noise becomes of concern because it may hinder communication between workers, thus leading to annoyance.

- This in turn may lead to poor job performance and compromise the safety of the worker.

- In addition, excessive exposure to high noise levels may cause loss of hearing.

Vibration

- It is important to note that exposure to vibration is more than just a nuisance.

- Whereas exposure to vibration may cause discomfort, intense vibration has been known to cause serious health problems such as back pain, carpal tunnel syndrome (a condition affecting the hand and wrist) and damage to bones and joints.

- There are two types of occupational vibration: whole-body and hand-arm vibration.

- Whereas whole-body vibration is transmitted through the supporting surface (feet, back, etc.), hand-arm vibration is transmitted to the hands and arms. Examples include: Mining equipment (whole-body vibration) and Hand-held power tools (hand-arm vibration).

Radiation

- The hazard associated with a particular type of radiation depends on its energy and ability to penetrate the body tissue.

- Cosmic, gamma and x-ray radiation can cause severe damage to the tissue.

- Infrared, ultraviolet, visible light, radio and microwaves on the other hand do not penetrate appreciably below the skin and the damage is mainly restricted to burns to the skin and eyes.

- Severe damage to eyes may result from excessive exposure to ultraviolet radiation.

Chemical Hazards in the Workplace

- A large variety of chemical agents are encountered in workplaces due to the diversity of raw materials and processes used.

- Chemical hazards in such environments arise due to the presence of certain chemical agents whose properties and/or toxicity may pose a potential risk to the health and safety of the worker coming in contact with it or handling it.

Chemical hazards may arise in operations involving:

- Compressed gasses

- Flammable and combustible materials

- Oxidizing materials

- Poisonous and infections materials

- Corrosive materials

- Dangerously reactive materials

Chemical agents may be present in the air as:

- Fine particles

- Gases and vapors

- Both as particles and vapors at the same time

- Adsorbed or absorbed gases and/or vapors on particles

Compressed gases

- In the workplace many hazardous chemicals (ex: chlorine gas are compressed and thus stored in cylinders under high pressure.

- In using such materials the worker is not only subjected to the chemical hazard but also the hazards associated with pressure extreme

Flammable and combustible materials

- Such materials may burn readily in the presence of sources of ignition.

- Many organic compounds such as gasoline, solvents, etc. fall in this category.

Oxidizing Materials

- These materials can contribute strongly to fire hazards and may possess the ability to oxidize and thus destroy the biomolecules in living systems.

- Some commonly encountered oxidizers in the workplace include: potassium, permanganate, hydrogen peroxide, etc.

Poisonous materials

- Such substances may be dangerous to life in very small amounts. For example, potassium cyanide, mercury salts.

Corrosive materials

- Such chemicals cause deterioration of materials including living tissues on contact. Some examples are, sulfuric acid, potassium hydroxide, etc.

Dangerously reactive materials

- The chemicals in this category may undergo rapid or violent reactions under certain conditions. For example, alkali metals react with water producing highly flammable hydrogen gas

The problem with fine particles

- The particles of size less than 5 microns are potentially the most hazardous because of their effective entry and retention in the lungs.

- The particles are classified as dust, fume and smoke (for solids) and mist (for liquids)

Sources of fine particles include:

- Spray painting (mist)

- Welding (fume)

- Incomplete combustion of oil and grease (smoke)

- Ore grinding (dust)

The problem with gases and vapors

- These may be generated as a result of various operations and can mix and distribute rapidly throughout the workplace.

- In view of their small size, the gases can readily enter the bloodstream through the lungs

Some sources of potentially toxic gases and vapors include:

- Solvent degreasing (vapors of solvent used)

- Spray painting (vapors of solvent used)

- Welding (gaseous combustion products)

Biological Agents

- Biological agents are living organisms, or substances produced by such organisms, that can cause illness or disease in humans.

- The adverse health effects due to biological agents may range from allergic reactions to serious medical conditions and even death.

- Food poisoning, rabies, tuberculosis and hepatitis are some of the infections caused by biological agents.

- These include: bacteria, fungi, viruses and other micro-organisms and their associated toxins

Workplace Exposure

- Many micro-organisms pose a potential danger in a variety of workplaces due to their ability to: reproduce rapidly and survive with minimum resources

- Workplace exposure to work-related biological agents such as bacteria, moulds, and viruses is limited to certain occupations such as health care workers (i.e., hospital workers and veterinarians), meat handlers (i.e., ranchers, farmers, and meat packers), and those who work in sanitation or sewage operations.

Ergonomic Stresses

- Ergonomics can be defined as the science or study of work. It is a way of designing a work environment to fit people.

- Over the recent years it has been fully realized that the traditional method of assigning people to jobs without accommodating their needs, limitations, sizes, strengths and weaknesses may lead to certain adverse job safety and health issues such as mental stress, loss of efficiency and accidents.

It has been established that ergonomics, properly applied, can help:

- Reduce workplace injuries and illnesses; particularly back injuries and cumulative trauma disorder (CTD) affecting, joints, muscles, nerves and tendons that can cause pain and swelling

- Improve productivity and quality of work

- Increase job satisfaction

- Satisfy government regulations

Toxicology: the science that involves the study of the properties and interactions of physical chemical or biological agents

Toxicity: the degree of danger of a material to injure a living organism by other than mechanical means

Toxic hazards (toxicity) posed by a chemical are influenced by a number of factors such as:

- Physical properties

- Chemical properties

- Intensity of exposure

- Mode of handling

- Routes of entry

- Susceptibility of the worker

Respiratory, digestive and cutaneous

- The interaction of physical, chemical or biological agents may produce adverse responses with our bodies.

- Such responses may range from minor irritation to dreaded diseases like cancer

- Most common natural routes of entry of chemical agents into the body include: respiratory tract (by inhalation), digestive tract (by ingestion) and cutaneous (skin and eyes by absorption)

- In occupational settings, inhalation is the most important route followed by absorption through skin and eyes

Dose

- Dose is defined as the amount of toxicant(s) actually delivered to the target organ.

- The units used to express dose are mg/kg (mass of toxicant/body mass).

Exposure

- Exposure on the other hand is the total amount of the toxicant(s) present in the workplace. However, exposure does provide an indication of the dose.

- Higher the exposure, greater the probability of larger amounts being delivered to the target organ and hence higher the dose.

- The units used to express exposure are: mg/m3 (mass of toxicant/volume of air) for particles and ppm (parts per million) for gases and vapors.

### Threshold Limit Values

- In order to provide guidelines for controlling occupational health hazards, Threshold Limit Values (TLVs) for various physical and chemical agents have been developed by the American Conference of Governmental Industrial Hygienists (ACGIH).

- According to ACGIH, one of the ways to specify these values is as Threshold Limit Value-Time-Weighted Average (TLV-TWA).

- It is the time-weighted average concentration of a substance for a normal 8-hour workday and a 40-hour workweek to which nearly all workers may be repeatedly exposed, day after day, without adverse effects.

- Therefore, the lower the TLV value, the more potentially dangerous (or high risk) the substance is.

- Example: Which of the following statements is true based upon the information that the TLVs for two chemicals X and Y are 15 ppm and 5 ppm respectively? Answer: Y poses more risk than X

The Effects of Toxicants

- Classification for the effects of over exposure to toxicants (or the commonly used term contaminants) can be based upon: duration of the effect and organs affected and physiological actions

Duration of the effect

- Acute: the observed effect of a toxicant is considered acute if it appears soon after exposure. The effect generally results from brief exposure to a high concentration of the contaminant. The effect is also easy to observe and relate

- Chronic: the effect is considered chronic if it is observed much later after exposure. Chronic effects result from low and repeated exposure over a long period of time, they may have latency periods. Chronic effects are more difficult to study and are very important to consider when dealing with hazardous wastes and pollution

Organs affected and physiological actions

- The toxicant may change the normal functioning of one organ or a set of organs that operate as a system. For example, hepatotoxins affect the liver whereas immunotoxins act upon the immune system.

- A variety of physiological effects have been observed due to the diversity of contaminants in the workplace.

Observed physiological effects

- Asphyxiants: such chemicals hinder the body in maintain an adequate supply of oxygen. This leads to suffocation due to lack of oxygen.

- Irritants: these substances cause eye, skin and mucous membraine irritation.

- Anathetics: these act as depressants.

- Narcotics: such chemicals are habit forming depressants

- Systemics: these cause organs or system damage

- Lung scarring agents: such materials can cause lung damage leading to lung cancer

Carcinogenic effect

- Carcinogens are chemicals that cause cancer

- They are indentified by their ability to cause cancer in exposed workers, other human populations, or in test animals.

- Many occupational cancers have a long latency period, meaning that cancer may develop 10-20 years or longer after exposure to carcinogen

Matagenic effect

- Mutagens are substances that cause changes in DNA

- Mutagens can cause changes/mutations in the genetic material of cells from people or test animals, which may result in disease or abnormalities in future generations.

- In WHMIS, mutagens are classified as very toxic if they are shown to affect cells of the reproductive systems.

- Mutaggens are classified as toxic if studies shown genetic changes only in cells that are not part of the reproductive system

Teratogenic effect

- Teratogens are chemicals that cause malformations in nerborns

- They can cause birth defects, abnormalities, developmental delays or death in animal offspring in the absence of significant harmful effect on the mother

- These materials are usually identified using test animals and may cause similar effects in humans

According to the WHMIS hazardous materials are divided into classes with a symbol for each class

CLASS A

- Compressed gas: contents under high pressure, cylinder may explode or burst when heated dropped or damaged

CLASS B

- Flammable and combustible material: may catch fire when exposed to heat, spark or flame and may burst into flames

CLASS C

- Oxidizing material: may cause fire or explosion when in contact with wood, fuels or other combustible material

CLASS D

- Poisonous and infectious material: poisonous substance, a single exposure may be fatal or cause serious or permanent damage to health, may cause irritation, repeated exposure may cause cancer, birth defects or other permanent damage

CLASS E

- Corrosive material: can cause burns to eyes, skin or respiratory system

CLASS F

- Dangerously reactive material: may react violently causing explosion, fire or release of toxic gases, when exposed to light, heat, vibration or extreme temperatures

Safety data sheet

- The Material Safety Data Sheet (MSDS) provides basic information on a chemical product.

- It contains information regarding, properties, potential hazards, safe use and emergency procedures

- It is important to note that these are not a complete source of information.

- In some cases it might be necessary to consult other relevant sources for further information and details on a given substance.

Worker Education

- Workers handling hazardous products must be instructed in:

- The information on labels and MSDSs as it applies to their work

- Safe use, handling, storage and disposal of hazardous materials

- Emergency procedures to be used in case of a spill or overexposure

Successful Implementation Requirements

- Successful implementation of WHMIS in the workplace requires cooperation of all parties involved. WHMIS sets the responsibilities of the parties involved:

- Suppliers: provide hazard information through labels and MSDS on all controlled products/containers of controlled products.

- Employers (Supervisors): ensure WHMIS labels, identifiers and MSDSs for all containers of controlled products. Employers also ensure availability and accessibility of MSDS information to employees and provide effective worker training to ensure understanding among employees of the labels, MSDSs, and precautionary measures for hazardous materials in their workplace.

- Employees: handle controlled products safely and inform supervisors of damaged/missing labels and missing MSDS information.

- Regulators: develop and administer WHMIS legislation.

GHS

- The Globally Harmonized System (GHS) for Hazard Classification and Labelling Systems similar to WHMIS (for hazard communication) exists in other countries.

- In 1992, the United Nations Conference on Environment and Development (UNCED) agreed to develop a Globally Harmonized System (GHS) for hazard classification and labelling. It is anticipated that such a system will facilitate safe use of chemicals and reduce trade barriers.

Some of the anticipated changes are:

- Inclusion of consumer, transport and workplace sectors rather than workplace only.

- Inclusion of all chemicals except pharmaceutical and cosmetic products intended for consumer use.

- Hazard classification format in the MSDSs.

- Hazard communication format in the labels