6.1 – Logic Gates

Objective

Objective of the lab was to construct simple combinational logic circuits. Determine the operation of the logic circuit constructed. Determine the logic function produced by different circuit configurations with the help of truth tables. Also to connect various gates together to create simple logic functions. To analyse the logic circuit and predict their operations using truth tables. To analyse simple logic circuit diagrams and find their logic function. To construct more complex logic circuits and test them. To analyse complex logic circuit diagrams and find their logic function and truth tables.

Equipment and Components

- QUARTUS 9.0 Service Pack 2 Web Edition

- Altera UP-1 Circuit Board

- AC adapter, minimum output: 9 VDC, 250 mA

- #22 solid-core wires

- Wire Strippers

Circuit Diagram

Figure 6.1.1 : One-chip logic circuit

Figure 6.1.3 : Three-chip logic circuit

Figure 6.1.5 : Exclusive OR circuit

Figure 6.1.8 : Multiple output circuit

Experimental Data and Data Processing

| Figure 6.1.1 ((A + B) + C) = R |  |  |  |  |
| --- | --- | --- | --- | --- |
| A | B | C | (A + B) | (A + B) + C |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 |

| Figure 6.1.3 ((AB) + (A’B’)) = K |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | A’ | B’ | AB | A’B’ | (AB) + (A’B’) |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 |

| Figure 6.1.5 (( )) = L |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | AB | AB' | (AB)’A | (AB)’B | ((AB)’A) + (B(AB’)) |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 |

| Figure 6.1.8 ((A ⊕ B) ⊕ C) = S) ( (AB)’((C (A ⊕ B))’ )’ = Co |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | A ⊕ B | S | AB | AB’ | ((A ⊕ B) C)’ | Co |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |

Comparison of Expected Data and Experimental Data

| Figure 6.1.1 Comparing Experimental with Expected |  |  |
| --- | --- | --- |
| Experimental | Expected | Result |
| (A + B) + C | (A + B) + C | The data is same as the expected data cause the board’s output is opposite. |
| 1 | 0 |  |
| 0 | 1 |  |
| 0 | 1 |  |
| 0 | 1 |  |
| 0 | 1 |  |
| 0 | 1 |  |
| 0 | 1 |  |
| 0 | 1 |  |

| Figure 6.1.3 Comparing Experimental with Expected |  |  |
| --- | --- | --- |
| Experimental | Expected | Result |
| (AB) + (A’B’) | (AB) + (A’B’) | The data is same as the expected data cause the board’s output is opposite. |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 0 |  |
| 0 | 1 |  |

| Figure 6.1.5 Comparing Experimental with Expected |  |  |
| --- | --- | --- |
| Experimental | Expected | Result |
| ((AB)’A) + (B(AB’)) | ((AB)’A) + (B(AB’)) | The data is same as the expected data cause the board’s output is opposite. |
| 1 | 0 |  |
| 0 | 1 |  |
| 0 | 1 |  |
| 1 | 0 |  |

| Figure 6.1.8 Comparing Experimental with Expected |  |  |  |  |
| --- | --- | --- | --- | --- |
| Experimental |  | Expected |  | Result |
| S | Co | S | Co | The data is same as the expected data cause the board’s output is opposite. |
| 1 | 1 | 0 | 0 |  |
| 0 | 1 | 1 | 0 |  |
| 0 | 1 | 1 | 0 |  |
| 1 | 0 | 0 | 1 |  |
| 0 | 1 | 1 | 0 |  |
| 1 | 0 | 0 | 1 |  |
| 1 | 0 | 0 | 1 |  |
| 0 | 0 | 1 | 1 |  |

Discussion and Conclusion

Laboratory data sheets