CORP ROUTER

security passwords min-length 10

enable secret ciscoclass

line con 0

exec-timeout 20 0

logging synchronous

login local

line vty 0 4

exec-timeout 20 0

login local

transport input ssh

login authentication TELNET_LINES

service password-encryption

no cdp run

ntp server 172.16.25.2

ntp update-calendar

service timestamps log datetime msec

logging 172.16.25.2

ip domain-name theccnas.com

crypto key zeroize rsa

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

ip ssh time-out 90

ip ssh authentication-retries 2

aaa new-model

aaa authentication login default local none

aaa authentication login TELNET_LINES local

SWITCH

interface fa0/1

storm-control broadcast level 50

interface range fa 0/1 – fa 0/23

no shutdown

spanning-tree portfast

spanning-tree bpduguard enable

#PortFast and BPDU guard can also be enabled globally with the spanning-tree portfast default and spanning-tree portfast bpduguard commands in global configuration mode.

switchport port-security

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security mac-address sticky

interface range fa 0/2 – fa 0/5

shutdown

interface range fa 0/7 – fa 0/10

shutdown

interface range fa 0/13 – fa 0/23

shutdown

CORP router

End

mkdir ipsdir

(enter)

ip ips config location flash:ips

ip ips name corpips

ip ips signature-category

category all

retired true

exit

category ios_ips basic

retired false

exit

exit

(enter y)

int fa 0/0

ip ips iosips out

exit

ip ips signature-definition

signature 2004 0

status

retired true

exit

exit

exit

(enter y)

ip ips signature-definition

signature 2004 0

status

enabled true

exit

exit

exit

(enter y)

ip ips signature-definition

signature 2004 0

engine

event-action produce-alert

event-action deny-packet-inline

event-action reset-tcp-connection

exit

exit

exit

(enter y)

Step 5

access-list 12 permit host 172.16.25.5

access-list 12 permit host 198.133.219.35

line vty 0 4

access-class 12 in