HYDROLOGY – GEOG 375/2

ASSIGNMENT #1; DIMENSIONS, UNIT, AND MODELS

Monday, September 30, 2013

- Dimensions

  - Show that the following equation is homogeneous:

Fr = V/(gY) ½

[1] = [LT-1] / ([LT-2][L])1/2)

[1] = [LT-1]/([L2T-2]1/2)

[1] = [LT-1]/[LT-1] or also [1]

[LT-1][1] = [LT-1]

[LT-1] = [LT-1]

  - What are the dimensions of the three constants k1, k2, k3; V= K1 + K2R + K3R2

    - Finding the constants:

| [L3] = K1 + [L2][L] +[L][L2] [L3] = K1 [L3]+[L3] [L3] = K1 [L3] [1] = K1 | [L3]=K2[L] [L3]=K2 [L] [L2] = K2 | [L3] = K3[L] [L3] = K3 [L2] [L] = K3 |
| --- | --- | --- |

    - [L3] = [1] + [L2][L] + [L][L2]

[L3] = [1] + [L3] + [L3]

    - According to rule #10; in a homogeneous equation all constants must be changed if the equation is to be used in another system of units.

  - Give the dimensions of X in the following equations and indicate the corresponding type of physical quantity in each case;

    - X = ϒ Y S

X = [ML-2T-2][L][1]

X = [ML-2+1T-2]

X = [ML-1T-2] = Pressure

    - X = 2.5 (g R S)1/2 ln(y/y0)

X =([LT-2][L][1])1/2 ln ([L]/[L])

X = ([L+1+1 T-2])1/2 ln ([1])

X = ([L2T-2])1/2

X = [LT-1] = Velocity

    - X = [3.48μ2 + 0.707 ρ g(ρs – ρ) r3 ] ½ - 1.87 μ

0.531 ρ r

X = ([ML-1T-1]2 + [ML-3][LT-2]([ML-3 – ML-3])[L]3)1/2 – [ML-1T-1]

[ML-3][L]

X = ([M2L-2T-2] + [ML-3+1T-2][ML-3][L3])1/2 – [ML-1T-1]

[ML-3+1]

X = ([M2L-2T-2] + [M1+1L-2-3T-2][L3])1/2 – [ML-1T-1]

[ML-2]

X = ([M2L-2T-2] + [M2L-5+3T-2])1/2 – [ML-1T-1]

[ML-2]

X = ([M2L-2T-2] + [M2L-2T-2])1/2 – [ML-1T-1]

[ML-2]

X = [ML-1T-1] + [ML-1T-1] – [ML-1T-1]

[ML-2]

X = [ML-1T-1]

[ML-2]

X = [L-1T-1]

[L-2]

X = [T-1] = [LT-1] = Velocity

[L-1]

- Units

What would be the diameter of the constant (0.048) in this equation if D50 were in mm rather than in m; n = 0.048 D50 1/6

n = [1]

D50 = [L] in m

    - Conversion factors:  1m = 1000mm

    - Change in units:

no = koDo1/6

nn = knDn1/6

Do = (Dn1/6)/1000

    - Substitution:

n = ko(Dn/1000)1/6 = KnDn1/6

0.048(Dn/1000)1/6 = KnDn1/6

0.048(Dn/1000)1/6 *(1/(Dn1/6)) = Kn

0.048(1/(10001/6)) = Kn

0.015 = Kn

- Reservoir Model

Vi(t) = 2 + 0.8t	Vo (t) = 0.8 h(t)	h(t) – h(t-1) = Vi(t) – Vo(t)

    - Substitution and isolation to find the h(t) and constant;

h(t) – h(t-1) = Vi(t) – Vo(t)

h(t) – h(t-1) = Vi(t) – 0.8h(t)

1h(t) + 0.8h(t) = Vi(t) + h(t-1)

h(t)(1+0.8) = Vi(t) + h(t-1)

h(t) = Vi(t) + h(t-1)

1.8

    - Use and excel sheet to find when the steady state is reached; a steady state is reached at time 11 minutes.

| Time (min) | Vi(t) (cm/min) | h(cm) | Vo(t) (cm/min) |
| --- | --- | --- | --- |
| 0 |  | 10 |  |
| 1 | 2.8 | 7.11 | 5.7 |
| 2 | 2.6 | 5.42 | 4.3 |
| 3 | 2.5 | 4.41 | 3.5 |
| 4 | 2.4 | 3.79 | 3.0 |
| 5 | 2.3 | 3.40 | 2.7 |
| 6 | 2.3 | 3.14 | 2.5 |
| 7 | 2.2 | 2.97 | 2.4 |
| 8 | 2.2 | 2.86 | 2.3 |
| 9 | 2.1 | 2.77 | 2.2 |
| 10 | 2.1 | 2.71 | 2.2 |
| 11 | 2.1 | 2.67 | 2.1 |
| 12 | 2.1 | 2.63 | 2.1 |
| 13 | 2.1 | 2.60 | 2.1 |
| 14 | 2.0 | 2.58 | 2.1 |
| 15 | 2.0 | 2.56 | 2.1 |
| 16 | 2.0 | 2.55 | 2.0 |
| 17 | 2.0 | 2.54 | 2.0 |
| 18 | 2.0 | 2.53 | 2.0 |
| 19 | 2.0 | 2.53 | 2.0 |
| 20 | 2.0 | 2.52 | 2.0 |
| 21 | 2.0 | 2.52 | 2.0 |
| 22 | 2.0 | 2.51 | 2.0 |
| 23 | 2.0 | 2.51 | 2.0 |
| 24 | 2.0 | 2.51 | 2.0 |
| 25 | 2.0 | 2.51 | 2.0 |