1.  a) Stream order 5

b) i) Rb Average

Order 1; 173		173/42=4.12		Rb Average = (4.12+5.25+4+2)/(4) = 3.84

Order 2; 42		42/8=5.25

Order 3; 8		8/2=4

Order 4; 2		2/1=2

Order 5; 1

ii) Regression line of the relationship log N-u

| Order | Number of segments | Lof of segments | Regression Slope |
| --- | --- | --- | --- |
| 1 | 172 | 2.236 | -0.652365466 |
| 2 | 42 | 1.623 |  |
| 3 | 8 | 0.903 |  |
| 4 | 2 | 0.301 |  |

c) Horton’s law of stream numbers states that a relationship exists between the number of streams of a given order and the corresponding. The formula for the law of stream numbers goes as follows:

Nu = Rb K – u

Where “Rb” is the bifurcation ratio, “K” is the max order and “u” is the stream order. This formula expedites the scientific process by allowing us to derive the number of streams in each order from the bifurcation ratio. The results go as follows:

N1 = 3.845 – 1 = 3.844 = 217 
N2 = 3.845 – 2 = 3.843 = 56 
N3 = 3.845 – 3 = 3.842 = 14 
N4 = 3.845 – 4 = 3.841 = 3 
N5 = 3.845 – 5 = 3.840 = 1

The results indicate that the law of stream numbers does not apply, as the numbers vary greatly from the observed stream numbers that were counted on the map

d) Surface area of entire water shed; 7509136894m2 x (1) 2 (km) 2  = 7509.14km2

(1000) 2(m) 2

e) The following table depicts Shreve’s Magnitude (M) for the Chaudiere watershed and its sub-basins:

| Watershed + Sub-basins | Shreve’s Magnitude |
| --- | --- |
| Famine | 28 |
| Du Loup | 20 |
| Beaurivage | 18 |
| St-Victor | 8 |
| Total magnitude | 172 |

f) The R2, or the coefficient of determination, indicates that there is a weak relationship between Shreve’s magnitude and the surface area of each of the sub-basins. This indicates that an increase in surface area does not necessarily equate to an increase in magnitude.

2. Average volume of precipitation using;

i) Arithmetic mean; Total # of rain from stations within water shed/# of stations within water shed

Average rainfall; 895.2/11 = 81.4mm

Average volume; 0.0814m x 7509136594m2 = 611 243 743.2m3

ii) Thiessen Polygons

| Station | Rainfall(mm) | Area m2 | Area km2 | Weighted rainfall (mm) |
| --- | --- | --- | --- | --- |
| Thetf ord-Mines | 100 | 102641202.9 | 102.64 | 10264.12 |
| Saint-Pierre-de-Broughton | 108.2 | 163541396.8 | 163.54 | 17695.18 |
| Saint-Camille | 64.8 | 230178293.1 | 230.18 | 14915.55 |
| Frampton-2 | 156.3 | 279669885.1 | 279.67 | 43712.40 |
| Beaurivage | 85 | 388285576.4 | 388.29 | 33004.27 |
| Saint-Flavien | 69.6 | 417402603.1 | 417.40 | 29051.22 |
| Saint-Séverin | 178 | 471278743.4 | 471.28 | 83887.62 |
| Saint-Côme-de-Linière | 50.8 | 500326097.8 | 500.33 | 25416.57 |
| Scott | 76.4 | 518151963.5 | 518.15 | 39586.81 |
| Saint-Georges | 64.8 | 533619044.4 | 533.62 | 34578.51 |
| Saint-Éphrem | 114.4 | 594286944.5 | 594.29 | 67986.43 |
| Saint-Prosper | 63.6 | 646718832.1 | 646.72 | 41131.32 |
| Saint-Ludger | 63.6 | 671237426.7 | 671.24 | 42690.70 |
| Saint-Théophile | 56.8 | 700526673.4 | 700.53 | 39789.92 |
| Lac Mégantic 2 | 73.2 | 1291272211 | 1291.27 | 94521.13 |
|  | Total | 7509136894 | 7509.14 | 618231.74 |

Average rainfall; 618231.74/7509.14 = 82.33 mm

Average volume; 0.0823m x 7509136894m2  = 618 001 966.6m3

iii) Isoheytal method

| Isohytes | Area enclosed (m2) | Area enclosed (km2) | Average rainfall | Rain Volume |
| --- | --- | --- | --- | --- |
|  | 88400000 | 88.4 | 23 | 2033.2 |
| 51 |  |  |  |  |
|  | 2519400000 | 2519.4 | 61.5 | 154943.1 |
| 72 |  |  |  |  |
|  | 2873000000 | 2873 | 82.5 | 237022.5 |
| 93 |  |  |  |  |
|  | 798000000 | 798 | 103.5 | 82593 |
| 114 |  |  |  |  |
|  | 287300000 | 287.3 | 125 | 35912.5 |
| 135 |  |  |  |  |
|  | 287300000 | 265.2 | 146 | 38719.2 |
| 156 |  |  |  |  |
|  | 265200000 | 265.2 | 167 | 44288.4 |
| Total | 7118600000 | 7096.5 | 708.5 | 595511.9 |

Average rainfall:  59551.9 / 7096.5 = 83.92 mm

Average volume: 0.0839m x 7118600000 m2 = 597 250 540 m3

3. a)

| Time (hr) | Rain (mm) | Cumulative (mm) | Intensity (2hr) | Time Interval |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 2.4 | (0-2hr) |
| 1 | 0.6 | 0.6 | 2.6 | (1-3hr) |
| 2 | 1.8 | 2.4 | 4.6 | (2-4hr) |
| 3 | 0.8 | 3.2 | 8.2 | (3-5hr) |
| 4 | 3.8 | 7 | 11.4 | (4-6hr) |
| 5 | 4.4 | 11.4 | 13.8 | (5-7hr) |
| 6 | 7 | 18.4 | 12.4 | (6-8hr) |
| 7 | 6.8 | 25.2 | 15.4 | (7-9hr) |
| 8 | 5.6 | 30.8 | 22.2 | (8-10hr) |
| 9 | 9.8 | 40.6 | 29 | (9-11hr) |
| 10 | 12.4 | 53 | 26.6 | (10-12hr) |
| 11 | 16.6 | 69.6 | 15.6 | (11-13hr) |
| 12 | 10 | 79.6 | 9.4 | (12-14hr) |
| 13 | 5.6 | 85.2 | 5.2 | (13-15hr) |
| 14 | 3.8 | 89 | 2.8 | (14-16hr) |
| 15 | 1.4 | 90.4 | 3 | (15-17hr) |
| 16 | 1.4 | 91.8 | 3.2 | (16-18hr) |
| 17 | 1.6 | 93.4 | 2.8 | (17-19hr) |
| 18 | 1.6 | 95 | 3 | (18-20hr) |
| 19 | 1.2 | 96.2 | 3.4 | (19-21hr) |
| 20 | 1.8 | 98 | 2 | (20-22hr) |
| 21 | 1.6 | 99.6 |  |  |
| 22 | 0.4 | 100 |  |  |

Mean intensity; 100ml/22hr = 4.55mm/hr

Max intensity for 2 hours; 26.6mm/2hr = 13.3mm/hr

b) Cumulative hyetograph