2) a)

b) Total infiltration if duration of event is 320 minutes at K 2.2 for Soil 1 is 34.70 and 8.0 for Soil 2.  (Discuss the results in terms of the impact of land use on infiltration).

3) a) Based on provided dataset from 1936 to 2008, what is the return period of a discharge equaling or exceeding 1500m3/s?

Return Period (T)  	= (N+1)/r			N: 72 years

= (72 +1)/14			Rank (for 1425 m3/s): 14

= 5.21

The return period of a discharge equaling or exceeding 1500m3/s is 5 or more years

b) The return period value for the Irene flood have (peak discharge of 1456 m3/s) would be 5.21 or more (see calculations above).

| StationID | Year | MAX annual Q (m3/s) | Rank | Return Period |
| --- | --- | --- | --- | --- |
| 23402 | 1991 | 2140 | 1 | 73.00 |
| 23402 | 1987 | 1980 | 2 | 36.50 |
| 23402 | 1998 | 1859 | 3 | 24.33 |
| 23402 | 1994 | 1723 | 4 | 18.25 |
| 23402 | 1974 | 1680 | 5 | 14.60 |
| 23402 | 1947 | 1650 | 6 | 12.17 |
| 23402 | 1955 | 1640 | 7 | 10.43 |
| 23402 | 1969 | 1640 | 8 | 9.13 |
| 23402 | 1982 | 1640 | 9 | 8.11 |
| 23402 | 1942 | 1580 | 10 | 7.30 |
| 23402 | 1976 | 1540 | 11 | 6.64 |
| 23402 | 1992 | 1538 | 12 | 6.08 |
| 23402 | 2006 | 1535 | 13 | 5.62 |
| 23402 | 2007 | 1425 | 14 | 5.21 |
| 23402 | 2008 | 1425 | 15 | 4.87 |
| 23402 | 1958 | 1420 | 16 | 4.56 |
| 23402 | 2001 | 1392 | 17 | 4.29 |
| 23402 | 1979 | 1390 | 18 | 4.06 |
| 23402 | 1986 | 1380 | 19 | 3.84 |
| 23402 | 1972 | 1360 | 20 | 3.65 |
| 23402 | 1989 | 1330 | 21 | 3.48 |
| 23402 | 1970 | 1320 | 22 | 3.32 |
| 23402 | 1981 | 1320 | 23 | 3.17 |
| 23402 | 1975 | 1310 | 24 | 3.04 |
| 23402 | 2002 | 1309 | 25 | 2.92 |
| 23402 | 1963 | 1300 | 26 | 2.81 |
| 23402 | 1990 | 1270 | 27 | 2.70 |
| 23402 | 1964 | 1260 | 28 | 2.61 |
| 23402 | 1996 | 1258 | 29 | 2.52 |
| 23402 | 1945 | 1250 | 30 | 2.43 |
| 23402 | 1993 | 1249 | 31 | 2.35 |
| 23402 | 2005 | 1231 | 32 | 2.28 |
| 23402 | 1939 | 1230 | 33 | 2.21 |
| 23402 | 1943 | 1230 | 34 | 2.15 |
| 23402 | 1971 | 1220 | 35 | 2.09 |
| 23402 | 1941 | 1140 | 36 | 2.03 |
| 23402 | 1950 | 1130 | 37 | 1.97 |
| 23402 | 1985 | 1130 | 38 | 1.92 |
| 23402 | 1951 | 1120 | 39 | 1.87 |
| 23402 | 1953 | 1120 | 40 | 1.83 |
| 23402 | 1978 | 1100 | 41 | 1.78 |
| 23402 | 1988 | 1090 | 42 | 1.74 |
| 23402 | 1938 | 1070 | 43 | 1.70 |
| 23402 | 2004 | 1066 | 44 | 1.66 |
| 23402 | 1961 | 1060 | 45 | 1.62 |
| 23402 | 1977 | 1060 | 46 | 1.59 |
| 23402 | 1948 | 1020 | 47 | 1.55 |
| 23402 | 2000 | 993.6 | 48 | 1.52 |
| 23402 | 1962 | 980 | 49 | 1.49 |
| 23402 | 1983 | 962 | 50 | 1.46 |
| 23402 | 1960 | 960 | 51 | 1.43 |
| 23402 | 1954 | 954 | 52 | 1.40 |
| 23402 | 1984 | 952 | 53 | 1.38 |
| 23402 | 1973 | 934 | 54 | 1.35 |
| 23402 | 1995 | 899 | 55 | 1.33 |
| 23402 | 1968 | 895 | 56 | 1.30 |
| 23402 | 1956 | 889 | 57 | 1.28 |
| 23402 | 1944 | 881 | 58 | 1.26 |
| 23402 | 1940 | 866 | 59 | 1.24 |
| 23402 | 1952 | 841 | 60 | 1.22 |
| 23402 | 1999 | 840.9 | 61 | 1.20 |
| 23402 | 2003 | 819.7 | 62 | 1.18 |
| 23402 | 1937 | 804 | 63 | 1.16 |
| 23402 | 1966 | 804 | 64 | 1.14 |
| 23402 | 1967 | 765 | 65 | 1.12 |
| 23402 | 1997 | 691.5 | 66 | 1.11 |
| 23402 | 1936 | 688 | 67 | 1.09 |
| 23402 | 1946 | 640 | 68 | 1.07 |
| 23402 | 1959 | 623 | 69 | 1.06 |
| 23402 | 1980 | 597 | 70 | 1.04 |
| 23402 | 1965 | 572 | 71 | 1.03 |
| 23402 | 1949 | 507 | 72 | 1.01 |
| 23402 | 1957 | 507 | 73 | 1.00 |