Chapter 05 Strategic Capacity Planning

1. The term capacity refers to a limit on the load an operating unit can handle. 
True    False

2. Capacity decisions are usually long-term decisions. 
True    False

3. If a company produces a variety of outputs, capacity has to be expressed as several partial measures, no overall measure of capacity is possible. 
True    False

4. Capacity decisions often involve a long-term commitment of resources which, when implemented, are difficult or impossible to modify without major added costs. 
True    False

5. Stating capacity in dollar amounts generally results in a consistent measure of capacity. 
True    False

6. Design capacity refers to the maximum output that can possibly be attained under ideal conditions. 
True    False

7. Design capacity refers to the maximum output possible given a product mix, operating hours, and machine maintenance. 
True    False

8. Efficiency is defined as the ratio of actual output to effective capacity. 
True    False

9. Utilization is defined as the ratio of effective capacity to design capacity. 
True    False

10. Facilities are a major factor influencing effective capacity. 
True    False

11. An example of an external factor which influences effective capacity is pollution standards for products. 
True    False

12. Evaluation of capacity alternatives involves economic calculations only. 
True    False

13. As forecasts are usually only accurate for the short term, forecasts are not useful in long-term capacity decisions. 
True    False

14. Capacity increases are usually acquired in fairly large "chunks" rather than smooth increments. 
True    False

15. In break-even analysis, costs that vary directly with volume of output are referred to as fixed costs because they are fixed to the level of output. 
True    False

16. The break-even quantity can be determined by dividing the fixed costs by the difference between the revenue per unit and the variable cost per unit. 
True    False

17. Among the assumptions of break-even analysis is that always only one product is involved. 
True    False

18. Which is not true about long-term capacity? 
A. excess capacity can serve as a barrier to entry
B. capacity may be difficult and costly to modify
C. exceeding capacity minimizes operating costs
D. capacity affects the ability to satisfy customer's demand
E. capacity is usually a major determinant of initial capital costs

19. Long-term capacity decisions are important for which of the following reasons?
 
I. the impact they have on the ability to meet future requirements
II. the increased reliance on debt rather than equity financing
III. as determinant of initial capital costs
IV. the effect on operating costs 
A. I and III
B. I, II, and III
C. II and IV
D. I, III, and IV
E. I, II, III, and IV

20. Capacity refers to the upper limit of: 
A. inventories
B. demand
C. supplies
D. rate of output
E. finances

21. The maximum possible output under ideal conditions is: 
A. design capacity
B. effective capacity
C. actual output
D. efficiency
E. utilization

22. The maximum possible output given a product mix, scheduling difficulties, operating hours, and so on, is: 
A. utilization
B. design capacity
C. efficiency
D. effective capacity
E. available capacity

23. Efficiency is defined as the ratio of: 
A. actual output to effective capacity
B. actual output to design capacity
C. design capacity to effective capacity
D. effective capacity to actual output
E. design capacity to actual output

24. Utilization is defined as the ratio of: 
A. actual output to effective capacity
B. actual output to design capacity
C. design capacity to effective capacity
D. effective capacity to actual output
E. design capacity to actual output

25. The ratio of actual output to effective capacity is: 
A. design capacity
B. effective capacity
C. actual capacity
D. efficiency
E. utilization

26. The ratio of actual output to design capacity is: 
A. design capacity
B. effective capacity
C. actual capacity
D. efficiency
E. utilization

27. Given the following information, the efficiency is:
 
Effective capacity = 80 units per day
Design capacity = 100 units per day
Utilization = 48% 
A. 20%
B. 35%
C. 48%
D. 60%
E. 80%

28. Given the following information, the efficiency is:
 
Effective capacity = 50 units per day
Design capacity = 100 units per day
Actual output = 30 units per day 
A. 40%
B. 50%
C. 60%
D. 80%
E. 90%

29. Given the following information, the utilization is:
 
Effective capacity = 20 units per day
Design capacity = 60 units per day
Actual output = 15 units per day 
A. 1/4
B. 1/3
C. 1/2
D. 3/4
E. none of these

30. Which of the following is not a determinant of effective capacity? 
A. facilities
B. product mix
C. actual output
D. human factors
E. external factors

31. Considerations in forecasting long-term demand include:
 
I) identifying demand trends
II) duration of demand trends
III) amplitude of demand cycles 
A. I
B. II
C. III
D. I and II
E. I, II and III

32. Which of the following is not a consideration for developing capacity alternatives? 
A. design rigidity into systems
B. taking a big-picture approach to capacity changes
C. preparing to deal with capacity in "chunks"
D. attempting to smooth out capacity requirements
E. identifying the optimal operating level

33. Seasonal variations are often easier to deal with in capacity planning than random variations because seasonal variations tend to be: 
A. smaller
B. larger
C. predictable
D. controllable
E. less frequent

34. Production units have an optimal rate of output where: 
A. total costs are minimum
B. unit costs are minimum
C. marginal costs are minimum
D. rate of output is maximum
E. total revenue is maximum

35. When the output is less than the optimal rate of output, the unit cost will be: 
A. lower
B. the same
C. higher
D. could be either higher or lower
E. could be either higher, lower or the same

36. At the break-even point: 
A. output equals capacity
B. total cost equals total revenue
C. total cost equals profit
D. variable cost equals fixed cost
E. variable cost equals total revenue

37. An alternative will have fixed costs of $10,000 per month, variable costs of $50 per unit, and revenue of $70 per unit. The break-even point volume is: 
A. 100
B. 2,000
C. 500
D. 1,000
E. none of these

38. For fixed costs of $1,000, revenue per unit of $1, and variable cost per unit of $0.80, the break-even quantity is: 
A. 1,000
B. 1,250
C. 2,250
D. 5,000
E. none of these

39. Which of the following are assumptions of the break-even model?
 
I. Usually one product is involved.
II. Everything that is produced can be sold.
III. The revenue per unit will be the same regardless of volume. 
A. I only
B. I and II only
C. II only
D. II and III only
E. I, II and III

40. Which of the following is not an assumption of the break-even model? 
A. One product is involved.
B. Everything that is produced can be sold.
C. Total variable cost is the same regardless of volume.
D. Fixed costs do not change with volume changes.
E. Revenue per unit is the same regardless of volume.

41. What is the break-even quantity for the following situation?
 
FC = $1,200 per week
VC = $2 per unit
Revenue (R) = $6 per unit 
A. 100
B. 200
C. 600
D. 1,200
E. 300

The owner of Firewood To Go is considering buying a hydraulic wood splitter which sells for $50,000. He figures it will cost an additional $100 per cord to purchase and split wood with this machine, while he can sell each cord of split wood for $125.

42. If, for this machine, design capacity is 50 cords per day, effective capacity is 40 cords per day, and actual output is anticipated to be 35 cords per day, what would be its utilization? 
A. 100%
B. 80%
C. 75%
D. 70%
E. 0%

43. If, for this machine, design capacity is 50 cords per day, effective capacity is 40 cords per day, and actual output is expected to be 32 cords per day, what would be its efficiency? 
A. 100%
B. 80%
C. 75%
D. 70%
E. 0%

44. What would the potential profit be if he were to split 4,000 cords of wood with this machine? 
A. $0
B. $200,000
C. $100,000
D. $75,000
E. $50,000

45. How many cords of wood would he have to split with this machine to break even? 
A. 5,000
B. 3,000
C. 2,000
D. 1,000
E. 0

46. How many cords of wood would he have to split with this machine to make a profit of $30,000? 
A. 3,200
B. 1,500
C. 2,000
D. 1,000
E. 500

The owner of a greenhouse and nursery is considering whether to spend $6,000 to acquire the licensing rights to grow a new variety of rosebush, which she could then sell for $6 each.

47. If her available land has design and effective capacities of 3,000 and 2,000 rosebushes per year respectively, and she plans to grow 1,200 rosebushes each year on this land, what will be the utilization of this land? 
A. 0%
B. 40%
C. 60%
D. 67%
E. 100%

48. If her available land has design and effective capacities of 3,000 and 2,000 rosebushes per year, respectively, and she expects to be 80% efficient in her use of this land, how many rosebushes does Rose plan to grow each year on this land? 
A. 1,600
B. 2,400
C. 3,000
D. 2,000
E. 1,000

49. What would the profit be if she were to produce and sell 5,000 rosebushes? 
A. $0
B. $9,000
C. $15,000
D. $10,000
E. $30,000

50. How many rosebushes would she have to produce and sell in order to break even? 
A. 1,600
B. 2,400
C. 2,000
D. 1,000
E. 1,500

51. How many rosebushes would she have to produce and sell in order to make a profit of $6,000? 
A. 1,600
B. 2,400
C. 3,000
D. 1,000
E. 4,000

A recruiter for a job placement agency is considering whether to pay $50,000 per year to lease a new recruiting facility in a prime location in Washington, D. C. He estimates it will cost $50 per recruit to process the paperwork at this new location. He receives a $75 commission for each new recruit he processes.

52. If the office space at this new location has design and effective capacities of 10,000 and 7,500 recruits processed annually, respectively, and 5,000 recruits will be processed per year, what will be the utilization of the office space? 
A. 0%
B. 30%
C. 50%
D. 60%
E. 100%

53. If his office space at this new location has design and effective capacities of 10,000 and 7,500 recruits processed annually, respectively, and he plans to be 80% efficient in his use of this space, how many recruits does he plan to process per year? 
A. 5,000
B. 8,000
C. 2,000
D. 4,000
E. 6,000

54. What would be his annual profit if he were to process 4,000 recruits per year at this new location? 
A. $0
B. $75,000
C. $50,000
D. $100,000
E. $300,000

55. How many recruits would he have to process annually to break even at this new location? 
A. 5,000
B. 8,000
C. 2,000
D. 4,000
E. 6,000

56. How many recruits would he have to process annually to make a profit of $100,000 at this new location? 
A. 5,000
B. 8,000
C. 2,000
D. 4,000
E. 6,000

Doctor J. is considering purchasing a new blood analysis machine for $60,000. He estimates that he could charge $25.00 for an office visit to have a patient's blood analyzed, while the actual cost of a blood analysis would be $5.00.

57. If this new blood analysis machine has design and effective capacities of 6,000 and 5,000 blood analyses per year, respectively, and Dr. J. expects to perform 4,500 blood analyses each year, what will be the utilization of this machine? 
A. 0%
B. 75%
C. 83%
D. 90%
E. 100%

58. If this new blood analysis machine has design and effective capacities of 6,000 and 5,000 blood analyses per year, respectively, and Dr. J. expects to be 80% efficient in his use of this machine, how many blood analyses does he plan to perform each year? 
A. 3,200
B. 4,800
C. 4,000
D. 1,000
E. 5,000

59. What would be his profit if he were to perform 5,000 blood analyses? 
A. $0
B. $40,000
C. $60,000
D. $25,000
E. $100,000

60. How many blood analyses would he have to perform in order to break even? 
A. 12,000
B. 2,400
C. 3,000
D. 1,000
E. 5,000

61. How many blood analyses would he have to perform in order to make a profit of $15,000? 
A. 3,000
B. 4,800
C. 5,000
D. 12,000
E. 3,750

62. The efficiency of a productive unit is 60%. The unit produces an average of 20 forklift trucks per day. Determine the effective capacity of the unit.

63. The utilization of a machine is 50%. The machine has a design capacity of 70 units per hour and an effective capacity of 60 units per hour. Find the efficiency of the machine.

64. An investment proposal will have annual fixed costs of $60,000, variable costs of $35 per unit of output, and revenue of $55 per unit of output.
 
(i) Determine the break-even quantity.
(ii) What volume of output will be necessary for an annual profit of $60,000?

65. A firm is considering three capacity alternatives: A, B, and C. Alternative A would have an annual fixed cost of $100,000 and variable costs of $22 per unit. Alternative B would have annual fixed costs of $120,000 and variable costs of $20 per unit. Alternative C would have fixed costs of $80,000 and variable costs of $30 per unit. Revenue is expected to be $50 per unit.
 
(i) Which alternative has the lowest break-even quantity?
(ii) Which alternative will produce the highest profits for an annual output of 10,000 units?
(iii) Which alternative would require the lowest volume of output to generate an annual profit of $50,000?

66. A small business owner is contemplating the addition of another product line. Capacity increases and equipment will result in an increase in annual fixed costs of $50,000. Variable costs will be $25 per unit.
 
(i) What unit selling price must the owner obtain to break-even on a volume of 2,500 units a year?
(ii) Because of market conditions, the owner feels a revenue of $47 is preferred to the value determined in part a. What volume of output will be required to achieve a profit of $16,000 using this revenue?

67. What is the anticipated utilization?

68. What is the anticipated efficiency?

69. What is the break-even quantity (produced and sold)?

70. What are total revenues for the break-even quantity?

71. What are total costs for the break-even quantity?

72. What quantity would be required for a profit of $2,000?

73. What profit (loss) would there be for a quantity of 27,000?

74. What profit (loss) would there be for a quantity of 10,000?

Chapter 05 Strategic Capacity Planning Key

1. The term capacity refers to a limit on the load an operating unit can handle. 
TRUE

Difficulty: Easy
Learning Objective: 1
Stevenson - Chapter 05 #1

2. Capacity decisions are usually long-term decisions. 
FALSE

Difficulty: Easy
Learning Objective: 1
Stevenson - Chapter 05 #2

3. If a company produces a variety of outputs, capacity has to be expressed as several partial measures, no overall measure of capacity is possible. 
FALSE

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #3

4. Capacity decisions often involve a long-term commitment of resources which, when implemented, are difficult or impossible to modify without major added costs. 
TRUE

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #4

5. Stating capacity in dollar amounts generally results in a consistent measure of capacity. 
FALSE

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #5

6. Design capacity refers to the maximum output that can possibly be attained under ideal conditions. 
TRUE

Difficulty: Easy
Learning Objective: 1
Stevenson - Chapter 05 #6

7. Design capacity refers to the maximum output possible given a product mix, operating hours, and machine maintenance. 
FALSE

Difficulty: Hard
Learning Objective: 1
Stevenson - Chapter 05 #7

8. Efficiency is defined as the ratio of actual output to effective capacity. 
TRUE

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #8

9. Utilization is defined as the ratio of effective capacity to design capacity. 
FALSE

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #9

10. Facilities are a major factor influencing effective capacity. 
TRUE

Difficulty: Easy
Learning Objective: 1
Stevenson - Chapter 05 #10

11. An example of an external factor which influences effective capacity is pollution standards for products. 
TRUE

Difficulty: Easy
Learning Objective: 1
Stevenson - Chapter 05 #11

12. Evaluation of capacity alternatives involves economic calculations only. 
FALSE

Difficulty: Medium
Learning Objective: 2
Stevenson - Chapter 05 #12

13. As forecasts are usually only accurate for the short term, forecasts are not useful in long-term capacity decisions. 
FALSE

Difficulty: Medium
Learning Objective: 2
Stevenson - Chapter 05 #13

14. Capacity increases are usually acquired in fairly large "chunks" rather than smooth increments. 
TRUE

Difficulty: Medium
Learning Objective: 2
Stevenson - Chapter 05 #14

15. In break-even analysis, costs that vary directly with volume of output are referred to as fixed costs because they are fixed to the level of output. 
FALSE

Difficulty: Hard
Learning Objective: 3
Stevenson - Chapter 05 #15

16. The break-even quantity can be determined by dividing the fixed costs by the difference between the revenue per unit and the variable cost per unit. 
TRUE

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #16

17. Among the assumptions of break-even analysis is that always only one product is involved. 
FALSE

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #17

18. Which is not true about long-term capacity? 
A. excess capacity can serve as a barrier to entry
B. capacity may be difficult and costly to modify
C. exceeding capacity minimizes operating costs
D. capacity affects the ability to satisfy customer's demand
E. capacity is usually a major determinant of initial capital costs

Difficulty: Hard
Learning Objective: 1
Stevenson - Chapter 05 #18

19. Long-term capacity decisions are important for which of the following reasons?
 
I. the impact they have on the ability to meet future requirements
II. the increased reliance on debt rather than equity financing
III. as determinant of initial capital costs
IV. the effect on operating costs 
A. I and III
B. I, II, and III
C. II and IV
D. I, III, and IV
E. I, II, III, and IV

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #19

20. Capacity refers to the upper limit of: 
A. inventories
B. demand
C. supplies
D. rate of output
E. finances

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #20

21. The maximum possible output under ideal conditions is: 
A. design capacity
B. effective capacity
C. actual output
D. efficiency
E. utilization

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #21

22. The maximum possible output given a product mix, scheduling difficulties, operating hours, and so on, is: 
A. utilization
B. design capacity
C. efficiency
D. effective capacity
E. available capacity

Difficulty: Easy
Learning Objective: 1
Stevenson - Chapter 05 #22

23. Efficiency is defined as the ratio of: 
A. actual output to effective capacity
B. actual output to design capacity
C. design capacity to effective capacity
D. effective capacity to actual output
E. design capacity to actual output

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #23

24. Utilization is defined as the ratio of: 
A. actual output to effective capacity
B. actual output to design capacity
C. design capacity to effective capacity
D. effective capacity to actual output
E. design capacity to actual output

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #24

25. The ratio of actual output to effective capacity is: 
A. design capacity
B. effective capacity
C. actual capacity
D. efficiency
E. utilization

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #25

26. The ratio of actual output to design capacity is: 
A. design capacity
B. effective capacity
C. actual capacity
D. efficiency
E. utilization

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #26

27. Given the following information, the efficiency is:
 
Effective capacity = 80 units per day
Design capacity = 100 units per day
Utilization = 48% 
A. 20%
B. 35%
C. 48%
D. 60%
E. 80%

Difficulty: Hard
Learning Objective: 1
Stevenson - Chapter 05 #27

28. Given the following information, the efficiency is:
 
Effective capacity = 50 units per day
Design capacity = 100 units per day
Actual output = 30 units per day 
A. 40%
B. 50%
C. 60%
D. 80%
E. 90%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #28

29. Given the following information, the utilization is:
 
Effective capacity = 20 units per day
Design capacity = 60 units per day
Actual output = 15 units per day 
A. 1/4
B. 1/3
C. 1/2
D. 3/4
E. none of these

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #29

30. Which of the following is not a determinant of effective capacity? 
A. facilities
B. product mix
C. actual output
D. human factors
E. external factors

Difficulty: Hard
Learning Objective: 1
Stevenson - Chapter 05 #30

31. Considerations in forecasting long-term demand include:
 
I) identifying demand trends
II) duration of demand trends
III) amplitude of demand cycles 
A. I
B. II
C. III
D. I and II
E. I, II and III

Difficulty: Medium
Learning Objective: 2
Stevenson - Chapter 05 #31

32. Which of the following is not a consideration for developing capacity alternatives? 
A. design rigidity into systems
B. taking a big-picture approach to capacity changes
C. preparing to deal with capacity in "chunks"
D. attempting to smooth out capacity requirements
E. identifying the optimal operating level

Difficulty: Easy
Learning Objective: 2
Stevenson - Chapter 05 #32

33. Seasonal variations are often easier to deal with in capacity planning than random variations because seasonal variations tend to be: 
A. smaller
B. larger
C. predictable
D. controllable
E. less frequent

Difficulty: Easy
Learning Objective: 2
Stevenson - Chapter 05 #33

34. Production units have an optimal rate of output where: 
A. total costs are minimum
B. unit costs are minimum
C. marginal costs are minimum
D. rate of output is maximum
E. total revenue is maximum

Difficulty: Medium
Learning Objective: 2
Stevenson - Chapter 05 #34

35. When the output is less than the optimal rate of output, the unit cost will be: 
A. lower
B. the same
C. higher
D. could be either higher or lower
E. could be either higher, lower or the same

Difficulty: Hard
Learning Objective: 2
Stevenson - Chapter 05 #35

36. At the break-even point: 
A. output equals capacity
B. total cost equals total revenue
C. total cost equals profit
D. variable cost equals fixed cost
E. variable cost equals total revenue

Difficulty: Easy
Learning Objective: 3
Stevenson - Chapter 05 #36

37. An alternative will have fixed costs of $10,000 per month, variable costs of $50 per unit, and revenue of $70 per unit. The break-even point volume is: 
A. 100
B. 2,000
C. 500
D. 1,000
E. none of these

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #37

38. For fixed costs of $1,000, revenue per unit of $1, and variable cost per unit of $0.80, the break-even quantity is: 
A. 1,000
B. 1,250
C. 2,250
D. 5,000
E. none of these

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #38

39. Which of the following are assumptions of the break-even model?
 
I. Usually one product is involved.
II. Everything that is produced can be sold.
III. The revenue per unit will be the same regardless of volume. 
A. I only
B. I and II only
C. II only
D. II and III only
E. I, II and III

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #39

40. Which of the following is not an assumption of the break-even model? 
A. One product is involved.
B. Everything that is produced can be sold.
C. Total variable cost is the same regardless of volume.
D. Fixed costs do not change with volume changes.
E. Revenue per unit is the same regardless of volume.

Difficulty: Hard
Learning Objective: 3
Stevenson - Chapter 05 #40

41. What is the break-even quantity for the following situation?
 
FC = $1,200 per week
VC = $2 per unit
Revenue (R) = $6 per unit 
A. 100
B. 200
C. 600
D. 1,200
E. 300

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #41

The owner of Firewood To Go is considering buying a hydraulic wood splitter which sells for $50,000. He figures it will cost an additional $100 per cord to purchase and split wood with this machine, while he can sell each cord of split wood for $125.

Stevenson - Chapter 05

42. If, for this machine, design capacity is 50 cords per day, effective capacity is 40 cords per day, and actual output is anticipated to be 35 cords per day, what would be its utilization? 
A. 100%
B. 80%
C. 75%
D. 70%
E. 0%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #42

43. If, for this machine, design capacity is 50 cords per day, effective capacity is 40 cords per day, and actual output is expected to be 32 cords per day, what would be its efficiency? 
A. 100%
B. 80%
C. 75%
D. 70%
E. 0%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #43

44. What would the potential profit be if he were to split 4,000 cords of wood with this machine? 
A. $0
B. $200,000
C. $100,000
D. $75,000
E. $50,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #44

45. How many cords of wood would he have to split with this machine to break even? 
A. 5,000
B. 3,000
C. 2,000
D. 1,000
E. 0

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #45

46. How many cords of wood would he have to split with this machine to make a profit of $30,000? 
A. 3,200
B. 1,500
C. 2,000
D. 1,000
E. 500

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #46

The owner of a greenhouse and nursery is considering whether to spend $6,000 to acquire the licensing rights to grow a new variety of rosebush, which she could then sell for $6 each.

Stevenson - Chapter 05

47. If her available land has design and effective capacities of 3,000 and 2,000 rosebushes per year respectively, and she plans to grow 1,200 rosebushes each year on this land, what will be the utilization of this land? 
A. 0%
B. 40%
C. 60%
D. 67%
E. 100%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #47

48. If her available land has design and effective capacities of 3,000 and 2,000 rosebushes per year, respectively, and she expects to be 80% efficient in her use of this land, how many rosebushes does Rose plan to grow each year on this land? 
A. 1,600
B. 2,400
C. 3,000
D. 2,000
E. 1,000

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #48

49. What would the profit be if she were to produce and sell 5,000 rosebushes? 
A. $0
B. $9,000
C. $15,000
D. $10,000
E. $30,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #49

50. How many rosebushes would she have to produce and sell in order to break even? 
A. 1,600
B. 2,400
C. 2,000
D. 1,000
E. 1,500

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #50

51. How many rosebushes would she have to produce and sell in order to make a profit of $6,000? 
A. 1,600
B. 2,400
C. 3,000
D. 1,000
E. 4,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #51

A recruiter for a job placement agency is considering whether to pay $50,000 per year to lease a new recruiting facility in a prime location in Washington, D. C. He estimates it will cost $50 per recruit to process the paperwork at this new location. He receives a $75 commission for each new recruit he processes.

Stevenson - Chapter 05

52. If the office space at this new location has design and effective capacities of 10,000 and 7,500 recruits processed annually, respectively, and 5,000 recruits will be processed per year, what will be the utilization of the office space? 
A. 0%
B. 30%
C. 50%
D. 60%
E. 100%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #52

53. If his office space at this new location has design and effective capacities of 10,000 and 7,500 recruits processed annually, respectively, and he plans to be 80% efficient in his use of this space, how many recruits does he plan to process per year? 
A. 5,000
B. 8,000
C. 2,000
D. 4,000
E. 6,000

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #53

54. What would be his annual profit if he were to process 4,000 recruits per year at this new location? 
A. $0
B. $75,000
C. $50,000
D. $100,000
E. $300,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #54

55. How many recruits would he have to process annually to break even at this new location? 
A. 5,000
B. 8,000
C. 2,000
D. 4,000
E. 6,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #55

56. How many recruits would he have to process annually to make a profit of $100,000 at this new location? 
A. 5,000
B. 8,000
C. 2,000
D. 4,000
E. 6,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #56

Doctor J. is considering purchasing a new blood analysis machine for $60,000. He estimates that he could charge $25.00 for an office visit to have a patient's blood analyzed, while the actual cost of a blood analysis would be $5.00.

Stevenson - Chapter 05

57. If this new blood analysis machine has design and effective capacities of 6,000 and 5,000 blood analyses per year, respectively, and Dr. J. expects to perform 4,500 blood analyses each year, what will be the utilization of this machine? 
A. 0%
B. 75%
C. 83%
D. 90%
E. 100%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #57

58. If this new blood analysis machine has design and effective capacities of 6,000 and 5,000 blood analyses per year, respectively, and Dr. J. expects to be 80% efficient in his use of this machine, how many blood analyses does he plan to perform each year? 
A. 3,200
B. 4,800
C. 4,000
D. 1,000
E. 5,000

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #58

59. What would be his profit if he were to perform 5,000 blood analyses? 
A. $0
B. $40,000
C. $60,000
D. $25,000
E. $100,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #59

60. How many blood analyses would he have to perform in order to break even? 
A. 12,000
B. 2,400
C. 3,000
D. 1,000
E. 5,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #60

61. How many blood analyses would he have to perform in order to make a profit of $15,000? 
A. 3,000
B. 4,800
C. 5,000
D. 12,000
E. 3,750

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #61

62. The efficiency of a productive unit is 60%. The unit produces an average of 20 forklift trucks per day. Determine the effective capacity of the unit.

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #62

63. The utilization of a machine is 50%. The machine has a design capacity of 70 units per hour and an effective capacity of 60 units per hour. Find the efficiency of the machine.

First, solve for actual output using the utilization formula.

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #63

64. An investment proposal will have annual fixed costs of $60,000, variable costs of $35 per unit of output, and revenue of $55 per unit of output.
 
(i) Determine the break-even quantity.
(ii) What volume of output will be necessary for an annual profit of $60,000?

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #64

65. A firm is considering three capacity alternatives: A, B, and C. Alternative A would have an annual fixed cost of $100,000 and variable costs of $22 per unit. Alternative B would have annual fixed costs of $120,000 and variable costs of $20 per unit. Alternative C would have fixed costs of $80,000 and variable costs of $30 per unit. Revenue is expected to be $50 per unit.
 
(i) Which alternative has the lowest break-even quantity?
(ii) Which alternative will produce the highest profits for an annual output of 10,000 units?
(iii) Which alternative would require the lowest volume of output to generate an annual profit of $50,000?

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #65

66. A small business owner is contemplating the addition of another product line. Capacity increases and equipment will result in an increase in annual fixed costs of $50,000. Variable costs will be $25 per unit.
 
(i) What unit selling price must the owner obtain to break-even on a volume of 2,500 units a year?
(ii) Because of market conditions, the owner feels a revenue of $47 is preferred to the value determined in part a. What volume of output will be required to achieve a profit of $16,000 using this revenue?

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #66

Stevenson - Chapter 05

67. What is the anticipated utilization?

80%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #67

68. What is the anticipated efficiency?

90%

Difficulty: Medium
Learning Objective: 1
Stevenson - Chapter 05 #68

69. What is the break-even quantity (produced and sold)?

[25,000 units]

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #69

70. What are total revenues for the break-even quantity?

$40,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #70

71. What are total costs for the break-even quantity?

$40,000

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #71

72. What quantity would be required for a profit of $2,000?

28,334 units

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #72

73. What profit (loss) would there be for a quantity of 27,000?

$1,200

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #73

74. What profit (loss) would there be for a quantity of 10,000?

$9,000 loss

Difficulty: Medium
Learning Objective: 3
Stevenson - Chapter 05 #74

Chapter 05 Strategic Capacity Planning Summary

| Category | # of Questions |
| --- | --- |
| Difficulty: Easy | 9 |
| Difficulty: Hard | 7 |
| Difficulty: Medium | 58 |
| Learning Objective: 1 | 36 |
| Learning Objective: 2 | 8 |
| Learning Objective: 3 | 30 |
| Stevenson - Chapter 05 | 79 |