Chapter 16 Job and Staff Scheduling

| 1. | Backward scheduling is based on working back from the due dates for jobs. True False |
| --- | --- |

| 2. | In the production planning hierarchy, scheduling decisions are the final step before the actual output is produced. True False |
| --- | --- |

| 3. | Forward scheduling is based on completing jobs exactly on their due date. True False |
| --- | --- |

| 4. | Scheduling work centre loading must be completed before starting materials requirement planning (MRP). True False |
| --- | --- |

| 5. | In job scheduling, most loading of work centres is based on infinite loading. True False |
| --- | --- |

| 6. | Finite loading does not account for the capacity of work centres. True False |
| --- | --- |

| 7. | Loading refers to the assignment of jobs to work centres. True False |
| --- | --- |

| 8. | A Gantt chart is a specialized scheduling tool specifically for planning construction projects. True False |
| --- | --- |

| 9. | A Gantt chart is a visual aid for loading, scheduling and control purposes. True False |
| --- | --- |

| 10. | The term loading, as used in scheduling, refers to choosing the sequence of jobs to process. True False |
| --- | --- |

| 11. | Infinite loading and finite loading are two major approaches used to load work centres. True False |
| --- | --- |

| 12. | Input/output (I/O) control refers to monitoring changes in productivity for various work centres in a process. True False |
| --- | --- |

| 13. | A schedule/control Gantt chart can be used to monitor job progress. True False |
| --- | --- |

| 14. | Sequencing is concerned with the order in which jobs are processed, while loading is concerned with assigning jobs to work centres or workstations. True False |
| --- | --- |

| 15. | All priority rules are designed to ensure that the most urgent jobs (earliest due date) are scheduled first. True False |
| --- | --- |

| 16. | Input/Output (I/O) control is a visual representation of the progress of jobs that indicates if they are ahead or behind schedule. True False |
| --- | --- |

| 17. | The Theory of Constraints focuses on developing schedules for part-time workers or overtime shifts to address capacity constraints. True False |
| --- | --- |

| 18. | Priority rules generally assume that job setup time is independent of the sequence of processing jobs. True False |
| --- | --- |

| 19. | First come, first served (FCFS) is an example of a priority rule for sequencing. True False |
| --- | --- |

| 20. | Make-span is the total time needed to complete a group of jobs. True False |
| --- | --- |

| 21. | For a one machine/station shop, the SPT priority rule will always result in the lowest average flow time. True False |
| --- | --- |

| 22. | All priority rules or dispatching rules involve complex computations intended to establish optimal processing sequence for production facilities. True False |
| --- | --- |

| 23. | The primary limitation of the first come, first served (FCFS) priority rule is that long jobs will tend to delay jobs that follow. True False |
| --- | --- |

| 24. | A major disadvantage of the shortest processing time (SPT) rule is that it tends to make very short jobs wait for a long time. True False |
| --- | --- |

| 25. | The shortest processing time (SPT) rule minimizes downstream idle time. True False |
| --- | --- |

| 26. | Johnson's rule is a technique used to sequence jobs through a two-step work sequence. True False |
| --- | --- |

| 27. | A major reason for scheduling difficulty is variability in setup times, processing times, interruptions, and changes in the set of jobs. True False |
| --- | --- |

| 28. | A schedule/control Gantt chart depicts jobs or their components versus time to illustrate if they are behind or ahead of schedule. True False |
| --- | --- |

| 29. | The output of the system is limited by the output of the bottleneck operation(s). True False |
| --- | --- |

| 30. | The elimination of idle time on both bottleneck and non-bottleneck operations is equally important. True False |
| --- | --- |

| 31. | As long as the bottleneck operations are used effectively, the idle time of non-bottleneck operations will not affect the overall productivity of the system. True False |
| --- | --- |

| 32. | For the Theory of Constraints, the transfer batch denotes a portion of the production lot that could be moved to the bottleneck operation. True False |
| --- | --- |

| 33. | Splitting a large lot at one or more operations after a bottleneck operation would reduce the waiting time of the bottleneck operation. True False |
| --- | --- |

| 34. | Bottlenecks occur at work centres when the idle time is equal to zero. True False |
| --- | --- |

| 35. | A basic difference between staff scheduling in services and manufacturing companies is the of demand for end products in manufacturing is more volatile in contrast to relatively uniform, predictable demand in services. True False |
| --- | --- |

| 36. | The inability to store or inventory services makes staff scheduling for services more complex than for manufacturing companies. True False |
| --- | --- |

| 37. | For services, the process of assigning work shifts to staff begins with forecasting the amount of work to be done during each hour (or shift) during the time period the schedule is being planned. True False |
| --- | --- |

| 38. | Job scheduling pertains to: A.; hiring workers. B.; capacity planning. C.; buying machinery. D.; timing the use of work centres. E.; all of the choices. |
| --- | --- |

| 39. | Which of the following is the last step in the production planning hierarchy before actual output is produced? A.; product planning. B.; process planning. C.; capacity planning. D.; aggregate planning. E.; scheduling. |
| --- | --- |

| 40. | An area in a production facility consisting of one or more workers and machines with similar capabilities that can be considered one unit for the purpose of scheduling is a: A.; job shop. B.; bottleneck. C.; work team. D.; cell layout. E.; work centre. |
| --- | --- |

| 41. | Assigning jobs to work centres without regard to the available capacity of the work centres describes: A.; backward loading. B.; forward loading. C.; finite loading. D.; infinite loading. E.; shop loading. |
| --- | --- |

| 42. | Assigning jobs to work centres and to various machines within each work centre describes: A.; loading. B.; staff scheduling. C.; Johnson's rule. D.; capacity planning. E.; capacity utilization. |
| --- | --- |

| 43. | Monitoring and managing the work flow and queue lengths at each work centre on the shop-floor of a production facility is called: A.; Lot sizing. B.; Inventory turnover. C.; Daily dispatch lists. D.; Gantt charting. E.; Input/output (I/O) control. |
| --- | --- |

| 44. | Which of the following is not a "tool" (technique) for shop-floor control? A.; Anticipated delay report. B.; Input/output (I/O) control. C.; Daily dispatch lists. D.; Product structure tree. E.; Schedule/control Gantt chart. |
| --- | --- |

| 45. | A schedule/control Gantt chart is a visual representation of progress on the shop-floor which shows _______ on a vertical axis, and _______ on the horizontal axis. A.; Jobs in progress/time. B.; Jobs in progress/total costs. C.; Staffing levels/hours worked. D.; Staffing levels/total costs. E.; Staffing levels/time. |
| --- | --- |

| 46. | The purpose of a Gantt chart is to: A.; Visually express the capacity of resources.. B.; Quantitatively analyze the optimum sequencing of resources.. C.; Visually represent the loading and timing of jobs for resources.. D.; Quantitatively analyze the capacity of production resources.. E.; None of the choices.. |
| --- | --- |

| 47. | Two different approaches to load work centres in job scheduling are: A.; economic order quantity (EOQ) and material requirements planning (MRP). B.; Gantt charts and make-span. C.; infinite loading and finite loading. D.; linear programming and make-span. E.; none of the choices. |
| --- | --- |

| 48. | Finite loading, as compared to infinite loading, assumes that: A.; There are a finite number of jobs to be done.. B.; Work centres can be loaded without regard to their capacity.. C.; Work centres' capacity must be taken into account in assigning jobs.. D.; Work centres' capacity will be increased as needed after assigning jobs.. E.; none of the choices. |
| --- | --- |

| 49. | In the drum-buffer-rope conceptualization used in the theory of constraints, the "drum" that sets the pace for production refers to: A.; work-in-process inventory before the bottleneck. B.; the bottleneck of the process. C.; total capacity of the process. D.; first stage in the process. E.; the master schedule. |
| --- | --- |

| 50. | In the drum-buffer-rope conceptualization used in the theory of constraints, the "buffer" refers to: A.; work-in-process inventory before the bottleneck. B.; the bottleneck of the process. C.; total capacity of the process. D.; first stage in the process. E.; the master schedule. |
| --- | --- |

| 51. | Scheduling that starts with the present time and continues into the future is called A.; priority scheduling. B.; global scheduling. C.; local scheduling. D.; forward scheduling. E.; backward scheduling. |
| --- | --- |

| 52. | Input/Output (I/O) control is used to: I) manage work flows and queues at a work centre II) prevent overloading or underutilization of a work centre III) minimize WIP at a work centre A.; I. B.; II. C.; III. D.; I and II. E.; I, II and III. |
| --- | --- |

| 53. | The priority rule where jobs are processed according to the smallest ratio of due date to processing time is: A.; critical ration (CR). B.; earliest due date (EDD). C.; first come, first serve (FCFS). D.; minimum slack time (MST). E.; shortest processing time (SPT). |
| --- | --- |

| 54. | Job flow time includes I) setup time II) processing time III) processing time plus idle time A.; I. B.; II. C.; II and III. D.; I and II. E.; I, II and III. |
| --- | --- |

| 55. | Which of the following is not a measure for judging the effectiveness of a schedule sequence? A.; average work-in-process. B.; total processing time. C.; average flow time. D.; average job lateness. E.; all of the choices are used. |
| --- | --- |

| 56. | The total time needed to complete a group of jobs from the beginning of the first job to the completion of the last job is: A.; job flow time. B.; make-span. C.; earliest due date (EDD). D.; delivery time. E.; lead time. |
| --- | --- |

| 57. | Average flow time for a schedule sequence at a work station is: A.; total processing time divided by the number of jobs. B.; total flow time divided by the number of jobs. C.; total flow time divided by total processing time. D.; total processing time plus total late time divided by number of jobs. E.; total flow time plus total late time divided by number of jobs. |
| --- | --- |

| 58. | Which sequencing rule is designed specifically to minimize job lateness? A.; critical ration (CR). B.; earliest due date (EDD). C.; first come, first serve (FCFS). D.; minimum slack time (MST). E.; shortest processing time (SPT). |
| --- | --- |

| 59. | The scheduling sequencing rule which always results in the lowest average flow (flow) time is the: A.; critical ration (CR). B.; earliest due date (EDD). C.; first come, first serve (FCFS). D.; minimum slack time (MST). E.; shortest processing time (SPT). |
| --- | --- |

| 60. | A scheduling rule used for sequencing jobs through two work centres is: A.; critical ratio rule. B.; Johnson's rule. C.; slack per operation rule. D.; shortest processing time rule. E.; due date rule. |
| --- | --- |

| 61. | For the jobs listed below, which processing sequence would result using Johnson's Rule? A.; c-a-d-b. B.; b-d-a-c. C.; a-b-c-d. D.; d-c-b-a. E.; c-a-b-d. |
| --- | --- |

| 62. | What is the optimum job sequence for the jobs listed below using Johnson's Rule? A.; e-g-f-d. B.; d-g-f-e. C.; d-f-g-e. D.; e-d-f-g. E.; f-e-d-g. |
| --- | --- |

| 63. | A major difference between manufacturing and service systems in terms of scheduling staff is: A.; variability in the nature and timing of service jobs. B.; processing cost per unit. C.; the number of units to be processed. D.; length of processing time. E.; output rate. |
| --- | --- |

| 64. | The Theory of Constraints focuses on _______. A.; work-in-process inventory. B.; total processing time. C.; bottleneck operations. D.; job sequencing. E.; none of the choices above. |
| --- | --- |

| 65. | Shift scheduling for services involves: I) forecasting the amount of work to be done per time period II) assigning shifts to staff based on skill and availability III) converting work per time period into number of workers required. A.; I. B.; II. C.; III. D.; II and III. E.; I, II and III. |
| --- | --- |

| 66. | A primary reason why staff scheduling in services can be more challenging than it is for manufacturing is: A.; Services have less processing variability.. B.; Services cannot be inventoried.. C.; People generally are willing to wait longer for manufactured products.. D.; Customers will not wait for services.. E.; All services use appointments that have to match customers' schedules. |
| --- | --- |

| 67. | In scheduling staff for services, flexible capacity is achieved by A.; part time workers and the timing of shifts. B.; using Johnson's rule. C.; offering incentives for customers to come during non-peak hours. D.; appointment schedules. E.; none of the choices. |
| --- | --- |

|  | The table below contains information about five jobs waiting to be processed at work station number three. |
| --- | --- |

| 68. | The sequence that would result using the SPT rule is: A.; e-b-c-d-a. B.; c-a-b-e-d. C.; a-d-c-e-b. D.; d-e-b-a-c. E.; e-d-b-a-c. |
| --- | --- |

| 69. | Using the EDD rule, the sequence of jobs would be: A.; d-e-b-a-c. B.; a-d-c-b-e. C.; e-d-b-a-c. D.; a-d-c-e-d. E.; e-b-c-d-a. |
| --- | --- |

| 70. | Using the CR rule, the sequence of jobs would be: A.; c-a-b-d-e. B.; a-b-d-e-c. C.; d-e-b-a-c. D.; c-a-b-e-d. E.; e-d-b-a-c. |
| --- | --- |

| 71. | Using the MST rule, the sequence of jobs would be: A.; c-a-b-d-e. B.; a-b-d-e-c. C.; d-e-b-a-c. D.; c-a-b-e-d. E.; e-d-b-a-c. |
| --- | --- |

|  | The owner/operator of the local franchise of Handyman, Inc., has four jobs to do today, shown in the order they were received: |
| --- | --- |

| 72. | If he uses the first come, first served (FCFS) priority rule to schedule these jobs, what will be the average flow time? A.; 7.5 hours. B.; 5 hours. C.; 3 hours. D.; 2.5 hours. E.; 2 hours. |
| --- | --- |

| 73. | If he uses the earliest due date first (EDD) priority rule to schedule these jobs, what will be the average job lateness? A.; 0 hours. B.; 1.5 hours. C.; 1.75 hours. D.; 2 hours. E.; 2.25 hours. |
| --- | --- |

| 74. | If he uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average flow time? A.; 7.5 hours. B.; 5 hours. C.; 3 hours. D.; 2.5 hours. E.; 2 hours. |
| --- | --- |

| 75. | If he uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average job lateness? A.; 0 hours. B.; 1.5 hours. C.; 1.75 hours. D.; 2 hours. E.; 2.25 hours. |
| --- | --- |

| 76. | If he uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average WIP in his shop today? A.; 0. B.; 1. C.; 2. D.; 3. E.; 4. |
| --- | --- |

|  | Jack, the owner of Jack's Electronic Repair, currently has five jobs to be scheduled, shown in order in which they arrived: |
| --- | --- |

| 77. | If Jack uses the first come, first served (FCFS) priority rule to schedule these jobs, what will be the average flow time? A.; 3 hours. B.; 5 hours. C.; 7 hours. D.; 7.2 hours. E.; 8 hours. |
| --- | --- |

| 78. | If Jack uses the earliest due date first (EDD) priority rule to schedule these jobs, what will be the average job lateness? A.; 2 hours. B.; 2.2 hours. C.; 2.33 hours. D.; 2.4 hours. E.; 3 hours. |
| --- | --- |

| 79. | If Jack uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average flow time? A.; 3 hours. B.; 5 hours. C.; 7 hours. D.; 7.2 hours. E.; 8 hours. |
| --- | --- |

| 80. | If Jack uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average job lateness? A.; 2 hours. B.; 2.2 hours. C.; 2.33 hours. D.; 2.4 hours. E.; 3 hours. |
| --- | --- |

| 81. | If Jack uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average WIP in his shop today? A.; 2 jobs. B.; 2.33 jobs. C.; 2.4 jobs. D.; 2.67 jobs. E.; 3 jobs. |
| --- | --- |

|  | The operations manager of a body and paint shop has five cars to schedule for repair. He would like to minimize the make-span to complete all work on these cars. Each car requires body work prior to painting. The estimates of the times required to do the work at each of the two stations are as follows: |
| --- | --- |

| 82. | Using Johnson's rule where in the optimum sequence should car E be scheduled? A.; first. B.; second. C.; third. D.; fourth. E.; fifth. |
| --- | --- |

| 83. | Using Johnson's rule where in the optimum sequence should car A be scheduled? A.; first. B.; second. C.; third. D.; fourth. E.; fifth. |
| --- | --- |

| 84. | Using Johnson's rule what is the optimum sequence (first car to last)? A.; A; B; C; D; E. B.; E; D; C; B; A. C.; E; B; D; C; A. D.; E; D; B; C; A. E.; E; B; D; A; C. |
| --- | --- |

| 85. | What is the make-span for the optimal schedule? A.; 24 hours. B.; 25 hours. C.; 26 hours. D.; 28 hours. E.; 50 hours. |
| --- | --- |

| 86. | What is the idle time at the paint work centre for the optimal schedule? A.; 1 hour. B.; 2 hours. C.; 4 hours. D.; 7 hours. E.; 10 hours. |
| --- | --- |

| 87. | Given the information below, determine the following: (i) Processing sequence using (1) SPT and (2) EDD. (ii) Average flow time and average job lateness using (1) SPT and (2) EDD. |
| --- | --- |

| 88. | Given the following information concerning jobs awaiting processing at a single work centre, what is the processing sequence using the critical ratio (CR) rule? |
| --- | --- |

| 89. | Given the following information concerning jobs awaiting processing at a single work station, what is the processing sequence using the critical ratio (CR) rule? |
| --- | --- |

| 90. | Using the information below, determine the following: (i) processing sequence using (1) SPT and (2) EDD and (3) MST rules (ii) average flow time and average job lateness under each rule |
| --- | --- |

| 91. | Determine the processing sequence for the six jobs shown below using Johnson's Rule. Chart makespan. Can total time be reduced by splitting the latest job? If so, by how much? |
| --- | --- |

| 92. | Use Johnson's Rule to determine the optimum processing sequence for the jobs listed below. Chart make-span. |
| --- | --- |

|  | The following questions refer to the table data for jobs awaiting processing at a single work station (jobs are shown in order of arrival): |
| --- | --- |

| 93. | What is the schedule sequence using the first come, first served (FCFS) priority rule? |
| --- | --- |

| 94. | What is the average flow time for the first come, first served (FCFS) priority rule schedule? Average job lateness? Average WIP at the centre? |
| --- | --- |

| 95. | What is the schedule sequence using the earliest due date (EDD) priority rule? |
| --- | --- |

| 96. | What is the average flow time for the earliest due date (EDD) priority rule schedule? Average job lateness? Average WIP at the centre? |
| --- | --- |

| 97. | What is the schedule sequence using the Minimum Slack Time (MST) priority rule? To break a tie, use the Critical Ratio (CR) rule. |
| --- | --- |

| 98. | What is the average flow time for the minimum slack time (MST) priority rule schedule (using the CR rule to break ties)? Average job lateness? Average WIP at the centre? |
| --- | --- |

| 99. | What is the schedule sequence using the shortest processing time (SPT) priority rule? |
| --- | --- |

| 100. | What is the average flow time for the shortest processing time (SPT) priority rule? Average job lateness? Average WIP at the centre? |
| --- | --- |

|  | Given the following data for jobs awaiting processing at a manufacturing cell in which jobs are first processed by machine A, then by Machine B: |
| --- | --- |

| 101. | If these jobs were scheduled in the sequence shown, what would be the make-span? |
| --- | --- |

| 102. | If these jobs were scheduled in the sequence shown, what would be the idle time at machine B? |
| --- | --- |

| 103. | If these jobs were scheduled in the sequence shown, when would idle time occur at machine B? |
| --- | --- |

| 104. | In developing the sequence which will minimize make-span, what is the initial job to be placed in the schedule sequence? |
| --- | --- |

| 105. | In developing the sequence that will minimize make-span, where in the schedule sequence should the job W be placed? |
| --- | --- |

| 106. | In developing the sequence which will minimize make-span, what is the second job to be added to the sequence? |
| --- | --- |

| 107. | In developing the sequence which will minimize throughput time, where should the job X be placed? |
| --- | --- |

| 108. | What is the sequence which will minimize makespan? |
| --- | --- |

| 109. | What is the makespan for the optimum schedule? |
| --- | --- |

| 110. | What is the idle time at machine B for the optimum schedule? |
| --- | --- |

| 111. | When does idle time occur at machine B for the optimum schedule? |
| --- | --- |

| 112. | Determine the minimum number of workers needed, and a schedule for the following staffing requirements, giving workers two consecutive days off per cycle. Day Mon Tue Wed Thu Fri Sat Sun Staff needed 2 4 2 3 5 4 4 |
| --- | --- |

| 113. | Determine the minimum number of workers needed, and a schedule for the following staffing requirements, giving workers two consecutive days off per cycle. Day Mon Tue Wed Thu Fri Sat Sun Staff needed 1 1 2 3 4 4 3 |
| --- | --- |

Chapter 16 Job and Staff Scheduling Key

| 1. | Backward scheduling is based on working back from the due dates for jobs. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 16 #1

| 2. | In the production planning hierarchy, scheduling decisions are the final step before the actual output is produced. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 16 #2

| 3. | Forward scheduling is based on completing jobs exactly on their due date. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 16 #3

| 4. | Scheduling work centre loading must be completed before starting materials requirement planning (MRP). FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 16 #4

| 5. | In job scheduling, most loading of work centres is based on infinite loading. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 16 #5

| 6. | Finite loading does not account for the capacity of work centres. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 16 #6

| 7. | Loading refers to the assignment of jobs to work centres. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 16 #7

| 8. | A Gantt chart is a specialized scheduling tool specifically for planning construction projects. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 16 #8

| 9. | A Gantt chart is a visual aid for loading, scheduling and control purposes. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 16 #9

| 10. | The term loading, as used in scheduling, refers to choosing the sequence of jobs to process. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 16 #10

| 11. | Infinite loading and finite loading are two major approaches used to load work centres. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 16 #11

| 12. | Input/output (I/O) control refers to monitoring changes in productivity for various work centres in a process. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 16 #12

| 13. | A schedule/control Gantt chart can be used to monitor job progress. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 16 #13

| 14. | Sequencing is concerned with the order in which jobs are processed, while loading is concerned with assigning jobs to work centres or workstations. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 16 #14

| 15. | All priority rules are designed to ensure that the most urgent jobs (earliest due date) are scheduled first. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #15

| 16. | Input/Output (I/O) control is a visual representation of the progress of jobs that indicates if they are ahead or behind schedule. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 16 #16

| 17. | The Theory of Constraints focuses on developing schedules for part-time workers or overtime shifts to address capacity constraints. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #17

| 18. | Priority rules generally assume that job setup time is independent of the sequence of processing jobs. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #18

| 19. | First come, first served (FCFS) is an example of a priority rule for sequencing. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 16 #19

| 20. | Make-span is the total time needed to complete a group of jobs. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #20

| 21. | For a one machine/station shop, the SPT priority rule will always result in the lowest average flow time. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #21

| 22. | All priority rules or dispatching rules involve complex computations intended to establish optimal processing sequence for production facilities. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #22

| 23. | The primary limitation of the first come, first served (FCFS) priority rule is that long jobs will tend to delay jobs that follow. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #23

| 24. | A major disadvantage of the shortest processing time (SPT) rule is that it tends to make very short jobs wait for a long time. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 16 #24

| 25. | The shortest processing time (SPT) rule minimizes downstream idle time. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 16 #25

| 26. | Johnson's rule is a technique used to sequence jobs through a two-step work sequence. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #26

| 27. | A major reason for scheduling difficulty is variability in setup times, processing times, interruptions, and changes in the set of jobs. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #27

| 28. | A schedule/control Gantt chart depicts jobs or their components versus time to illustrate if they are behind or ahead of schedule. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 16 #28

| 29. | The output of the system is limited by the output of the bottleneck operation(s). TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 5 Stevenson - Chapter 16 #29

| 30. | The elimination of idle time on both bottleneck and non-bottleneck operations is equally important. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #30

| 31. | As long as the bottleneck operations are used effectively, the idle time of non-bottleneck operations will not affect the overall productivity of the system. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #31

| 32. | For the Theory of Constraints, the transfer batch denotes a portion of the production lot that could be moved to the bottleneck operation. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 5 Stevenson - Chapter 16 #32

| 33. | Splitting a large lot at one or more operations after a bottleneck operation would reduce the waiting time of the bottleneck operation. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #33

| 34. | Bottlenecks occur at work centres when the idle time is equal to zero. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #34

| 35. | A basic difference between staff scheduling in services and manufacturing companies is the of demand for end products in manufacturing is more volatile in contrast to relatively uniform, predictable demand in services. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 16 #35

| 36. | The inability to store or inventory services makes staff scheduling for services more complex than for manufacturing companies. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 16 #36

| 37. | For services, the process of assigning work shifts to staff begins with forecasting the amount of work to be done during each hour (or shift) during the time period the schedule is being planned. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 16 #37

| 38. | Job scheduling pertains to: A.; hiring workers. B.; capacity planning. C.; buying machinery. D.; timing the use of work centres. E.; all of the choices. |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 16 #38

| 39. | Which of the following is the last step in the production planning hierarchy before actual output is produced? A.; product planning. B.; process planning. C.; capacity planning. D.; aggregate planning. E.; scheduling. |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 16 #39

| 40. | An area in a production facility consisting of one or more workers and machines with similar capabilities that can be considered one unit for the purpose of scheduling is a: A.; job shop. B.; bottleneck. C.; work team. D.; cell layout. E.; work centre. |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 16 #40

| 41. | Assigning jobs to work centres without regard to the available capacity of the work centres describes: A.; backward loading. B.; forward loading. C.; finite loading. D.; infinite loading. E.; shop loading. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 16 #41

| 42. | Assigning jobs to work centres and to various machines within each work centre describes: A.; loading. B.; staff scheduling. C.; Johnson's rule. D.; capacity planning. E.; capacity utilization. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 16 #42

| 43. | Monitoring and managing the work flow and queue lengths at each work centre on the shop-floor of a production facility is called: A.; Lot sizing. B.; Inventory turnover. C.; Daily dispatch lists. D.; Gantt charting. E.; Input/output (I/O) control. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 16 #43

| 44. | Which of the following is not a "tool" (technique) for shop-floor control? A.; Anticipated delay report. B.; Input/output (I/O) control. C.; Daily dispatch lists. D.; Product structure tree. E.; Schedule/control Gantt chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 16 #44

| 45. | A schedule/control Gantt chart is a visual representation of progress on the shop-floor which shows _______ on a vertical axis, and _______ on the horizontal axis. A.; Jobs in progress/time. B.; Jobs in progress/total costs. C.; Staffing levels/hours worked. D.; Staffing levels/total costs. E.; Staffing levels/time. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 16 #45

| 46. | The purpose of a Gantt chart is to: A.; Visually express the capacity of resources.. B.; Quantitatively analyze the optimum sequencing of resources.. C.; Visually represent the loading and timing of jobs for resources.. D.; Quantitatively analyze the capacity of production resources.. E.; None of the choices.. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 16 #46

| 47. | Two different approaches to load work centres in job scheduling are: A.; economic order quantity (EOQ) and material requirements planning (MRP). B.; Gantt charts and make-span. C.; infinite loading and finite loading. D.; linear programming and make-span. E.; none of the choices. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 16 #47

| 48. | Finite loading, as compared to infinite loading, assumes that: A.; There are a finite number of jobs to be done.. B.; Work centres can be loaded without regard to their capacity.. C.; Work centres' capacity must be taken into account in assigning jobs.. D.; Work centres' capacity will be increased as needed after assigning jobs.. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 16 #48

| 49. | In the drum-buffer-rope conceptualization used in the theory of constraints, the "drum" that sets the pace for production refers to: A.; work-in-process inventory before the bottleneck. B.; the bottleneck of the process. C.; total capacity of the process. D.; first stage in the process. E.; the master schedule. |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 16 #49

| 50. | In the drum-buffer-rope conceptualization used in the theory of constraints, the "buffer" refers to: A.; work-in-process inventory before the bottleneck. B.; the bottleneck of the process. C.; total capacity of the process. D.; first stage in the process. E.; the master schedule. |
| --- | --- |

Difficulty: Easy Learning Objective: 5 Stevenson - Chapter 16 #50

| 51. | Scheduling that starts with the present time and continues into the future is called A.; priority scheduling. B.; global scheduling. C.; local scheduling. D.; forward scheduling. E.; backward scheduling. |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 16 #51

| 52. | Input/Output (I/O) control is used to: I) manage work flows and queues at a work centre II) prevent overloading or underutilization of a work centre III) minimize WIP at a work centre A.; I. B.; II. C.; III. D.; I and II. E.; I, II and III. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 16 #52

| 53. | The priority rule where jobs are processed according to the smallest ratio of due date to processing time is: A.; critical ration (CR). B.; earliest due date (EDD). C.; first come, first serve (FCFS). D.; minimum slack time (MST). E.; shortest processing time (SPT). |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 16 #53

| 54. | Job flow time includes I) setup time II) processing time III) processing time plus idle time A.; I. B.; II. C.; II and III. D.; I and II. E.; I, II and III. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #54

| 55. | Which of the following is not a measure for judging the effectiveness of a schedule sequence? A.; average work-in-process. B.; total processing time. C.; average flow time. D.; average job lateness. E.; all of the choices are used. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 16 #55

| 56. | The total time needed to complete a group of jobs from the beginning of the first job to the completion of the last job is: A.; job flow time. B.; make-span. C.; earliest due date (EDD). D.; delivery time. E.; lead time. |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 16 #56

| 57. | Average flow time for a schedule sequence at a work station is: A.; total processing time divided by the number of jobs. B.; total flow time divided by the number of jobs. C.; total flow time divided by total processing time. D.; total processing time plus total late time divided by number of jobs. E.; total flow time plus total late time divided by number of jobs. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #57

| 58. | Which sequencing rule is designed specifically to minimize job lateness? A.; critical ration (CR). B.; earliest due date (EDD). C.; first come, first serve (FCFS). D.; minimum slack time (MST). E.; shortest processing time (SPT). |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #58

| 59. | The scheduling sequencing rule which always results in the lowest average flow (flow) time is the: A.; critical ration (CR). B.; earliest due date (EDD). C.; first come, first serve (FCFS). D.; minimum slack time (MST). E.; shortest processing time (SPT). |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #59

| 60. | A scheduling rule used for sequencing jobs through two work centres is: A.; critical ratio rule. B.; Johnson's rule. C.; slack per operation rule. D.; shortest processing time rule. E.; due date rule. |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 16 #60

| 61. | For the jobs listed below, which processing sequence would result using Johnson's Rule? A.; c-a-d-b. B.; b-d-a-c. C.; a-b-c-d. D.; d-c-b-a. E.; c-a-b-d. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #61

| 62. | What is the optimum job sequence for the jobs listed below using Johnson's Rule? A.; e-g-f-d. B.; d-g-f-e. C.; d-f-g-e. D.; e-d-f-g. E.; f-e-d-g. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #62

| 63. | A major difference between manufacturing and service systems in terms of scheduling staff is: A.; variability in the nature and timing of service jobs. B.; processing cost per unit. C.; the number of units to be processed. D.; length of processing time. E.; output rate. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 16 #63

| 64. | The Theory of Constraints focuses on _______. A.; work-in-process inventory. B.; total processing time. C.; bottleneck operations. D.; job sequencing. E.; none of the choices above. |
| --- | --- |

Difficulty: Easy Learning Objective: 5 Stevenson - Chapter 16 #64

| 65. | Shift scheduling for services involves: I) forecasting the amount of work to be done per time period II) assigning shifts to staff based on skill and availability III) converting work per time period into number of workers required. A.; I. B.; II. C.; III. D.; II and III. E.; I, II and III. |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 16 #65

| 66. | A primary reason why staff scheduling in services can be more challenging than it is for manufacturing is: A.; Services have less processing variability.. B.; Services cannot be inventoried.. C.; People generally are willing to wait longer for manufactured products.. D.; Customers will not wait for services.. E.; All services use appointments that have to match customers' schedules. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 16 #66

| 67. | In scheduling staff for services, flexible capacity is achieved by A.; part time workers and the timing of shifts. B.; using Johnson's rule. C.; offering incentives for customers to come during non-peak hours. D.; appointment schedules. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 16 #67

|  | The table below contains information about five jobs waiting to be processed at work station number three. |
| --- | --- |

Stevenson - Chapter 16

| 68. | The sequence that would result using the SPT rule is: A.; e-b-c-d-a. B.; c-a-b-e-d. C.; a-d-c-e-b. D.; d-e-b-a-c. E.; e-d-b-a-c. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #68

| 69. | Using the EDD rule, the sequence of jobs would be: A.; d-e-b-a-c. B.; a-d-c-b-e. C.; e-d-b-a-c. D.; a-d-c-e-d. E.; e-b-c-d-a. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #69

| 70. | Using the CR rule, the sequence of jobs would be: A.; c-a-b-d-e. B.; a-b-d-e-c. C.; d-e-b-a-c. D.; c-a-b-e-d. E.; e-d-b-a-c. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #70

| 71. | Using the MST rule, the sequence of jobs would be: A.; c-a-b-d-e. B.; a-b-d-e-c. C.; d-e-b-a-c. D.; c-a-b-e-d. E.; e-d-b-a-c. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #71

|  | The owner/operator of the local franchise of Handyman, Inc., has four jobs to do today, shown in the order they were received: |
| --- | --- |

Stevenson - Chapter 16

| 72. | If he uses the first come, first served (FCFS) priority rule to schedule these jobs, what will be the average flow time? A.; 7.5 hours. B.; 5 hours. C.; 3 hours. D.; 2.5 hours. E.; 2 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #72

| 73. | If he uses the earliest due date first (EDD) priority rule to schedule these jobs, what will be the average job lateness? A.; 0 hours. B.; 1.5 hours. C.; 1.75 hours. D.; 2 hours. E.; 2.25 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #73

| 74. | If he uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average flow time? A.; 7.5 hours. B.; 5 hours. C.; 3 hours. D.; 2.5 hours. E.; 2 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #74

| 75. | If he uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average job lateness? A.; 0 hours. B.; 1.5 hours. C.; 1.75 hours. D.; 2 hours. E.; 2.25 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #75

| 76. | If he uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average WIP in his shop today? A.; 0. B.; 1. C.; 2. D.; 3. E.; 4. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #76

|  | Jack, the owner of Jack's Electronic Repair, currently has five jobs to be scheduled, shown in order in which they arrived: |
| --- | --- |

Stevenson - Chapter 16

| 77. | If Jack uses the first come, first served (FCFS) priority rule to schedule these jobs, what will be the average flow time? A.; 3 hours. B.; 5 hours. C.; 7 hours. D.; 7.2 hours. E.; 8 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #77

| 78. | If Jack uses the earliest due date first (EDD) priority rule to schedule these jobs, what will be the average job lateness? A.; 2 hours. B.; 2.2 hours. C.; 2.33 hours. D.; 2.4 hours. E.; 3 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #78

| 79. | If Jack uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average flow time? A.; 3 hours. B.; 5 hours. C.; 7 hours. D.; 7.2 hours. E.; 8 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #79

| 80. | If Jack uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average job lateness? A.; 2 hours. B.; 2.2 hours. C.; 2.33 hours. D.; 2.4 hours. E.; 3 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #80

| 81. | If Jack uses the shortest processing time first (SPT) priority rule to schedule these jobs, what will be the average WIP in his shop today? A.; 2 jobs. B.; 2.33 jobs. C.; 2.4 jobs. D.; 2.67 jobs. E.; 3 jobs. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #81

|  | The operations manager of a body and paint shop has five cars to schedule for repair. He would like to minimize the make-span to complete all work on these cars. Each car requires body work prior to painting. The estimates of the times required to do the work at each of the two stations are as follows: |
| --- | --- |

Stevenson - Chapter 16

| 82. | Using Johnson's rule where in the optimum sequence should car E be scheduled? A.; first. B.; second. C.; third. D.; fourth. E.; fifth. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #82

| 83. | Using Johnson's rule where in the optimum sequence should car A be scheduled? A.; first. B.; second. C.; third. D.; fourth. E.; fifth. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #83

| 84. | Using Johnson's rule what is the optimum sequence (first car to last)? A.; A; B; C; D; E. B.; E; D; C; B; A. C.; E; B; D; C; A. D.; E; D; B; C; A. E.; E; B; D; A; C. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #84

| 85. | What is the make-span for the optimal schedule? A.; 24 hours. B.; 25 hours. C.; 26 hours. D.; 28 hours. E.; 50 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #85

| 86. | What is the idle time at the paint work centre for the optimal schedule? A.; 1 hour. B.; 2 hours. C.; 4 hours. D.; 7 hours. E.; 10 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #86

| 87. | Given the information below, determine the following: (i) Processing sequence using (1) SPT and (2) EDD. (ii) Average flow time and average job lateness using (1) SPT and (2) EDD. (i) (1) C-D-B-E-A (2) A-C-B-D-E (ii) SPT: EDD: |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #87

| 88. | Given the following information concerning jobs awaiting processing at a single work centre, what is the processing sequence using the critical ratio (CR) rule? |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #88

| 89. | Given the following information concerning jobs awaiting processing at a single work station, what is the processing sequence using the critical ratio (CR) rule? |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #89

| 90. | Using the information below, determine the following: (i) processing sequence using (1) SPT and (2) EDD and (3) MST rules (ii) average flow time and average job lateness under each rule (i) SPT: D-C-A-E-B, EDD and MST: C-B-E-D-A SPT: (ii) EDD and MST: |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #90

| 91. | Determine the processing sequence for the six jobs shown below using Johnson's Rule. Chart makespan. Can total time be reduced by splitting the latest job? If so, by how much? |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #91

| 92. | Use Johnson's Rule to determine the optimum processing sequence for the jobs listed below. Chart make-span. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #92

|  | The following questions refer to the table data for jobs awaiting processing at a single work station (jobs are shown in order of arrival): |
| --- | --- |

Stevenson - Chapter 16

| 93. | What is the schedule sequence using the first come, first served (FCFS) priority rule? A-B-C-D-E |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #93

| 94. | What is the average flow time for the first come, first served (FCFS) priority rule schedule? Average job lateness? Average WIP at the centre? 11.6 days; 5.6 days; 2.9 jobs |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #94

| 95. | What is the schedule sequence using the earliest due date (EDD) priority rule? A-B-D-E-C |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #95

| 96. | What is the average flow time for the earliest due date (EDD) priority rule schedule? Average job lateness? Average WIP at the centre? 10.2 days; 4.2 days; 2.55 jobs |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #96

| 97. | What is the schedule sequence using the Minimum Slack Time (MST) priority rule? To break a tie, use the Critical Ratio (CR) rule. A-B-D-C-E |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #97

| 98. | What is the average flow time for the minimum slack time (MST) priority rule schedule (using the CR rule to break ties)? Average job lateness? Average WIP at the centre? 11 days; 5 days; 2.75 jobs |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #98

| 99. | What is the schedule sequence using the shortest processing time (SPT) priority rule? A-D-E-B-C |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #99

| 100. | What is the average flow time for the shortest processing time (SPT) priority rule? Average job lateness? Average WIP at the centre? 10 days; 4 days; 2.5 jobs |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #100

|  | Given the following data for jobs awaiting processing at a manufacturing cell in which jobs are first processed by machine A, then by Machine B: |
| --- | --- |

Stevenson - Chapter 16

| 101. | If these jobs were scheduled in the sequence shown, what would be the make-span? 20 hours |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #101

| 102. | If these jobs were scheduled in the sequence shown, what would be the idle time at machine B? 5 hours |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #102

| 103. | If these jobs were scheduled in the sequence shown, when would idle time occur at machine B? 4 hours at the outset; 1 hour between W and X |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #103

| 104. | In developing the sequence which will minimize make-span, what is the initial job to be placed in the schedule sequence? Y |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #104

| 105. | In developing the sequence that will minimize make-span, where in the schedule sequence should the job W be placed? The second |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #105

| 106. | In developing the sequence which will minimize make-span, what is the second job to be added to the sequence? W |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #106

| 107. | In developing the sequence which will minimize throughput time, where should the job X be placed? The third |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #107

| 108. | What is the sequence which will minimize makespan? Y-W-X-Z |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #108

| 109. | What is the makespan for the optimum schedule? 19 hours |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #109

| 110. | What is the idle time at machine B for the optimum schedule? 4 hours |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #110

| 111. | When does idle time occur at machine B for the optimum schedule? 1 hour at the outset; 2 hours between Y and W; 1 hour between W and X |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 16 #111

| 112. | Determine the minimum number of workers needed, and a schedule for the following staffing requirements, giving workers two consecutive days off per cycle. Day Mon Tue Wed Thu Fri Sat Sun Staff needed 2 4 2 3 5 4 4 (Bold = days off) A minimum of 6 workers is needed. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 16 #112

| 113. | Determine the minimum number of workers needed, and a schedule for the following staffing requirements, giving workers two consecutive days off per cycle. Day Mon Tue Wed Thu Fri Sat Sun Staff needed 1 1 2 3 4 4 3 (Bold = days off) A minimum of 4 workers is needed. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 16 #113

Chapter 16 Job and Staff Scheduling Summary

| Category | # of Questions |
| --- | --- |
| Difficulty: Easy | 25 |
| Difficulty: Hard | 5 |
| Difficulty: Medium | 83 |
| Learning Objective: 1 | 8 |
| Learning Objective: 2 | 12 |
| Learning Objective: 3 | 65 |
| Learning Objective: 4 | 8 |
| Learning Objective: 5 | 11 |
| Learning Objective: 6 | 9 |
| Stevenson - Chapter 16 | 119 |