INTRODUCTION:

In this lab, we were to learn more about chemical kinetics and apply our knowledge in the lab and our final reports. Chemical kinetics is a field of study in which the process of transforming reactings into products is studied. The Instantaneous Rate of reaction is the rate at which hthe reactants are becoming products at any specific time throughout the reaction. To find that rate, we must graph the reaction with concentration of the reactant on the y-axis as a function of time in minutes.

The formulas used in my calculations were:

%T = I / Io x 100%, where we were to convert the percent transmittance from the spectrophotometer to its real value.

A = – log T = – log (I / Io ), where the absorbance of the transmittance was calculated.

ACr(III) = A - At, where we calculated the amount of unreacted Cr(III) at any time.

As mentioned above, a spectrophotometer was used and it is a device that shines light onto a solution, in this experiment it was through the three different EDTA solutions of pH 4, 4.5 and 5, and measures its spectra in the form of percent transmittance. Transmittance is the ratio of the intensity of light after it passes through the medium.

RAW DATA COLLECTION & OBSERVATIONS:

| pH | 4.0 | 4.5 | 5.0 |
| --- | --- | --- | --- |
| Volume of EDTA | 50mL | 50mL | 50mL |
| time at which Cr was added | 3 min | 3 min | 3 min |
| # of drops of Cr | 7 | 6 | 5 |
| Temperature of water (°C) | 23 | 23 | 23 |

|  | % Transmittance |  |  |  |
| --- | --- | --- | --- | --- |
| time (min) | pH 4.0 | pH 4.5 | pH 5.0 |  |
| 5 | 85.0 | 82.0 | 72.0 |  |
| 10 | 82.0 | 75.0 | 57.0 |  |
| 15 | 82.5 | 68.5 | 47.0 |  |
| 20 | 80.5 | 63.0 | 40.0 |  |
| 25 | 78.0 | 58.0 | 35.0 |  |
| 30 | 77.0 | 54.0 | 31.5 |  |
| 35 | 75.0 | 49.5 | 28.0 |  |
| 40 | 73.5 | 47.5 | 26.0 |  |
| 45 | 71.5 | 44.5 | 24.5 |  |
| 50 | 71.0 | 42.5 | 23.0 |  |
| 55 | 69.5 | 41.0 | 22.0 |  |
| 60 | 68.5 | 40.0 | 21.0 |  |
| 65 | 68.0 | 39.0 | 20.0 |  |
| 70 | 65.0 | 36.0 | 19.5 |  |
| 75 | 64.0 | 34.0 | 18.5 |  |
| 80 | 63.5 | 33.5 | 18.0 |  |
| 85 | 62.0 | 33.0 | 18.0 |  |
| 90 | 61.0 | 32.0 | 17.5 |  |
| 95 | 60.0 | 31.0 | 17.0 |  |
| 100 | 59.5 | 30.5 | 16.5 |  |

Observations:

- EDTA at starting was clear, colorless liquid

- Solution, Cr (III) ion solution was dark purple

- pH 4.0 upon mixing no color change, progressively turned dark purple

- pH 4.5 turned dark purple

- pH 5.0 turned dark purple

| pH 4.0 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| time (min) |  | transmittance |  | absorbance |  | absorbance to ∞ |  | Absorbance [Cr(III)] |  | log(Absorbance [Cr(III)]) |  |
| 5 |  | 0.850 |  | 0.071 |  | 1.097 |  | 1.026 |  | 0.011 |  |
| 10 |  | 0.820 |  | 0.086 |  | 1.097 |  | 1.011 |  | 0.005 |  |
| 15 |  | 0.825 |  | 0.084 |  | 1.097 |  | 1.013 |  | 0.006 |  |
| 20 |  | 0.805 |  | 0.094 |  | 1.097 |  | 1.003 |  | 0.001 |  |
| 25 |  | 0.780 |  | 0.108 |  | 1.097 |  | 0.989 |  | -0.005 |  |
| 30 |  | 0.770 |  | 0.114 |  | 1.097 |  | 0.983 |  | -0.007 |  |
| 35 |  | 0.750 |  | 0.125 |  | 1.097 |  | 0.972 |  | -0.012 |  |
| 40 |  | 0.735 |  | 0.134 |  | 1.097 |  | 0.963 |  | -0.016 |  |
| 45 |  | 0.715 |  | 0.146 |  | 1.097 |  | 0.951 |  | -0.022 |  |
| 50 |  | 0.710 |  | 0.149 |  | 1.097 |  | 0.948 |  | -0.023 |  |
| 55 |  | 0.695 |  | 0.158 |  | 1.097 |  | 0.939 |  | -0.027 |  |
| 60 |  | 0.685 |  | 0.164 |  | 1.097 |  | 0.933 |  | -0.030 |  |
| 65 |  | 0.680 |  | 0.167 |  | 1.097 |  | 0.929 |  | -0.032 |  |
| 70 |  | 0.650 |  | 0.187 |  | 1.097 |  | 0.910 |  | -0.041 |  |
| 75 |  | 0.640 |  | 0.194 |  | 1.097 |  | 0.903 |  | -0.044 |  |
| 80 |  | 0.635 |  | 0.197 |  | 1.097 |  | 0.900 |  | -0.046 |  |
| 85 |  | 0.620 |  | 0.208 |  | 1.097 |  | 0.889 |  | -0.051 |  |
| 90 |  | 0.610 |  | 0.215 |  | 1.097 |  | 0.882 |  | -0.054 |  |
| 95 |  | 0.600 |  | 0.222 |  | 1.097 |  | 0.875 |  | -0.058 |  |
| 100 |  | 0.595 |  | 0.225 |  | 1.097 |  | 0.871 |  | -0.060 |  |
| afterheating | 0.080 |  | 1.097 |  |  |  |  |  |  |  |

CALCULATIONS:

| pH 4.5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| time (min) | transmittance | absorbance | absorbance to ∞ | Absorbance [Cr(III)] | log(Absorbance [Cr(III)]) |
| 5 | 0.820 | 0.086 | 1.071 | 0.984 | -0.007 |
| 10 | 0.750 | 0.125 | 1.071 | 0.946 | -0.024 |
| 15 | 0.685 | 0.164 | 1.071 | 0.906 | -0.043 |
| 20 | 0.630 | 0.201 | 1.071 | 0.870 | -0.061 |
| 25 | 0.580 | 0.237 | 1.071 | 0.834 | -0.079 |
| 30 | 0.540 | 0.268 | 1.071 | 0.803 | -0.095 |
| 35 | 0.495 | 0.305 | 1.071 | 0.765 | -0.116 |
| 40 | 0.475 | 0.323 | 1.071 | 0.747 | -0.127 |
| 45 | 0.445 | 0.352 | 1.071 | 0.719 | -0.143 |
| 50 | 0.425 | 0.372 | 1.071 | 0.699 | -0.156 |
| 55 | 0.410 | 0.387 | 1.071 | 0.683 | -0.165 |
| 60 | 0.400 | 0.398 | 1.071 | 0.673 | -0.172 |
| 65 | 0.390 | 0.409 | 1.071 | 0.662 | -0.179 |
| 70 | 0.360 | 0.444 | 1.071 | 0.627 | -0.203 |
| 75 | 0.340 | 0.469 | 1.071 | 0.602 | -0.220 |
| 80 | 0.335 | 0.475 | 1.071 | 0.596 | -0.225 |
| 85 | 0.330 | 0.481 | 1.071 | 0.589 | -0.230 |
| 90 | 0.320 | 0.495 | 1.071 | 0.576 | -0.240 |
| 95 | 0.310 | 0.509 | 1.071 | 0.562 | -0.250 |
| 100 | 0.305 | 0.516 | 1.071 | 0.555 | -0.256 |
| after heating | 0.085 |  |  |  |  |

| pH 5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| time (min) | transmittance | absorbance | absorbance to ∞ | Absorbance [Cr(III)] | log(Absorbance [Cr(III)]) |
| 5 | 0.720 | 0.143 | 0.886 | 0.743 | -0.129 |
| 10 | 0.570 | 0.244 | 0.886 | 0.642 | -0.193 |
| 15 | 0.470 | 0.328 | 0.886 | 0.558 | -0.253 |
| 20 | 0.400 | 0.398 | 0.886 | 0.488 | -0.311 |
| 25 | 0.350 | 0.456 | 0.886 | 0.430 | -0.366 |
| 30 | 0.315 | 0.502 | 0.886 | 0.384 | -0.415 |
| 35 | 0.280 | 0.553 | 0.886 | 0.333 | -0.477 |
| 40 | 0.260 | 0.585 | 0.886 | 0.301 | -0.521 |
| 45 | 0.245 | 0.611 | 0.886 | 0.275 | -0.560 |
| 50 | 0.230 | 0.638 | 0.886 | 0.248 | -0.606 |
| 55 | 0.220 | 0.658 | 0.886 | 0.228 | -0.641 |
| 60 | 0.210 | 0.678 | 0.886 | 0.208 | -0.681 |
| 65 | 0.200 | 0.699 | 0.886 | 0.187 | -0.728 |
| 70 | 0.195 | 0.710 | 0.886 | 0.176 | -0.754 |
| 75 | 0.185 | 0.733 | 0.886 | 0.153 | -0.815 |
| 80 | 0.180 | 0.745 | 0.886 | 0.141 | -0.850 |
| 85 | 0.180 | 0.745 | 0.886 | 0.141 | -0.850 |
| 90 | 0.175 | 0.757 | 0.886 | 0.129 | -0.889 |
| 95 | 0.170 | 0.770 | 0.886 | 0.117 | -0.934 |
| 100 | 0.165 | 0.783 | 0.886 | 0.104 | -0.985 |
| after heating | 0.130 | 0.886 |  |  |  |

pH 4.5

pH 4.0

| Δtime (min) | Absorbance [Cr(III)] | IR | log(IR) |
| --- | --- | --- | --- |
| 2 | 1.026 | 0.513 | -0.290 |
| 2 | 1.011 | 0.505 | -0.296 |
| 2 | 1.013 | 0.507 | -0.295 |
| 2 | 1.003 | 0.501 | -0.300 |
| 2 | 0.989 | 0.495 | -0.306 |
| 2 | 0.983 | 0.492 | -0.308 |
| 2 | 0.972 | 0.486 | -0.313 |
| 2 | 0.963 | 0.482 | -0.317 |
| 2 | 0.951 | 0.476 | -0.323 |
| 2 | 0.948 | 0.474 | -0.324 |
| 2 | 0.939 | 0.469 | -0.328 |
| 2 | 0.933 | 0.466 | -0.331 |
| 2 | 0.929 | 0.465 | -0.333 |
| 2 | 0.910 | 0.455 | -0.342 |
| 2 | 0.903 | 0.452 | -0.345 |
| 2 | 0.900 | 0.450 | -0.347 |
| 2 | 0.889 | 0.445 | -0.352 |
| 2 | 0.882 | 0.441 | -0.355 |
| 2 | 0.875 | 0.438 | -0.359 |
| 2 | 0.871 | 0.436 | -0.361 |

| Δtime (min) | Absorbance [Cr(III)] | IR | log(IR) |
| --- | --- | --- | --- |
| 2 | 0.984 | 0.492 | -0.308 |
| 2 | 0.946 | 0.473 | -0.325 |
| 2 | 0.906 | 0.453 | -0.344 |
| 2 | 0.870 | 0.435 | -0.362 |
| 2 | 0.834 | 0.417 | -0.380 |
| 2 | 0.803 | 0.401 | -0.396 |
| 2 | 0.765 | 0.383 | -0.417 |
| 2 | 0.747 | 0.374 | -0.428 |
| 2 | 0.719 | 0.359 | -0.444 |
| 2 | 0.699 | 0.349 | -0.457 |
| 2 | 0.683 | 0.342 | -0.466 |
| 2 | 0.673 | 0.336 | -0.473 |
| 2 | 0.662 | 0.331 | -0.480 |
| 2 | 0.627 | 0.313 | -0.504 |
| 2 | 0.602 | 0.301 | -0.521 |
| 2 | 0.596 | 0.298 | -0.526 |
| 2 | 0.589 | 0.295 | -0.531 |
| 2 | 0.576 | 0.288 | -0.541 |
| 2 | 0.562 | 0.281 | -0.551 |
| 2 | 0.555 | 0.277 | -0.557 |

SAMPLE CALCULATIONS:

- Transmittance = %T / 100

= 85.0% / 100

= 0.850

- Absorbance = - log (transmittance)

=   - log (0.850)

= 0.07058107429  = 0.071

- Absorbance∞ = - log (transmittance after heating)

= - log (0.080)

= 1.096910013  = 1.10

- Cr(III) Absorbance (ACr(III)) = absorbance∞ - absorbancet

= absorbance∞ - absorbance5

= 1.096910013 – 0.07058107429

= 1.026328939  = 1.03

- log (Cr(III) Absorbance) = log (Cr(III) Absorbance)

= log (1.026328939)

= 0.011

- Calculating the instantaneous rate at a certain time (2min): (pH 4.0 @ t = 5min)

Instantaneous rate at a certain time:rate:		IR = Δy ÷ Δx ≈ y ÷ Δx

IR = ACr(IIII) ÷ Δtime

IR = 1.026 ÷ 2 min

IR = 0.513 M/min

- Calculating the log of the Instantaneous rate

logIR = log(IR)

logIR = log(0.513)

logIR = -0.290

DISCUSSION:

Throughout this lab, the goal was to find the absorbance of all three solutions throughout the whole 100 minutes of data collecting with a percent transmittance.

As seen in graph 3, the slopes appear to be very alike even through their pH was different (4 and 4.5. The value of the two slopes were 1 which proves that the partial order with respect to Cr(III) is 1. In graph 2, it is linear which also proves that the reaction is a pseudo-first order reaction.

It was also observed through the experiment and then alanysing the data that the EDTA of pH 4 had the highest percent transmittance and pH 5 had the lowest. This is true because the solution with pH 4 has more hydronium ions and the creation of the Cr(III) complex ion is inversely proportional to the concentration of the hydronium ions. Which means that if you have a lower [ ] of H+, you get a higher rate.

The sources of error were noted to be that during the experiment, my partner and I left the test tube rack holding the cuvettes for the first 10 minutes in the water bath. This would slow down the reaction and not letting it be exposed completely to the water temperature. Also, the machine appeared to be a little faulty and would un-calibrate often. Also, it was impossible to keep the temperature of the water bath constant as it kept fluctuating from 1 or 2 degrees Celcius. As the temperature decreased, the rate of reaction would have also decreased due to the fact that temperature in this reaction is directly proportional to the rate of reaction.

In order to improve on these possible sources of error, I would first start out by leaving the cuvettes suspended completely in the water bath (as instructed to do, but forgot) to ensure better results. Also, perhaps using a more modern spectrophotometer that doesn’t get too faulty would definitely improve results. And unfortunately, nothing could be done about the temperature not being constant.

CONCLUSION:

After completing this lab, I conclude by saying that the EDTA soltuon of oH 4 had the highest percent transmittance while the pH 5 solution had the lowest,. However, their final % transmittance after the heating, the EDTA solution of pH 5 had the highest percent transmittance with a value of  13.0%.

Also through the graphs, it was determined that between pH 4 and 4.5, the solution of pH 4 had the higher IR of reaction.