Problem 1		Material Requirements Planning

The product structure tree of product A is given below. The lead times and available inventories of each item are found in the following table.

|  | A |  |  |  |  |  |  |  |  |  |  | Table 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Item | Lead Time |  | Available Inventory |
| E(1) |  |  | C(2) |  |  |  | B(3) |  |  |  |  | A | 2 weeks |  | 200 |
|  |  |  |  |  |  |  |  |  |  |  |  | B | 2 weeks |  | 1000 |
|  |  |  |  |  | C(1) |  |  |  | D(2) |  |  | C | 1 week |  | 1500 |
|  |  |  |  |  |  |  |  |  |  |  |  | D | 3 weeks |  | 8500 |
|  |  |  |  |  |  |  |  |  |  |  |  | E | 4 weeks |  | 900 |

[3]	(a)	Given the available inventories, how many additional units of E must be ordered if a total of 2,000 units of A are required? Show your work. (Hint: You do not need to develop an MRP plan; also, do not specify when the units of E must be ordered: we only want the total number of E to order).

A= 2,000 – 200

= 1,800

E= 1,800 – 900

= 900

Therefore, 900 additional units of E are required.

[6]	(b)	Given the available inventories, how many additional units of C must be ordered if a total of 2,000 units of A are required? Show your work. (Hint: You do not need to develop an MRP plan; also, do not specify when the units of C must be ordered: we only want the total number of C to order).

A = 2,000 – 200 				C = C1+C2 - 1,500

= 1,800						   = 3,600 + 4,400 – 1,500

= 6,500

C1 = 2 x 1,800

= 3,600					Therefore, 6,500 additional units of C are required.

B = 3 x 1,800

= 5,400 – 1,000

= 4,400

C2 = 1 x 4,400

= 4,400

|  | A |  |  |  |  |  |  |  |  |  |  | Table 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Item | Lead Time |  | Available Inventory |
| E(1) |  |  | C(2) |  |  |  | B(3) |  |  |  |  | A | 2 weeks |  | 200 |
|  |  |  |  |  |  |  |  |  |  |  |  | B | 2 weeks |  | 1000 |
|  |  |  |  |  | C(1) |  |  |  | D(2) |  |  | C | 1 week |  | 1500 |
|  |  |  |  |  |  |  |  |  |  |  |  | D | 3 weeks |  | 8500 |
|  |  |  |  |  |  |  |  |  |  |  |  | E | 4 weeks |  | 900 |

[6]	(c)	Treat the following questions independently; show your work for each answer. Given the available inventories and assembly lead times, what is the maximum amount of product A that could be supplied to customers:

[2]	i)	now (i.e. at the beginning of week 1)?  Briefly Justify.

At the beginning of week 1, only the 200 units available in inventory for Product A can be supplied to customers. This is because the minimum lead time to produce more of Product A is 2 weeks, and thus there is not enough time to produce more at the beginning of week 1.

[4]	ii)		within the next two weeks (i.e. on the beginning of week 3, assuming we are at the beginning of week 1)?  Show your calculations.

Lead time is 2 weeks for A, Max what we have 200 units. Min (1000/3, 1500/2, 900/1) = 333 units

200 + 333 = 533 units

[6]	(d)	Given the available inventories and assembly lead times, complete the following table.

| Week: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item: A |  |  |  |  |  |  |  |  |
| Gross Requirements |  |  |  |  |  |  |  | 1,200 |
| Available | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |
| Net Requirements |  |  |  |  |  |  |  | 1,000 |
| Plan. Order Receipts |  |  |  |  |  |  |  | 1,000 |
| Plan. Order Releases |  |  |  |  |  | 1,000 |  |  |
| Item: B |  |  |  |  |  |  |  |  |
| Gross Requirements |  |  |  |  |  | 3,000 |  |  |
| Available | 1,000 | 1,000 | 1,000 | 1,000 | 1,000 | 1,000 | 0 | 0 |
| Net Requirements |  |  |  |  |  | 2,000 |  |  |
| Plan. Order Receipts |  |  |  |  |  | 2,000 |  |  |
| Plan. Order Releases |  |  |  | 2,000 |  |  |  |  |
| Item: C |  |  |  |  |  |  |  |  |
| Gross Requirements |  |  |  | 2,000 |  | 2,000 |  |  |
| Available | 1,500 | 1,500 | 1,500 | 1,500 | 0 | 0 | 0 | 0 |
| Net Requirements |  |  |  | 500 |  | 2,000 |  |  |
| Plan. Order Receipts |  |  |  | 500 |  | 2,000 |  |  |
| Plan. Order Releases |  |  | 500 |  | 2,000 |  |  |  |
| Item: D |  |  |  |  |  |  |  |  |
| Gross Requirements |  |  |  | 4,000 |  |  |  |  |
| Available | 8,500 | 8,500 | 8,500 | 8,500 | 4,500 | 4,500 | 4,500 | 4,500 |
| Net Requirements |  |  |  | 0 |  |  |  |  |
| Plan. Order Receipts |  |  |  | 0 |  |  |  |  |
| Plan. Order Releases | 0 |  |  |  |  |  |  |  |
| Item: E |  |  |  |  |  |  |  |  |
| Gross Requirements |  |  |  |  |  | 1,000 |  |  |
| Available | 900 | 900 | 900 | 900 | 900 | 900 | 0 | 0 |
| Net Requirements |  |  |  |  |  | 100 |  |  |
| Plan. Order Receipts |  |  |  |  |  | 100 |  |  |
| Plan. Order Releases |  | 100 |  |  |  |  |  |  |

[2]	(e)	Given the available inventories and assembly lead times, how many weeks would be needed to meet a gross requirement of 1,200 units of A? At the earliest, at the beginning of what week could an order of 1,200 units of A be delivered? Justify and be precise.

6 weeks are needed to meet a gross requirement of 1,200 units of A (8-2).

At the earliest, you could deliver 1,200 units of A by the beginning of week 7 or the end of week 6, if you start at 0.

[7]	Problem 2		Inventory Management: Deterministic Demand

QUESTION (a) AND (b) ARE INDEPENDENTS

[3]	(a)	The Dine Corporation is both a producer and a user of brass couplings. The firm operates 220 days per year and uses the couplings at a steady rate of 50 per day. Couplings can be produced at a rate of 200 per day. Assume storage costs are $1 per coupling per year, and machine setup costs (or ordering costs) are $35.

What is the optimal production order quantity of brass couplings?

d= 50

D = 50 x 220 = 11,000

p= 200

H = $1

S = $35

Q* = √(2DS/H)*(p/(p-d))

= √(2*11,000*$35/$1)*(200/(200-50))

=√ $770,000 * 1.333

= 1,013.25 ~ 1,013

Therefore, the optimal production order quantity of brass couplings is 1,013.

[4]	(b)	Burger Prince orders paper cups from a supplier, and uses the basic Economic Order Quantity model to determine the order quantity. The ordering cost is constant and fixed irrespective of the order quantity, while the holding cost is determined as a certain percentage of the purchase cost of paper cups. Burger Prince was told that the cost of paper cups would increase by 10%.

[2]	i)	Should the order quantity change and if so, Should Burger Prince order more or less than before? Briefly Justify.

The order quantity should change. Burger Prince should order less than before. The theory behind this lies in the equation itself, as when solving for the optimal order quantity, your holding cost should be equal to (or very close to) your set up costs. Therefore, if the holding cost increases, according to the equation you would have to decrease the order quantity in order to make your set up cost increase.

If it decrese by 15%: H will decrease thus the EOQ will increse  as Q*, he should order more than before.

[2]	ii)	By what percentage the order quantity will increase or decrease following the increase of 10% in the cost of paper cups?  show your calculations.

Let C = 10			1. √2DS/H 	2. √2DS/H

H = 1				  = √2000/1	 = √2000/1.1

S= 10				  = 44.72	 = 42.64

D = 100

H’= 1 x 10%

42.64/44.72 = ~ 0.95

1 – 0.95 – 0.05

Therefore, the order quantity would decrease by 5%. He should order less by 5%.

[17]	Problem 3		Inventory Management: Probabilistic Demand

Weekly demand of a product is stationary and has been forecasted using exponential smoothing. The expected demand is 196 units per week and the standard deviation of weekly demand is 25. The item is purchased from a vendor at a cost of $10 per unit: the ordering cost is $50 with a lead time of one (1) week; The holding cost is $6 per unit per year. Assume there are exactly 52 weeks per year. The following questions are all based on these data, but are independent.

[4]	(a)	Assume the manager decided to adopt a fixed-order-quantity model, but decided not to carry any safety stock (e.g. the size of the safety stock is 0). Calculate the probability of stocking out during the lead time, and the expected demand units lost during the lead time. Show your work and give sufficient details.

[2]	Probability of stocking out during lead time:

R = µL +zσL

µL = dL = 196 x 1 = 196

z = (x - µ) / σ = 196 – 196 / 0 = 0

σL = √σd x L = √25 x 1 = 25

Since z = 0, P(stockout) = 0.5 – 0 = 0.5

Therefore, the probability of stocking out during lead time is 50%

[2]	Expected demand units lost during the lead time:

For z = 0, E(z) = 0.399  E(z) table

Expected demand units lost during L = σL x (E)z = 25 x 0.399 = 9.975

Therefore, the expected demand units lost during lead time is approximately 10 units.

[4]	(b)	Assume the manager decided to adopt a fixed-order-quantity model, and wishes to maintain a 90% service level (type 1 service). Calculate the optimal order quantity and the reorder point. Show your work and give sufficient details. (Reminder: this question is independent of question (a) ).

[2]	Optimal Order Quantity:

Q* = √2DS/H

= √2(196x52)(50) / 6

= 412.12

Therefore, optimal order quantity is 413 units.

[2]	Reorder Point:

ROP = dL + z σL

= 196 x 1 + z (25)

z = .90 – .50 = .4

z = 1.285

= 196 + 1.285(25)

= 196 + 32.125

= ~ 228

Therefore, reorder when inventory drops to 228 units.

[5]	(c)	Assume now the manager decided to adopt a fixed-time-period model with T = 4 weeks as review period (i.e. time between orders). Assume the beginning inventory is 300 units. What should the order quantity be so that 98% of the demand will be satisfied? Show your work and give sufficient details.

Q = S - I

S = d(T+L) + E(z) σT+L

d(T+L) = 196(4+1) = 980

σT+L = σ √T+L = 25 √4+1 = 55.9

E(z) = dT(1-p) / σT+L

= 196(4)(1-0.98)/55.9 = 0.2805 table

z = 0.28

S = 980 + 0.28(55.9)

= 995.6 ~ 996

Q = 996 – 300 = 696

Therefore, the order quantity to satify 98% of demand is 696 units after you take into account the 300 units in inventory.

[4]	(d)	The production manager wishes to set up a fixed-time-period inventory system so that the probability of stockout is at most 2.5%. However, given storage space constraints, the size of the safety stock must be at most equal to the expected demand over one week which is 196 units. What is the maximum time between orders (e.g. T is the time between two orders) that would meet these criteria? Show all your work.

p = 0.975

ss = 196

z = 1.96

SS = z σT+L

196 = 1.96 σT+L

σT+L  = 196/1.96 = 100

σT+L  = 25 √T + L

100 = 25 √T +1

4 = √T + 1

16 = T + 1

T = 15

Therefore, the maximum time between orders is 15 weeks.
[9]	Problem 4		Just-in-Time Philosophy

To illustrate the process of "continuous improvement" and "elimination of waste", a "Just-in-Time Production Game" was played in class: an inefficient production system was simulated, the inefficiencies were analyzed and a discussion followed, during which possible changes were proposed that would facilitate the transition of the production system to a "Just-in-Time" system.

State three of the seven JIT success factors which were implemented and briefly explain how they were used to eliminate inefficiencies. In each case, refer specifically to something that was done in the game.

JIT success factor number 1: _______________________________________________

Change to the system and resulting elimination of inefficiencies (please refer to the game):

JIT success factor number 2:_______________________________________________

Change to the system and resulting elimination of inefficiencies (please refer to the game):

JIT success factor number 3:_

Change to the system and resulting elimination of inefficiencies (please refer to the game):

[10]	Problem 5		Quality Control: Control Charts

Forty-eight (48) samples of 100 units each were taken on the production of an assembly line manufacturing microchips. The system operates around the clock, and there are two 12-hour shifts: the day shift begins work at 7:00 and the night shift begins work at 19:00. One sample was taken at the end of each production hour for a period of two consecutive days. The sample results are summarized in the Table below (samples taken between 8:00 and 19:00 inclusive were taken on the production of the day shift):

| Time | Number of Defective Chips |  | Time | Number of Defective Chips |  |
| --- | --- | --- | --- | --- | --- |
| Day shift | Day 1 | Day 2 | Night Shift | Day 1 | Day 2 |
| 8:00 | 2 | 1 | 20:00 | 2 | 2 |
| 9:00 | 2 | 2 | 21:00 | 1 | 2 |
| 10:00 | 0 | 1 | 22:00 | 3 | 3 |
| 11:00 | 3 | 4 | 23:00 | 3 | 4 |
| 12:00 | 3 | 2 | 0:00 | 3 | 3 |
| 13:00 | 5 | 4 | 1:00 | 4 | 3 |
| 14:00 | 4 | 5 | 2:00 | 5 | 5 |
| 15:00 | 5 | 5 | 3:00 | 4 | 5 |
| 16:00 | 5 | 6 | 4:00 | 5 | 5 |
| 17:00 | 7 | 6 | 5:00 | 6 | 7 |
| 18:00 | 6 | 5 | 6:00 | 7 | 8 |
| 19:00 | 7 | 7 | 7:00 | 9 | 8 |
| Total | 49 | 48 | Total | 52 | 55 |

[4]	(a)	Using these data, calculate 99% upper and lower control limits.

pdoublehat = 49+48+52+55 / (48)(100)

= 204 / 4800 = 0.0425

99% = 2.33 standard deviations

Upper Control Limits (UCL):

pdoublehat + σ√ pdoublehat (1 – pdoublehat) / n

= 0.0425 + 2.33 √ 1 – 0.0425 / 100

= 0.089 = 8.9%

Lower Control Limits (LCL):

pdoublehat - σ√ pdoublehat (1 – pdoublehat) / n

= 0.0425 - 2.33 √ 1 – 0.0425 / 100

= - 0.0045 ~ 0%

[6]	(b)	Below you will find a plot of the percentage of defectives for each of the forty-eight (48) samples taken during the two days of the study. On the graph, show clearly the average proportion of defective chips and the 99% control limits to complete the control chart. From this chart, what are your conclusions and recommendations?

[2]	Control limits and average proportion on graph

[2]	Conclusions (interpretation of the the control chart graph):

According to the upper and lower control limits, there is one point that is out of control at 7pm at the end of the day 1 night shift. There also seems to be a pattern present. For those reasons, my conclusion would call for an investigation to see what the problem is and solving it.

The night shift have more defective.

[2]	Recommendations (action to take based on your conclusions):

During the time when defects are higher than average, especially at the end of their shift, I would recommend to include more breaks for the workers, or even change the policy from two 12 hour shifts to three eight hour shifts. Also I would provide coffee for them.

Provide incentive, like award, or more pay.

[4]	Problem 6		Quality Control: Process Capability

In an assembly line, resistances for micro-chips are manufactured. Random samples taken regularly on the production of theses resistances have shown that the average resistance is 1,000 ohms, and that the standard deviation is 3 ohms.  According to specifications, only resistances between 994 and 1,006 ohms are acceptable.

[2]	(a)	Calculate the Process Capability.  What can you conclude?

Cp= tolerance/6σ

= 1,006 – 994 / 6(3)

= 12/18

= 0.666

Therefore, since 0.666 < 1, we can conclude that this process is not capable of meeting the specifications.

[2]	(b)	What proportion of all resistances produced meet the specifications? Show all your calculations.

z = x - µ / σ

= 1006 – 1000 / 3

= 2  z table = 0.4772

Therefore, 47.72% of the resistances produced meet the specifications.