Gravimetric Analysis of a Chloride Salt

Lab partner:

11/09/10

Purpose:

To understand gravimetric techniques by determining the amount of chloride in an unknown soluble salt.

Theory:

A common way of precipitating a chloride ion is by adding a silver ion, to create silver chloride.

Ag+(aq) + Cl-(aq) → AgCl(s)

Solid silver chloride (AgCl(s)) is very insoluble; however, its low solubility can be expressed by:

AgCl(s) → Ag+(aq) + Cl-(aq)				Ksp = 1.6x10-10

This precipitation reaction of silver chloride happens rapidly, which may cause other ions or precipitates to be present during precipitation. For example, a weak acid such as carbonate (also known as carbonic acid) could form a precipitate with excess silver ions, causing the precipitate to have a slightly higher mass, thus skewing the results of further calculations. To avoid any excess ions interfering with this process is to allow precipitation to carry out as slowly as possible in the presence of an acid solution. The acid intermediate helps prevent co-precipitation.

Such quick precipitation also causes silver chloride to form into very small particles, called a colloid. To make it into a more definite form, the precipitate has to be heated and stirred gently in the presence of nitric acid.

Ag+(aq) + Cl-(aq) + heat → AgCl(s)

The nitric acid prevents the precipitate from reverting back to a colloid state. If the sample was to re-colloid, it would easily wash right through the filter, causing the analytical results to be low.

Once the precipitate is dried, the silver and chlorine can be separated by simply being in the presence of light, illustrated in the below equation:

AgCl(s) → Ag(s) + 12Cl2(g)

This decomposition happens quite quickly in strong light; however, it mostly occurs on the surface of the precipitate because its opaqueness protects all the material below the surface.

If there are excess silver ions during the decomposition, another reaction may occur:

3Cl2(g) + 3H2O(l) + 5Ag+(aq) → 5AgCl(s) + ClO3-(aq) + 6H+(aq)

In this reaction, some of the chloride ions (Cl-) become chlorate ions (ClO3-), resulting in less precipitate, causing poor analytical results.

Prior to the formation of the precipitate, the unknown chloride salt was first dissolved in 100 mL of water before subsequently adding the silver. This method is effective because though the precipitate is fairly insoluble in water, small traces of AgCl residue can still dissolve due to the small solubility constant that the precipitate does have.

Ksp = [Ag+][Cl-] = 1.6x10-6

[Ag+] = [Cl-] = (1.6x10-10)1/2 = 1.3x10-5 mol/L = 1.3x10-6 mol/100mL

MW AgCl = 143.321 g/mol

m lost = (1.3x10-6 mol/100mL)(413.321 g/mol)

= 1.9x10-4 g/100mL

Thus, 1.9x10-4 g of the precipitate is lost by washing with 100 mL of distilled water. This value is extremely small and thus is negligible as it would not impact the final mass of the precipitate.

There is a possibility that other cations in the solution could form precipitates with the chloride, such as potassium or sodium:

K+(aq) + 2Cl-(aq) + Ag+(aq) → AgCl(s) + KCl(s)

Na+(aq) + 2Cl-(aq) + Ag+(aq) → AgCl(s) + NaCl(s)

This co-precipitation would affect the results differently, depending on the weight of the other precipitates.

Procedure:

An unknown salt sample was obtained, as well as a clean, dry, and cooled glass filter crucible. Using an analytical balance, 0.1358 g (±0.0001 g)of the salt sample was weighed out by difference into a clean 250 mL beaker. 100 mL of distilled water and 1mL of 6M nitric acid (HNO3) was added to the beaker and stirred until the salt sample had dissolved completely. Approximately 20.5 mL of 0.1M silver nitrate (AgNO3) was also added and gently stirred.

Whilst being stirred, the prepared solution was then slowly heated to a temperature slightly below the boiling point. To check for the completeness of precipitation of the AgCl, a few drops of AgNO3 was added to the solution. If no further precipitate became visible, the precipitation reaction was determined to be complete; however, if more precipitate did become visible after adding the silver nitrate, the precipitation reaction was deemed incomplete. This process was repeated until the precipitation reaction had gone to completion. Once complete, the mixture was placed in a drawer to prevent exposure to light, also preventing further coagulation (as the coagulation of the precipitate into a more crystalline structure is initiated by heat).

The glass filter crucibles were weighed using an analytical balance, and direct contact with the crucibles was avoided at all times, using paper towels or tongs to hold them instead. The mixture was then decanted through the glass crucibles using vacuum filtration apparatus. A few millilitres of 0.01M HNO3 was added to the precipitate remaining in the beaker, then was swirled gently and decanted once more through the sintered glass filter – this was repeated three times. Again, the precipitate was washed with the nitric acid solution; however, this time the precipitate was washed into the filter as well, instead of remaining in the beaker. Any precipitate sticking to the sides of the beaker was washed off with distilled water.

To ensure the filtrate had no silver ion remaining, some of the washings were collected in a test tube, and a few drops of HCl were added, to see whether or not additional precipitate had formed. The washing was repeated until no silver was detected, and then the precipitate was washed with 5 mL portions of acetone three times, successively.

The glass crucible filter with the precipitate was removed from the vacuum filtration apparatus, and was heated in an oven at 110°C for 30 minutes, and was then let to cool for approximately 5 minutes, and was weighed on an analytical balance. This process was repeated until a constant weight was obtained.

Data/Observations:

The sample number obtained was 360. It was a light purple colour, solid, and found in small, coarse chunks. The precipitates were white in colour, and were in small, finer solid pieces than the salt sample

Two trials were done on this salt sample. When testing for the completion of precipitation, an additional 1.3 mL of AgNO3 was added to the first trial, and an additional 6 mL was added to the second. When checking for the completeness of the washings, no extra HCl needed to be added for either trial.

.

Table 1 – Data for trial 1:

| Mass of vial (initial) | 14.1994 g ± 0.0001 g |
| --- | --- |
| Mass of vial (final) | 14.0636 g ± 0.0001 g |
| Mass of sample | 0.1358 g ± 0.0002 g |
| Mass of crucible | 31.3401 g ± 0.0001 g |
| Amount of AgNO3 added | 22.8 mL ± 0.5 mL |
| Oven temperature (before and after) (1st time) | 112 °C, 81 °C |
| Drying time (1st time) | 5:01 – 5:31 (30 minutes) |
| Cooling time (1st time) | 5:31 – 5:35 (4 minutes) |
| Mass of crucible after drying (1st time) | 31.5873 g ± 0.0001 g |
| Oven temperature (before and after) (2nd time) | 90 °C, 119 °C |
| Drying time (2nd time) | 5:35 – 6:05 (30 minutes) |
| Cooling time (2nd time) | 6:05 – 6:10 (5 minutes) |
| Mass of crucible after drying (2nd time) | 31.5869 g ± 0.0001 g |
| Mass of precipitate | 0.2468 g ± 0.0002 g |

Table 2 – Data for trial 2:

| Mass of vial (initial) | 14.3373 g ± 0.0001 g |
| --- | --- |
| Mass of vial (final) | 14.1994 g ± 0.0001 g |
| Mass of sample | 0.1379 g ± 0.0002 g |
| Mass of crucible | 31.3590 g ± 0.0001 g |
| Amount of AgNO3 added | 26.0 mL ± 0.5 mL |
| Oven temperature (before and after) (1st time) | 112 °C, 81 °C |
| Drying time (1st time) | 5:01 – 5:31 (30 minutes) |
| Cooling time (1st time) | 5:31 – 5:35 (4 minutes) |
| Mass of crucible after drying (1st time) | 31.8624 g ± 0.0001 g |
| Oven temperature (before and after) (2nd time) | 90 °C, 119 °C |
| Drying time (2nd time) | 5:35 – 6:05 (30 minutes) |
| Cooling time (2nd time) | 6:05 – 6:10 (5 minutes) |
| Mass of crucible after drying (2nd time) | 31.8624 g ± 0.0001 g |
| Mass of precipitate | - g ± 0.0002 g |

Calculations:

1. Amount of AgNO3 required:

n Cl- = 0.055 g35.5 g/mol

= 0.001549 mol

= 1.55x10-3 mol

n Ag+  = n Cl-

= 1.55x10-3 mol

C = 0.1 mol/L

V = nC

= 1.55x10-3 mol0.1 mol/L

= 1.55x10-2 L

= 15.5 mL

15.5 mL + 5.0 mL = 20.5 mL

2. Percent chloride in sample (trial 1):

m AgCl = 0.2468 g ± 0.0002 g

MW AgCl = 143.3321 g/mol

n AgCl = mMW1

= 0.2468 g143.21 g/mol

= 1.722x10-3 mol

n Cl- = n AgCl

= 1.722x10-3 mol

MW Cl- = 35.45 g/mol

m Cl- = nMW

= (1.722x10-3 mol)(35.45 g/mol)

= 0.06104 g

% Cl- = mass of Cl-mass of sample x 100

= 0.06104 g0.1358 g x 100

= 44.95 %

3. Percentage uncertainty (trial 1):

= ± 0.0002 g0.2468 g x 100

= ± 0.0810 %

± 0.0810 % = x0.06104 g x 100%

x = ± 0.00004944 g (for the mass of Cl-)

± 0.00004883 g0.06104 g Cl- x 100+± 0.0002 g0.1358 g (sample) x 100

= 0.0810 % + 0.147 %

= 0.228 %

0.228 % = x44.95 % x 100%

x = ± 0.102 %

Therefore, the percentage uncertainty for the percent of chloride in the sample is ± 			0.102 %.

4. The average of the two trials:

Average = Trial 1+Trial 22

= 44.95 %+90.31 %2

= 67.63 %

5. The relative error of the average between the two trials:

Percentage error = actual value-theoretical valueactual value x 100

= 53.35 %-67.63 %53.35 % x 100

= 26.77 %

6. The relative spread:

Trial 2-Trial 1Average value x 1000

= 90.31%-44.95%67.63 % x 1000

= 670.7 ppt

Discussion

The results for trial one were a slightly lower than the accepted value of percentage chloride in the salt sample, whereas the results for trial two were fairly high. The results obtained from trail one were low because as the salt sample was being weighed out by difference, some of the salt was spilled, resulting in slightly less salt than what was recorded. Another possible reason for such low results may have been the beaker being exposed to light for long periods of time causing some chloride ions to react, becoming soluble chlorate ions. The higher results in trial two may have resulted because the precipitate was not washing with the 0.1M HNO3. This may have resulted in co-precipitation leading to a larger-than-expected mass of precipitate. Also, a substantial amount of AgNO3 was added to the solution in the second trial, to ensure the completeness of precipitation. This was likely due to human error, thus the amount of HNO3 added to dissolve the salt sample was not measured correctly, making it possible for co-precipitates to form.

Conclusion

The percentage chloride by mass in the salt sample was calculated to be an average of 67.63% between the two trials, and the accepted value for that specific sample was 53.35%. The percentage uncertainty for the value obtained in trial one was found to be ± 0.102%, and for trial two it was ± 0.207%. These results for trial one had high precision, but low accuracy, obtaining a value only about 8.4% lower than the accepted value. The results for trial two were greater than the accepted value by almost 37%, which shows that there was lower precision and low accuracy.

Bibliography

D.W. Archer et al. Chemistry 1001/1002 & 1005/1006 Introductory Chemistry Laboratory 	Manual 2010-2011, Carleton University, Ottawa, 2010, 57-52.