NAME: Student Number:   Lab Section:

Spectrophotometric Determination of Iron

Calculations:

Prior to any calculations, the Vernier spectra photometer and its appropriate software were needed for a few necessary steps. The software allowed a graph of Absorbance vs. Wavelength to be produced which provided information regarding which wavelength would be optimal to program the spectra photometer with. The optimum wavelength was found to be 486.1nm and is indicated by the first local maximum shown in the graph below:

Graph 1.1: Absorbance vs. Wavelength

Table 1.1: Absorbance Reading vs. Concentration Value for the Solutions

| Solution | Absorbance Reading | Concentration Value (mg Fe/50mL) |
| --- | --- | --- |
| 0.00 | 0 | 0 |
| 0.05 | 0.183 | 0.0483 |
| 0.10 | 0.359 | 0.0947 |
| 0.15 | 0.553 | 0.146 |
| 0.20 | 0.753 | 0.199 |
| 0.25 | 0.934 | 0.246 |
| Unknown #1 | 0.881 | 0.232 |
| Unknown #2 | 0.782 | 0.206 |

*note that the values in red are calculations that will be explained below

The Beer-Lambert’s Law is typically used to determine the absorbance of a substance, as shown below:

A=ε∙c∙l

Where A = Absorbance of intensity from the beam by the sample, ε = Molar extinction coefficient (or the slope of the absorbance vs. concentration graph), l = Solution path length (in cm), and c = Concentration in mol/L. With this formula the concentration of the solution can be solved for in mol/L:

c=Aε∙l

From Table 1.1, a graph was produced plotting absorbance readings versus concentration (in mg Fe/50mL) (Graph 1.2). The line of best fit was added and the equation of the line was found to be y=3.789x. The form y=mx+b (where m is the slope and b is the y-intercept) suggests that the slope of the line (or the molar extinction coefficient, ε) is 3.789.

Graph 1.2: Absorbance vs. Concentration (mg/50mL)

Through the analysis of Graph 1.1, it is possible to say that y=3.789x, where y equals the absorbance reading and x is equal to the concentration in mg Fe/mol. Using this information the concentrations of the solutions (including the two unknown solutions) can be calculated:

Calculation 1.1: For Unknown #1

y=3.789→x=y3.789

Since the absorbance reading for unknown sample 1 is 0.881, then

x=0.8813.789=0.2325=0.232

∴for Unknown #1,concentration mgFe/50mL=x=0.232 mgFe/50mL

Calculation 1.2: For Unknown #2

y=3.789x→x=y3.789

Since the absorbance reading for unknown sample 1 is 0.683, then

x=0.7823.789=0.2063=0.206

∴for Unknown #2,concentration mgFe/50mL=x=0.206 mgFe/50mL

The accepted value for the Unknown #1 was given and found to be: 0.2092 g/L. To calculate the relative error between the accepted value and the determined value for Unknown #1:

0.2325mg50mL∙1000mLL∙g1000mg∙50 (dilution factor)

=0.2325gL

relative error=|accepted-determined|sum of accepted and determinedx100%                              =|0.2092-0.2325|0.2092+0.2325x100%                                                                                        =~5.28%

Discussion:

The results gathered appear to be accurate to ~5.28 percent. Relative error is a crucial method that must take place in all laboratory experiments. The validity of a result is directly related to the relative error that may have accumulated throughout the laboratory experiment.

Possible discrepancies may have occurred during the blanking process, where the colorimeter was being zeroed. Oils and other residue from hands may have skewed the readings when finding the optimum wavelength at which the absorbance was to be recorded. Prior to handling the casings used in the blanking process, an individual is recommended to wear rubber gloves in order to reduce residue and contamination. Another possible error may have occurred if an individual did not use distilled water to dilute their solutions of the Iron sample. Trace substances (which may include iron itself) could potentially have changed the iron content in the solution, thus increasing or decreasing its concentration.

Conclusion:

With limited error, the iron content of a sample was successfully determined by spectrophotometric analysis. The concentrations of the two unknown solutions (expressed in mg Fe/50mL) were found to be ~0.232 for unknown sample 1 and ~0.206 for unknown sample 2. Relatively, this is correct to approximately 5.28% due to potential residue contamination.