Arc Welding

- Welding process which coalesces materials through the use of heat from an arc

  - SMAW (Stick welding)

    - About ½ of all large-scale industrial welding use this

    - Arc is created around the electrode and work piece

    - When the electrode melts a flux melts and shields the weld

    - Constant current is used

  - Submerged Arc Welding

    - Similar to SMAW but designed to be automatic

    - Spool of wire is used instead of electrode and sand is poured around the weld and reusable

    - No sparks, everything is under flux

    - ONLY FLAT SURFACES

  - GMAW (MIG)

    - Arc is produced by a continuous wire electrode

    - Shielding gas is used

    - Constant VOLTAGE is used

    - Fast

  - FCAW (Flux)

    - Same as MIG but no shielding gas is used

    - A flux in the core of the wire is vaporized and creates the shielding gas

    - Large amount of fumes

  - GTAW (TIG)

    - Constant current

    - Electrode is not consumable

    - Uses a ground tungsten electrode

    - If no filler is added to the weld it is called autogenous

  - Plasma Welding

    - Inert plasma is created and is used to fuse the metal

    - Creates deep narrow welds

Oxyfuel Gas

- 3 types of flames are used Neutral, oxidizing, and carburizing. Uses oxy fuel and large pressures to push the material togeather

  - Air acetylene welding

  - Oxyacetylene welding

  - Oxygen/propane welding

  - Oxyhydrogen welding

  - Pressure gas welding

Solid State Welding

- Welding processes that coalescence at temperatures below the melting point of the base materials being joined without the use of brazing filler metal

  - Roll Bonding

  - Ultrasonic Welding

    - Metallic tip vibrating at ultrasonic frequency to join a thin piece to a thicker one

    - Consists of power source and a transducer

    - Either single point or roller

    - 20-60 kHz is typically used

  - Friction Welding

    - High speeds and pressures are used to create the bond

    - Material should be capable of forging and generate sufficient heat at interface

    - Not suitable for brittle materials

    - Not suitable for alloys with graphite and lead

    - Used to join similar and dissimilar materials

    - Stir welding – a rotating tool is inserted into the work piece and moved around the weld line

  - Diffusion Welding

    - Materials are simply pressed together at a high temperature

    - No deformation

    - Very slow process

  - Explosion Welding

    - Weld is achieved by making one part strike the other at a very high but subsonic velocity

    - Process is completed in microseconds

Resistance Welding

- Current flows through work piece

- Pressure forces the two work pieces together

- Time varies the amount of heat generated

  - Resistance Spot Welding

    - Fuck

  - Resistance Seam Welding

    - Off

  - Resistance Projection Welding

    - One piece has a flange that creates weld nuggets

    - Uses lower currents, forces, and cycle times than spot welding

    - Greater electrode life than spot welding due to larger contact surface

  - Flash Welding

    - Arc is created between part and substrate

    - Once weld pool is established the stud is pressed onto the substrate

    - Flash welding is welding two studs together

  - Upset Welding

    - Produces coalescence over the entire area of faying surfaces or progressively along a butt joint by the heat from the resistance to the flow

    - Pressure used to complete the weld

Other Welding

  - Electrogas Welding

    - Vertical arc welding

    - Shielding gas is used

    - Like arc welding, no application of pressure

  - Electroslag Welding

    - Vertical arc welding

    - Done from the top down

  - Electron Beam Welding

    - Beam is used to heat and fuse the material

    - Extremely high quality welds

    - Very expensive

    - Can weld much deeper than laser but must be done in a vaccuum

  - Laser Beam Welding

    - Beam is used to heat and fuse the material

    - Main application is for repairing welding defects

  - Thermite Welding

    - Mixture of aluminium powder and metal oxide that result in a non-explosive exothermic reaction when ignited.

    - Molten metal is used for joining metal parts by pouring it between them resulting in cast weld joint