Textbook Notes: Landslide

Lesson 1: Landslide Mechanics

Socio-Economic Impact

Vulnerability to landslides is increased by:

- Increase in population density

- Use of marginal land

- Rapid land-use change

- Global warming

- Elm Event, Switzerland 1881

- Sudden release of 10 million m3 of rocks

- 15 people dead

- Stemmed from extraction of slate from a quarry

- La Conchita, California 1995 and 2005

- Cased by heavy rainfall

- On complex combination of faulting, tectonic uplift, very weak foundation, steep slopes and presence of spring

- Haiyan, Ningxia Earthquake and Landslides 1920

- 200,000 deaths

- direct: destruction

- indirect: exposure to winter climate

- Rockfall and Slide Events

- involve a smaller rock mass that disintegrates into numerous blocks that fall

Effects of Gravity

Mass movement (mass wasting): downslope motion of soil and/or rock materials under the influence of gravity

Landslide – general term for mass movements.

- occur when gravitational force pulling the slope mater down > slope materials internal resistance to shear or failure

- Gravity and Shear Stress

Gp – the component of gravity that acts perpendicular to the slope

- helps to hold an object or slope materials in place on the slope (shear strength)

Gt – the component of gravity that acts tangential to the slope surface or parallel to and down the slope

- pulls an object or slope material in a down-slope direction (shear stress)

- Shear Strength

- Forces that resist the down-slope movement

- Frictional resistance

- Cohesion

- Internal resistance of a body to shear stress

Affected by a number of factors:

- Type of soil or rock

- Condition of the geologic materials (how eroded)

- Presence of weak surfaces (joints and faults)

- Pore water pressure within the slope

- Factor of Safety Fs

- failure occurs when

shear stress > shear strength

Fs = Sf/T where Fs = Factor of safety; Sf = Shear strength at failure; T = average shear stress

- numerical value that indicates slope stability

When Fs >= 1.0 (shear strength>shear stress = stable)

When Fs < 1.0 (Shear strength< shear stress = unstable)

Downslope motion is favored by:

- Steeper slope angles

- Decrease of shear strength

- Effective Stress

- Composed of cohesion and frictional resistance

Factor determine the resistance of failure:

- Weight of material itself acting perpendicular to the slope, Gp (normal stress)

- Addition of space between grains (primarily water pressure) = pore water pressure, u

Effective Stress = [normal stress] – [pore water pressure]

Lesson 2: Landslide Causes and Triggers

- Discerning a cause versus a trigger

cause – makes slopes susceptible to movement without actually initiating a landslide

trigger – event that actually initiates movement by driving the Fs < 1.0

ex. Earthquake, volcano, rainfall, etc.

Landslides can have many causes, but 1 trigger

Trigger Frequency – how often an even occurs that set off a landslide

- Slope Stability and Erosion

Erosion – the transport of material away from its source

Erosion at the base of a slope can cause failure:

- Driving mass becomes > then resisting mass

- Slope steepness at the base increases (Shear stress> Shear strength)

Rapid Erosion – can be caused by heavy rainfall or large waves

- Slope Stability and Vegetation

- vegetation increases slope strength

- increases cohesion, binds material

- decreases surface erosion and helps absorb excess water

- Slope Stability and Water Content

- An excessive amount of water increases mass, increasing shear stress

- Fluid Pressure; can reduce friction, reduce shear strength resulting in slope failure

- Water increases weathering of rocks, causes volume change that can wedge apart rocks that causes them to fail.

- Slope Stability and Seasonal Precipitation

- landslides occur more frequently after periods of heavy rainfall

- different types of rainfall can cause different type of landslides (eg. La Conchita)

- Slope Stability and Volcanic Activity

- loose volcanic ash followed by accelerated erosion and mud/debris flow triggered by intense rainfall = landslides

- Nevado Del Ruiz; Columbian Volcano

- No warning

- Volcano melted glacier causing lahars

- Mt. St. Helens

- earthquake shook loose flank causing landslide

- Slope Stability and Earthquakes

- strong ground shaking during seismic activity is a very common triggering mechanism

- Slope Stability and Anthropogenic Activity

Ways in which humans can increase likelihood of a landslide:

- Excavation of a slope at its base/toe

- Loading of a slope at its crest

- Deforestation

- Irrigation

- Mining

- Water leakage from utilities

- Artificial vibration

- Vaiont Dam

Initial filling: long tension crack opened near top of the slope – large landslides

Second: Water stopped filling at 648m

Third: Raised third time and slope movement resumed. Water lowered to stop slope movement. Velocities continued acceleration

Imminent Filling: Water being lowered not helping

The Landslide: Huge landslide generating huge wave of water that passed over the dam – the landslide was reported as not predictable

Lesson 3: Landslide Classification

- The Principle of Classification (landslide)

Velocity

7 extremely rapid – none (sudden death)

6 very rapid – none (death)

5 rapid – evacuation

4 moderate – evacuation

3 slow – maintenance

2 very slow – maintenance

1 extremely slow – none (minimal disruption)

- Falls

Fall – sudden, vertical movement of material

Bedrock Falling – rock fall

Unsorted soil – debris fall

Velocity = /2gz

G = acceleration due to gravity (9.8m/s2)

Z = height of fall

- Topples

Topples – require forward rotation of material (rocks) about a pivot point below the center of gravity of the unit or in the slope

- fractured material oriented perpendicular and parallel to the slope face

- material is broken up into coherent pieces that roll forward sown a slope

- cant be debris topple because blocks must be coherent to topple

- Slides

- involve large volumes of rock or soil material moving as an initially coherent mass on a sliding surface

- Rotational Slide: blocks of soil or weak rock move along a curved rupture

- formed because shear stress> strength

- blocks of material rotate and life

- Translational slides: blocks of strong rock move upon weak plane or pairs of intersecting weak planes in a rock

- Spreads

- Lateral Spreads – occur where there is a slow to rapid extensional movement of material

- Usually related to sudden liquefaction of a weak soil layer

- Often triggered by earthquakes and explosions

- Can occur on relatively shallow slopes

Liquefaction – sudden loss of strength in the soil transforming its behavior into that of a liquid

- loose, saturated soil and seismic activity can cause water pressures to dramatically increase

FS = [ Normal Stress – Pore Water Pressure ] x tan0 + cohesion/ Shear Stress

- Factor of safety sharply decreases because of the dramatic increase in pore water pressures

- Sensitive Clays and Spreads

Sensitive Clays: young marine clays that are deposited with a House-Of-Cards like structure

- during an earthquake, this may collapse and liquefy

Canada’s Quick Clays: St. Lawrence Rivers valleys are lined with thick beds of sensitive marine clays, known as Leda Clays

- earthquakes can cause liquefaction of clay (loss of shear strength), causing devastation to large areas of flat terrain.

- Flows, Creep and Long Run-out Debris Flows

- Flows occur when there is a relatively large volume of water present in mixture of soil and debris

- flows downhill as a chaotic, viscous mixture

- Rock or Soil Creep: type of flow where soil and upper bedrock zones move downslope extremely slow

- usually measured in mm or cm per year

- creates the most long term economic problems

- Debris Flow: rapid flow of fully saturated debris in a steep channel

(Flows into a debris fan/colluvial fan)

- Debris Avalanche: Rapid and shallow flow of partially-fully saturated debris on a steep slope, not confined to an established channel

(Flows into a run out/ colluvial apron)

- Debris Flow and Avalanches are found in mountainous areas with high rainfall

- Can move at speeds > 330 km/h

- Depend on viscosity (amount of water) and density (abundance of rocks)

- Complex Landslides

- formed by combination of 2 or more landslide types

- eg. long run-out debris flows, sturzstroms

Sturzstroms: rare category of dry rock avalanche that travel vast horizontal distances with only small vertical drop

- mass movement of dry rock debris

- volume > 10^6m3

- mobility increases with volume

Landslide Mitigation

- Mitigation – trying to fix the problem

- Avoidance – relocating a facility, structure or route where landslide hazards are

- Landslide Prevention – reduce driving forces or increase resisting forces

- Landslide Protection – minimizing hazards present

- Landslide Frequency in BC

(Central and Eastern Canada)

f = (0.0005 – 0.0020 events/year)/(10000km2 mountainous terrain)

Return Period = 1/F = 2000 to 500 years

Western Canada

F = 0.01 to 0.02 events/year

RP = 25-100years

RP = (time span of recorded data)/(# of cases of Magnitude, M)

- Anthropogenic Effects

People heighten landslide natural disasters by:

1.Developing on steeper slopes – response to overpopulating

2. Removing Vegetation

3. Excavation and Slope Loading

4. Altering water drainage

5. Altering climate

- Case: Frankslide

- 82 million tonnes of limestone buried town of Frank

- Complex Slide

- Large volume rock fall – long run-out debris flow

- Limestone is easily weathered when it is chemically reacted with slightly acidic rainwater

- Cavities form that reduce shear strength

- Excavation and mining partially at fault

- Case: Sea to Sky, Mt. Garibadi and Rubble Creek

- Steep slopes, weak rock, volcanic and seismic activity present at Mt. Garbidi

- Mt. Garibadi: composite cones where hot, acidic water permeated through the fractured cone (altering the weak clays through hydro thermal alteration)

- Rubble creek is the outflow from the springs that originate at the base of the barrier

- Case: Lions Bay

- heavy rainstorm triggered

- form debris fan

- Case; Lions Bay Retention Dam

- modified dam with 2 central outflow conduits

mitigates by:

- Remove debris flow from the flow mass

- Prevent more debris from being incorporated into flow mass

- Decrease the flows velocity and erosive capabilities

- Prevent debris from overstepping the channel

- Living With Landslides

Engineering an unstable slope: excavation, grading, pumping, draining, installing surface drainage devices, planting, vegetation, retention walls, reinforcing the slide mass.

Landslides Textbook

Frank Slide – Canadian rock avalanche

Rock avalanche – sudden failure of large mass of rock that fragments rapidly and travels as streaming mass

Contributing factors:

- cause is a internal or external factor that causes a reduction in slope stability over time

- trigger is an event that sets off landslide

- Geology

- Glaciation

- Mining

- Weather

- Mass wasting/landslide: downslope movement of rock or sediment due to gravity

To classify a landslide:

- mechanism of movement (fall, topple, etc.)

- type of material

- amount of water

- rate of movement

Types:

Fall – bounding of rock or blocks of sediment from the face of a cliff

Slide – downslide movement of a coherent block of rock/sediment along a discrete failure plane

Slump – type of slide that takes place on a failure plane that is curved upwards

Flow – slow to rapid downslope movement of sediment in which particular move semi-independently (commonly aided with water)

Debris Flow – mixtures of mud, debris and water

Creep – very slow movement/flow

Sackung – huge, slow-moving flow

Topple – rock mass pivots on a point

Rock avalanches – high velocity flows of rock

Subaqueous (underwater) landslides – most often complex events

Forces on Slopes:

Shear strength – resisting force of landslide

Factor of Safety (Fs) – evaluation of slope stability

If Fs <= 1; stable

If Fs > 1; failure expected

Forces determined by variables:

- type of material

- slope and topography

- climate

- vegetation

- water

- time

Collovium – mixture of weathered rock and debris

Thaw Flow Slides – shallow landslide; typically occur during warm spells

La Conchita

Vulnerability: Presence of steep high slopes, weak rocks, numerous historic and prehistoric landslides, periodic prolonged and intense rainfall

Humans Causing Landslides

- Timber Harvesting

- Urbanization

Minimizing Risk

- Identification of Potential Landslides

- Prevention: Drainage Control, Grading Slope Supports and Landslide Warning Systems