Q1. What is compaction?

The compaction is the densification of the soil through external compactive effort by applying mechanical energy.

Q2.Why is it important to have well compacted soil? Name three

consequences of poor soil compaction.

It is important to have well compacted soil because a well compacted soil will have higher strength and lower permeability. It will be good to have the requested soil resistance and density for the foundation of buildings. The three consequences of poor soil compaction are erosion of foundation, failure of road and collapse of old excavation.

Q3. What is optimum water content (moisture content) of a compacted

soil?

The optimum water content of a compacted soil is the water content that will make the soil to have maximum degree of compaction.

Q4. What factors can affect the soil compaction?

The factors can affect the soil compaction are compaction energy or effort, the type and the grain size distribution of soil and water content and dry density.

Q5.Name three different techniques of soil compaction that the engineers

can use to conduct deep ground compaction in a large area.

The three different techniques are deep dynamic compaction, vibro-flotation and the dynamic technique compaction.

Q6.The results of a compaction test performed on clayey silt are presented

in the table given below. Find the maximal dry unit weight and the optimal

Water content of the soil. Draw the compaction curve, the saturation curve

(S = 100%) and the S = 90% saturation curve (Gs = 2,65).

Water content (%) Dry unit weight (kN/m3)

Simple Calculation:

| Unit Weight of Water(9.8 KN/m3) |  | Gs(2.65) |  |
| --- | --- | --- | --- |
| Water Content (%) | Dry unit weight (kN/m3) | Saturation(90%) (kN/m3) | Saturation(100%) (kN/m3) |
| 5 | 15.2 | 22.6 | 22.9 |
| 7 | 17.2 | 21.5 | 21.9 |
| 8 | 18.9 | 21.0 | 21.4 |
| 10 | 19.5 | 20.1 | 20.5 |
| 11 | 19.1 | 19.6 | 20.1 |
| 13 | 17 | 18.8 | 19.3 |

According to the table, we find the maximal dry unit weight and the optimal

Water content of the soil are 19.5 kN/m3 and 10%.