Question 1  (1 point)
   When selecting a sequence of views for an othographic drawing, you should choose: 
       1.          The sequence with the least number of hidden features.   
       2.          The standard views every time (Top, Front, Right Side).   
       3.          The easiest to visualize.   
       4.          The sequence is arbitrary   
       5.          None of the above   
 
Question 2  (1 point)
   In what types of drawings can true length measurements be made on the front plane? 
       1.          Perspective   
       2.          Oblique   
       3.          Orthographic Projections   
       4.          Both answers 1 and 2   
       5.          Both answers 2 and 3   
       6.          None of the above   
 
Question 3  (1 point)
 Given the pictorial shown below, what are the minimum number of views that are necessary to completely describe the object for manufacturing purposes?        
       1.          An isometric view   
       2.          One orthographic view   
       3.          Two orthographic views   
       4.          Three orthographic views   

 
Question 4  (1 point)
   Which of the following is an oblique pictorial:
1.          Figure A   
2.          Figure B   
3.          Figure C   
4.          None of the above   
 
Question 5  (1 point)
   How are layers suppressed in IntelliCAD viewports? 
1.          By activating the erase feature   
            2.          By activating a "hide" command   
3.                    By activating a "freeze" command in the Explorer window  
4.                    Both answers 1 and 2 above   
5.                    Both answers 1 and 3 above   
6.                    None of the above   
 
Question 6  (1 point)
   The magnitude of the cross product of two vectors is: 
1.          Equal to 1/2 the area of the parallelogram spanned by the two vectors   
            2.          Equal to the area of the parallelogram spanned by the two vectors   
            3.          Equal to twice the area of the parallelogram spanned by the two vectors   

            4.          Not a valid vector operation   
 
Question 7  (1 point)
  Which of the following statements is false? A system of linear equations can have: 
       1.          No finite solution   
       2.          A finite number of distinct solutions   False
       3.          Infinitely many solutions   
       4.          Exactly one solution   
       5.          None of the above is false   
 
Question 8  (1 point)
   What is the Euclidean norm of the vector 
        v = (2,-1,-2)
       1.      &radic;(9)   
       2.          &radic;(5)   E
       3.          &radic;(-1)   
       4.          &radic;(6)   
       5.          &radic;(-8)   
       6.          &radic;(12)   
       7.          None of the above.   
 
Question 9  (1 point)
   Let a = (1,3,1) and b = (1,1,1). What does the vector equation      (a · b) - (b · a) 
evaluate to? 
1.          5   
2.         -5   
3.          3   
4.         -3   
5.          6   
6.        -4   
7.         None of the above.   
 
Question 10  (1 point)
In the Maple software system the operator := does what? 
1.          The variable on the left-hand-side (LHS) of the := is assigned to whatever is evaluated on the right-hand-side (RHS)   
 2.          The variable on the LHS of the := is reset to its initial value   
 3.          Evaluates the RHS of an _expression using double precision floating point numbers   
 4.          Evaluates the RHS of an _expression using either single, or double precision floating point numbers   
 5.          None of the above is a correct answer   
 
Question 11  (1 point)
   Given the following four matrices, which of the 5 matrix-matrix multiplications listed below are valid? a=[1,2;3,4] b=[1,2;3,4;5,6]   c=[7,8,9;4,5,6;1,2,3] d=[-1,5,0;2,-5,6]
   1) a*b
   2) b*d
   3) a*c
   4) c*b
   5) d*a
       1.          Operations 1,3,5   
       2.          Operations 2,4   
       3.          all of the above are valid operations   
       4.          Operations 1,2,4   
       5.          Operations 2,3,4,5   
       6.          Operations 3,5   
       7.          None of the above   
 
Question 12  (1 point)
What does MATLAB stand for? 
1.          Mathematics Laboratory   
2.          Matrix Laboratory.   
3.          Matrix Linear Algebra Basic.   
4.          Matrix Algebra Theory Laboratory.  
 
Question 13  (1 point)
   Let A and B be matrices defined in MATLAB such that
        A = [1,2,3;4,5,6;7,8,9] and 
        B = [0,1,1;1,0,1;1,1,0]
   Given the matrix product P = AB, what is element P(3,2)? 
       1.          9   
       2.          4   
       3.          16   
       4.          17   

       5.          11   
       6.          12   
       7.          None of the above.   
 
Question 14  (1 point)
   Let A and B be matrices defined in MATLAB such that
        A = [1,2;3,4;5,6;7,8] and 
        B = [0,1;0,1;1,0;0,1]
   Given the matrix product P = AB, what is element P(1,2)? 
       1.          3   
       2.          6   
       3.         14   
       4.          10   
       5.          9   
       6.          2   
       7.          None of the above   
 

Question 15  (1 point)
   Which of the following is not a requirement to become registered in PEO (Professional Engineers of Ontario) 
       1.          Must be 18 years old   
       2.          Must be sponsored by an organization doing engineering work   
       3.          Must be a Canadian citizen or permenant resident   
       4.          Must meet the educational standards   
       5.          Must have four years experience in an engineering capacity   
       6.          Must pass a professional practice examination   
       7.          All of the above are requirements for registration   
 

Question 16  (1 point)
   What is the first priority of the Legislative mandate of PEO (Professional Engineers of Ontario)? 
1.          Service for its members   
2.          Public safety   
3.          Advisory to the government with regard to engineering matters   

4.          None of the above   
 
Question 17  (1 point)
   What is the axonometric projection of an object? 
   1. A special case of oblique projection.   
   2. A single-view orthogonal projection showing more than one face of the object.   
   3. A single-view orthogonal projection showing more than one face of the object, with one face in true shape.   
    4.  A single-view orthogonal projection showing more than one face of the object in true shape.   
    5.  A single-view orthogonal projection showing all faces of the object in true shape.   
    6.  A single-view orthogonal projection showing more than one face of the object, with one face perspectively distorted.   
    7.          None of the above.   
 

Question 18  (1 point)
 Which of the following statements best describes a three-point perspective projection? 
1. The projection plane is parallel to two principal axes of the reference coordinate system.   
2. The projection plane is perpendicular to two principal axes of the reference coordinate system.   
3. The projection plane is defined by three vanishing points.   
4. The projection plane intersects exactly two principal axes of the reference coordinate system.   
5. The projection is not parallel to any principal axis of the reference coordinate system.   
6. All but one of the projectors are orthogonal to the projection plane.   
7. None of the above.   
 
Question 19  (1 point)
   What is a detail drawing? 
1.          A drawing which includes a list of materials.   
2.          A drawing where all parts of an assembly are identified   
3.          A working drawing of a single part   
4.          A drawing detailing how parts fit together   

 
Question 20  (1 point)
 In Descriptive Geometry, a line is seen in true length if, and only if it is viewed in a projection plane that is: 
1.          Perpendicular to the line   
2.          Parallel to the line   
3.          Inclined to the line   

4.         Oblique to the line   
5.          None of the above will serve to show the line in true length   
 
Question 21  (1 point)
   Which of the following does not belong in a final engineering report? 
1.          Letter of transmittal   
2.          Progress section   
3.          Acknowledgements   

4.          Recommendations   
5.          All of the above may be part of a final engineering report   
6.          None of the above belongs in a final engineering report   
 
Question 22  (1 point)
   Three planes which intersect in a unique point at infinity, but in no finite points, must have which of the following properties? 
       1.          All three are parallel, but distinct.   
       2.          All three are mutually orthogonal.   
       3.          The planar intersections form a pair of distinct parallel lines.   
       4.          Two intersect in a line that is contained in the third.   
       5.          Two intersect in a point that is contained in the third.   
       6.          All of the above.   
       7.          None of the above.   
 
Question 23  (1 point)
Let vector u = (5,5) and vector v = (3,1). What is the magnitude of the orthogonal projection of u in the direction of v? 
1.          4   
2.          2&radic;(10)   
3.          0.8   

4.          2&radic;(2)   
5.          4&radic;(5)   
6.          25   
7.          None of the above   
 
Question 24  (1 point)
   Let w = (1,-1,1), x = (-2,2,-2), a = (1,0,1). What is the magnitude of the cross product of w and x (indicated by ||w × x||) in the   direction of a? 
1.          &radic;(2)   
2.          2&radic;(2)   
3.          ½&radic;(2)   
4.          &radic;(3)   
5.          ½&radic;(3)   
6.          0   
7.          None of the above.   
.
 
 
Question 25  (1 point)
   Consider the following MATLAB m-file code: 
        index = 0; 
        M = [1,2,3;4,5,6;7,8,9];
        m = M(:,index) 
   When the m-file is run, what will appear in the command window? 
1.          m = 
              1
              4
              7
2.          m = 1 2 3
3.          m = 1
4.          m = 0
5.          An error message.   
6.          None of the above.   
 
Question 26  (1 point)
   Which of the following areas of study is not an area of responsibility that belongs to the disciplines of Civil or Environmental   Engineering? 
    1.          Building and bridge design   
       2.          Air pollution   
       3.          Airport design   
       4.          Public transportation problems   
       5.          Waste water treatment   
       6.          Geotechnical problems   
       7.          All of the above are Civil or Environmental Engineering areas of responsibility  
 

   Question 27  (1 point)
   In the system of equations shown below the coefficients may be thought of as amn in matrix form. Calculate a23 times a32 in the
   coefficient matrix A.
   x + 2y + 3z = 4 
   5x + 6y + 7z = 8 
   9x +10y + 11z = 12
      1.          66   
       2.          67   
       3.          22   
       4.          45   
       5.          70   
       6.          None of the above    

 
Question 28  (1 point)
   Given the voltage V and the resistances R1, R2, and R3 in the circuit shown below what is the size of the coefficient matrix required to solve for all currents. 
       1.          6 by 6   
       2.          1 by 6   
       3.          6 by 1   
       4.          3 by 3   
       5.          1 by 3   

       6.          3 by 1   
       7.          None of the above   
 
Question 29  (1 point)
   In MATLAB the number 10 is assigned to the variable a. What is the output from the MATLAB command 2a^2? 
       1.          400   
       2.          200   
        3.          4a^2   
       4.          None of the above   
 

Question 30  (1 point)
  In the third angle projection, "glass box" coordinate system, the profile projection is 
       1.          the x/y plane   
        2.          the x/z plane   
       3.          the y/z plane   
       4.          the y/minus z plane   

       5.          the x/minus z plane   
       6.          none of the above   
 
 1    When selecting a sequence of views for an othographic drawing, you should choose: 
	[image: image1.wmf]
	1.
	The sequence with the least number of hidden features.   

	[image: image2.wmf]
	2.
	The standard views every time (Top, Front, Right Side).   

	[image: image3.wmf]
	3.
	The easiest to visualize.   

	[image: image4.wmf]
	4.
	The sequence is arbitrary   

	[image: image5.wmf]
	5.
	None of the above   


 
2
Choose the answer that has the correct line type precedence (‘&’ indicates an equal precedence): 
	[image: image6.wmf]
	1.
	(iii), then (ii) & (iv), then (i)   

	[image: image7.wmf]
	2.
	(iii) & (ii), then (iv), then (i)   

	[image: image8.wmf]
	3.
	(iii), then (ii), then (iv) & (i)   

	[image: image9.wmf]
	4.
	(iii), then (ii), then (i) & (iv)   

	[image: image10.wmf]
	5.
	No line is given preference in engineering drawings   


 
3
The following diagram is an isometric pictorial of a cube with one corner removed. Determine which 2 of the 3 principal orthographic views will look the same. 
	[image: image11.wmf]
	1.
	Top and Right   

	[image: image12.wmf]
	2.
	Top and Front   

	[image: image13.wmf]
	3.
	Front and Right   

	[image: image14.wmf]
	4.
	They will all be different   


 
4
Given the pictorial shown below, what are the minimum number of views that are necessary to completely describe the object for manufacturing purposes? 
	[image: image15][image: image16.wmf]
	1.
	An isometric view   

	[image: image17.wmf]
	2.
	One orthographic view   

	[image: image18.wmf]
	3.
	Two orthographic views   

	[image: image19.wmf]
	4.
	Three orthographic views   

	[image: image20.wmf]
	5.
	Both answers 1 and 4 are correct   


 
5
Which statement below best describes an Engineering Code as discussed in class? 
	[image: image21.wmf]
	1.
	A set of rules defining a minimum level of performance   

	[image: image22.wmf]
	2.
	A set of specifications   

	[image: image23.wmf]
	3.
	A set of working drawings   

	[image: image24.wmf]
	4.
	A set of design criteria   

	[image: image25.wmf]
	5.
	All of the above   

	[image: image26.wmf]
	6.
	None of the above   


 
6
What is the transpose of an m-by-n matrix? 
	[image: image27.wmf]
	1.
	An m-by-n matrix.   

	[image: image28.wmf]
	2.
	An m-by-m matrix.   

	[image: image29.wmf]
	3.
	An n-by-n matrix.   

	[image: image30.wmf]
	4.
	An n-by-m matrix   

	[image: image31.wmf]
	5.
	None of the above   


 
7
The magnitude of the cross product of two vectors is: 
	[image: image32.wmf]
	1.
	Equal to 1/2 the area of the parallelogram spanned by the two vectors   

	[image: image33.wmf]
	2.
	Equal to the area of the parallelogram spanned by the two vectors   

	[image: image34.wmf]
	3.
	Equal to twice the area of the parallelogram spanned by the two vectors   

	[image: image35.wmf]
	4.
	Not a valid vector operation   


 
8
Let a = (1,0) and b = (1,1). What is the cosine of the angle between the two vectors? 
	[image: image36.wmf]
	1.
	½√(2)   

	[image: image37.wmf]
	2.
	√(2)   

	[image: image38.wmf]
	3.
	½√(3)   

	[image: image39.wmf]
	4.
	2√(2)   

	[image: image40.wmf]
	5.
	¾√(3)   

	[image: image41.wmf]
	6.
	¼√(3)   

	[image: image42.wmf]
	7.
	None of the above.   


 
9
What is the area of the parallelogram spanned by the two vectors u and v, where 
u = (1,2,3) and v = (1,1,1) 
	[image: image43.wmf]
	1.
	√(2)   

	[image: image44.wmf]
	2.
	√(3)   

	[image: image45.wmf]
	3.
	2√(2)   

	[image: image46.wmf]
	4.
	2√(6)   

	[image: image47.wmf]
	5.
	2√(3)   

	[image: image48.wmf]
	6.
	4√(2)   

	[image: image49.wmf]
	7.
	None of the above   


 
10
Which of the following Maple commands must be used to force the axes of a plot to have the same scale? 
	[image: image50.wmf]
	1.
	scaling=scaled   

	[image: image51.wmf]
	2.
	scaling=constrained   

	[image: image52.wmf]
	3.
	scaling=unconstrained   

	[image: image53.wmf]
	4.
	scaling=common   

	[image: image54.wmf]
	5.
	None of the above   


 
11
Choose the appropriate Matlab command for assigning 5 to the variable ECOR1000 in the list below. 
	[image: image55.wmf]
	1.
	ECOR1000 := 5   

	[image: image56.wmf]
	2.
	ECOR1000 = 5   

	[image: image57.wmf]
	3.
	ecor1000 := 5   

	[image: image58.wmf]
	4.
	ecor1000 = 5   

	[image: image59.wmf]
	5.
	Both answers 1 and 3   

	[image: image60.wmf]
	6.
	Both answers 2 and 4   

	[image: image61.wmf]
	7.
	None of the above   


 
12
What does MATLAB stand for? 
	[image: image62.wmf]
	1.
	Mathematics Laboratory   

	[image: image63.wmf]
	2.
	Matrix Laboratory.   

	[image: image64.wmf]
	3.
	Matrix Linear Algebra Basic.   

	[image: image65.wmf]
	4.
	Matrix Algebra Theory Laboratory.   


 
13
Let A and B be matrices defined in MATLAB such that 
A = [1,2;3,4;5,6;7,8] and 
B = [0,1;0,1;1,0;0,1]
Given the matrix product P = AB, what is element P(1,2)? 
	[image: image66.wmf]
	1.
	3   

	[image: image67.wmf]
	2.
	6   

	[image: image68.wmf]
	3.
	14   

	[image: image69.wmf]
	4.
	10   

	[image: image70.wmf]
	5.
	9   

	[image: image71.wmf]
	6.
	2   

	[image: image72.wmf]
	7.
	None of the above   


 
14
Let A and B be matrices defined in MATLAB such that 
A = [1,2,3,4;5,6,7,8] and 
B = [0,1;0,0;1,0;1,0]
Given the matrix product P = AB, what is element P(2,1)? 
	[image: image73.wmf]
	1.
	15   

	[image: image74.wmf]
	2.
	7   

	[image: image75.wmf]
	3.
	5   

	[image: image76.wmf]
	4.
	4   

	[image: image77.wmf]
	5.
	11   

	[image: image78.wmf]
	6.
	17   

	[image: image79.wmf]
	7.
	None of the above   


 
15
What is the general defence for an engineer charged under the Occupational Health and Safety Act of Ontario for an alleged safety violation? 
	[image: image80.wmf]
	1.
	A financial settlement   

	[image: image81.wmf]
	2.
	Confirmation that the engineer is a registered member of the profession   

	[image: image82.wmf]
	3.
	Due diligence   

	[image: image83.wmf]
	4.
	Demonstration that the engineer was under a great deal of employer pressure during the time of the violation   

	[image: image84.wmf]
	5.
	None of the above   


 
16
Professions are governed by: 
	[image: image85.wmf]
	1.
	An act of the Federal Government   

	[image: image86.wmf]
	2.
	An act of the Provincial Government   

	[image: image87.wmf]
	3.
	A code of ethics   

	[image: image88.wmf]
	4.
	Provincially appointed authorities   

	[image: image89.wmf]
	5.
	None of the above   


 
17
Consider a three-view drawing done in first-angle projection. Where does the top view appear relative to the front view? 
	[image: image90.wmf]
	1.
	To the left.   

	[image: image91.wmf]
	2.
	To the right.   

	[image: image92.wmf]
	3.
	Below.   

	[image: image93.wmf]
	4.
	On top.   

	[image: image94.wmf]
	5.
	In front.   

	[image: image95.wmf]
	6.
	Behind.   

	[image: image96.wmf]
	7.
	None of the above.   


 
18
Which of the following statements best describes a two-point perspective projection? 
	[image: image97.wmf]
	1.
	The projection plane is parallel to two principal axes of the reference coordinate system.   

	[image: image98.wmf]
	2.
	The projection plane is perpendicular to two principal axes of the reference coordinate system.   

	[image: image99.wmf]
	3.
	The projection plane is defined by three vanishing points.   

	[image: image100.wmf]
	4.
	The projection plane intersects exactly two principal axes of the reference coordinate system.   

	[image: image101.wmf]
	5.
	The projection is not parallel to any principal axis of the reference coordinate system.   

	[image: image102.wmf]
	6.
	All but one of the projectors are orthogonal to the projection plane.   

	[image: image103.wmf]
	7.
	None of the above.   


 
19
An oblique projection presents the true shape of: 
	[image: image104.wmf]
	1.
	One face of a projected object   

	[image: image105.wmf]
	2.
	Two faces of a projected object   

	[image: image106.wmf]
	3.
	All visible faces of a projected object   

	[image: image107.wmf]
	4.
	No visible faces of a projected object are in true vie


 
20
What is a detail drawing? 
	[image: image108.wmf]
	1.
	A drawing which includes a list of materials.   

	[image: image109.wmf]
	2.
	A drawing where all parts of an assembly are identified   

	[image: image110.wmf]
	3.
	A working drawing of a single part   

	[image: image111.wmf]
	4.
	A drawing detailing how parts fit together   


_1443018751.unknown

_1443018767.unknown

_1443018775.unknown

_1443018779.unknown

_1443018781.unknown

_1443018782.unknown

_1443018780.unknown

_1443018777.unknown

_1443018778.unknown

_1443018776.unknown

_1443018771.unknown

_1443018773.unknown

_1443018774.unknown

_1443018772.unknown

_1443018769.unknown

_1443018770.unknown

_1443018768.unknown

_1443018759.unknown

_1443018763.unknown

_1443018765.unknown

_1443018766.unknown

_1443018764.unknown

_1443018761.unknown

_1443018762.unknown

_1443018760.unknown

_1443018755.unknown

_1443018757.unknown

_1443018758.unknown

_1443018756.unknown

_1443018753.unknown

_1443018754.unknown

_1443018752.unknown

_1443018719.unknown

_1443018735.unknown

_1443018743.unknown

_1443018747.unknown

_1443018749.unknown

_1443018750.unknown

_1443018748.unknown

_1443018745.unknown

_1443018746.unknown

_1443018744.unknown

_1443018739.unknown

_1443018741.unknown

_1443018742.unknown

_1443018740.unknown

_1443018737.unknown

_1443018738.unknown

_1443018736.unknown

_1443018727.unknown

_1443018731.unknown

_1443018733.unknown

_1443018734.unknown

_1443018732.unknown

_1443018729.unknown

_1443018730.unknown

_1443018728.unknown

_1443018723.unknown

_1443018725.unknown

_1443018726.unknown

_1443018724.unknown

_1443018721.unknown

_1443018722.unknown

_1443018720.unknown

_1443018703.unknown

_1443018711.unknown

_1443018715.unknown

_1443018717.unknown

_1443018718.unknown

_1443018716.unknown

_1443018713.unknown

_1443018714.unknown

_1443018712.unknown

_1443018707.unknown

_1443018709.unknown

_1443018710.unknown

_1443018708.unknown

_1443018705.unknown

_1443018706.unknown

_1443018704.unknown

_1443018695.unknown

_1443018699.unknown

_1443018701.unknown

_1443018702.unknown

_1443018700.unknown

_1443018697.unknown

_1443018698.unknown

_1443018696.unknown

_1443018687.unknown

_1443018691.unknown

_1443018693.unknown

_1443018694.unknown

_1443018692.unknown

_1443018689.unknown

_1443018690.unknown

_1443018688.unknown

_1443018683.unknown

_1443018685.unknown

_1443018686.unknown

_1443018684.unknown

_1443018679.unknown

_1443018681.unknown

_1443018682.unknown

_1443018680.unknown

_1443018677.unknown

_1443018678.unknown

_1443018675.unknown

_1443018676.unknown

_1443018674.unknown

_1443018673.unknown

