Chem 235 Practice questions

- What is the difference between distillation and refluxing?

- Distillation is the process involving heating a substrate until it vaporizes, which is then condensed by cooling and then collecting the condensate and putting it into a receiver

- Refluxing uses the same processes as distillation but the condensate is collected in the same container

- What are the purposes of performing a distillation?

- Purification of liquid

- Useful for separating a mixture when the components of the mixture have different boiling points

- What conditions must be met in order for distillation to be successful?

- At least one of the compounds in the mixture must be volatile

- If 2 or more volatile liquids are to be separated, then there must be at least a 50 degree difference in their boiling point

- The components of the mixture must not form an azeotrope

- What is the term used for a mixture that distils at a constant temperature and a constant composition even though the components having differing BP?

- Azeotrope

- What cause the above situation to occur?

- When there is intermolecular forces between the components such as hydrogen bonding ex) 95 percent ethanol and 5 percent water form an azeotrope

- In Chem 235, what are the criteria’s used to determine the purity of a liquid?

- The boiling point of a substance

- The refractive index

- What is the purpose of a liquid-liquid extraction?

- To separate mixtures using their differences in solubility of the components in the 2 immiscible solvents

- Name 3 characteristics required for the solvent used when separating solids?

- Both solvents must be immiscible with each other

- One solvent must dissolve both substances well

- The other solvent must only dissolve one of the compounds well

- Why are successive extractions performed during the liquid-liquid extractions?

- The second solvent has the ability to still dissolve the first, so successive separation is done to ensure the best possible separation of the mixture

- What is the purpose of recrystallization?

- To purify a crude compound isolated from chemical synthesis

- Name 4 requirements that must be met in order for recrystallization to occur?

- The solvent must dissolve the compound sparingly at room temp. but must have the ability to dissolve the compound at boiling point

- The solvent must have a lower boiling point than the melting point of the compound being dissolved

- The compound must not react chemically with the solvent

- Should dissolve impurities well or not at all

- When one solvent does not meet the requirements stated in the above question, a ‘solvent pair’ must be used. Name 2 characteristics the solvent pair must have in order for recrystallization to be possible.

- One solvent must dissolve the compound well at all temperature

- The other solvent should not dissolve the compound at all temperature

- What is the proper amount of solvent that should be used in a recrystallization?

- Small amount of solvents should be added till the solid just dissolves

- What is the advantage to using a condenser in recrystallization?

- The condenser prevents the solution from drying out and also allows for refluxing, therefore, minimizing the amount of solvent needed

- What effect does impurities have on the melting point of a compound?

- Decreases the melting point

- Widens the melting range of the compound

- What is the purpose of Thin Layer Chromatography?

- Used to determine the number of compounds in a mixture

- Can also establish if 2 compounds are identical using Rf values

- Can check the effectiveness of separation by column, recrystallization or extraction

- How can TLC determine if 2 compounds are Identical?

- Using Rf values

- Why is a half-moon shaped liner used in TLC?

- The liner helps saturate the chamber with vapours of the developing solvent, therefore, reducing it’s evaporation from the TLC plates as well as maintaining the rate of the irrigation process

- What is to be done to the developing solvent if a compound is too polar?

- A small amount of acetic acid is added

- Define Capillary action.

- Capillary action is the ability of water to flow up a narrow tube, without the assistance of, or opposition of external forces such as gravity. It uses adhesion, cohesion and intermolecular forces.

- Why are flask’s warmed before hot filtrations?

- So the solution does not precipitate prematurely in the funnel or in the receiver

- What is the use of celite in gravity hot filtration?

- To prevent norit from going through the filter paper

True/False?

- Boiling point of pure substances remains constant. True

- In a simple distillation, the observed boiling points are always between the boiling points of the pure components, rising slowly as the distillation proceeds. True

- A distinct separation of the components can happen with simple distillation. False. However, with fractional distillation, an almost distinct separation of the components in a mixture can be achieved.

- NaCl can be used to break up emulsions if the solvent used was water. True.

- Thin layer chromatography can be used with volatile compounds. False