Lab test, Elec 364 2015, prof. M. Amer

Create a summation of a^n with a user defined input for a and n = 0:10. Each term in the summation should be stored in an array. Each term of the array should be output to the console as it’s created.   Create a second array that stores the values of the difference between successive terms in your first array, we’ll call this the error between terms.  Term 1 of this array should equal absolute value of  firstTerm – secondTerm from the original array, term 2 equals absolute value of secondTerm-thirdTerm and so on.  In order to find the 11th term of this second array you will need to define a 12th term of the first array.  For your user input use a = 0.5.

Plot both arrays in the same figure using subplot(),  they will ask you to label all plots exactly as shown in the figure provided.  I have included the correct labels in my code.

Starting on page  2 is the MATLAB code you will use, it has been tested in MATLAB and will give correct output.

n = [0:10];

a = input('enter a value for a: ')

x = a.^n;

sum = 0; % this is for your summation

disp('each term in the summation: ')

for index = 1:11

sum = sum + x(index) % a semicolon here would not show each value of the

% summation as it's being created, as asked

end

subplot(2,1,1)

stem(n,x)

%they will specify the exact labels they want, see below

xlabel('n')

ylabel('Value')

title('Terms of the Series')

error =[]; % our second array holding there error between terms of the first

for index = 1:10

error(index) = abs( x(index) - x(index+1) ); % don't have to output every term here

end

% now we have to define the 12th term of the first array to get the 11th

% term of the error [], error(11) = abs( x(12) - x(11) )

term12 = a^12;

error(11) = abs( x(11) - term12 );

% plot the second array in same figure as asked

subplot( 2,1,2)

stem( n, error )

xlabel('n')

ylabel('Error')

title('Error between each term')

Using a value of .5 for a when asked by the program our output to the console should look like this:

enter a value for a: .5

a =

0.5000

each term in the summation:

sum =

1

sum =

1.5000

sum =

1.7500

sum =

1.8750

sum =

1.9375

sum =

1.9688

sum =

1.9844

sum =

1.9922

sum =

1.9961

sum =

1.9980

sum =

1.9990

>>