CONCORDIA UNIVERSITY

Faculty of Engineering and Computer Science

ENGR 202 SUSTAINABLE DEVELOPMENT AND ENVIRONMENTAL STEWARDSHIP

(Fall 2015)

Assignment No. 1

Yuen Shan, Chan

27749104

October 15th 2015

- Question 1.5
The basis of green design, industrial ecology, and sustainable development is in a way a new approach, system or sets of rules by which engineers want to abide to in order increase their responsibility on the environmental impacts that result from their design. Engineers have always been obsessed with innovation and making life easier and more efficient for the population, however in the past they will not take into consideration the effects that will be produced from their design. As environmental problems arise, engineers become more aware of the importance to preserve, or at least slowdown the uptake, of the energy resource and the safe environment that is the Earth. An example stated in the book is the obligation of manufacturers in Germany to collect back their discarded computers and properly dispose of. This major setback for the manufacturers has lead to a new way of designing their product in the first place. There are different ways in which environmental impacts can be changed and these include material selection, manufacturing processes and energy use. Engineers are now trying to find alternatives in each respect to reduce the negative impacts. An example in material selection is the global change of the water bottle. In recent years we have seen the design of plastic water bottles become more and more space efficient while maintaining its capacity and reducing the percent of plastic used per bottle. It is suggested that Aquafina is now using 50% less plastic for their half-liter water bottles.

- Using the Canadian National Pollutant Release Inventory, choose any 3 chemicals and determine their quantities released to the air, water and land.

| Chemical name | Released to Air | Released to Water | Released to Land |
| --- | --- | --- | --- |
| Carbon Monoxide release in 2014 | 875 343 tonnes | 0 | 0 |
| Lead (and components) release in 2014 | 102 664 kg | 146 669 kg | 102 888 kg |
| Sulphuric Acid release in 2014 | 4229 tonnes | 34 tones | 0,621 tonne |

- Question 2.11
According to PetroStrategie.Inc the world crude oil supply remaining is at 1687.9 billions barrels. According to their calculations on the annual worldwide consumption and change in oil reserve quantity, there are around 53.3 years left until the depletion of oil reserve. However according to an article from EIA, there is no possibilities this could happen. It is because there is a limit to which oils can be extracted. In fact there are oil reserves that cannot be reached. The use of oil may have to stop, but not because running out of it. It is most likely be the prices of oil increasing so high due to difficulty of extraction, as suggested in the EIA report. Environmental effect that results from further extraction of hard to reach oils is the increased use of other energy resource to do the extraction of oil. Additionally, reaching the oil may require disruption of the land that sits above the oil source. The natural habitat of biodiversity will be disturbed; people living in the vicinity will be greatly affected as well.

- Question 12.2
Using the equilibrium condition that the solar energy should be equal in both of the equations, we can write the following equality:
So×Surface area of Earth Sphere=So*×Perpendicular area or Earth(disk)
So×4πR2=So*×πR2
So=So*×πR24πR2
So=So*4

- Question 12.15

| Greenhouse Gas | Emissions (Mt/yr) | 20-Year GWP | Equiv. CO2 (Mt/yr) |
| --- | --- | --- | --- |
| CO2 | 29800 | 1 | 29800 |
| CH4 | 375 | 56 | 21000 |
| N2O | 5.7 | 280 | 1596 |
| CFC-11 | 0.23 | 5000 | 1150 |
| CFC-12 | 0.23 | 7900 | 1817 |
| CFC-113 | 0.23 | 5000 | 1150 |
| HCFC-22 | 0.2 | 4300 | 860 |
| Total |  |  | 57373 |

Percent actualCO2 contribution=Actual CO2 emmisonsTotal equivalent CO2  ×100
Percent actual CO2 contribution=2980057373 ×100
Percent actual CO2 contribution=51.94%

The next most important greenhouse gas based on its effect is Methane CH4. Although its yearly emissions are much lower than carbon dioxide, its effect is much higher.

The results from the 20-year calculations revealed a much higher equivalent CO2 rate (57373 vs 43860). For a shorter period of time, the CO2 equivalent value for methane seems to be much higher in the 20-year than in the 100-year study.

# Bibliography

Are We Running Out of Oil and Gas? (n.d.). Retrieved from PetroStrategies.Inc : http://www.petrostrategies.org/Learning_Center/are_we_running_out_of_oil_and_gas.htm

Long-Term World Oil Supply Scenarios. (n.d.). Retrieved from EIA: http://large.stanford.edu/publications/coal/references/wood/

New Aquafina Bottles Reduce Plastic Use by 50 Percent. (n.d.). Retrieved from Earth911: http://www.earth911.com/food/new-aquafina-bottles-reduce-plastic-use-by-50-percent/