Midterm2: CST8103 Networking Basics

Fall 2014

Time: 45 minutes;  42 questions, total Marks available:  41 marks

Version D

Instructions:

- BEFORE answering any questions, please check that your copy of the test has all pages (as indicated in the footer at the bottom of each page).  Please read all questions carefully, then answer question 1 first!

- Be sure to mark your name, student id and version of this midterm on the scantron answer sheet.

- All answers should be circled on this test paper and then marked on the scantron answer sheet.

- If you do not find an answer which is clearly the correct choice, choose the best answer.

1. What is your:

NAME?  					 	Student Id?

(Continued on next page)

1. How many broadcast domains in the following network topology:

| a. | 7 | c. | 5 |
| --- | --- | --- | --- |
| b. | 4 | d. | 6 |

2.	Which 802.11 security protocol offers the best protection?

| a. | All of the above | c. | WEP |
| --- | --- | --- | --- |
| b. | WPA2-PSK with AES | d. | WPA-PSK with TKIP |

3.	Decompress the following IPv6 address: 1::

| a. | 0000:0000:0000:0000:0000:0000:0000:0001 |
| --- | --- |
| b. | 0001:0000:0000:0000:0000:0000:0000:0000:0000 |
| c. | 0001:0:0:0:0:0:0:0 |

4.	Which of the following is NOT a valid subnet mask?

| a. | 248.0.0.0 | d. | 255.250.0.0 |
| --- | --- | --- | --- |
| b. | 255.255.254.0 | e. | 255.255.255.255 |
| c. | 224.0.0.0 |

5.	Can a router have a default route?

| a. | Yes, but only if it is not directly connected to a switch | c. | Yes |
| --- | --- | --- | --- |
| b. | Yes, but only if it is directly connected to a switch | d. | No |

6.	What Windows command gives your adapter’s MAC address?

| a. | tracert | c. | ifconfig |
| --- | --- | --- | --- |
| b. | ipconfig | d. | ipconfig/all |

7.	Are hosts 172.10.10.124/25 and 172.10.10.130/25 part of the same network segment?

| a. | Yes | b. | No |
| --- | --- | --- | --- |

8.	Which statement is not true about ping?

| a. | The two Ping messages are Echo request and Echo reply | c. | Ping uses the ICMP protocol |
| --- | --- | --- | --- |
| b. | Only works with IPv4 hosts | d. | Ping is often blocked by a host’s firewall |

9.	Analyse the network topology below to determine which entry is missing from Router2’s routing table.

Router2 Routing Table

Gateway of last resort is not set

C 172.16.1.160/27 is directly connected, FastEthernet0/0

C 172.16.1.224/27 is directly connected, FastEthernet0/1

C 172.16.1.192/27 is directly connected, FastEthernet0/2

S 172.16.1.0/27 [1/0] via 172.16.1.162

S 172.16.1.96/27 [1/0] via 172.16.1.162

| a. | 0.0.0.0/0 via 172.16.1.162 | c. | S 172.16.1.64/27 [1/0] via 172.16.1.162 |
| --- | --- | --- | --- |
| b. | S 172.16.1.32/27 [1/0] via 172.16.1.162 | d. | S 172.16.1.128/27 [1/0] via 172.16.1.162 |

10.	Which of the following prefix is equivalent to subnet mask 255.255.192.0?

| a. | /17 | d. | /26 |
| --- | --- | --- | --- |
| b. | /10 | e. | /19 |
| c. | /18 |

11.	What is the correct statement about this address: 2001:4860:4860::8888/64

| a. | This is a loopback address | c. | This is a link-local address |
| --- | --- | --- | --- |
| b. | The network id is: 2001:4860:4860:0000 The interface id is: 0000:0000:0000:8888 | d. | The network id is: 2001:4860:4860 The interface id is: ::8888 |

12.	What identifies the wireless network and is contained in the beacons sent out by the Access Points?

| a. | IBSS | c. | SID |
| --- | --- | --- | --- |
| b. | BSS | d. | SSID |

13.	Examine the exhibit and select the incorrect statement?

| a. | The IPv6 is a link-local address | d. | The IPv6 address enables local communication only. |
| --- | --- | --- | --- |
| b. | The Ethernet adapter is dual-stacked | e. | The network segment on which this host is connected offers SLAAC and DHCPv6 services |
| c. | The ping ::1 should succeed! |

14.	Which of the following is not an Internet IPv4 routable address?

| a. | 15.33.44.67 | d. | They are all public addresses |
| --- | --- | --- | --- |
| b. | 192.168.1.1 | e. | 128.122.99.22 |
| c. | 196.168.1.2 |

15.	When does a host use a default route?

| a. | It completely depends on the protocol being used. | c. | Never |
| --- | --- | --- | --- |
| b. | Always (for every network interaction) | d. | For packets of unknown networks |

16.	Which of the following is NOT a valid method for obtaining an IPv6 address?

| a. | DHCPv6 | d. | SLAAC (RA) |
| --- | --- | --- | --- |
| b. | NDP | e. | SLAAC (RA) and DHCPv6 |
| c. | Static (manual) |

17.	What is not true about an IPv6 address?

| a. | 8 hextets | c. | 32 hexadecimal digits |
| --- | --- | --- | --- |
| b. | 32 octets | d. | 128 bits |

18.	Compress the following IPv6 address: 0000:0000:0000:0000:0001:0000:0000:0000

| a. | 0:0:0:0:1: |
| --- | --- |
| b. | ::1:: |
| c. | ::1:0:0:0 |

19.	Which network diagnostic tool is used to display the route (path) and measuring transit delays of packets across an Internet Protocol (IP) network.

| a. | ping | c. | icmp |
| --- | --- | --- | --- |
| b. | route -r | d. | tracert |

20.	What is NOT a characteristic of hosts on the same network segment?

| a. | They share the same network id | c. | They share the same broadcast address |
| --- | --- | --- | --- |
| b. | They share the same subnet mask | d. | Two hosts may have the same host ids |

21.	Which of the following is not a Wi-Fi issue?

| a. | Interference | c. | Mobility |
| --- | --- | --- | --- |
| b. | Range | d. | Security |

22.	What protocol helps discover the MAC for an IPv6 address?

| a. | DHCP | c. | ARP |
| --- | --- | --- | --- |
| b. | NDP | d. | DNS |

23.	Which media access control method is used with 802.11?

| a. | CSMA/CD | c. | None |
| --- | --- | --- | --- |
| b. | Shared Media Access | d. | CSMA/CA |

24.	Which of the following is not a valid IPv4 communication type?

| a. | Multicast | c. | Broadcast |
| --- | --- | --- | --- |
| b. | Unicast | d. | Anycast |

25.	What protocol helps discover the MAC for an IPv4 address?

| a. | ARP | c. | DHCP |
| --- | --- | --- | --- |
| b. | NDP | d. | DNS |

26.	What is not a characteristic of the IPv4 protocol?

| a. | Low overhead | c. | Reliable |
| --- | --- | --- | --- |
| b. | Connectionless | d. | Media independent |

27.	Which of the following is not a valid IPv6 Unicast address type?

| a. | Unique-Local | d. | Multicast |
| --- | --- | --- | --- |
| b. | Global | e. | Link-Local |
| c. | unspecified |

28.	For IP 177.15.126.66/26, calculate: the network and broadcast address; first and last host IP address; number of hosts that can exist in the subnet.

| a. | 177.15.126.0, 177.15.126.255, 177.15.126.1, 177.15.126.254, 254 | c. | 177.15.126.64, 177.15.126.127, 177.15.126.65, 177.15.126.126, 62 |
| --- | --- | --- | --- |
| b. | 177.15.126.66, 177.15.126.128, 177.15.126.65, 177.15.126.126, 62 | d. | 177.15.126.64, 177.15.126.127, 177.15.126.65, 177.15.126.126, 64 |

29.	Analyse the network topology below to determine which entry is missing from PC2’s host routing table.  Assume PC2’s IPv4 address is 172.16.1.130/27 and the router’s address is 172.16.1.129/27.   Hint:  PC2 must be able to communicate with remote and local hosts!

PC2’s Routing Table

| a. | 172.16.1.128 255.255.255.224 On-Link 172.16.1.130 |
| --- | --- |
| b. | 0.0.0.0 0.0.0.0 172.16.1.129 172.16.1.130 172.16.1.128 255.255.255.0 On-Link 172.16.1.130 |
| c. | 0.0.0.0 0.0.0.0 172.16.1.129 172.16.1.130 172.16.1.128 255.255.255.224 On-Link 172.16.1.130 |
| d. | 0.0.0.0 0.0.0.0 172.16.1.130 172.16.1.130 172.16.1.129 255.255.255.224 On-Link 172.16.1.130 |

30.	A wireless access point shares the same features as which of the following devices:

| a. | switch (dedicated bandwidth) | c. | hub (shared bandwidth) |
| --- | --- | --- | --- |
| b. | router |

31.	Examine the exhibit to determine the lease on the IPv6 address?

| a. | 1 second | d. | Impossible to tell |
| --- | --- | --- | --- |
| b. | It’s a static address, therefore there is no lease!!! | e. | 1 year |
| c. | 1 day |

32.	How many usable host addresses exist in subnet 10.11.12.32/27?

| a. | 64 | d. | 30 |
| --- | --- | --- | --- |
| b. | 16 | e. | 62 |
| c. | 32 |

33.	PC0 and Server0 have been assigned the last and second last addresses in their subnet.  What are their IP addresses and the dotted-decimal subnet mask.

| a. | 172.16.1.255 and 172.16.1.254 Subnet Mask: 255.255.255.224 | c. | 172.16.1.255 and 172.16.1.254 Subnet Mask: /27 |
| --- | --- | --- | --- |
| b. | 172.16.1.254 and 172.16.1.253 Subnet Mask: /27 | d. | 172.16.1.254 and 172.16.1.253 Subnet Mask: 255.255.255.224 |

34.	What are the three non-overlapping (22 Mhz) channels for the 2.4Ghz band?

| a. | 1, 6 and 11 | c. | 1, 6, and 14 |
| --- | --- | --- | --- |
| b. | 0, 7 and 11 | d. | none of the answers |

35.	What are the features for 802.11n?

| a. | Maximum speed 7Gb/s Frequency 2.4 or 5 Ghz Backward Compatible 802.a/b/g | c. | Maximum speed 600Mb/s Frequency 2.4 or 5 Ghz Backward Compatible 802.a/b/g |
| --- | --- | --- | --- |
| b. | Maximum speed 600Mb/s Frequency 2.4 or 5 Ghz Backward Compatible No | d. | Maximum speed 600Mb/s Frequency 2.4 Backward Compatible 802.a/b/g |

36.	What is the first and last usable address for 172.138.1.44/30?

| a. | 172.138.1.45 and 172.138.1.49 | d. | 172.138.1.44 and 172.138.1.46 |
| --- | --- | --- | --- |
| b. | 172.138.1.44 and 172.138.1.47 | e. | 172.138.1.44 and 172.138.1.46 |
| c. | 172.138.1.45 and 172.138.1.46 |

37.	Which of the following is not a valid IPv6 address type?

| a. | Unicast | c. | Multicast |
| --- | --- | --- | --- |
| b. | Broadcast | d. | Anycast |

38.	Examine the exhibit to determine the lease on the IPv4 address?

| a. | 1 second | d. | It’s a static address, therefore there is no lease!!! |
| --- | --- | --- | --- |
| b. | 1 day | e. | Impossible to tell |
| c. | 1 year |

39.	What entry will be added to PC1’s ARP cache following a ping to PC2.  Assume PC1 and PC2 are in the subnet?

| a. | PC2’s MAC address | c. | PC1 and PC2’s MAC addresses |
| --- | --- | --- | --- |
| b. | PC1’s MAC address | d. | none of the answers |

40.	You have been tasked with subnetting IP address 200.10.23.0/24 into 4 networks.  How many host bits are left after the subnetting process?

| a. | 3 | c. | 4 |
| --- | --- | --- | --- |
| b. | 5 | d. | 6 |

41.	What dotted-decimal subnet mask would be used for hosts in the 128.176.0.0/15 network?

| a. | 255.238.0.0 | d. | 255.252.0.0 |
| --- | --- | --- | --- |
| b. | 255.255.254.0 | e. | 255.254.0.0 |
| c. | 255.0.0.0 |

42.	Which version of the midterm are you writing (check the cover page)?

| a. | A | c. | C |
| --- | --- | --- | --- |
| b. | B | d. | D |

Use this page for rough work     Your Name: ___