Thermodynamics I                               FINAL                      
                                ENGR 251/2 Fall 07


CONCORDIA UNIVERSITY

Faculty of Engineering and Computer Science

Department of Mechanical Engineering
	Course

Thermodynamics I
	Number

ENGR 251
	Section

All Sections

	Examination

Final
	Date

17- December-2007
	Time

9:00 – 12:00
	Number of Pages

	Professor
L. Kadem 

	Materials Allowed:  NO
	

	Special Instructions

Answer all questions.

Return all material with the answer books.

Only non-programmable calculators are allowed.


Student’s Name: 

I.D.:

PROBLEM I [20 pts]
A steam power plant is proposed to operate between the pressures of 10 kPa and 2 MPa with a maximum temperature of 400°C.

What is the maximum efficiency possible from the power cycle?

PROBLEM II [20 pts]
In an air-standard Brayton cycle the air enters the compressor at 0.1 MPa and 15°C. The pressure leaving the compressor is 1.0 MPa, and the maximal temperature in the cycle is 1100°C. 
- Sketch the T-s diagram.

- Determine the pressure and temperature at each point in the cycle.

- Determine the compressor work, turbine work, and cycle efficiency.

PROBLEM III [20 pts]
The compression ratio in an air-standard Otto cycle is 10. At the beginning of the compression stroke the pressure is 0.1 MPa and the temperature is 15°C. The heat transfer to air per cycle is 1800 kJ/kg. 

- Sketch the P-v Diagram.

- Determine the thermal efficiency.

- Determine the mean effective pressure.

PROBLEM IV [20 pts]

A tank of 2 m3 volume contains saturated ammonia at a temperature of 40°C. Initially the tank contains 50% liquid and 50% vapor by volume. Vapor is withdrawn from the top of the tank until the temperature is 10°C. Assuming that only vapor leaves and that the process is adiabatic:
- Determine the initial mass of liquid in the tank.

- Determine the initial mass of vapor in the tank.

- Calculate the mass of ammonia that is withdrawn.
NOTE: The enthalpy of vapor leaving the tank is considered as constant and equal to: 1461.1 kJ/kg.

PROBLEM V [20 pts]

- Explain physically (only using words):

· The zeroth law of thermodynamics

· The first law of thermodynamics

· The second law of thermodynamics

- The efficiency of a modified Brayton cycle is given by:


[image: image1.wmf]k

k

p

r

T

T

1

3

1

1

-

-

=

h


Where T1 is the lowest temperature; T3 is the highest temperature and rp is the pressure ratio. Under which conditions the efficiency of this cycle is equal to the efficiency of a Carnot cycle? Demonstrate why? 
- For a polytropic compression in a piston-cylinder device to be possible, what are the limitations on the polytropic exponent (n)? 
Formulae and Constants
For air: k= 1.4; Cp = 1.004 kJ/kg K; Cv = 0.717 kJ/kg K

[image: image2.wmf]1

21

12

1

22

11

21

12

k

scte

k

k

scte

k

scte

p

v

Tv

Tv

TP

TP

Pv

Pv

C

k

C

-

=

-

=

=

æöæö

=

ç÷ç÷

èøèø

æöæö

=

ç÷ç÷

èøèø

æöæö

=

ç÷ç÷

èøèø

=


PAGE  
1/2

_1259311871.unknown

_1219037809.unknown

