CVG2132A

Matea Ljubenkovic

7746736

Assignment #1

2015-10-02

Question 1:

(a) What was the main finding of “Silent Spring”?

The main finding of Silent Spring is the negative impact humans have posed on the natural environment.  More so, the environmental book focuses on the damages caused by the arbitrary use of the pesticide DDT. It encourages the United States to reconsider its pesticide policies (Griswold 2012).

(b) What happened during the Donora episode?

The Donora episode became a historic air inversion that resulted in a wall of smog killing about 20 people and made over 7,000 people ill in 1948. This occurred in Donora, Pennsylvania, which was a mill town on the Monongahela River.  It is known to be one of the worst air pollution disasters (Hamill 2008).

(c) What policy changes were triggered by the Cuyahoga River fire?

The Cuyahoga River has caught fire at least 13 times but the biggest fire was in 1952 and caused over $1 million in damages (Adler 2014). In 1969 the river caught the attention of media and new policies were triggered. The "Clean Water Act", "Great Lakes Water Quality Agreement", "Environmental Protection Agency", and the "Ohio Environmental Protection Agency" were all created. This resulted in improved water quality of this river (Griffith2004).

(d) What was the Love Canal incident?

The Love Canal incident occurred in 1978. The Love Canal was a town located near Niagara Falls in upstate New York. It was home to hundreds of houses and schools. However, this little rural area sat upon over 20 tonnes of toxic waste that was buried underground in the 1940's and 1950's (Beck 1979).

(e) What changes occurred as a result of the Love Canal incident?

By 1978, all of the toxic waste under the Love Canal began to bubble and leak into the backyards and cellars of homes and buildings. It became an unavoidable and unfixable problem. Residents were forced to sell their homes and move elsewhere. In 1980 a foundation was formed called the "Superfund program" which helps pay for cleanups at toxic areas like the Love Canal (Beck 1979).

Question 2:

Other than the greenhouse effect what are other environmental impacts of the combustion of fossil fuels?

Other environmental impacts of the combustion fossil fuels are things such as: air pollution, acid rain, oil spills and climate change. Along with these environmental impacts, fossil fuels have also been the cause for smog (Chmielewski 1999).

Question 3:

What is the most common, used method for disinfection of drinking water in the Canada?  Why is it important to remove the naturally occurring organic matter (NOM) before disinfecting drinking water?

One of the most commonly used methods for disinfecting drinking water in Canada is the use of chlorine as a disinfectant. Chlorine is used to disinfect water as it can kill most microorganisms that are found in water. Since chlorine is a commonly used method for disinfecting water, the NOMs can react with the chlorine and forms of chlorination by-products which are linked to cancerous diseases. It is important to be sure to remove the NOMs before disinfecting drinking water to avoid this problem (Garcia 2011).

Question 4:

In the water samples, what is the aqueous concentration of Mirex in units of:

(a) ppb

1mgL=1000μgL

0.002mgL x 1000μgL=2μgL=2 ppb

(b) ppt

1 ppm=10-6;  ppt=10-12

1ppm=106ppt

ppt=106ppm

ppt=0.002ppmx106

=2000 ppt

(c) µM

0.002mgL x 1 g1000 mg x 1 mol540 g=3.70∙10-9μM

In the fish samples, what is the concentration of Mirex in fish in:

(a) ppm

0.002mg of body massgx1000gkg=2mg of body masskg

2 mgkg=2 ppm

(b)ppb

1 ppm=1000 ppb and ppb=ppm x 1000

ppb=2 x 1000=2000 ppb

Question 5:

A water sample contains 10 mg NO3-L. What is the concentration in a dilute water in units of:

(a) ppm

1mgL=1ppm	so, 10 mg NO3-L = 10 ppm

(b) molesL

10 mg NO3-L; MW=62gmol

10 mgLx1g1000mg=0.01gL; 0.01 gLx162gmol=0.000161molL=1.61∙10-4molL

(c) mg NO3-LN

10 mg NO3-Lx10mg∙N62 mg NO3- =2.26mg of NO3-L

(d) ppb

10mgLx1000μgL=10,000ppb

Question 6:

Determine the equilibrium pH of aqueous solutions of the following strong acids or bases:

(a)  15 mg Lof HSO4-

15 mg of HSO4-  Lx1 g1000 mgx197.07gmol=1.54∙10-4molL

To find the equilibrium pH, use the following equation:

pH=-log⁡[H+]

pH=-log⁡[1.54∙10-4molL]

pH=3.81

(b) 10 mM NaOH

10milli molLx1 mol1000 millimol=0.01 molL

To find the equilibrium pH, use the following equation:

pOH=-log⁡[OH-]

pOH=-log⁡[0.01 molL]

pOH=2

pH=14-2=12 pH level.

(c) 2500 µgL of HNO3

2500 µgL x 1 g0.000001µgx1 63.02gmol=3.97∙10-5molL

Question 7:

(a) A liquid sample has an iron concentration of 5.6 mg/L. The density of the liquid is 2000g/L. What is the iron concentration in ppm?

5.6 mgLxL2000gx1000 gkg=2.8mgkg=2.8 ppm

(b) Near three rural wells nitrate concentrations were reported as 12 mg NO3- NL,  12 mg NO3L , 20 mg NO3NL.  Do any of these three wells exceed the 44.3 ppm level?

(i) 12 mg NO3- N/L

12 mg NLx1 g1000 mgx1 mol14.01 g N=8.57∙10-3molL

8.57∙10-3molLx6 kgmol=0.0531 gL=53mgL=53 ppm

This exceeds the 44.3 ppm level, not safe.

(ii) 12 mg NO3L

12 mg NO3L=12 ppm

This does not exceed the 44.3 ppm, safe.

(iii) 20 mg NO3N/L

20 mgLxmol14.01 gxg1000 mgx62g NO3=mol=8.85∙10-2=88.5mgL=88 ppm

This exceeds the 44.3 ppm limit. Not safe.

Question 8:

(a) What is the hardness of the water in units of mg/L as  CaCO3?

The molar mass of CaCO3, Ca2+ and Mg2+ and Fe2+  and Cd2+ respectively:

MCaCO3=100gmol;MCa=24.3gmol;MMg=40.1gmol;MFe=55.8gmol;MCd=112gmol

The ratios of the molar masses are:

MCaCO3MCa=100gmol 40.1gmol=2.5;MCaCO3MMg=100gmol 24.3gmol=4.1;MCaCO3MFe=100gmol 55.8gmol=1.8;

MCaCO3MCd=100gmol 112gmol=0.892

The total hardness is calculated by the following formula:

TOTAL HARDNESS (TH) = SUM OF ALL POLYVALENT CATIONS

The hardness for the groundwater is as follows:

[CaCO3]= (Molar Mass Ratio)[Mg+2]+(Molar Mass Ratio)[Ca]+(Molar Mass Ratio)[Fe2+]

CaCO3=4.184gm3+2.5190gm3+1.80.1gm3+0.892[0.2gm3]

CaCO3=820mgL

(b) Determine the alkalinity of the water in units of mg/L as CaCO3.

MW of CO32-=60 g	z of CO32-= 2	EW of CO32-= 30

MW of HCO3-=61 g	z of HCO3-= 1	EW of HCO3-= 61

Converting to molL:

CO32-=30gm3xmol CO32-60000 mg CO32-=5.0∙10-4molL

HCO3=260gm3xmol HCO361000 mg HCO3=4.26∙10-3molL

To calculate alkalinity, use the following formula:

AlkalinityeqL=zHCO3HCO3+(zCO32-)[CO32-]

AlkalinityeqL=4.26∙10-3molL+25.0∙10-4molL=5.26∙10-3eqL

To represent in units of mgL as CaCO3:

EW of CaCO3=MW of CaCO3z of CaCO3; EW of CaCO3=100 g CaCO3mol CaCO32eqmol; EW of CaCO3=50g  CaCO3eq

Alkalinity mg as CaCO3L=5.26∙10-3eqLx 50000 mg  CaCO3eq

Alkalinity mg as CaCO3L=263  mg as CaCO3L

(c) Estimate the sample pH.

pH=-log[H+]

pH=-log[4.27 x 10-10]= 9.36

(d) Check if the analysis meets the acceptable error limits.

Using the given information, each cation and anion is converted to concentration of M eqL.

[anions in meqL-cations in meqL]

19.6-23.0=-3.4=3.4

| 0.11+0.01 x anions in meqL|=0.11+0.01 x 19.6=0.30

As 3.4 is not smaller or equal to 0.30, the analysis does not meet the error limits.

Work Cited

Adler, Jonathan A. "The Fable of the Burning River, 45 Years Later." Washington Post. The Washington Post, 22 June 2014. Web. 29 Sept. 2015. <https://www.washingtonpost.com/news/volokh-conspiracy/wp/2014/06/22/the-fable-of-the-burning-river-45-years-later/>.

Beck, Eckardt C. "EPA." The Love Canal Tragedy. EPA, Jan. 1979. Web. 29 Sept. 2015. <http://www2.epa.gov/aboutepa/love-canal-tragedy>.

Chmielewski, Andrzej Grzegorz. Evironmental! Effects of Fossil Fuel Combustion. Warszawa: Instytut Chemii I Techniki Jądrowej, 1999. Web. 29 Sept. 2015. <http://www.eolss.net/sample-chapters/c09/e4-23-02.pdf>.

Garcia, Indiana. "Trihalomethanes in Drinking Water: Effect of Natural Organic Matter Distribution." Water SA WSA 39.1 (2013): n. pag. Removal of Natural Organic Matter to Reduce the Presence of Trihalomethanes in Drinking Water. School of Chemical Science and Engineering, 18 Feb. 2011. Web. 29 Sept. 2015. <http://kth.diva-portal.org/smash/get/diva2:397508/FULLTEXT01.pdf>.

Griffith, Susan. "Myths Surrounding Cuyahoga River Fire 35 Years Ago." EurekAlert! CASE WESTERN RESERVE UNIVERSITY, 17 June 2004. Web. 29 Sept. 2015. <http://www.eurekalert.org/pub_releases/2004-06/cwru-msc061704.php>.

Griswold, Eliza. "How ‘Silent Spring’ Ignited the Environmental Movement." The New York Times. The New York Times, 22 Sept. 2012. Web. 29 Sept. 2015. <http://www.nytimes.com/2012/09/23/magazine/how-silent-spring-ignited-the-environmental-movement.html?pagewanted=all&_r=0>

Hamill, Sean D. "Unveiling a Museum, a Pennsylvania Town Remembers the Smog That Killed 20." The New York Times. The New York Times, 01 Nov. 2008. Web. 29 Sept. 2015. <http://www.nytimes.com/2008/11/02/us/02smog.html>.