Questions from Last Lecture

- •Are geologists licensed like doctors? Yes There is a provincial license and only licensed geoscientists can certified geoscience work.

- •What units are m/a? (Meters per year; a=year)

- •Explain the Unit Runoff concept?

- •What is the base level in a graded stream?

Unit Runoff / Discharge (difference between what goes in and what goes out)

What is the average discharge of the Ottawa River at the Carillon Dam (~Montreal)?

– Flow = 1940 m^3 sec^-1

– Drainage Area = 146,300 km 2

What is the annual average UNIT discharge of the Ottawa River?

– Unit Discharge = Flow/Drainage Area

– Unit Discharge = flow m^3 a^-1 / Drainage Area (m2)

– Unit Discharge = 0.42 m a^-1

Think of unit runoff like precipitation (m/a)

Base Flow and Channel Formation

- All rivers eventually drain into the ocean

- The ultimate base level would be sea level

How would you form Grand Canyon using the base flow concept?

- The Colorado plateau gets lifted up by other tectonic activity

- It’s now way above base level

- Erosion than erodes it down to base level

- Need sedimentary basin, need to be uplifted, then you have down cutting that’s occurred over time

- Ground water is small quantity of the total but it’s extremely important

- 9-14 million people rely on groundwater

Groundwater

- The water that lies beneath the ground surface = moves slowly

- Hydrosphere's fresh water:

  - Glaciers (~77%)

  - Groundwater (~22%)

- ~ 15% of total precipitation infiltrates ground and ends up as groundwater

- Groundwater is slow moving, but not static:

  - recharges (rain)

  - discharges (feeds rivers)

Misconception: groundwater is found in underground lakes and rivers (true but quite rare)

- There are spaces in between dry soil particles

- Both pores and fractures can hold water

Porosity

- Fraction of voids or pore spaces compared to the total volume of rock

- The more uniform the grain size (the larger the grain size)=high porosity

- Not uniform (large mixed with small grains)=low porocity

- Determines how much groundwater can be stored.

  - • Φ = f(grain roundness, sorting, and cementation) → texture

Measurement of Porosity

- •Volume of sand column = 1000mL

- •Column contains sand plus porosity

  - –In dry sand the porosity is filled with air

- •Replace all the air with water

- •Porosity = volume water / total volume

Permeability or Conductivity:

- Capacity of a rock or sediment to transmit a fluid. Function of pore SIZES and INTERCONNECTIVITY.

How much water can you get from a material? “Yield” of Water

Sand Experiment

- Porosity =35%

- Water Yield =very low

  - Would have to pump hard to get water oi

Gravel Experiment

- Porosity =  48%

- Water yield =96%

  - Connectivity is high

  - Why did we not get all of it back?

    - Some of water molecules are participating in surface tension

Porosity and permeability

- Which one will produce best water?

  - Shale

    - High concentrations of sulfur and other organic materials

  - Quartz sandstone

  - Granite

  - Fractured crystalline rock

Distribution of groundwater

1. Unsaturated zone

- •voids filled by water and air; infiltration and percolation

- •water is under suction (negative pressure) due to capillary forces

- •water cannot be pumped by wells

2. Water table:

- upper limit of zone of saturation; surface of the water level in the ground

3. Zone of saturation

- •zone where all of the voids in soil, sediment and rock are completely filled with water

- •water is under + pressure and can be extracted by wells

- Water table can fluctuate by few centimeters per year

What factors control infiltration?

1. Precipitation (qty / time)

2. Slope

- If we got flat system, there is lots of time for water to infiltrate

3. Geologic Material

4. Vegetation

- Taken water that can reach the well and evaporate it off

Where is groundwater stored?

- Aquifer: permeable rock strata or sediment that stores and transmits groundwater. (e.g.: sand, gravel)

- Acquitard: semi-permeable rock layer (… retards water)

- Aquiclude: impermeable rock layer (… excludes water)

  - Both are confining layers (barriers to flow ex. shale)

  - Prevents water form going up and prevents it from entering

  - So the water has been there since long time ago and you are drawing water from it

  - Doesn’t get recharged

  - Not sustainable

  - Unconfined = sustainable

Aquifers

• Unconfined aquifer: aquifer uncapped by a confining layer; groundwater is not under pressure; top of aquifer (water table) moves freely up and down.

• Confined aquifer (syn.: artesian aquifer): aquifer sandwiched in between two confining layers (usually inclined to allow recharge); groundwater is under pressure and level of ‘projected’ water table (pressure or potentiometric surface) is above the top of the aquifer.

Well: a hole (usually lined) drilled into the ground to penetrate an aquifer (goal : extract water).

Non-flowing artesian well: when the water table (pressure surface) of a confined aquifer rests BELOW the ground level

Flowing artesian well: when the water table (pressure surface) of a confined aquifer rests ABOVE the ground level

- Artesian well is flowing cause of more pressure

General pattern of movement

Recharge (high) → discharge (low)

Balance between:

- potential energy (high to low elevations)

- pressure (weight of water above)

Velocity in order of cm/day

- Faster near surface

- But still very slow

Henry Darcy (French engineer) – equation to calculate fluid velocity through a porous media (circa 1850).

- Darcy’s Law – states that if permeability remains uniform, the velocity of groundwater will increase as the slope of the water table (hydraulic gradient) increases.

- If we want to calculate velocity in aquifers of sand or gravel these are the things we need to know

- H=10m at one well and 8m at another well

- Distance between wells is 50m

Two basic types of interactions:

Exchange of water between groundwater and streams depends on the positions of the water table (w.t.) with respect to the stream bed

- gaining streams:

  - gain water from the inflow of groundwater through the streambed

- losing streams:

  - lose water to the groundwater system by outflow through the streambed

Sample calculation 2

- Getsitall – ground water flows into it

- Leakalot – ground water leaks out cause it’s higher at elevation

Ground water contamination

Other sources and types of contamination include:

  - Highway road salt

    - Number 1 contaminant in our area

  - Fertilizers

  - Pesticides

  - Chemical and industrial materials

  - Bacteria and viruses

Problems associated with groundwater withdrawal

Sewage contamination: sewage often becomes purified as it passes through tens of meters of an aquifer composed of sand or permeable sandstone.

- In extremely permeable aquifers, such as coarse gravel or fractured bedrock, groundwater may travel long distances without being cleaned.

- To ensure a continuous supply of water, a well must penetrate below the w.t.

- Excessive pumping of wells can cause:

  - • drawdown (lowering) of the w.t.

  - • cone of depression in the w.t.

Other problems

- Treating groundwater as a non-renewable resource –

  - in many places the water available to recharge the aquifer falls significantly short of the amount being withdrawn

  - Hydrologic mass balance is out of Balance!

  - Can lead to subsidense

  - Subsidence: ground sinks when water is pumped from wells faster than natural recharge processes can replace it.

- Saltwater contamination: excessive groundwater withdrawal causes saltwater to be drawn into wells

Features associated with groundwater

- Geysers: intermittent surface emission of hot water

  - Water erupts with great force, often rising 30-60 m into the air

  - Occur when complex network of fractures in hot igneous rock

  - Groundwater heats, expands, changes to steam, and erupts

- Erosional features

  - Caves: large underground cavity

  - Karst topography: feature created when ground or surface water dissolves rock; typically forms in limestone bedrock

- Depositional features

  - Groundwater is often mildly acidic → dissolves rock (particularly limestone – CaCO3)

Summary

- GROUNDWATER is widely distributed and a key source of potable water for many people.

- Groundwater is in the porosity of the rock or soil

- Groundwater movement controlled by gradient and hydraulic conductivity of the porous media (Darcy’s Law).

- Groundwater can dissolve carbonate rocks leading to sink holes and other geologic features.

- Excess usage of groundwater is depleting the resource in many areas or contaminating it with chemical / biological wastes.