Chapter 10 Quality Control

| 1. | Statistical quality control that occurs during the production process is known as acceptance sampling. True False |
| --- | --- |

| 2. | Quality control efforts that occur during production are referred to as statistical process control. True False |
| --- | --- |

| 3. | Statistical quality control is assuring that processes are performing in an acceptable manner. True False |
| --- | --- |

| 4. | Typically the production of high-cost, low-volume items require little quality control inspection. True False |
| --- | --- |

| 5. | Low-cost, high-volume items often require more intensive inspection. True False |
| --- | --- |

| 6. | The amount of inspection can range from no inspection at all to inspecting each item numerous times. True False |
| --- | --- |

| 7. | The optimum level of inspection occurs when the cost of passing defectives is minimized. True False |
| --- | --- |

| 8. | The optimum level of inspection minimizes the sum of inspection costs and the cost of passing defectives. True False |
| --- | --- |

| 9. | The frequency and quantity of inspection relates primarily to the cost of inspection. True False |
| --- | --- |

| 10. | The amount of inspection needed is governed by the costs of inspection and the expected costs of passing defective items. True False |
| --- | --- |

| 11. | Most operations have very few places where it is possible to inspect the product during the production process. True False |
| --- | --- |

| 12. | The HACCP system provides guidelines for determining critical inspection points. True False |
| --- | --- |

| 13. | The primary purpose of statistical process control is to detect defects in finished products before they are shipped to customers. True False |
| --- | --- |

| 14. | Statistical quality control is based on comparing samples from a process to predetermined limits. True False |
| --- | --- |

| 15. | Variation in a sample statistic collected from a process may be either random variation or assignable variation. True False |
| --- | --- |

| 16. | The main task in process control is to distinguish assignable from random variation. True False |
| --- | --- |

| 17. | The variability of a process is equivalent to the distribution of samples of any size taken from the process. True False |
| --- | --- |

| 18. | The distribution of sample means from a process has less variability than the process distribution. True False |
| --- | --- |

| 19. | The sampling distribution can be assumed to be approximately normal even when the underlying process distribution is not normally distributed. True False |
| --- | --- |

| 20. | Approximately 99.7% of sample means will fall within ± two standard deviations of the process mean. True False |
| --- | --- |

| 21. | "Assignable variation" is variation due to a specific cause, such as tool wear. True False |
| --- | --- |

| 22. | Control limits distinguish between non-random and assignable variability. True False |
| --- | --- |

| 23. | Control limits are directly related to design specification limits. True False |
| --- | --- |

| 24. | A sample statistic that falls outside the control limits suggests that the process mean has changed. True False |
| --- | --- |

| 25. | A sample statistic that falls inside the control limits suggests non-random, assignable variation in the process. True False |
| --- | --- |

| 26. | Concluding a process has not changed when it has is known as a Type I error. True False |
| --- | --- |

| 27. | Concluding a process has changed when it has not is known as a Type I error. True False |
| --- | --- |

| 28. | Concluding a process has changed when it has not is known as a Type II error. True False |
| --- | --- |

| 29. | The probability of a Type I error is the probability of concluding that only randomness is present when assignable variation exists. True False |
| --- | --- |

| 30. | If a point on a control chart falls outside one of the control limits, this suggests that the process output is non-random. True False |
| --- | --- |

| 31. | The sample size chosen for control charts depends on the cost of inspection, and the cost of Type I and II errors. True False |
| --- | --- |

| 32. | Range control charts are used to monitor process dispersion. True False |
| --- | --- |

| 33. | Control charts based on attribute data are for process characteristics that are counted rather than measured. True False |
| --- | --- |

| 34. | Attributes are process characteristics that must be measured rather than counted. True False |
| --- | --- |

| 35. | Range charts are used to measure shifts in the process mean. True False |
| --- | --- |

| 36. | Sample mean charts are used to measure shifts in process dispersion. True False |
| --- | --- |

| 37. | The number of defective parts in a sample is a process characteristic that is counted rather than measured. True False |
| --- | --- |

| 38. | A p-chart is used to monitor the fraction of defectives in the output of a process. True False |
| --- | --- |

| 39. | A c-chart is used to monitor the number of defectives in the output of a process. True False |
| --- | --- |

| 40. | A c-chart is used to monitor the number of defects per unit for process output. True False |
| --- | --- |

| 41. | Design specifications are equivalent to control limits. True False |
| --- | --- |

| 42. | Control limits are specifications established by engineering design or customer requirements. True False |
| --- | --- |

| 43. | "Process capability" compares "process variability" to the "design specifications". True False |
| --- | --- |

| 44. | Control limits and process variability are directly related. True False |
| --- | --- |

| 45. | There is no direct link between design specifications and statistical quality control limits. True False |
| --- | --- |

| 46. | The output of a process may not conform to specifications even though the process may be statistically "in control." True False |
| --- | --- |

| 47. | Control limits are statistical limits that reflect the extent to which sample statistics such as means and ranges can vary due to randomness alone. True False |
| --- | --- |

| 48. | Process variability is a key factor in process capability. True False |
| --- | --- |

| 49. | A process is "capable" if the process output falls within the acceptable range of variability allowed by the design specification. True False |
| --- | --- |

| 50. | Tolerances are specified in terms of the natural range of variation of a process. True False |
| --- | --- |

| 51. | The process capability index (indicated by Cp) is calculated as the ratio design specification width to the process variation width. True False |
| --- | --- |

| 52. | The process capability index (indicated by Cpk) is used only when the process is centred. True False |
| --- | --- |

| 53. | When a process is not centred, its capability should be based on considering the upper and lower design specifications separately, not just the overall design specification width. The symbol for this case is Cpk. True False |
| --- | --- |

| 54. | In order for a process to be capable, it must have a capability ratio of at least 1.00. True False |
| --- | --- |

| 55. | A process capability index (Cp) of 0.70 indicates that a process is capable of producing to design specifications. True False |
| --- | --- |

| 56. | Six sigma is similar to continuous improvement of TQM. True False |
| --- | --- |

| 57. | The optimum level of inspection for quality control is where the: A.; cost of inspection is minimum. B.; cost of passing defectives is minimum. C.; total cost of inspection and defectives is maximum. D.; total cost of inspection and defectives is minimum. E.; difference in inspection and defectives costs is minimum. |
| --- | --- |

| 58. | The purpose of control charts is to: A.; estimate the proportion of output that is acceptable. B.; weed out defective items. C.; determine if the output is within specifications/specifications. D.; distinguish between random variation and assignable variation in the process. E.; all of these. |
| --- | --- |

| 59. | Which of the following is not an inspection point in manufacturing? A.; Raw materials and purchased parts at the beginning of the process. B.; Finished products at the end of the process. C.; Before a costly operation in the process. D.; Before a covering operation in the process. E.; All of the choices are inspection points. |
| --- | --- |

| 60. | Which of the following is not a step in the quality control planning process? A.; define the quality characteristic to be controlled. B.; determine a quality control point in the process. C.; eliminate all defects found. D.; plan how inspection is to be done. E.; all of the choices are steps.. |
| --- | --- |

| 61. | The term "2-sigma limits" in the context of a control chart refers to the variability of the: A.; process. B.; population. C.; sample. D.; sampling distribution. E.; none of the choices. |
| --- | --- |

| 62. | The probability of concluding that assignable variation exists when only random variation is present is: I. the probability of a Type I error II. known as the alpha risk III. highly unlikely IV. the sum of probabilities in the two tails of the distribution A.; I and II. B.; I and IV. C.; II and III. D.; I, II, and IV. E.; I, III, and IV. |
| --- | --- |

| 63. | Three sigma control limits (compared to two sigma limits) tend to have what impact on error probabilities? A.; increase the Type II and decrease Type I. B.; decrease the Type II and increase Type I. C.; decrease the Type I and Type II. D.; increase the Type I and Type II. E.; have little or no impact. |
| --- | --- |

| 64. | A control chart used to monitor the process mean is the: A.; p-chart. B.; R-chart. C.; x-bar chart. D.; c-chart. E.; Gantt chart. |
| --- | --- |

| 65. | Sample mean control charts can be created using: I. the process standard deviation II. the standard deviation of the sampling distribution III. the sample range IV. the average of sample ranges A.; I or II. B.; I or III. C.; II or III. D.; II or IV. E.; I or II or IV. |
| --- | --- |

| 66. | A shift in the process mean for a measured characteristic would most likely be detected by a: A.; p-chart. B.; x-bar chart. C.; c-chart. D.; R-chart. E.; s-chart. |
| --- | --- |

| 67. | A point which is outside of the lower control limit on an R-chart: A.; is an indication that no cause of variation is present. B.; should be ignored because it signifies better than average quality. C.; should be investigated because it could indicate an improved method. D.; should be ignored unless another point is outside that limit. E.; is impossible since the lower limit is always zero. |
| --- | --- |

| 68. | A point that plots below the lower control limit on the range chart: I. should be ignored since lower variation is desirable. II. may be an indication that process variation has decreased. III. should be investigated for assignable cause. A.; I and II. B.; I and III. C.; II and III. D.; II only. E.; I, II, and III. |
| --- | --- |

| 69. | The range chart (R-chart) is most likely to detect a change in: A.; proportion. B.; mean. C.; number defective. D.; process variability. E.; sample size. |
| --- | --- |

| 70. | Which of the following quality control sample statistics indicates a quality characteristic which is an attribute? A.; mean. B.; variance. C.; standard deviation. D.; range. E.; proportion. |
| --- | --- |

| 71. | A control chart used to monitor the fraction of defectives generated by a process is the: A.; p-chart. B.; R-chart. C.; x-bar chart. D.; c-chart. E.; Gantt chart. |
| --- | --- |

| 72. | For which of the following would a p-chart be used? A.; monitor average shrinkage. B.; monitor dispersion in sample data. C.; monitor the fraction defective in a sample. D.; monitor the number of defects per unit. E.; monitor the range of values. |
| --- | --- |

| 73. | A c-chart is used for: A.; means. B.; ranges. C.; percent defective. D.; fraction defective per unit. E.; number of defects per unit. |
| --- | --- |

| 74. | A control chart used to monitor the number of defects per unit is the: A.; p-chart. B.; R-chart. C.; x-bar chart. D.; c-chart. E.; Gantt chart. |
| --- | --- |

| 75. | Which statement is an accurate description of a process that is ``capable``? A.; no process variability is evident. B.; process variability is greater than variation allowed by design specifications. C.; process variability is less than variation allowed by design specifications. D.; assignable variation is less than random variation. E.; assignable variation is greater than random variation. |
| --- | --- |

| 76. | Which of the following relationships must never be correct? A.; Specifications > process variability > control limits. B.; Process variability > specifications > control limits. C.; Specifications > control limits > process variability. D.; Process variability > control limits > specifications. E.; All of the choices may be correct.. |
| --- | --- |

| 77. | Studies on a bottle-filling machine indicates it fill bottles to a mean of 16 ounces with a standard deviation of 0.10 ounces. What is the process specification, assuming the Cpindex of 1? A.; 0.10 ounces. B.; 0.60 ounces. C.; 16.0 ounces. D.; 16.0 ounces plus or minus 0.30 ounces. E.; none of the choices. |
| --- | --- |

| 78. | The six sigma improvement methodology is called: A.; PDSA. B.; DMAIC. C.; 5W2H. D.; Continuous improvement system. E.; None of the choices. |
| --- | --- |

| 79. | The DMAIC methodology involves all but which of the following? A.; Define. B.; Measure. C.; Analyze. D.; Improvise. E.; Control. |
| --- | --- |

|  | A design engineer wants to construct a sample mean chart for controlling the service life of one of the types of light bulbs his company produces. He knows from numerous previous samples that this service life is normally distributed with a mean of 500 hours and a standard deviation of 20 hours. On three recent production batches, he tested service life on random samples of four bulbs, with these results: |
| --- | --- |

| 80. | What is the sample mean service life for sample 2? A.; 460 hours. B.; 495 hours. C.; 500 hours. D.; 515 hours. E.; 525 hours. |
| --- | --- |

| 81. | What is the mean of the sampling distribution of sample means for whenever service life is in control? A.; 250 hours. B.; 470 hours. C.; 495 hours. D.; 500 hours. E.; 515 hours. |
| --- | --- |

| 82. | What is the standard deviation of the sampling distribution of sample means for whenever service life is in control? A.; 5 hours. B.; 6.67 hours. C.; 10 hours. D.; 11.55 hours. E.; 20 hours. |
| --- | --- |

| 83. | If he uses upper and lower control limits of 520 and 480 hours, what is his risk (alpha) of concluding service life is out of control when it is actually under control (Type I error)? A.; 0.0026. B.; 0.0456. C.; 0.3174. D.; 0.6826. E.; 0.9544. |
| --- | --- |

| 84. | If he uses upper and lower control limits of 520 and 480 hours, on what sample(s) (if any) does service life appear to be out of control? A.; sample 1. B.; sample 2. C.; sample 3. D.; both samples 2 and 3. E.; none. |
| --- | --- |

|  | A Quality Analyst wants to construct a sample mean chart for controlling a packaging process. He knows from past experience that whenever this process is under control, package weight is normally distributed with a mean of twenty ounces and a standard deviation of two ounces. Each day last week, he randomly selected four packages and weighed each: |
| --- | --- |

| 85. | What is the sample mean package weight for Thursday? A.; 19 ounces. B.; 20 ounces. C.; 20.6 ounces. D.; 21 ounces. E.; 23 ounces. |
| --- | --- |

| 86. | What is the mean of the sampling distribution of sample means for whenever this process is under control? A.; 18 ounces. B.; 19 ounces. C.; 20 ounces. D.; 21 ounces. E.; 22 ounces. |
| --- | --- |

| 87. | What is the standard deviation of the sampling distribution of sample means for whenever this process is under control? A.; 0.1 ounces. B.; 0.4 ounces. C.; 0.5 ounces. D.; 1 ounce. E.; 2 ounces. |
| --- | --- |

| 88. | If he uses upper and lower control limits of 22 and 18 ounces, what is his risk (alpha) of concluding this process is out of control when it is actually in control (Type I error)? A.; 0.0026. B.; 0.0456. C.; 0.3174. D.; 0.6826. E.; 0.9544. |
| --- | --- |

| 89. | If he uses upper and lower control limits of 22 and 18 ounces, on what day(s), if any, does this process appear to be out of control? A.; Monday. B.; Tuesday. C.; Monday and Tuesday. D.; Monday, Tuesday, and Thursday. E.; none. |
| --- | --- |

|  | A Quality Analyst wants to construct a control chart for determining whether three machines, all producing the same product, are under control with regard to a particular quality variable. Accordingly, he sampled four units of output from each machine, with the following results: |
| --- | --- |

| 90. | What is the sample mean for machine #1? A.; 15. B.; 16. C.; 17. D.; 21. E.; 23. |
| --- | --- |

| 91. | What is the estimate of the process mean for whenever it is under control? A.; 16. B.; 19. C.; 20. D.; 21. E.; 23. |
| --- | --- |

| 92. | What is the estimate of the sample average range based upon this limited sample? A.; 13.0. B.; 4.33. C.; 5.4. D.; 4.2. E.; 2.0. |
| --- | --- |

| 93. | What are the x-bar chart three sigma upper and lower control limits? A.; 22 and 18. B.; 23.29 and 16.71. C.; 23.5 and 16.5. D.; 23.16 and 16.84. E.; 24 and 16. |
| --- | --- |

| 94. | For upper and lower control limits of 23.29 and 16.71, which machine(s), if any, appear(s) to have an out-of-control process mean? A.; machine #1. B.; machine #2. C.; machine #3. D.; all of the machines. E.; none of the machines. |
| --- | --- |

|  | The Chair of the Operations Management Department at Quality University wants to construct a p-chart for determining whether the four faculty teaching the basic P/OM course are under control with regard to the number of students who fail the course. Accordingly, he sampled 100 final grades from last year for each instructor, with the following results: |
| --- | --- |

| 95. | What is the sample proportion of failures (p) for Prof. D? A.; 0. B.; .04. C.; .11. D.; .13. E.; .16. |
| --- | --- |

| 96. | What is the estimate of the mean proportion of failures for these instructors? A.; .10. B.; .11. C.; .13. D.; .16. E.; .40. |
| --- | --- |

| 97. | What is the estimate of the standard deviation of the sampling distribution for an instructor's sample proportion of failures? A.; .0075. B.; .03. C.; .075. D.; .3. E.; .75. |
| --- | --- |

| 98. | What are the .95 (5% risk of Type I error) upper and lower control limits for the p-chart? A.; .95 and .05. B.; .13 and .07. C.; .1588 and .0412. D.; .16 and .04. E.; .1774 and .0226. |
| --- | --- |

| 99. | Using .95 control limits, (5% risk of Type I error), which instructor(s), if any, should he conclude is (are) out of control? A.; none. B.; Prof. B. C.; Prof. D. D.; both Prof. B and Prof. D. E.; all. |
| --- | --- |

|  | A Quality Analyst wants to construct a control chart for determining whether four machines, all producing the same product, are under control with regard to a particular quality attribute. Accordingly, she inspected 1,000 units of output from each machine in random samples, with the following results: |
| --- | --- |

| 100. | What is the sample proportion of defectives for machine #1? A.; .023. B.; .02. C.; .0115. D.; .0058. E.; .005. |
| --- | --- |

| 101. | What is the estimate of the process proportion of defectives for whenever it is under control? A.; .08. B.; .06. C.; .04. D.; .02. E.; .01. |
| --- | --- |

| 102. | What is the estimate of the standard deviation of the sampling distribution of sample proportions for whenever this process is under control? A.; .016. B.; .00016. C.; .04. D.; .0044. E.; .00002. |
| --- | --- |

| 103. | What are the control chart upper and lower control limits for an alpha risk of .05? A.; .0272 and .0128. B.; .0287 and .0113. C.; .029 and .013. D.; .0303 and .0097. E.; .0332 and .0068. |
| --- | --- |

| 104. | For upper and lower control limits of .026 and .014, which machine(s), if any, appear(s) to be out-of-control for process proportion of defectives? A.; machine #3. B.; machine #4. C.; machines #3 and #4. D.; machines #2 and #3. E.; none of the machines. |
| --- | --- |

| 105. | Studies on a machine that molds plastic water pipe indicate that when it is injecting 1-inch diameter pipe, the process standard deviation is 0.05 inches. The one-inch pipe has a specification of 1-inch plus or minus 0.10 inch. What is the process capability index (Cp)? A.; 0.50. B.; 0.67. C.; 1.00. D.; 2.00. E.; none of the choices. |
| --- | --- |

| 106. | The specification limit for a product is 8 cm and 10 cm. A process that produces the product has a mean of 9.5 cm and a standard deviation of 0.2 cm. What is the process capability, Cpk? A.; 3.33. B.; 1.67. C.; 0.83. D.; 2.50. E.; none of the choices. |
| --- | --- |

| 107. | The specifications for a product are 6 mm 0.1 mm. The process is known to operate at a mean of 6.05 with a standard deviation of 0.01 mm. What is the Cpkfor this process? A.; 3.33. B.; 1.67. C.; 5.00. D.; 2.50. E.; none of the choices. |
| --- | --- |

| 108. | A process that produces explosion-proof fittings has an output that is normally distributed with a mean of 6 cm. and a standard deviation of .01 cm. A job is to be run that requires 200 fittings. (i) Determine three sigma control limits for an x-bar chart assuming a sample size of 10. (ii) If specifications are 5.98 to 6.02, what run size should be used for this job so that the expected number of good pieces is 200, assuming the process is in control? |
| --- | --- |

| 109. | Four samples of three observations each have been taken, with actual measurements (in centimetres) shown below. Construct three sigma mean and range charts, and determine if corrective action is needed. |
| --- | --- |

| 110. | A town's department of public works is concerned about adverse public reaction to a sewer project that is currently in progress. Because of this, the Commissioner of Public Works has authorized a weekly survey to be conducted of town residents. Each week, a sample of 100 residents is questioned on their feelings towards the project. The results to date are shown below. Analyze this data using an appropriate control chart with a 5% risk of Type I error. Is the community sentiment stable? |
| --- | --- |

| 111. | Construct the appropriate control chart for the sample observations listed below, and determine if the process is in control using two sigma limits. |
| --- | --- |

| 112. | The data below represent sample means which were taken at periodic intervals and plotted on a control chart. Is the output random? |
| --- | --- |

| 113. | Given the following control chart, would you say that the process appears to be performing appropriately? |
| --- | --- |

|  | Given the following process control data for a normally distributed quality variable (three samples of size four each): |
| --- | --- |

| 114. | What is the sample mean for sample #1? #2? #3? |
| --- | --- |

| 115. | If the process is known to have a mean of 15 and a standard deviation of 3, what is the mean of the sampling distribution of sample means for whenever this process is under control? The standard deviation? |
| --- | --- |

| 116. | If the process is known to have a mean of 15 and a standard deviation of 3, what is the alpha risk (probability of Type I error) for upper and lower control limits of 16.5 and 13.5 respectively? 18 and 12? 19.5 and 10.5? |
| --- | --- |

| 117. | If the process is known to have a mean of 15 and a standard deviation of 3, what are the three sigma upper and lower control limits for an x-bar chart? |
| --- | --- |

| 118. | If the process is known to have a mean of 15 and a standard deviation of 3, using three sigma control limits, do any of the sample means indicate an out-of-control process mean? |
| --- | --- |

|  | Given the following process control data for a quality attribute (three samples of size 400 each): |
| --- | --- |

| 119. | What is the sample proportion of defectives for sample #1? #2? #3? |
| --- | --- |

| 120. | If the process is known to produce 11 percent defectives on average, what is the mean of the sampling distribution of sample proportions for whenever this process is under control? The standard deviation? |
| --- | --- |

| 121. | If the process is known to produce 11 percent defectives on average, what is the alpha risk (probability of Type I error) for upper and lower control limits of .1256 and .0944 respectively? .1412 and .0788? .1568 and .0632? |
| --- | --- |

| 122. | If the process is known to produce 11 percent defectives on average, what are the upper and lower control limits for an alpha risk of .10? .05? .01? |
| --- | --- |

| 123. | If the process is known to produce 11 percent defectives on average, using three sigma control limits, do any of the sample proportions indicate an out-of-control process proportion of defectives? |
| --- | --- |

| 124. | If the process proportion of defectives is unknown, what is the estimate of it? |
| --- | --- |

| 125. | If the process proportion of defectives is unknown, what is the alpha risk (probability of Type I error) for upper and lower control limits of .115 and .085 respectively? .13 and .07? .145 and .055? |
| --- | --- |

| 126. | If the process proportion of defectives is unknown, what are the upper and lower control limits for an alpha risk of .10? .05? .01? |
| --- | --- |

| 127. | If the process proportion of defectives is unknown, using .10 alpha risk control limits, do any of the sample proportions indicate an out-of-control process proportion of defectives? |
| --- | --- |

Chapter 10 Quality Control Key

| 1. | Statistical quality control that occurs during the production process is known as acceptance sampling. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #1

| 2. | Quality control efforts that occur during production are referred to as statistical process control. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #2

| 3. | Statistical quality control is assuring that processes are performing in an acceptable manner. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 10 #3

| 4. | Typically the production of high-cost, low-volume items require little quality control inspection. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #4

| 5. | Low-cost, high-volume items often require more intensive inspection. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #5

| 6. | The amount of inspection can range from no inspection at all to inspecting each item numerous times. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 10 #6

| 7. | The optimum level of inspection occurs when the cost of passing defectives is minimized. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #7

| 8. | The optimum level of inspection minimizes the sum of inspection costs and the cost of passing defectives. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 10 #8

| 9. | The frequency and quantity of inspection relates primarily to the cost of inspection. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 1 Stevenson - Chapter 10 #9

| 10. | The amount of inspection needed is governed by the costs of inspection and the expected costs of passing defective items. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 10 #10

| 11. | Most operations have very few places where it is possible to inspect the product during the production process. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 10 #11

| 12. | The HACCP system provides guidelines for determining critical inspection points. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #12

| 13. | The primary purpose of statistical process control is to detect defects in finished products before they are shipped to customers. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #13

| 14. | Statistical quality control is based on comparing samples from a process to predetermined limits. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #14

| 15. | Variation in a sample statistic collected from a process may be either random variation or assignable variation. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #15

| 16. | The main task in process control is to distinguish assignable from random variation. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #16

| 17. | The variability of a process is equivalent to the distribution of samples of any size taken from the process. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #17

| 18. | The distribution of sample means from a process has less variability than the process distribution. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #18

| 19. | The sampling distribution can be assumed to be approximately normal even when the underlying process distribution is not normally distributed. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #19

| 20. | Approximately 99.7% of sample means will fall within ± two standard deviations of the process mean. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #20

| 21. | "Assignable variation" is variation due to a specific cause, such as tool wear. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #21

| 22. | Control limits distinguish between non-random and assignable variability. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #22

| 23. | Control limits are directly related to design specification limits. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #23

| 24. | A sample statistic that falls outside the control limits suggests that the process mean has changed. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #24

| 25. | A sample statistic that falls inside the control limits suggests non-random, assignable variation in the process. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #25

| 26. | Concluding a process has not changed when it has is known as a Type I error. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #26

| 27. | Concluding a process has changed when it has not is known as a Type I error. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #27

| 28. | Concluding a process has changed when it has not is known as a Type II error. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #28

| 29. | The probability of a Type I error is the probability of concluding that only randomness is present when assignable variation exists. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #29

| 30. | If a point on a control chart falls outside one of the control limits, this suggests that the process output is non-random. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #30

| 31. | The sample size chosen for control charts depends on the cost of inspection, and the cost of Type I and II errors. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #31

| 32. | Range control charts are used to monitor process dispersion. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #32

| 33. | Control charts based on attribute data are for process characteristics that are counted rather than measured. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #33

| 34. | Attributes are process characteristics that must be measured rather than counted. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #34

| 35. | Range charts are used to measure shifts in the process mean. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #35

| 36. | Sample mean charts are used to measure shifts in process dispersion. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #36

| 37. | The number of defective parts in a sample is a process characteristic that is counted rather than measured. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #37

| 38. | A p-chart is used to monitor the fraction of defectives in the output of a process. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #38

| 39. | A c-chart is used to monitor the number of defectives in the output of a process. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #39

| 40. | A c-chart is used to monitor the number of defects per unit for process output. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #40

| 41. | Design specifications are equivalent to control limits. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #41

| 42. | Control limits are specifications established by engineering design or customer requirements. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #42

| 43. | "Process capability" compares "process variability" to the "design specifications". TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #43

| 44. | Control limits and process variability are directly related. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 10 #44

| 45. | There is no direct link between design specifications and statistical quality control limits. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #45

| 46. | The output of a process may not conform to specifications even though the process may be statistically "in control." TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #46

| 47. | Control limits are statistical limits that reflect the extent to which sample statistics such as means and ranges can vary due to randomness alone. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #47

| 48. | Process variability is a key factor in process capability. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 10 #48

| 49. | A process is "capable" if the process output falls within the acceptable range of variability allowed by the design specification. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #49

| 50. | Tolerances are specified in terms of the natural range of variation of a process. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #50

| 51. | The process capability index (indicated by Cp) is calculated as the ratio design specification width to the process variation width. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #51

| 52. | The process capability index (indicated by Cpk) is used only when the process is centred. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #52

| 53. | When a process is not centred, its capability should be based on considering the upper and lower design specifications separately, not just the overall design specification width. The symbol for this case is Cpk. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 10 #53

| 54. | In order for a process to be capable, it must have a capability ratio of at least 1.00. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 10 #54

| 55. | A process capability index (Cp) of 0.70 indicates that a process is capable of producing to design specifications. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #55

| 56. | Six sigma is similar to continuous improvement of TQM. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 10 #56

| 57. | The optimum level of inspection for quality control is where the: A.; cost of inspection is minimum. B.; cost of passing defectives is minimum. C.; total cost of inspection and defectives is maximum. D.; total cost of inspection and defectives is minimum. E.; difference in inspection and defectives costs is minimum. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #57

| 58. | The purpose of control charts is to: A.; estimate the proportion of output that is acceptable. B.; weed out defective items. C.; determine if the output is within specifications/specifications. D.; distinguish between random variation and assignable variation in the process. E.; all of these. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #58

| 59. | Which of the following is not an inspection point in manufacturing? A.; Raw materials and purchased parts at the beginning of the process. B.; Finished products at the end of the process. C.; Before a costly operation in the process. D.; Before a covering operation in the process. E.; All of the choices are inspection points. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 10 #59

| 60. | Which of the following is not a step in the quality control planning process? A.; define the quality characteristic to be controlled. B.; determine a quality control point in the process. C.; eliminate all defects found. D.; plan how inspection is to be done. E.; all of the choices are steps.. |
| --- | --- |

Difficulty: Hard Learning Objective: 1 Stevenson - Chapter 10 #60

| 61. | The term "2-sigma limits" in the context of a control chart refers to the variability of the: A.; process. B.; population. C.; sample. D.; sampling distribution. E.; none of the choices. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #61

| 62. | The probability of concluding that assignable variation exists when only random variation is present is: I. the probability of a Type I error II. known as the alpha risk III. highly unlikely IV. the sum of probabilities in the two tails of the distribution A.; I and II. B.; I and IV. C.; II and III. D.; I, II, and IV. E.; I, III, and IV. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #62

| 63. | Three sigma control limits (compared to two sigma limits) tend to have what impact on error probabilities? A.; increase the Type II and decrease Type I. B.; decrease the Type II and increase Type I. C.; decrease the Type I and Type II. D.; increase the Type I and Type II. E.; have little or no impact. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #63

| 64. | A control chart used to monitor the process mean is the: A.; p-chart. B.; R-chart. C.; x-bar chart. D.; c-chart. E.; Gantt chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #64

| 65. | Sample mean control charts can be created using: I. the process standard deviation II. the standard deviation of the sampling distribution III. the sample range IV. the average of sample ranges A.; I or II. B.; I or III. C.; II or III. D.; II or IV. E.; I or II or IV. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #65

| 66. | A shift in the process mean for a measured characteristic would most likely be detected by a: A.; p-chart. B.; x-bar chart. C.; c-chart. D.; R-chart. E.; s-chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #66

| 67. | A point which is outside of the lower control limit on an R-chart: A.; is an indication that no cause of variation is present. B.; should be ignored because it signifies better than average quality. C.; should be investigated because it could indicate an improved method. D.; should be ignored unless another point is outside that limit. E.; is impossible since the lower limit is always zero. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #67

| 68. | A point that plots below the lower control limit on the range chart: I. should be ignored since lower variation is desirable. II. may be an indication that process variation has decreased. III. should be investigated for assignable cause. A.; I and II. B.; I and III. C.; II and III. D.; II only. E.; I, II, and III. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #68

| 69. | The range chart (R-chart) is most likely to detect a change in: A.; proportion. B.; mean. C.; number defective. D.; process variability. E.; sample size. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #69

| 70. | Which of the following quality control sample statistics indicates a quality characteristic which is an attribute? A.; mean. B.; variance. C.; standard deviation. D.; range. E.; proportion. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #70

| 71. | A control chart used to monitor the fraction of defectives generated by a process is the: A.; p-chart. B.; R-chart. C.; x-bar chart. D.; c-chart. E.; Gantt chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #71

| 72. | For which of the following would a p-chart be used? A.; monitor average shrinkage. B.; monitor dispersion in sample data. C.; monitor the fraction defective in a sample. D.; monitor the number of defects per unit. E.; monitor the range of values. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #72

| 73. | A c-chart is used for: A.; means. B.; ranges. C.; percent defective. D.; fraction defective per unit. E.; number of defects per unit. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #73

| 74. | A control chart used to monitor the number of defects per unit is the: A.; p-chart. B.; R-chart. C.; x-bar chart. D.; c-chart. E.; Gantt chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 10 #74

| 75. | Which statement is an accurate description of a process that is ``capable``? A.; no process variability is evident. B.; process variability is greater than variation allowed by design specifications. C.; process variability is less than variation allowed by design specifications. D.; assignable variation is less than random variation. E.; assignable variation is greater than random variation. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #75

| 76. | Which of the following relationships must never be correct? A.; Specifications > process variability > control limits. B.; Process variability > specifications > control limits. C.; Specifications > control limits > process variability. D.; Process variability > control limits > specifications. E.; All of the choices may be correct.. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #76

| 77. | Studies on a bottle-filling machine indicates it fill bottles to a mean of 16 ounces with a standard deviation of 0.10 ounces. What is the process specification, assuming the Cpindex of 1? A.; 0.10 ounces. B.; 0.60 ounces. C.; 16.0 ounces. D.; 16.0 ounces plus or minus 0.30 ounces. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #77

| 78. | The six sigma improvement methodology is called: A.; PDSA. B.; DMAIC. C.; 5W2H. D.; Continuous improvement system. E.; None of the choices. |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 10 #78

| 79. | The DMAIC methodology involves all but which of the following? A.; Define. B.; Measure. C.; Analyze. D.; Improvise. E.; Control. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 10 #79

|  | A design engineer wants to construct a sample mean chart for controlling the service life of one of the types of light bulbs his company produces. He knows from numerous previous samples that this service life is normally distributed with a mean of 500 hours and a standard deviation of 20 hours. On three recent production batches, he tested service life on random samples of four bulbs, with these results: |
| --- | --- |

Stevenson - Chapter 10

| 80. | What is the sample mean service life for sample 2? A.; 460 hours. B.; 495 hours. C.; 500 hours. D.; 515 hours. E.; 525 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #80

| 81. | What is the mean of the sampling distribution of sample means for whenever service life is in control? A.; 250 hours. B.; 470 hours. C.; 495 hours. D.; 500 hours. E.; 515 hours. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #81

| 82. | What is the standard deviation of the sampling distribution of sample means for whenever service life is in control? A.; 5 hours. B.; 6.67 hours. C.; 10 hours. D.; 11.55 hours. E.; 20 hours. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #82

| 83. | If he uses upper and lower control limits of 520 and 480 hours, what is his risk (alpha) of concluding service life is out of control when it is actually under control (Type I error)? A.; 0.0026. B.; 0.0456. C.; 0.3174. D.; 0.6826. E.; 0.9544. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #83

| 84. | If he uses upper and lower control limits of 520 and 480 hours, on what sample(s) (if any) does service life appear to be out of control? A.; sample 1. B.; sample 2. C.; sample 3. D.; both samples 2 and 3. E.; none. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #84

|  | A Quality Analyst wants to construct a sample mean chart for controlling a packaging process. He knows from past experience that whenever this process is under control, package weight is normally distributed with a mean of twenty ounces and a standard deviation of two ounces. Each day last week, he randomly selected four packages and weighed each: |
| --- | --- |

Stevenson - Chapter 10

| 85. | What is the sample mean package weight for Thursday? A.; 19 ounces. B.; 20 ounces. C.; 20.6 ounces. D.; 21 ounces. E.; 23 ounces. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #85

| 86. | What is the mean of the sampling distribution of sample means for whenever this process is under control? A.; 18 ounces. B.; 19 ounces. C.; 20 ounces. D.; 21 ounces. E.; 22 ounces. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #86

| 87. | What is the standard deviation of the sampling distribution of sample means for whenever this process is under control? A.; 0.1 ounces. B.; 0.4 ounces. C.; 0.5 ounces. D.; 1 ounce. E.; 2 ounces. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #87

| 88. | If he uses upper and lower control limits of 22 and 18 ounces, what is his risk (alpha) of concluding this process is out of control when it is actually in control (Type I error)? A.; 0.0026. B.; 0.0456. C.; 0.3174. D.; 0.6826. E.; 0.9544. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #88

| 89. | If he uses upper and lower control limits of 22 and 18 ounces, on what day(s), if any, does this process appear to be out of control? A.; Monday. B.; Tuesday. C.; Monday and Tuesday. D.; Monday, Tuesday, and Thursday. E.; none. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #89

|  | A Quality Analyst wants to construct a control chart for determining whether three machines, all producing the same product, are under control with regard to a particular quality variable. Accordingly, he sampled four units of output from each machine, with the following results: |
| --- | --- |

Stevenson - Chapter 10

| 90. | What is the sample mean for machine #1? A.; 15. B.; 16. C.; 17. D.; 21. E.; 23. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #90

| 91. | What is the estimate of the process mean for whenever it is under control? A.; 16. B.; 19. C.; 20. D.; 21. E.; 23. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #91

| 92. | What is the estimate of the sample average range based upon this limited sample? A.; 13.0. B.; 4.33. C.; 5.4. D.; 4.2. E.; 2.0. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #92

| 93. | What are the x-bar chart three sigma upper and lower control limits? A.; 22 and 18. B.; 23.29 and 16.71. C.; 23.5 and 16.5. D.; 23.16 and 16.84. E.; 24 and 16. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #93

| 94. | For upper and lower control limits of 23.29 and 16.71, which machine(s), if any, appear(s) to have an out-of-control process mean? A.; machine #1. B.; machine #2. C.; machine #3. D.; all of the machines. E.; none of the machines. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #94

|  | The Chair of the Operations Management Department at Quality University wants to construct a p-chart for determining whether the four faculty teaching the basic P/OM course are under control with regard to the number of students who fail the course. Accordingly, he sampled 100 final grades from last year for each instructor, with the following results: |
| --- | --- |

Stevenson - Chapter 10

| 95. | What is the sample proportion of failures (p) for Prof. D? A.; 0. B.; .04. C.; .11. D.; .13. E.; .16. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #95

| 96. | What is the estimate of the mean proportion of failures for these instructors? A.; .10. B.; .11. C.; .13. D.; .16. E.; .40. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #96

| 97. | What is the estimate of the standard deviation of the sampling distribution for an instructor's sample proportion of failures? A.; .0075. B.; .03. C.; .075. D.; .3. E.; .75. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #97

| 98. | What are the .95 (5% risk of Type I error) upper and lower control limits for the p-chart? A.; .95 and .05. B.; .13 and .07. C.; .1588 and .0412. D.; .16 and .04. E.; .1774 and .0226. |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #98

| 99. | Using .95 control limits, (5% risk of Type I error), which instructor(s), if any, should he conclude is (are) out of control? A.; none. B.; Prof. B. C.; Prof. D. D.; both Prof. B and Prof. D. E.; all. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #99

|  | A Quality Analyst wants to construct a control chart for determining whether four machines, all producing the same product, are under control with regard to a particular quality attribute. Accordingly, she inspected 1,000 units of output from each machine in random samples, with the following results: |
| --- | --- |

Stevenson - Chapter 10

| 100. | What is the sample proportion of defectives for machine #1? A.; .023. B.; .02. C.; .0115. D.; .0058. E.; .005. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #100

| 101. | What is the estimate of the process proportion of defectives for whenever it is under control? A.; .08. B.; .06. C.; .04. D.; .02. E.; .01. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #101

| 102. | What is the estimate of the standard deviation of the sampling distribution of sample proportions for whenever this process is under control? A.; .016. B.; .00016. C.; .04. D.; .0044. E.; .00002. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #102

| 103. | What are the control chart upper and lower control limits for an alpha risk of .05? A.; .0272 and .0128. B.; .0287 and .0113. C.; .029 and .013. D.; .0303 and .0097. E.; .0332 and .0068. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #103

| 104. | For upper and lower control limits of .026 and .014, which machine(s), if any, appear(s) to be out-of-control for process proportion of defectives? A.; machine #3. B.; machine #4. C.; machines #3 and #4. D.; machines #2 and #3. E.; none of the machines. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #104

| 105. | Studies on a machine that molds plastic water pipe indicate that when it is injecting 1-inch diameter pipe, the process standard deviation is 0.05 inches. The one-inch pipe has a specification of 1-inch plus or minus 0.10 inch. What is the process capability index (Cp)? A.; 0.50. B.; 0.67. C.; 1.00. D.; 2.00. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 10 #105

| 106. | The specification limit for a product is 8 cm and 10 cm. A process that produces the product has a mean of 9.5 cm and a standard deviation of 0.2 cm. What is the process capability, Cpk? A.; 3.33. B.; 1.67. C.; 0.83. D.; 2.50. E.; none of the choices. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #106

| 107. | The specifications for a product are 6 mm 0.1 mm. The process is known to operate at a mean of 6.05 with a standard deviation of 0.01 mm. What is the Cpkfor this process? A.; 3.33. B.; 1.67. C.; 5.00. D.; 2.50. E.; none of the choices. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 10 #107

| 108. | A process that produces explosion-proof fittings has an output that is normally distributed with a mean of 6 cm. and a standard deviation of .01 cm. A job is to be run that requires 200 fittings. (i) Determine three sigma control limits for an x-bar chart assuming a sample size of 10. (ii) If specifications are 5.98 to 6.02, what run size should be used for this job so that the expected number of good pieces is 200, assuming the process is in control? |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #108

| 109. | Four samples of three observations each have been taken, with actual measurements (in centimetres) shown below. Construct three sigma mean and range charts, and determine if corrective action is needed. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #109

| 110. | A town's department of public works is concerned about adverse public reaction to a sewer project that is currently in progress. Because of this, the Commissioner of Public Works has authorized a weekly survey to be conducted of town residents. Each week, a sample of 100 residents is questioned on their feelings towards the project. The results to date are shown below. Analyze this data using an appropriate control chart with a 5% risk of Type I error. Is the community sentiment stable? |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #110

| 111. | Construct the appropriate control chart for the sample observations listed below, and determine if the process is in control using two sigma limits. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #111

| 112. | The data below represent sample means which were taken at periodic intervals and plotted on a control chart. Is the output random? |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #112

| 113. | Given the following control chart, would you say that the process appears to be performing appropriately? A process appears to be performing appropriately if two conditions are met: (1) all points are within the limits and (2) no patterns. These conditions are met, based on visual inspection. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #113

|  | Given the following process control data for a normally distributed quality variable (three samples of size four each): |
| --- | --- |

Stevenson - Chapter 10

| 114. | What is the sample mean for sample #1? #2? #3? 14; 17.5; 16.5 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #114

| 115. | If the process is known to have a mean of 15 and a standard deviation of 3, what is the mean of the sampling distribution of sample means for whenever this process is under control? The standard deviation? 15; 1.5 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #115

| 116. | If the process is known to have a mean of 15 and a standard deviation of 3, what is the alpha risk (probability of Type I error) for upper and lower control limits of 16.5 and 13.5 respectively? 18 and 12? 19.5 and 10.5? .3174; .0456; .0026 |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #116

| 117. | If the process is known to have a mean of 15 and a standard deviation of 3, what are the three sigma upper and lower control limits for an x-bar chart? Lower limit = 10.5 Upper limit = 19.5 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #117

| 118. | If the process is known to have a mean of 15 and a standard deviation of 3, using three sigma control limits, do any of the sample means indicate an out-of-control process mean? No, all are within the limits. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #118

|  | Given the following process control data for a quality attribute (three samples of size 400 each): |
| --- | --- |

Stevenson - Chapter 10

| 119. | What is the sample proportion of defectives for sample #1? #2? #3? .09; .08; .13 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #119

| 120. | If the process is known to produce 11 percent defectives on average, what is the mean of the sampling distribution of sample proportions for whenever this process is under control? The standard deviation? .11; .0156 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #120

| 121. | If the process is known to produce 11 percent defectives on average, what is the alpha risk (probability of Type I error) for upper and lower control limits of .1256 and .0944 respectively? .1412 and .0788? .1568 and .0632? .3174; .0456; .0026 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #121

| 122. | If the process is known to produce 11 percent defectives on average, what are the upper and lower control limits for an alpha risk of .10? .05? .01? .1357 and .0843; .1406 and .0794; .1502 and .0698 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #122

| 123. | If the process is known to produce 11 percent defectives on average, using three sigma control limits, do any of the sample proportions indicate an out-of-control process proportion of defectives? No, all are within limits. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #123

| 124. | If the process proportion of defectives is unknown, what is the estimate of it? .10 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #124

| 125. | If the process proportion of defectives is unknown, what is the alpha risk (probability of Type I error) for upper and lower control limits of .115 and .085 respectively? .13 and .07? .145 and .055? .3174; .0456; .0026 |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 10 #125

| 126. | If the process proportion of defectives is unknown, what are the upper and lower control limits for an alpha risk of .10? .05? .01? .1247 and .0753; .1294 and .0706; .1387 and .0613 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #126

| 127. | If the process proportion of defectives is unknown, using .10 alpha risk control limits, do any of the sample proportions indicate an out-of-control process proportion of defectives? Yes: #3 |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 10 #127

Chapter 10 Quality Control Summary

| Category | # of Questions |
| --- | --- |
| Difficulty: Easy | 27 |
| Difficulty: Hard | 31 |
| Difficulty: Medium | 69 |
| Learning Objective: 1 | 15 |
| Learning Objective: 2 | 88 |
| Learning Objective: 3 | 21 |
| Learning Objective: 4 | 3 |
| Stevenson - Chapter 10 | 134 |