Chapter 14 Material Requirements Planning and Enterprise Resource Planning

| 1. | MRP usually involves inventory items that have independent demand. True False |
| --- | --- |

| 2. | Dependent demand is derived demand. True False |
| --- | --- |

| 3. | Independent demand tends to be lumpier than dependent demand. True False |
| --- | --- |

| 4. | Lumpy demand for components results primarily from the periodic scheduling of batch production. True False |
| --- | --- |

| 5. | Dependent demand tends to be intermittent, whereas independent demand tends to be more stable. True False |
| --- | --- |

| 6. | The three basic inputs of MRP are: 1) the bill of material; 2) the inventory level, lead times and open orders; and 3) planned order releases. True False |
| --- | --- |

| 7. | The master production schedule (MPS) states which end items are to be produced. True False |
| --- | --- |

| 8. | The master production schedule states (MPS) which product components are to be produced. True False |
| --- | --- |

| 9. | The planning horizon for the master production schedule (MPS) should cover the stacked or cumulative lead time necessary to produce the end items. True False |
| --- | --- |

| 10. | A phantom bill is a special type of bill of material that is a combination of the bill of materials for a number of subassemblies. True False |
| --- | --- |

| 11. | The MRP provides a list of the individual parts that will be converted to finished goods. True False |
| --- | --- |

| 12. | The bill of material indicates how much material will be needed for a given master production schedule. True False |
| --- | --- |

| 13. | A bill of material contains a listing of all the raw materials, parts, and subassemblies needed to produce one unit of an end item. True False |
| --- | --- |

| 14. | The bill of material contains information on lead times. True False |
| --- | --- |

| 15. | MRP requires information on inventory level, lead times and purchase orders for each item in stock. True False |
| --- | --- |

| 16. | An assembly-time chart indicates gross and net requirements. True False |
| --- | --- |

| 17. | For a given component in an MRP plan, under lot-for-lot ordering, planned order sizes are equal to net requirements. True False |
| --- | --- |

| 18. | The gross requirements at one level of an MRP plan determine the gross requirements at the next lower level. True False |
| --- | --- |

| 19. | Gross requirements for components are equal to the gross requirements of their immediate parent multiplied by the quantity per parent. True False |
| --- | --- |

| 20. | The term pegging refers to identifying the parent items that have generated a given set of material requirements for a part or subassembly. True False |
| --- | --- |

| 21. | A net-change MRP is one that is updated periodically. True False |
| --- | --- |

| 22. | Processing costs are less in a regenerative MRP than in a net-change MRP. True False |
| --- | --- |

| 23. | A regenerative MRP is one that is updated continuously. True False |
| --- | --- |

| 24. | One of the primary reports of MRP concerns changes to open orders. True False |
| --- | --- |

| 25. | Safety time is sometimes used in MRP rather than safety stock. True False |
| --- | --- |

| 26. | Lot-for-lot ordering in MRP provides coverage for some predetermined number of periods. True False |
| --- | --- |

| 27. | The part-period method for lot sizing attempts to balance setup and holding costs. True False |
| --- | --- |

| 28. | The part-period method for lot sizing breaks the time periods into smaller units of time to create daily or hourly production schedules. True False |
| --- | --- |

| 29. | Fixed-interval ordering in MRP places orders that cover a predetermined number of periods. True False |
| --- | --- |

| 30. | Load reports show capacity requirements for departments or work centres. True False |
| --- | --- |

| 31. | MRP II involves a broader approach to production planning and scheduling than MRP. True False |
| --- | --- |

| 32. | MRP II is simply an improved version of software for MRP. True False |
| --- | --- |

| 33. | Load reports of capacity may be misleading as they do not consider scheduling and queuing delays. True False |
| --- | --- |

| 34. | Load reports compare expected capacity requirements with planned capacity requirements. True False |
| --- | --- |

| 35. | ERP systems are more popular than MRP because they are less complex to use. True False |
| --- | --- |

| 36. | ERP involves software that is used to manage and coordinate all the resources, information, and functions of an organization from a shared database. True False |
| --- | --- |

| 37. | Typically the total cost of ownership (TCO) of ERP systems is clearly detailed and certain at the time a company makes the investment to acquire a system. True False |
| --- | --- |

| 38. | ERP packages are intended to fit established ways of doing business, but some modifications to business processes or the ERP software are typically required. True False |
| --- | --- |

| 39. | Any company should be able to use an ERP system. True False |
| --- | --- |

| 40. | ERP implementations that fully transform a company's operations typically take from one to three years. True False |
| --- | --- |

| 41. | ERP automates the tasks involved in performing a business process, such as order fulfilment and financial reporting. True False |
| --- | --- |

| 42. | Independent demand tends to be __________, while dependent demand tends to be _________. A.; stable, unstable. B.; unstable, stable. C.; stable, lumpy. D.; smooth, sawtooth. E.; sawtooth, smooth. |
| --- | --- |

| 43. | Which of the following most closely describes dependent demand? A.; demand generated by suppliers. B.; estimates of demand using regression analysis of independent variables. C.; derived demand. D.; demands placed on suppliers by their customers. E.; net material requirements. |
| --- | --- |

| 44. | A process designed to handle ordering and scheduling of dependent-demand components is: A.; economic part-period (EPP). B.; lot-for-lot (LFL). C.; economic order quantity (EOQ). D.; material requirements planning (MRP). E.; economic run size (ERS). |
| --- | --- |

| 45. | Which of these items would be most likely to have dependent demand? A.; electric starters. B.; toy trains. C.; flowers. D.; chocolate chip cookies. E.; none of the choices. |
| --- | --- |

| 46. | The development and application of MRP is primarily associated with upon two developments: (1) managers recognizing the need to differentiate between independent and dependent demand, and (2) __________. A.; computers. B.; development of the EOQ model. C.; inventory control systems. D.; all of the choices. E.; none of the choices. |
| --- | --- |

| 47. | Which one of the following is not an input in an MRP system? A.; planned-order releases. B.; bill of material. C.; master production schedule. D.; inventory levels and lead times. E.; All are inputs. |
| --- | --- |

| 48. | The MRP input stating which end items are to be produced, when they are needed, and what quantities are needed, is the: A.; master production schedule. B.; bill-of-materials. C.; inventory-records. D.; assembly-time chart. E.; net-requirements chart. |
| --- | --- |

| 49. | In an MRP master production schedule, the planning horizon is often separated into a series of times periods called: A.; pegs. B.; lead times. C.; stacked lead times. D.; time buckets. E.; none of the choices. |
| --- | --- |

| 50. | The input used in MRP that lists the assemblies, subassemblies, parts, and raw materials needed to produce one unit of finished product is the: A.; master production schedule. B.; bill-of-material. C.; inventory-records. D.; assembly-time chart. E.; net-requirements chart. |
| --- | --- |

| 51. | A hierarchical diagram that depicts the subassemblies and components that are needed to produce and/or assemble a product is called a(n): A.; assembly time chart. B.; product structure tree. C.; MRP II. D.; pegging. E.; none of the choices. |
| --- | --- |

| 52. | Ignoring the possibility of safety stock requirements, which one of the following most closely describes the calculation of net requirements for an item in MRP? A.; gross requirements - amount on-hand - scheduled receipts. B.; gross requirements - planned receipts. C.; gross requirements - order releases + amount on-hand. D.; gross requirements - planned order releases. E.; gross requirements - amount on-hand + planned order releases. |
| --- | --- |

| 53. | In MRP, "scheduled receipts" are: A.; identical to "planned-order receipts". B.; identical to "planned-order releases". C.; open orders already scheduled but not delivered yet. D.; "net requirements". E.; none of the choices. |
| --- | --- |

| 54. | In MRP, under lot-for-lot ordering, "planned-order receipts" are: A.; identical to "scheduled receipts". B.; identical to "planned-order releases". C.; open orders already scheduled but not delivered yet. D.; "gross requirements". E.; none of the choices. |
| --- | --- |

| 55. | The gross requirements of a given part or component are determined from: A.; net requirements + amount on-hand.. B.; gross requirements of the immediate parent(s). C.; planned orders of the end item. D.; net requirements of end item. E.; planned orders of the immediate parent(s). |
| --- | --- |

| 56. | Which of these products would be most likely to have dependent demand? A.; refrigerators. B.; automobile engines. C.; televisions. D.; brownies. E.; none of the choices. |
| --- | --- |

| 57. | Pegging is used primarily for which one of the following purposes? A.; identify parent items that have generated a certain set of requirements for an item. B.; identify the supplier of a given component. C.; identify the supplier of a given component's parent. D.; periodically update material requirements as conditions change. E.; obtain exception reports due to periodic changes in material requirements. |
| --- | --- |

| 58. | Periodic updating of an MRP system to account for all changes which have occurred within a given time interval is called: A.; pegging. B.; planned order releases. C.; net-change MRP. D.; regenerative MRP. E.; exception report. |
| --- | --- |

| 59. | An MRP system whose records are updated immediately after changes occur is referred to as a(n): A.; regenerative MRP. B.; batch-type MRP. C.; Plossl-Wright system. D.; net-change MRP. E.; gross-change MRP. |
| --- | --- |

| 60. | Which is true of a net-change MRP? A.; It is a batch-type system which is updated periodically.. B.; It generates exception reports.. C.; The basic production plan is modified to reflect changes as they occur.. D.; It is used to authorize the execution of planned orders.. E.; It indicates the amount and timing of future changes.. |
| --- | --- |

| 61. | Which of the following are primary reports provided by MRP systems? I) planned order releases II) immediate order releases III) demand history reports IV) changes to open orders A.; I and II. B.; I, II, III. C.; I, II, IV. D.; II, III, IV. E.; I, II, III, IV. |
| --- | --- |

| 62. | Under fixed-interval lot sizing, order sizes for component parts are determined based on which one of the following? A.; gross requirements for one period. B.; net requirements for some predetermined number of periods. C.; economic order quantity for one period. D.; gross requirements - net requirements for a predetermined number of periods. E.; net requirements - amount on-hand for a predetermined number of periods. |
| --- | --- |

| 63. | Which one of the following most closely describes the MRP approach that is used for components or subassemblies to compensate for variations in lead time? A.; pegging. B.; safety stock. C.; increased order sizes. D.; safety time. E.; low-level coding. |
| --- | --- |

| 64. | Which of the following are accurate statements concerning MPS, MRP and capacity requirements planning (CRP)? I) CRP is equivalent to a master production schedule (MPS) II) CRP is needed to determine if an MPS is feasible III) CRP is an input to MRP IV) planned-order releases from MRP are the inputs for CRP A.; I and II. B.; I, II, III. C.; II and IV. D.; II, III, IV. E.; I, II, III, IV. |
| --- | --- |

| 65. | Which action can managers take if capacity requirements planning (CRP) indicate one or more work stations are projected to be underloaded or overloaded? A.; Alternative routings. B.; Changing lot sizing. C.; Lot splitting. D.; Moving production forward or backward. E.; All of the choices are options. |
| --- | --- |

| 66. | A report that compares known and expected future capacity requirements with projected available capacity is a: A.; master production schedule. B.; bill of material report. C.; inventory status report. D.; MRP schedule. E.; load report. |
| --- | --- |

| 67. | What is the correct sequence to go from the broadest to the most detailed form of operations planning? A.; MRP » MPS » ERP. B.; ERP » MRP » MPS. C.; ERP » MPS » MRP. D.; MPS » ERP » MRP. E.; MPS » MRP » ERP. |
| --- | --- |

| 68. | Budget overruns when implementing ERP systems are most likely to be caused by: A.; High equipment costs. B.; High training costs. C.; High capital interest charges. D.; Employee resistance to change. E.; All of the above choices. |
| --- | --- |

| 69. | If 17 Ps are needed, and no on-hand inventory exists, how many Cs will be needed? A.; 8. B.; 16. C.; 136. D.; 204. E.; 272. |
| --- | --- |

| 70. | If 17 Ps are needed, and on-hand inventory consists of 10 As, 15 Bs, 20 Cs, 12 Ms, and 5 Ns, how many Cs are needed? A.; 48. B.; 144. C.; 192. D.; 212. E.; 272. |
| --- | --- |

| 71. | If 40 Ps are needed, and on-hand inventory consists of 15 Ps and 10 each of all other components and subassemblies, how many Cs are needed? A.; 340. B.; 350. C.; 380. D.; 400. E.; 590. |
| --- | --- |

|  | Great Product, Ltd. has just received an order for 100 Whozats, which must be ready for delivery at the start of week six. An MRP planner has prepared the following table showing product structure, lead times (orders are lot-for-lot), and quantities on hand: |
| --- | --- |

| 72. | What is the net requirement for Whatzats to fill this order? A.; 100. B.; 140. C.; 150. D.; 180. E.; 200. |
| --- | --- |

| 73. | How many Wherezats should be ordered? A.; 400. B.; 450. C.; 500. D.; 550. E.; 600. |
| --- | --- |

| 74. | When should an order for Wherezats be released? A.; at the start of week 2. B.; at the start of week 3. C.; at the start of week 4. D.; at the start of week 5. E.; at the start of week 6. |
| --- | --- |

|  | Given an order cost of $100, a per period holding cost of $.50, and the following requirements schedule: |
| --- | --- |

| 75. | What is the economic part-period (EPP) for ordering this item? A.; 2. B.; 50. C.; 100. D.; 150. E.; 200. |
| --- | --- |

| 76. | What is the part-period method lot size for the order for the first period? A.; 120. B.; 200. C.; 280. D.; 350. E.; 450. |
| --- | --- |

| 77. | For which period will the second order be submitted using the part period lot sizing method? A.; period 2. B.; period 3. C.; period 4. D.; period 5. E.; period 6. |
| --- | --- |

| 78. | What is the lot size for the second order using the part-period method? A.; 80. B.; 150. C.; 170. D.; 250. E.; 230. |
| --- | --- |

| 79. | If the firm is using a fixed period lot size of two periods, what is the order size for the first order? A.; 120. B.; 200. C.; 280. D.; 160. E.; 150. |
| --- | --- |

| 80. | Using the product-tree shown, determine the following: (i) the quantity of component K that will be needed to assemble 80 units of P, assuming no on-hand inventory exists. (ii) the quantity of component K needed to assemble 80 units of P, given on-hand inventory of 30 A's, 50 B's and 20 C's. |
| --- | --- |

| 81. | The following is a list of components required to produce one unit of end item P: P: 2 A's, 3 B's, 3 C's A: 5 M's, 2 R's B: 1 D, 3 N's. C: 1 T, 4 N's M: 1 N Determine the number of N's that will be needed to make 60 P's in each of these cases: (i) There are currently 10 P's on hand. (ii) On-hand inventory consists of 15 P's, 10A's, 20 B's, 10 C's, 100 N's, 300 T's, and 200 M's. |
| --- | --- |

| 82. | Given the following information, construct a product tree diagram and develop a material requirements plan that will yield 400 units of product P at the start of week 7. Use the following form: |
| --- | --- |

| 83. | Given the following data, construct a material requirements plan which will result in 100 units of Parent #1 (P1) at the beginning of week 6, and 200 units of Parent #2 (P2) at the beginning of week 8: |
| --- | --- |

| 84. | Develop a material requirements plan for end item P and its components, given the tree below. Assume that all lead times are one week, and that lot-for-lot ordering is used except for item F, which is ordered in multiples of 400 units. One hundred units of P are needed at the start of each of weeks 4 and 8. Beginning inventories are: 20 P, 100 A, and 200 F. Scheduled receipts are: 800 F at the start of week 1. |
| --- | --- |

| 85. | The following table lists the components needed to assemble an end item, their lead-times, and quantities on-hand. (i) If 50 units of the end item are to be assembled, how many additional units of S will be needed? All parts can be ordered using lot-for lot ordering, except M, which must be ordered in multiples of 40 units. [Hint: You don't need to do an MRP plan.] (ii) An order for the end item is scheduled to be shipped at the start of week 9. When is the latest week that the initial component of this order can be started and still be ready to ship? [Hint: You don't need to do an MRP plan.] |
| --- | --- |

Chapter 14 Material Requirements Planning and Enterprise Resource Planning Key

| 1. | MRP usually involves inventory items that have independent demand. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 14 #1

| 2. | Dependent demand is derived demand. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 14 #2

| 3. | Independent demand tends to be lumpier than dependent demand. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 14 #3

| 4. | Lumpy demand for components results primarily from the periodic scheduling of batch production. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 14 #4

| 5. | Dependent demand tends to be intermittent, whereas independent demand tends to be more stable. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 14 #5

| 6. | The three basic inputs of MRP are: 1) the bill of material; 2) the inventory level, lead times and open orders; and 3) planned order releases. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 14 #6

| 7. | The master production schedule (MPS) states which end items are to be produced. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #7

| 8. | The master production schedule states (MPS) which product components are to be produced. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 14 #8

| 9. | The planning horizon for the master production schedule (MPS) should cover the stacked or cumulative lead time necessary to produce the end items. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #9

| 10. | A phantom bill is a special type of bill of material that is a combination of the bill of materials for a number of subassemblies. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 14 #10

| 11. | The MRP provides a list of the individual parts that will be converted to finished goods. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 14 #11

| 12. | The bill of material indicates how much material will be needed for a given master production schedule. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 2 Stevenson - Chapter 14 #12

| 13. | A bill of material contains a listing of all the raw materials, parts, and subassemblies needed to produce one unit of an end item. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #13

| 14. | The bill of material contains information on lead times. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 14 #14

| 15. | MRP requires information on inventory level, lead times and purchase orders for each item in stock. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #15

| 16. | An assembly-time chart indicates gross and net requirements. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #16

| 17. | For a given component in an MRP plan, under lot-for-lot ordering, planned order sizes are equal to net requirements. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #17

| 18. | The gross requirements at one level of an MRP plan determine the gross requirements at the next lower level. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #18

| 19. | Gross requirements for components are equal to the gross requirements of their immediate parent multiplied by the quantity per parent. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #19

| 20. | The term pegging refers to identifying the parent items that have generated a given set of material requirements for a part or subassembly. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #20

| 21. | A net-change MRP is one that is updated periodically. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 14 #21

| 22. | Processing costs are less in a regenerative MRP than in a net-change MRP. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #22

| 23. | A regenerative MRP is one that is updated continuously. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #23

| 24. | One of the primary reports of MRP concerns changes to open orders. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #24

| 25. | Safety time is sometimes used in MRP rather than safety stock. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #25

| 26. | Lot-for-lot ordering in MRP provides coverage for some predetermined number of periods. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #26

| 27. | The part-period method for lot sizing attempts to balance setup and holding costs. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #27

| 28. | The part-period method for lot sizing breaks the time periods into smaller units of time to create daily or hourly production schedules. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 14 #28

| 29. | Fixed-interval ordering in MRP places orders that cover a predetermined number of periods. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 14 #29

| 30. | Load reports show capacity requirements for departments or work centres. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #30

| 31. | MRP II involves a broader approach to production planning and scheduling than MRP. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 14 #31

| 32. | MRP II is simply an improved version of software for MRP. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 14 #32

| 33. | Load reports of capacity may be misleading as they do not consider scheduling and queuing delays. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #33

| 34. | Load reports compare expected capacity requirements with planned capacity requirements. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #34

| 35. | ERP systems are more popular than MRP because they are less complex to use. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 14 #35

| 36. | ERP involves software that is used to manage and coordinate all the resources, information, and functions of an organization from a shared database. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 5 Stevenson - Chapter 14 #36

| 37. | Typically the total cost of ownership (TCO) of ERP systems is clearly detailed and certain at the time a company makes the investment to acquire a system. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 14 #37

| 38. | ERP packages are intended to fit established ways of doing business, but some modifications to business processes or the ERP software are typically required. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 5 Stevenson - Chapter 14 #38

| 39. | Any company should be able to use an ERP system. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 5 Stevenson - Chapter 14 #39

| 40. | ERP implementations that fully transform a company's operations typically take from one to three years. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 14 #40

| 41. | ERP automates the tasks involved in performing a business process, such as order fulfilment and financial reporting. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 14 #41

| 42. | Independent demand tends to be __________, while dependent demand tends to be _________. A.; stable, unstable. B.; unstable, stable. C.; stable, lumpy. D.; smooth, sawtooth. E.; sawtooth, smooth. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 14 #42

| 43. | Which of the following most closely describes dependent demand? A.; demand generated by suppliers. B.; estimates of demand using regression analysis of independent variables. C.; derived demand. D.; demands placed on suppliers by their customers. E.; net material requirements. |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 14 #43

| 44. | A process designed to handle ordering and scheduling of dependent-demand components is: A.; economic part-period (EPP). B.; lot-for-lot (LFL). C.; economic order quantity (EOQ). D.; material requirements planning (MRP). E.; economic run size (ERS). |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 14 #44

| 45. | Which of these items would be most likely to have dependent demand? A.; electric starters. B.; toy trains. C.; flowers. D.; chocolate chip cookies. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 14 #45

| 46. | The development and application of MRP is primarily associated with upon two developments: (1) managers recognizing the need to differentiate between independent and dependent demand, and (2) __________. A.; computers. B.; development of the EOQ model. C.; inventory control systems. D.; all of the choices. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 14 #46

| 47. | Which one of the following is not an input in an MRP system? A.; planned-order releases. B.; bill of material. C.; master production schedule. D.; inventory levels and lead times. E.; All are inputs. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #47

| 48. | The MRP input stating which end items are to be produced, when they are needed, and what quantities are needed, is the: A.; master production schedule. B.; bill-of-materials. C.; inventory-records. D.; assembly-time chart. E.; net-requirements chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #48

| 49. | In an MRP master production schedule, the planning horizon is often separated into a series of times periods called: A.; pegs. B.; lead times. C.; stacked lead times. D.; time buckets. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 14 #49

| 50. | The input used in MRP that lists the assemblies, subassemblies, parts, and raw materials needed to produce one unit of finished product is the: A.; master production schedule. B.; bill-of-material. C.; inventory-records. D.; assembly-time chart. E.; net-requirements chart. |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 14 #50

| 51. | A hierarchical diagram that depicts the subassemblies and components that are needed to produce and/or assemble a product is called a(n): A.; assembly time chart. B.; product structure tree. C.; MRP II. D.; pegging. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 14 #51

| 52. | Ignoring the possibility of safety stock requirements, which one of the following most closely describes the calculation of net requirements for an item in MRP? A.; gross requirements - amount on-hand - scheduled receipts. B.; gross requirements - planned receipts. C.; gross requirements - order releases + amount on-hand. D.; gross requirements - planned order releases. E.; gross requirements - amount on-hand + planned order releases. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #52

| 53. | In MRP, "scheduled receipts" are: A.; identical to "planned-order receipts". B.; identical to "planned-order releases". C.; open orders already scheduled but not delivered yet. D.; "net requirements". E.; none of the choices. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #53

| 54. | In MRP, under lot-for-lot ordering, "planned-order receipts" are: A.; identical to "scheduled receipts". B.; identical to "planned-order releases". C.; open orders already scheduled but not delivered yet. D.; "gross requirements". E.; none of the choices. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #54

| 55. | The gross requirements of a given part or component are determined from: A.; net requirements + amount on-hand.. B.; gross requirements of the immediate parent(s). C.; planned orders of the end item. D.; net requirements of end item. E.; planned orders of the immediate parent(s). |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #55

| 56. | Which of these products would be most likely to have dependent demand? A.; refrigerators. B.; automobile engines. C.; televisions. D.; brownies. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 14 #56

| 57. | Pegging is used primarily for which one of the following purposes? A.; identify parent items that have generated a certain set of requirements for an item. B.; identify the supplier of a given component. C.; identify the supplier of a given component's parent. D.; periodically update material requirements as conditions change. E.; obtain exception reports due to periodic changes in material requirements. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #57

| 58. | Periodic updating of an MRP system to account for all changes which have occurred within a given time interval is called: A.; pegging. B.; planned order releases. C.; net-change MRP. D.; regenerative MRP. E.; exception report. |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 14 #58

| 59. | An MRP system whose records are updated immediately after changes occur is referred to as a(n): A.; regenerative MRP. B.; batch-type MRP. C.; Plossl-Wright system. D.; net-change MRP. E.; gross-change MRP. |
| --- | --- |

Difficulty: Easy Learning Objective: 3 Stevenson - Chapter 14 #59

| 60. | Which is true of a net-change MRP? A.; It is a batch-type system which is updated periodically.. B.; It generates exception reports.. C.; The basic production plan is modified to reflect changes as they occur.. D.; It is used to authorize the execution of planned orders.. E.; It indicates the amount and timing of future changes.. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #60

| 61. | Which of the following are primary reports provided by MRP systems? I) planned order releases II) immediate order releases III) demand history reports IV) changes to open orders A.; I and II. B.; I, II, III. C.; I, II, IV. D.; II, III, IV. E.; I, II, III, IV. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #61

| 62. | Under fixed-interval lot sizing, order sizes for component parts are determined based on which one of the following? A.; gross requirements for one period. B.; net requirements for some predetermined number of periods. C.; economic order quantity for one period. D.; gross requirements - net requirements for a predetermined number of periods. E.; net requirements - amount on-hand for a predetermined number of periods. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #62

| 63. | Which one of the following most closely describes the MRP approach that is used for components or subassemblies to compensate for variations in lead time? A.; pegging. B.; safety stock. C.; increased order sizes. D.; safety time. E.; low-level coding. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 14 #63

| 64. | Which of the following are accurate statements concerning MPS, MRP and capacity requirements planning (CRP)? I) CRP is equivalent to a master production schedule (MPS) II) CRP is needed to determine if an MPS is feasible III) CRP is an input to MRP IV) planned-order releases from MRP are the inputs for CRP A.; I and II. B.; I, II, III. C.; II and IV. D.; II, III, IV. E.; I, II, III, IV. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 14 #64

| 65. | Which action can managers take if capacity requirements planning (CRP) indicate one or more work stations are projected to be underloaded or overloaded? A.; Alternative routings. B.; Changing lot sizing. C.; Lot splitting. D.; Moving production forward or backward. E.; All of the choices are options. |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 14 #65

| 66. | A report that compares known and expected future capacity requirements with projected available capacity is a: A.; master production schedule. B.; bill of material report. C.; inventory status report. D.; MRP schedule. E.; load report. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #66

| 67. | What is the correct sequence to go from the broadest to the most detailed form of operations planning? A.; MRP » MPS » ERP. B.; ERP » MRP » MPS. C.; ERP » MPS » MRP. D.; MPS » ERP » MRP. E.; MPS » MRP » ERP. |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 14 #67

| 68. | Budget overruns when implementing ERP systems are most likely to be caused by: A.; High equipment costs. B.; High training costs. C.; High capital interest charges. D.; Employee resistance to change. E.; All of the above choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 14 #68

Stevenson - Chapter 14

| 69. | If 17 Ps are needed, and no on-hand inventory exists, how many Cs will be needed? A.; 8. B.; 16. C.; 136. D.; 204. E.; 272. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #69

| 70. | If 17 Ps are needed, and on-hand inventory consists of 10 As, 15 Bs, 20 Cs, 12 Ms, and 5 Ns, how many Cs are needed? A.; 48. B.; 144. C.; 192. D.; 212. E.; 272. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #70

| 71. | If 40 Ps are needed, and on-hand inventory consists of 15 Ps and 10 each of all other components and subassemblies, how many Cs are needed? A.; 340. B.; 350. C.; 380. D.; 400. E.; 590. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #71

|  | Great Product, Ltd. has just received an order for 100 Whozats, which must be ready for delivery at the start of week six. An MRP planner has prepared the following table showing product structure, lead times (orders are lot-for-lot), and quantities on hand: |
| --- | --- |

Stevenson - Chapter 14

| 72. | What is the net requirement for Whatzats to fill this order? A.; 100. B.; 140. C.; 150. D.; 180. E.; 200. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #72

| 73. | How many Wherezats should be ordered? A.; 400. B.; 450. C.; 500. D.; 550. E.; 600. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #73

| 74. | When should an order for Wherezats be released? A.; at the start of week 2. B.; at the start of week 3. C.; at the start of week 4. D.; at the start of week 5. E.; at the start of week 6. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #74

|  | Given an order cost of $100, a per period holding cost of $.50, and the following requirements schedule: |
| --- | --- |

Stevenson - Chapter 14

| 75. | What is the economic part-period (EPP) for ordering this item? A.; 2. B.; 50. C.; 100. D.; 150. E.; 200. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #75

| 76. | What is the part-period method lot size for the order for the first period? A.; 120. B.; 200. C.; 280. D.; 350. E.; 450. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #76

| 77. | For which period will the second order be submitted using the part period lot sizing method? A.; period 2. B.; period 3. C.; period 4. D.; period 5. E.; period 6. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #77

| 78. | What is the lot size for the second order using the part-period method? A.; 80. B.; 150. C.; 170. D.; 250. E.; 230. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #78

| 79. | If the firm is using a fixed period lot size of two periods, what is the order size for the first order? A.; 120. B.; 200. C.; 280. D.; 160. E.; 150. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 14 #79

| 80. | Using the product-tree shown, determine the following: (i) the quantity of component K that will be needed to assemble 80 units of P, assuming no on-hand inventory exists. (ii) the quantity of component K needed to assemble 80 units of P, given on-hand inventory of 30 A's, 50 B's and 20 C's. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #80

| 81. | The following is a list of components required to produce one unit of end item P: P: 2 A's, 3 B's, 3 C's A: 5 M's, 2 R's B: 1 D, 3 N's. C: 1 T, 4 N's M: 1 N Determine the number of N's that will be needed to make 60 P's in each of these cases: (i) There are currently 10 P's on hand. (ii) On-hand inventory consists of 15 P's, 10A's, 20 B's, 10 C's, 100 N's, 300 T's, and 200 M's. The product-structure tree: |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #81

| 82. | Given the following information, construct a product tree diagram and develop a material requirements plan that will yield 400 units of product P at the start of week 7. Use the following form: Product structure: |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #82

| 83. | Given the following data, construct a material requirements plan which will result in 100 units of Parent #1 (P1) at the beginning of week 6, and 200 units of Parent #2 (P2) at the beginning of week 8: Product structures: |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #83

| 84. | Develop a material requirements plan for end item P and its components, given the tree below. Assume that all lead times are one week, and that lot-for-lot ordering is used except for item F, which is ordered in multiples of 400 units. One hundred units of P are needed at the start of each of weeks 4 and 8. Beginning inventories are: 20 P, 100 A, and 200 F. Scheduled receipts are: 800 F at the start of week 1. |
| --- | --- |

Difficulty: Hard Learning Objective: 3 Stevenson - Chapter 14 #84

| 85. | The following table lists the components needed to assemble an end item, their lead-times, and quantities on-hand. (i) If 50 units of the end item are to be assembled, how many additional units of S will be needed? All parts can be ordered using lot-for lot ordering, except M, which must be ordered in multiples of 40 units. [Hint: You don't need to do an MRP plan.] (ii) An order for the end item is scheduled to be shipped at the start of week 9. When is the latest week that the initial component of this order can be started and still be ready to ship? [Hint: You don't need to do an MRP plan.] |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 14 #85

Chapter 14 Material Requirements Planning and Enterprise Resource Planning Summary

| Category | # of Questions |
| --- | --- |
| Difficulty: Easy | 21 |
| Difficulty: Hard | 18 |
| Difficulty: Medium | 46 |
| Learning Objective: 1 | 11 |
| Learning Objective: 2 | 15 |
| Learning Objective: 3 | 30 |
| Learning Objective: 4 | 20 |
| Learning Objective: 5 | 9 |
| Stevenson - Chapter 14 | 88 |