Chapter 15 Just-in-Time and Lean Production

| 1. | JIT refers to a production system in which goods arrive for processing just as the preceding batch is completed. True False |
| --- | --- |

| 2. | In a broad sense, JIT is identical to lean production. True False |
| --- | --- |

| 3. | In a JIT environment, anything superfluous is viewed as waste. True False |
| --- | --- |

| 4. | JIT was developed at Ford Motor Company by Taiichi Ohno. True False |
| --- | --- |

| 5. | The four building blocks of JIT are: product design, process design, personnel/organizational elements, and planning and controls. True False |
| --- | --- |

| 6. | Quality and flexible machinery are requirements for successful JIT and lean systems. True False |
| --- | --- |

| 7. | A major difference between kanban and MRP in scheduling the products to be built is that kanban is primarily a manual system. True False |
| --- | --- |

| 8. | Waiting time is not considered a waste in the JIT philosophy. True False |
| --- | --- |

| 9. | The ultimate goal of a lean system is to achieve balanced, rapid flow of materials through the system. True False |
| --- | --- |

| 10. | Fixed capacity and set schedules are goals of JIT and lean systems. True False |
| --- | --- |

| 11. | A benefit of small lot sizes in JIT systems is each product is produced less frequently. True False |
| --- | --- |

| 12. | In the JIT philosophy, the smaller the lot size, the better. True False |
| --- | --- |

| 13. | A benefit of JIT is the flexibility to handle a mix of products within a short time frame. True False |
| --- | --- |

| 14. | The small lot-sizing policy works well in a situation where both holding costs and setup costs are high. True False |
| --- | --- |

| 15. | A JIT system uses long production runs as this reduces setup time required. True False |
| --- | --- |

| 16. | The use of small lot sizes in JIT is in conflict with the EOQ approach. True False |
| --- | --- |

| 17. | Reductions in setup time are necessary to make a JIT system feasible. True False |
| --- | --- |

| 18. | Setup time and cost can often be reduced by the use of group technology. True False |
| --- | --- |

| 19. | Manufacturing cells are commonly used in JIT systems True False |
| --- | --- |

| 20. | "Group technology" can be used to reduce setup cost and time in a JIT conversion. True False |
| --- | --- |

| 21. | Manufacturing cells in JIT systems process similar parts for a product. True False |
| --- | --- |

| 22. | "Autonomation" indicates that the firm has reduced the number of manual operations by increasing the use of robotics. True False |
| --- | --- |

| 23. | The very first step of JIT implementation is to cut inventories everywhere in the system. True False |
| --- | --- |

| 24. | In the JIT approach, inventories are pared down gradually, instead of reducing inventories as rapidly as possible. True False |
| --- | --- |

| 25. | The JIT philosophy recognizes that buffer inventory is necessary. True False |
| --- | --- |

| 26. | A fundamental tenet of the JIT philosophy is that workers are assets, not a cost element. True False |
| --- | --- |

| 27. | A responsibility of the JIT cross-trained worker is to help other workers when bottlenecks occur. True False |
| --- | --- |

| 28. | JIT/lean production systems need to be directed by strong leadership roles based on top-down communication to ensure production schedules are implemented accurately. True False |
| --- | --- |

| 29. | It is mainly manufacturing personnel that are required to support a JIT system. True False |
| --- | --- |

| 30. | A JIT system is compatible with a chase demand strategy. True False |
| --- | --- |

| 31. | The ideal scheduling for JIT/lean operations follows a push system of planning. True False |
| --- | --- |

| 32. | In a "pull system," a pile of inventory in front of a workstation indicates problems at that workstation. True False |
| --- | --- |

| 33. | In a "push system," a pile of inventory in front of a workstation indicates problems at that workstation. True False |
| --- | --- |

| 34. | The control of the flow of work in JIT systems can be characterized as a backward communication approach. True False |
| --- | --- |

| 35. | A kanban card is used to signal that work is ready to be moved to the next station. True False |
| --- | --- |

| 36. | JIT systems typically require that suppliers be able to provide large lots at fairly regular intervals. True False |
| --- | --- |

| 37. | JIT purchasing requires frequent contract bidding by multiple sources to ensure the buyer of competitive prices. True False |
| --- | --- |

| 38. | A JIT/lean system is not effective when the industry consists of a tiered supplier network. True False |
| --- | --- |

| 39. | For JIT/lean companies, certification of suppliers using a system such as ISO 9001 provides a cost effective an alternative to having to develop close supplier relationships. True False |
| --- | --- |

| 40. | Kanban is a system of lights used to signal problems or slowdowns. True False |
| --- | --- |

| 41. | The purpose of an andon is to provide an immediate visual signal when and where problems are occurring. True False |
| --- | --- |

| 42. | "Preventive maintenance" is extra-special maintenance done after a breakdown has occurred to prevent any further breakdowns. True False |
| --- | --- |

| 43. | For JIT/lean companies the responsibility for preventive maintenance and repairs is often assigned to workers that operate production equipment. True False |
| --- | --- |

| 44. | The sole purpose of housekeeping in JIT/lean operations is to enhance safety in the workplace. True False |
| --- | --- |

| 45. | In a JIT system, long-term relationships with suppliers are unusual. True False |
| --- | --- |

| 46. | Which of the following is not an element of the building blocks of JIT/lean operations? A.; high levels of quality in products and processes. B.; highly specialized process systems. C.; a program on continuous improvement. D.; level loading. E.; close supplier relationships. |
| --- | --- |

| 47. | In contrast to MRP systems, JIT systems A.; are more complex. B.; require extensive and detailed shop floor controls. C.; rely on visual or audible signals to trigger production. D.; are more expensive to develop. E.; all of the choices are true. |
| --- | --- |

| 48. | Which one of the following is not one of the foundational building blocks of JIT? A.; product design. B.; process design. C.; personnel/organizational elements. D.; planning and controls. E.; kanban. |
| --- | --- |

| 49. | The ultimate goal of lean systems is to have: A.; no in-process inventories. B.; cross-trained workers. C.; a balanced, rapid flow of materials and products through the system. D.; reduced setup times. E.; none of the choices. |
| --- | --- |

| 50. | The ultimate objective in a JIT system is: A.; low inventory. B.; high quality. C.; minimal production cost. D.; a balanced, rapid flow of materials and products through the system. E.; all of the choices. |
| --- | --- |

| 51. | Which of the following is not a goal of JIT/lean production? A.; System flexibility. B.; Elimination of waste. C.; Continuous improvement. D.; Preventative maintenance. E.; Elimination of disruptions. |
| --- | --- |

| 52. | Continuous improvement (Kaizen) is based on all the tenets except: A.; Involvement of everyone in the organization. B.; Applied throughput the organization. C.; Learning by doing. D.; Direct observation and data collection. E.; Rapid, major breakthroughs in improvement. |
| --- | --- |

| 53. | Which one of the following is not affiliated with product design to support JIT/lean operations? A.; concurrent engineering. B.; design for manufacturing and assembly. C.; value analysis of product functionality. D.; material requirements planning (MRP). E.; quality function deployment (QFD) to capture "voice" of customers. |
| --- | --- |

| 54. | Which of the following is not a benefit of small lot sizes in JIT systems? A.; In-process inventory is considerably less.. B.; Each product is produced less frequently.. C.; Holding costs are reduced.. D.; There is less clutter in the workplace.. E.; Inspection and rework costs are less.. |
| --- | --- |

| 55. | Which of the following is a benefit of small lot sizes in JIT systems? A.; Quicker response to changing customer demands.. B.; Reduced setup costs.. C.; Inventory is eliminated.. D.; EOQ models are not required.. E.; All of the choices are benefits.. |
| --- | --- |

| 56. | In the JIT philosophy, the ideal lot size is: A.; the economic order quantity. B.; the economic run size. C.; one unit. D.; the capacity of the standard container. E.; N = (DT(1+X))/C. |
| --- | --- |

| 57. | Which of the following does not contribute to reduced setup time and cost? A.; standardized setup tools. B.; standardized setup equipment. C.; specialized setup procedures for each product. D.; use of multipurpose equipment or attachments. E.; use of group technology. |
| --- | --- |

| 58. | Which of the following is not a method for reducing setup time? A.; streamline the setup process. B.; separate external (off-line) and internal (cannot be done off-line) setup activities. C.; multipurpose equipment or attachments. D.; group technology so parts requiring similar setup procedures are grouped. E.; all of the choices reduce setup time. |
| --- | --- |

| 59. | The technique intended to increase communications among functions, reducing the need for changes and speed up the design process is: A.; design for manufacturing and assembly (DFM and DFA). B.; Quality function deployment (QFD). C.; Concurrent engineering. D.; Group technology. E.; Takt time reduction. |
| --- | --- |

| 60. | A set of tools and methods for reducing setup time developed by Shigeo Shingo based on using presses and dies is called: A.; Shingo Prize. B.; DFMA. C.; SMED. D.; group technology. E.; none of the choices. |
| --- | --- |

| 61. | Which of the following is not associated with converting to cellular layouts? A.; determining a family of products to group together. B.; designing the process for the product family. C.; balancing the cell. D.; reducing the variety of end products produced. E.; determining the capacity of the cell. |
| --- | --- |

| 62. | The term used in Japan that refers to quality at the source, meaning stopping to fix problems to avoid passing defective products on to the next workstation is: A.; poka-yoke. B.; autonomation. C.; jidoka. D.; andon. E.; kanban. |
| --- | --- |

| 63. | A term that refers to the automatic detection of defects is: A.; poka-yoke. B.; autonomation. C.; jidoka. D.; andon. E.; kanban. |
| --- | --- |

| 64. | A basic requirement for operating with low inventories in JIT systems is: A.; Inventory space must be saved.. B.; Inventory investment must be saved.. C.; Major problems must be uncovered.. D.; Major problems must have been solved.. E.; Inventories must be reduced rapidly.. |
| --- | --- |

| 65. | JIT philosophy tries to eliminate work-in-process inventory because: A.; of the savings in capital requirements.. B.; inventory covers up recurring problems.. C.; it free up space for kanban storage bins.. D.; to increase dependency on suppliers.. E.; all of the choices are true.. |
| --- | --- |

| 66. | Which of the following is not an important element of personnel and organization for JIT? A.; Cross-trained workers. B.; Authoritative leaders. C.; Support from non-manufacturing personnel. D.; Participation in Kaizen teams and suggestion systems. E.; all of the choices are elements. |
| --- | --- |

| 67. | Which of the following is not true of worker/management roles and relations for JIT? A.; Stress on workers is reduced by requiring less of them.. B.; Workers are considered a key asset of a JIT system.. C.; Two way communication between workers and management is encouraged.. D.; Employees are empowered in selecting and using methods.. E.; all of the choices are true. |
| --- | --- |

| 68. | Which of the following are potentially negative aspects of cross-functionally trained workers? I. cost and time for highly skilled workers II. worker resistance due to perceived loss of job security III. higher stress levels from added responsibility and a continued push to improve A.; I and III. B.; I and II. C.; II only. D.; II and III. E.; I, II, and III. |
| --- | --- |

| 69. | Which of the following are benefits of level capacity loading? I. minimizes work-in-process inventory II. minimizes time to setup for different products III. maintains flexibility to meet customer demand A.; I. B.; II. C.; III. D.; I and III. E.; I, II, and III. |
| --- | --- |

| 70. | Material requirements planning (MRP) is an example of a: A.; push system. B.; pull system. C.; forward communications approach. D.; inventory accumulation approach. E.; none of the choices. |
| --- | --- |

| 71. | The control of the flow of work in JIT systems is best characterized as a: A.; push system. B.; pull system. C.; forward communications approach. D.; inventory accumulation approach. E.; none of the choices. |
| --- | --- |

| 72. | A kanban card is used to signal that: A.; work is needed from the preceding station. B.; work is ready to be moved to the next station. C.; a worker has run out of parts. D.; a machine has broken down. E.; a machine is ready for preventive maintenance. |
| --- | --- |

| 73. | When using kanban cards, long trip times may require: A.; more containers. B.; fewer containers. C.; more replenishment parts in inventory. D.; additional cross training. E.; none of the choices. |
| --- | --- |

| 74. | With regard to suppliers, JIT systems typically require: A.; delivery of large lots at regular intervals. B.; buyer inspection of goods and materials. C.; multiple sources from which to purchase. D.; long-term relationships and commitments. E.; the lowest price possible. |
| --- | --- |

| 75. | With regard to suppliers, JIT systems typically involve: A.; delivery of large lots on short notice. B.; the highest quality at the lowest price. C.; long-term relationships. D.; multiple suppliers. E.; all of the choices. |
| --- | --- |

| 76. | Which of the following is characteristic of the JIT philosophy? A.; Inventories are a valued asset.. B.; Few, large deliveries of orders from suppliers.. C.; Suppliers become partners.. D.; Queues of WIP inventory are a necessary investment.. E.; all of the choices. |
| --- | --- |

| 77. | A system of lights used at each workstation to signal problems or slowdowns is: A.; jidoka. B.; automation. C.; andon. D.; pull system. E.; kanban. |
| --- | --- |

| 78. | An extended version of process flow diagrams that includes information flows and the activity times refers to: A.; value stream mapping. B.; kanban control. C.; the 5S principles. D.; push analysis. E.; pull analysis. |
| --- | --- |

| 79. | Which of the following is not characteristic of preventive maintenance in JIT systems? A.; maintaining equipment in good operating condition. B.; replacing parts before they fail. C.; workers maintaining their own equipment. D.; eliminating supplies of spare parts. E.; all of the choices are true. |
| --- | --- |

| 80. | Which of the following is not affiliated with the implementation of JIT/lean systems? A.; Inventories are reduced.. B.; Space requirements are reduced.. C.; Quality is increased.. D.; Fewer orders from suppliers need to be processed.. E.; Closer relationships with suppliers.. |
| --- | --- |

| 81. | Which of the following contributes to the competitive advantage enjoyed by firms using JIT/lean production? I. lower processing costs II. fewer employees III. fewer defectives IV. greater flexibility. A.; I, II, and III. B.; II, III, and IV. C.; I and IV. D.; I, III, and IV. E.; I, II, III, and IV. |
| --- | --- |

| 82. | The All Seasonings Company uses 3,200 glass jars at one of its jar-filling workstations each eight hours of production. The waiting time for a standard container, which holds 90 jars, averages 45 minutes. If management uses a safety factor of twenty percent, how many containers should be used? |
| --- | --- |

| 83. | The Four Star Publishing Company uses sixty reams of paper each hour at one of its high speed printing stations. The waiting time for a standard container, which holds 15 reams of paper, averages forty minutes. If management has decided to use three containers, what safety factor was applied? |
| --- | --- |

| 84. | A production system uses kanban cards to control production and movement of parts. One work centre uses an average of 40 pieces per hour of a certain part. Standard containers hold 10 parts. The waiting time for parts containers is about 36 minutes. Management has assigned a safety factor of .20 to this work centre. How many containers should be used to support this operation? |
| --- | --- |

| 85. | A production cell uses 240 pounds of plastic resin each 8-hour day. Resin is transported in drums that hold 100 pounds each. The material is obtained from a nearby supplier, and has a 12-hour waiting time. A safety factor of .15 has been assigned to this cell. What is the optimum number of containers to support this operation? |
| --- | --- |

Chapter 15 Just-in-Time and Lean Production Key

| 1. | JIT refers to a production system in which goods arrive for processing just as the preceding batch is completed. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 15 #1

| 2. | In a broad sense, JIT is identical to lean production. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 15 #2

| 3. | In a JIT environment, anything superfluous is viewed as waste. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 15 #3

| 4. | JIT was developed at Ford Motor Company by Taiichi Ohno. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 15 #4

| 5. | The four building blocks of JIT are: product design, process design, personnel/organizational elements, and planning and controls. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 1 Stevenson - Chapter 15 #5

| 6. | Quality and flexible machinery are requirements for successful JIT and lean systems. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 15 #6

| 7. | A major difference between kanban and MRP in scheduling the products to be built is that kanban is primarily a manual system. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #7

| 8. | Waiting time is not considered a waste in the JIT philosophy. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 15 #8

| 9. | The ultimate goal of a lean system is to achieve balanced, rapid flow of materials through the system. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 15 #9

| 10. | Fixed capacity and set schedules are goals of JIT and lean systems. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 2 Stevenson - Chapter 15 #10

| 11. | A benefit of small lot sizes in JIT systems is each product is produced less frequently. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #11

| 12. | In the JIT philosophy, the smaller the lot size, the better. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #12

| 13. | A benefit of JIT is the flexibility to handle a mix of products within a short time frame. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #13

| 14. | The small lot-sizing policy works well in a situation where both holding costs and setup costs are high. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #14

| 15. | A JIT system uses long production runs as this reduces setup time required. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #15

| 16. | The use of small lot sizes in JIT is in conflict with the EOQ approach. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #16

| 17. | Reductions in setup time are necessary to make a JIT system feasible. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #17

| 18. | Setup time and cost can often be reduced by the use of group technology. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #18

| 19. | Manufacturing cells are commonly used in JIT systems TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #19

| 20. | "Group technology" can be used to reduce setup cost and time in a JIT conversion. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #20

| 21. | Manufacturing cells in JIT systems process similar parts for a product. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #21

| 22. | "Autonomation" indicates that the firm has reduced the number of manual operations by increasing the use of robotics. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #22

| 23. | The very first step of JIT implementation is to cut inventories everywhere in the system. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #23

| 24. | In the JIT approach, inventories are pared down gradually, instead of reducing inventories as rapidly as possible. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #24

| 25. | The JIT philosophy recognizes that buffer inventory is necessary. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #25

| 26. | A fundamental tenet of the JIT philosophy is that workers are assets, not a cost element. TRUE |
| --- | --- |

Difficulty: Easy Learning Objective: 5 Stevenson - Chapter 15 #26

| 27. | A responsibility of the JIT cross-trained worker is to help other workers when bottlenecks occur. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 15 #27

| 28. | JIT/lean production systems need to be directed by strong leadership roles based on top-down communication to ensure production schedules are implemented accurately. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 15 #28

| 29. | It is mainly manufacturing personnel that are required to support a JIT system. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 5 Stevenson - Chapter 15 #29

| 30. | A JIT system is compatible with a chase demand strategy. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #30

| 31. | The ideal scheduling for JIT/lean operations follows a push system of planning. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #31

| 32. | In a "pull system," a pile of inventory in front of a workstation indicates problems at that workstation. TRUE |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #32

| 33. | In a "push system," a pile of inventory in front of a workstation indicates problems at that workstation. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #33

| 34. | The control of the flow of work in JIT systems can be characterized as a backward communication approach. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #34

| 35. | A kanban card is used to signal that work is ready to be moved to the next station. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #35

| 36. | JIT systems typically require that suppliers be able to provide large lots at fairly regular intervals. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #36

| 37. | JIT purchasing requires frequent contract bidding by multiple sources to ensure the buyer of competitive prices. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #37

| 38. | A JIT/lean system is not effective when the industry consists of a tiered supplier network. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #38

| 39. | For JIT/lean companies, certification of suppliers using a system such as ISO 9001 provides a cost effective an alternative to having to develop close supplier relationships. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #39

| 40. | Kanban is a system of lights used to signal problems or slowdowns. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 15 #40

| 41. | The purpose of an andon is to provide an immediate visual signal when and where problems are occurring. TRUE |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #41

| 42. | "Preventive maintenance" is extra-special maintenance done after a breakdown has occurred to prevent any further breakdowns. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 15 #42

| 43. | For JIT/lean companies the responsibility for preventive maintenance and repairs is often assigned to workers that operate production equipment. FALSE |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #43

| 44. | The sole purpose of housekeeping in JIT/lean operations is to enhance safety in the workplace. FALSE |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #44

| 45. | In a JIT system, long-term relationships with suppliers are unusual. FALSE |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 15 #45

| 46. | Which of the following is not an element of the building blocks of JIT/lean operations? A.; high levels of quality in products and processes. B.; highly specialized process systems. C.; a program on continuous improvement. D.; level loading. E.; close supplier relationships. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 15 #46

| 47. | In contrast to MRP systems, JIT systems A.; are more complex. B.; require extensive and detailed shop floor controls. C.; rely on visual or audible signals to trigger production. D.; are more expensive to develop. E.; all of the choices are true. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 15 #47

| 48. | Which one of the following is not one of the foundational building blocks of JIT? A.; product design. B.; process design. C.; personnel/organizational elements. D.; planning and controls. E.; kanban. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 15 #48

| 49. | The ultimate goal of lean systems is to have: A.; no in-process inventories. B.; cross-trained workers. C.; a balanced, rapid flow of materials and products through the system. D.; reduced setup times. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 15 #49

| 50. | The ultimate objective in a JIT system is: A.; low inventory. B.; high quality. C.; minimal production cost. D.; a balanced, rapid flow of materials and products through the system. E.; all of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 1 Stevenson - Chapter 15 #50

| 51. | Which of the following is not a goal of JIT/lean production? A.; System flexibility. B.; Elimination of waste. C.; Continuous improvement. D.; Preventative maintenance. E.; Elimination of disruptions. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 15 #51

| 52. | Continuous improvement (Kaizen) is based on all the tenets except: A.; Involvement of everyone in the organization. B.; Applied throughput the organization. C.; Learning by doing. D.; Direct observation and data collection. E.; Rapid, major breakthroughs in improvement. |
| --- | --- |

Difficulty: Medium Learning Objective: 2 Stevenson - Chapter 15 #52

| 53. | Which one of the following is not affiliated with product design to support JIT/lean operations? A.; concurrent engineering. B.; design for manufacturing and assembly. C.; value analysis of product functionality. D.; material requirements planning (MRP). E.; quality function deployment (QFD) to capture "voice" of customers. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 15 #53

| 54. | Which of the following is not a benefit of small lot sizes in JIT systems? A.; In-process inventory is considerably less.. B.; Each product is produced less frequently.. C.; Holding costs are reduced.. D.; There is less clutter in the workplace.. E.; Inspection and rework costs are less.. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #54

| 55. | Which of the following is a benefit of small lot sizes in JIT systems? A.; Quicker response to changing customer demands.. B.; Reduced setup costs.. C.; Inventory is eliminated.. D.; EOQ models are not required.. E.; All of the choices are benefits.. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #55

| 56. | In the JIT philosophy, the ideal lot size is: A.; the economic order quantity. B.; the economic run size. C.; one unit. D.; the capacity of the standard container. E.; N = (DT(1+X))/C. |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #56

| 57. | Which of the following does not contribute to reduced setup time and cost? A.; standardized setup tools. B.; standardized setup equipment. C.; specialized setup procedures for each product. D.; use of multipurpose equipment or attachments. E.; use of group technology. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #57

| 58. | Which of the following is not a method for reducing setup time? A.; streamline the setup process. B.; separate external (off-line) and internal (cannot be done off-line) setup activities. C.; multipurpose equipment or attachments. D.; group technology so parts requiring similar setup procedures are grouped. E.; all of the choices reduce setup time. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #58

| 59. | The technique intended to increase communications among functions, reducing the need for changes and speed up the design process is: A.; design for manufacturing and assembly (DFM and DFA). B.; Quality function deployment (QFD). C.; Concurrent engineering. D.; Group technology. E.; Takt time reduction. |
| --- | --- |

Difficulty: Medium Learning Objective: 3 Stevenson - Chapter 15 #59

| 60. | A set of tools and methods for reducing setup time developed by Shigeo Shingo based on using presses and dies is called: A.; Shingo Prize. B.; DFMA. C.; SMED. D.; group technology. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 4 Stevenson - Chapter 15 #60

| 61. | Which of the following is not associated with converting to cellular layouts? A.; determining a family of products to group together. B.; designing the process for the product family. C.; balancing the cell. D.; reducing the variety of end products produced. E.; determining the capacity of the cell. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #61

| 62. | The term used in Japan that refers to quality at the source, meaning stopping to fix problems to avoid passing defective products on to the next workstation is: A.; poka-yoke. B.; autonomation. C.; jidoka. D.; andon. E.; kanban. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #62

| 63. | A term that refers to the automatic detection of defects is: A.; poka-yoke. B.; autonomation. C.; jidoka. D.; andon. E.; kanban. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #63

| 64. | A basic requirement for operating with low inventories in JIT systems is: A.; Inventory space must be saved.. B.; Inventory investment must be saved.. C.; Major problems must be uncovered.. D.; Major problems must have been solved.. E.; Inventories must be reduced rapidly.. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #64

| 65. | JIT philosophy tries to eliminate work-in-process inventory because: A.; of the savings in capital requirements.. B.; inventory covers up recurring problems.. C.; it free up space for kanban storage bins.. D.; to increase dependency on suppliers.. E.; all of the choices are true.. |
| --- | --- |

Difficulty: Hard Learning Objective: 4 Stevenson - Chapter 15 #65

| 66. | Which of the following is not an important element of personnel and organization for JIT? A.; Cross-trained workers. B.; Authoritative leaders. C.; Support from non-manufacturing personnel. D.; Participation in Kaizen teams and suggestion systems. E.; all of the choices are elements. |
| --- | --- |

Difficulty: Hard Learning Objective: 5 Stevenson - Chapter 15 #66

| 67. | Which of the following is not true of worker/management roles and relations for JIT? A.; Stress on workers is reduced by requiring less of them.. B.; Workers are considered a key asset of a JIT system.. C.; Two way communication between workers and management is encouraged.. D.; Employees are empowered in selecting and using methods.. E.; all of the choices are true. |
| --- | --- |

Difficulty: Hard Learning Objective: 5 Stevenson - Chapter 15 #67

| 68. | Which of the following are potentially negative aspects of cross-functionally trained workers? I. cost and time for highly skilled workers II. worker resistance due to perceived loss of job security III. higher stress levels from added responsibility and a continued push to improve A.; I and III. B.; I and II. C.; II only. D.; II and III. E.; I, II, and III. |
| --- | --- |

Difficulty: Hard Learning Objective: 5 Stevenson - Chapter 15 #68

| 69. | Which of the following are benefits of level capacity loading? I. minimizes work-in-process inventory II. minimizes time to setup for different products III. maintains flexibility to meet customer demand A.; I. B.; II. C.; III. D.; I and III. E.; I, II, and III. |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #69

| 70. | Material requirements planning (MRP) is an example of a: A.; push system. B.; pull system. C.; forward communications approach. D.; inventory accumulation approach. E.; none of the choices. |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 15 #70

| 71. | The control of the flow of work in JIT systems is best characterized as a: A.; push system. B.; pull system. C.; forward communications approach. D.; inventory accumulation approach. E.; none of the choices. |
| --- | --- |

Difficulty: Easy Learning Objective: 6 Stevenson - Chapter 15 #71

| 72. | A kanban card is used to signal that: A.; work is needed from the preceding station. B.; work is ready to be moved to the next station. C.; a worker has run out of parts. D.; a machine has broken down. E.; a machine is ready for preventive maintenance. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #72

| 73. | When using kanban cards, long trip times may require: A.; more containers. B.; fewer containers. C.; more replenishment parts in inventory. D.; additional cross training. E.; none of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #73

| 74. | With regard to suppliers, JIT systems typically require: A.; delivery of large lots at regular intervals. B.; buyer inspection of goods and materials. C.; multiple sources from which to purchase. D.; long-term relationships and commitments. E.; the lowest price possible. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #74

| 75. | With regard to suppliers, JIT systems typically involve: A.; delivery of large lots on short notice. B.; the highest quality at the lowest price. C.; long-term relationships. D.; multiple suppliers. E.; all of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #75

| 76. | Which of the following is characteristic of the JIT philosophy? A.; Inventories are a valued asset.. B.; Few, large deliveries of orders from suppliers.. C.; Suppliers become partners.. D.; Queues of WIP inventory are a necessary investment.. E.; all of the choices. |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #76

| 77. | A system of lights used at each workstation to signal problems or slowdowns is: A.; jidoka. B.; automation. C.; andon. D.; pull system. E.; kanban. |
| --- | --- |

Difficulty: Easy Learning Objective: 4 Stevenson - Chapter 15 #77

| 78. | An extended version of process flow diagrams that includes information flows and the activity times refers to: A.; value stream mapping. B.; kanban control. C.; the 5S principles. D.; push analysis. E.; pull analysis. |
| --- | --- |

Difficulty: Medium Learning Objective: 7 Stevenson - Chapter 15 #78

| 79. | Which of the following is not characteristic of preventive maintenance in JIT systems? A.; maintaining equipment in good operating condition. B.; replacing parts before they fail. C.; workers maintaining their own equipment. D.; eliminating supplies of spare parts. E.; all of the choices are true. |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #79

| 80. | Which of the following is not affiliated with the implementation of JIT/lean systems? A.; Inventories are reduced.. B.; Space requirements are reduced.. C.; Quality is increased.. D.; Fewer orders from suppliers need to be processed.. E.; Closer relationships with suppliers.. |
| --- | --- |

Difficulty: Medium Learning Objective: 7 Stevenson - Chapter 15 #80

| 81. | Which of the following contributes to the competitive advantage enjoyed by firms using JIT/lean production? I. lower processing costs II. fewer employees III. fewer defectives IV. greater flexibility. A.; I, II, and III. B.; II, III, and IV. C.; I and IV. D.; I, III, and IV. E.; I, II, III, and IV. |
| --- | --- |

Difficulty: Medium Learning Objective: 7 Stevenson - Chapter 15 #81

| 82. | The All Seasonings Company uses 3,200 glass jars at one of its jar-filling workstations each eight hours of production. The waiting time for a standard container, which holds 90 jars, averages 45 minutes. If management uses a safety factor of twenty percent, how many containers should be used? N = ? D = 3200/8 = 400 jars per hour T = 45/60 = .75 hour X = .20 C = 90 jars |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #82

| 83. | The Four Star Publishing Company uses sixty reams of paper each hour at one of its high speed printing stations. The waiting time for a standard container, which holds 15 reams of paper, averages forty minutes. If management has decided to use three containers, what safety factor was applied? X = ? N = 3 containers D = 60 reams per hour T = 40/60 = 2/3 hour C = 15 reams |
| --- | --- |

Difficulty: Hard Learning Objective: 6 Stevenson - Chapter 15 #83

| 84. | A production system uses kanban cards to control production and movement of parts. One work centre uses an average of 40 pieces per hour of a certain part. Standard containers hold 10 parts. The waiting time for parts containers is about 36 minutes. Management has assigned a safety factor of .20 to this work centre. How many containers should be used to support this operation? N = ? D = 40/hour X = .20 C = 10 |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #84

| 85. | A production cell uses 240 pounds of plastic resin each 8-hour day. Resin is transported in drums that hold 100 pounds each. The material is obtained from a nearby supplier, and has a 12-hour waiting time. A safety factor of .15 has been assigned to this cell. What is the optimum number of containers to support this operation? N = ? D = 240/8 = 30 pounds/hour T = 12 hours C = 100 pounds |
| --- | --- |

Difficulty: Medium Learning Objective: 6 Stevenson - Chapter 15 #85

Chapter 15 Just-in-Time and Lean Production Summary

| Category | # of Questions |
| --- | --- |
| Difficulty: Easy | 24 |
| Difficulty: Hard | 19 |
| Difficulty: Medium | 42 |
| Learning Objective: 1 | 10 |
| Learning Objective: 2 | 6 |
| Learning Objective: 3 | 2 |
| Learning Objective: 4 | 28 |
| Learning Objective: 5 | 7 |
| Learning Objective: 6 | 29 |
| Learning Objective: 7 | 3 |
| Stevenson - Chapter 15 | 85 |