Section #1-- Just-In-Time Systems:

Just-In-Time (JIT) Manufacturing To produce only what is needed, when it is needed

- JIT inventory

- JIT purchasing

- Lean manufacturing

JIT Philosophy Eliminate all waste in the organization

JIT System A management system that aims to improve the manufacturing or service process by eliminating waste

Origin of JIT Systems

- 1950s, the Japanese were short on capital and space

- For Toyota, to improve performance meant to reduce inventory

- Inventory hides problems

- Reducing inventory led to reducing all kinds of waste

  - Poor quality

  - Machine breakdowns

  - Poor design

  - Poor vendors

  - Inefficient layout

  - Long setups

Types of Waste

Waste Anything other than the minimum amount of resources that is essential to add value to the product

- Process

  - Scrap

  - Non-value-added cost

  - Wrong tools/equipment

- Over-production

  - Extra inventory

  - Inappropriate use of resources

- Inventory

  - Storage

  - Capital Costs

- Product Defect

  - Interrupted flow

  - Lost capacity

  - Wait for replacement

- Waiting Time

  - Unplanned

  - Planned queue

  - Waiting for other parts in batch

- Methods

  - Searching for tools

  - Poor layout

  - Walking

- Movement

  - Material handling

  - Receiving

  - Storing

  - Retrieving

Key Elements of JIT Systems

Work Cells Efficiency within a process, multi-functional workers

Cell Grouping of products based on similar requirements

- Eliminate worker inefficiency increase worker productivity

- Operation of a number of different machines highly utilized, mutli-functional workers

- Cellular Facility Layout Work is moved with a cell (mostly U shaped) according to a prescribed path

  - Traditionally automobile production is mass production standardized products using assembly lines

- Flexible Machines

  - Automated, general purpose machines

  - Quick setup time

  - Flexible Manufacturing System (FMS)

Pull System Coordination between processes

- To improve the pull system, reduce the variability associated with supply and demand

- To control supply variability

  - Small lot sizes

  - Preventive maintenance

  - High quality

  - Reliable suppliers

- To control demand variability

  - Uniform loading

Pull Production System

- Improve poor coordination between processes eliminate the need for large inventory

  - Major problem in automobile manufacturing

- Push System Each workstation produces according to a schedule and “pushes” its completed work to the next workstation

  - Traditional approach to product; builds inventory

- Pull System Requires workstation requests, or “pulls”, items from the previous workstation only when needed

  - Prevent overproduction and underproduction

  - Kanban System Force coordination between processes

- Kanban System A visible production control system which authorizes the production or movement of the next batch of matiral only when needed

  - Kanban: a Japanese work for “signal” or “visible record”

  - Most common form: card, container

  - Kanbans are not schedules

- Work the same way as a fixed-quantity inventory system where order quantity (Q) equals the reorder point (R)

  - Only inventory maintained is the amount needed to cover usage until the next order arrives

- MRP vs. Kanban

  - Schedule vs. Cue

  - Complex vs. Repetitive

  - Higher-level vs. Shop-level control

Supply Variability #1: Lot Sizes

- Each Kanban “container” represents the production or order lot size

- Small lot sizes provide many benefits

  - Better coordination

  - Better material handling

  - Reduce average inventory level

  - Reduce inventory space

  - Avoid buildup of defective items

  - Flexibility in reacting to problems

- Quick set-up is crucial

Supply Variability #2: Machine Breakdowns

- Maintenance To keep facilities and equipment in good working order

- Machine downtimes are waste fix fast fixing is also waste

- Preventive Maintenance Periodic inspection and maintenance designed to avoid breakdowns

  - Preventive vs. breakdown maintenance

  - Necessary due to small inventory and high automation

- Work Environment: 5 S’s

  - Seiri Housekeeping

  - Seiton Workplace Organization

  - Seiso Clean up

  - Seiketsu Keep clean

  - Shitsuke Discipline

Supply Variability #3: Quality

- Consistently high quality products are necessary due to small inventory

- Quality at the source: uncover the root cause

  - Andon lights to signal problems

  - Empowerment Authority given to workers to stop the production line if a problem occurs

- Preventive

  - Poka-yoke Devices that are designed to prevent mistakes/defects from happening

  - Standard parts, modular design: reduce variability

- Continuous improvement (“kaizen”)

  - Quality Circle Team-based process improvement

  - Under capacity scheduling to deal with problems daily

Supply Variability #4: Suppliers

- Need reliable suppliers for on-time, frequent deliveries

- Must build long-term, close relationships with few suppliers

  - Single Sourcing Entire family of parts provided by 1 supplier

  - JIT II Supplier working in the manufacturer’s plant

- Physical proximity is preferred but not necessary

- From the Supplier’s point of view

  - Guaranteed, steady demand

  - Advanced notice of volume changes

  - Minimal design changes

  - Lots of requirements

  - Competitive vs. Cooperative Dilemma

  - ‘All eggs in one basket’

Demand Variability

- Cells can deal with changes in product mix and volume to a certain extent

  - Volume adjustment by a number of workers

  - Volume adjustment by integrating or separating cells

- Kanban system can absorb +10% variability in demand by manipulating the number of kanbans

  - Fewer kanbans, the less production

  - At Toyota, container size is at most 10% of daily demand

- Uniform loading Arrange daily production mix in the same ratio as monthly demand

  - Always have some quantities of each product

  - Steady demand on components

Before & After JIT

- Before

  - Inventory to protect against problems & uncertainty

  - Assembly lines

  - Push Manufacturing

  - Tolerate Defects

  - Tolerate set up times

  - Emphasize work of individuals, following manager instructions

  - Treat supplier as independent entities

- After

  - Reduce Inventory

  - Cells

  - Pull Manufacturing

  - Zero defects

  - Reduce setup time

  - Emphasize team-oriented employee involvement

  - Suppliers as partners

Benefits of JIT Systems

- Reduced inventory

- Reduced space requirements

- Improved quality

- Improved process

- Better use of human resources

- Better relations with suppliers

- Increased employee participation

- Shorter cycle time

- Shorter lead time

- Greater flexibility

- More product variety

- Increased productivity

- Increased machine utilization

JIT Implementation Issues

- All key elements must be present and integrated for the system to work well

- Must balance technical and behavioural aspects

- Each organization must mould a JIT system to suit its own environment

- JIT system requires a fundamental change in the organization

- Not suit for:

  - Very high volume

  - Very low volume, unique products

  - Highly fluctuating demands or true make-to-order products

Job Design

- Job Design Specifies the contents of the job

- Automation

  - Good for repetitive, computational, precise or physical tasks

  - Not good for people interaction, creativity, multiple variables

- Specialization Performing single or limited tasks

- Management

  - Readily available labour 	High absenteeism

  - Minimal training 		High turnover rates

  - Reasonable wage 		High scrap rates

  - High productivity		Grievances Filed

- Workers

  - Minimal credentials 		Boredom

  - Minimal Responsibilities 	Little growth opportunity

  - Minimal mental effort		Little control or initiative

  - Reasonable wage		Little intrinsic satisfaction

- Job Rotation Shifts workers to different jobs to increase understanding of the total process

- Job Enlargement Expansion of the job through increasing the scope of the work assigned

- Job Enrichment Expansion of the job through increasing the worker responsibility

- Teams

  - Problem-solving teams

  - Special-purpose teams

  - Self-directed teams

- Method Analysis The study of how a job is done

- Time Study Method Sets a standard time based on timed observations of one worker over several cycles

Section #2-- Quality:

- What is Quality

- Quality: Degree of excellence

- Product Based

  - Quality of Design: Degree to which quality dimensions are designed into the product

- Producer’s Perspective

  - Quality of Conformance: Degree to which a product conforms to required specifications

- Consumer’s Perspective

  - Fitness For Use: Degree to which a product satisfies consumer’s wants

  - Value-Based: Degree to which a product provides acceptable quality at a reasonable price

- American Society for Quality

  - The totality of features and characteristics that satisfy needs

- Dimensions of Quality

- Products

  - Performance: basic characteristics

  - Features: “extra” items

  - Conformance: meeting pre-set standards

  - Reliability: Frequency of failure

  - Durability: Length of product’s life

  - Serviceability: Ease of getting repair

  - Aesthetics: Appearance

  - Safety: Not causing injury or harm

  - Perceived quality: Reputation & intangibles

- Services

  - Time and timeliness

  - Accessibility and convenience

  - Accuracy and competence

  - Completeness

  - Consistency

  - Courtesy

  - Responsiveness to unusual circumstances

  - Communication

  - Security

  - Credibility

- Quality Measures

  - Scrap, rework, number of errors, premium shipping cost

  - Product Yield:

    - Yield= (# of units to produce) x (% Good units) + (# of units to produce) x (1 - % Good Units) x (% Reworked)

  - Quality Index: report quality cost relative to some base value (indexing)

    - Labor Index= (Quality Cost) / (Direct Labor Hours)

    - Cost Index= (Quality Cost) / (Production Cost)

    - Sales Index= (Quality Cost) / (Sales)

    - Production Index= (Quality Cost) / (Production Yield)

    - Quality-Productivity Index= (Product Yield) / (Product Cost)

- Costs of Quality

  - All costs, tangible and intangible, relating to managing the quality of a good or service

  - Costs of poor quality (failure costs):

    - Internal Failure:

    - External Failure:

    - As quality increases failure decreases

    - As internal Failure decreases, external costs decrease

  - Costs of good quality (control costs):

    - Appraisal: Costs of treating and inspection (equipment, operators)

    - Prevention: Costs of preparing and implementing a quality plan

      - Product/product design, training, information costs

    - As product quality increases, appraisal costs decrease

- Costs of good quality vs. Costs of poor quality (Graphs)

  - Pre-“Zero Defects” Mentality

  - “Zero Defects” Mentality

- Quality and Profitability

  - Improved Quality Leads to:

    - Higher Sales:

      - Improved response

      - Improved reputation

      - Higher prices

      - Increased market share

    - Lower Costs:

      - Lower rework & scrap

      - Lower warranty & liability

      - Increased productivity

  - Which both lead to higher profits

- Quality Improvements

  - Process Improvements: Final product inspection is too late

    - Must improve the production process

      - Eliminate common causes (e.g. Poor design)

      - Eliminate special causes (e.g. Specific equipment)

  - Continuous Improvement: A philosophy of never-ending pursuit of high product quality

    - Innovation vs. Continuous Improvement

    - Big Jumps vs. Small Steps

    - Dramatic vs. Not Dramatic

    - Specialists vs. Everyone

  - Quality of Design: Built-in quality in the product or service design

    - Quality Function Deployment (QFD): A tool to translate the customer preferences into specific technical requirements

    - House of quality

- Process Improvement Tools

  - Check Sheet

    - To initiate process improvement by recording quality problems

    - Fact-finding tool

    - Tallies the number of problems

  - Histogram, Pareto Chart

    - To prioritize quality problems

    - Shows the frequency of quality problems

  - Cause-and-effect (fishbone, Ishikawa) diagram

    - To identify the cause of a quality problem by tracking the production

    - Identifies all possible areas to which quality problems may be related

  - Scatter Diagram

    - To narrow down to one cause

    - Shows the relationship between two variables

  - Flow Chart

    - To identify the cause of a quality problem by tracing the production process

    - Focus on where in a process a quality problem might exist

  - Statistical Process Control (SPC) Chart

    - Quality Monitoring

    - Monitors whether or not the process is in control over time

- Continual Improvement Tool

  - Deming Wheel: 4 stage-process for continual quality improvement

  - AKA PDSA or PDCA Cycle

  - 4 Steps:

    - Plan

      - Identify problem and develop plan for improvement

    - Do

      - Implement plan on a test basis

    - Study/Check

      - Check to see if the plan is working

    - Act

      - Institutionalize improvement

  - Quality Circle: Team-based approach to continuous quality improvement

  - EX. All in a circle so no specific order

    - Organization

      - 8-10 members

      - Same area

      - Moderator

    - Training

      - Group processes

      - Data collection

      - Problem analysis

    - Problem ID

      - List alternatives

      - Consensus

      - Brainstorming

    - Problem Analysis

      - Cause & Effect

      - Data Collection & Analysis

    - Solution

      - Problem Results

    - Presentation

      - Implementation

      - Monitoring

Section #3-- Statistical Process Control (SPC):

- Quality Management

  - 4 Stages:

    - Quality Inspection: Focus on providing information

      - When?

        - Upon receipt of resources

        - Before transformation operations (especially bottleneck)

        - The first few items out of an automated operation

        - Final inspection

        - Customer complaints and returned goods

      - How Much? and How Often?

        - Complete inspection vs. sampling

        - Cost of inspection vs. cost of not detecting defects

        - Cost, time and physical possibility

    - Quality Control: Focus on monitoring and controlling--> SPC

    - Quality Assurance: Management programs aimed at ensuring good product quality

    - Total Quality Management (TQM): Management philosophy about ensuring and improving product quality throughout the entire organization

- Acceptance Sampling

  - The method of randomly inspecting a sample of goods and deciding whether to accept the entire lot based on the results

    - Take a random sample from a lot (batch) of items

    - Test the sample items for the specified quality characteristics

    - Accept all items in the lot if

(Number of defective items in the sample) < (Maximum number of defective items

allowed in a sample)

Otherwise, reject all items in the lot

  - Historical mentality: “some degree of poor quality will occur and that is acceptable”

- Statistical Process Control (SPC)

  - To prevent poor product quality

  - Process improvement tool

  - Method of randomly inspecting a sample of goods and deciding whether the production process is in control

    - Monitor the production process (data pattern)

    - Provide a statistical signal when the process (quality) charges

- General Steps in SPC

  - Define the quality characteristic to measure

    - Attributes: Characteristics that are measured qualitatively, thus have discrete values

      - E.g. Defective/non-defective, # of scratches or blemishes

      - Count both defective and non-defective items--> p-chart

      - Count defective features in an item--> c-chart

    - Variables: Characteristics that are measure quantitatively, thus have continuous values

      - E.g. Weight, length, volume, temperature

      - Difference (range) between smallest and largest values--> B-chart

      - Average--> e-chart

  - Set up a control chart

    - Chart pg.88

  - Take a random sample and plot the quality measure

    - Chart pg. 89

  - Is an sample point outside of the control limits?

    - Yes--> the process is out of control (step 6)

    - No--> Go to step 5

  - Does any pattern exist?

    - Yes--> the process is out of control (step 6)

    - No--> the process is in control--> continue monitoring (step 3)

  - The process is out of control. Stop the process until the quality problem is identified and fix

    - Process improvement tools

    - Discard out-of-control sample observations

    - May need to revise the control chart

- Key Ideas Behind the Control Limits

- In spite of inherent random variation in a production process, the average of the distribution of the quality characteristic should be stable if the process is in control

- The range between the UCL and LCL indicate the existence of abnormal variation

- Observations falling outside the UCL or LCL indicating the existence of an abnormal variation

- Control limit standard= 3-sign a limits

  - Control limits too narrow--> Type I error: random variation mistaken for an abnormal variation

  - Control limits too wide--> Type II error: abnormal variation may not be detected

- p-Chart: Control Chart for Attributes

- Quality to be measured: proportion of defective items

- Sample size (n): 30-100 items per sample (guideline)

- Center Line:       = historical average proportion of defects

Sample

Standard Deviation

Z= number of standard deviations corresponding to the sigma limit

c-Chart: Control Chart for Attributes

- Quality to be measured: # of defective features per product

- Sample size: 1 item per sample

- Used when it is not possible to count non-defective items

  - i.e. no proportion of defectives can be computed

- Center Line:       = Historical average # of defective features

Sample

Standard Deviation

R-Chart: Control Chart for Variable

- Quality to be measured:

R= (Largest value in the sample) – (Smallest value)

- Sample size (n): 2-10 items per sample

- Center Line:       = Historical average of R values

D3 and D4= Range value constants based on sample size

- Specifically developed to determine control limits for R-chart

x-Chart: Control Chart for Variable

- Quality to be measured: Sample average

- Sample size (n): 2-10 items per sample

- Center Line:       = Historical average of x values

A2= Mean value constant based on sample size

- Specifically developed to determine control limits for x-chart

R-chart and x-chart should be used together

Control Limit Factors—for R-chart and x-chart with 3-sigma limits

Pattern Tests

- To detect a non-random pattern within the control limits

- Run tests: check for the runs

  - Run A sequence of observations with a certain characteristic

  - Pattern #1: 8 Consecutive points on one side of the center line

  - Pattern #2: 8 Consecutive points up/or down

  - Pattern #3: 16 Consecutive alternating up and down

- Zone tests: check for the zones

  - 3 zones for 3 sigma limit control chart

  - Pattern #4: 2 out of 3 consecutive points in Zone A

  - Pattern #5: 4 out of 5 consecutive points in Zone A or B

Process Capability

- SPC monitors natural vs. non-random variation within a process  Producer’s Perspective

- Process Capability The ability of a process to satisfy a product’s or service’s tolerances

  - Tolerances Design specifications that reflect customer requirements

  - Not statistically determined

  - Not a result of the production process

- Production process must be in control (check SPC) and must meet design specifications (check process capability)

- Control chart data can be used to determine process capability

  - UCL Upper Tolerance Limit (UTL)

  - LCL Lower Tolerance Limit (LTL)

- Rough Guideline:

  - Process is capable of meeting specifications if:

    - (UCL – LCL) = (UTL – LTL)

  - Process is capable of exceeding specifications if:

    - (UCL – LCL) < (UTL – LTL)

  - Process is not capable of meeting specifications if:

    - (UCL – LCL) > (UTL – LTL)

Section #4—Total Quality Management (TQM):

- What’s TQM?

  - An organization-wide effort to achieve high product quality based on 3 guiding principles:

- Customer Focus

- Quality from the customer perspective; obtain customer feedback

- Continual Improvement

- Constantly seek to improve processes, products, productivity, effectiveness, responsiveness, etc.

- Total Participation and Teamwork

- Workers are the inspections: providing training, authority, rewards

- Cross-functional, process improvement teams

- Top management commitment

- Supplier Relationship

- Stages in QM

  - Quality Inspection

  - Quality Control

      - Obtain Management Support

    - Management Commitment

    - Customer Focus

      - Choose a SPC Leader

      - Select a Process for Pilot Study

      - Provide SPC Training

    - Empowered Workers

      - Construct Control Charts

    - Continual Improvement

    - Process Improvement

  - Quality Assurance

      - Project-by-project

      - Process-by-process

    - Cross-functional

  - TQM

      - Company-wide

    - Supplier Relationship

- Quality Evolution

|  |  | Events/People | Ideas |
| --- | --- | --- | --- |
| Industrial Revolution/Early 1900s | Quality Inspection |  |  |
| 1920s-1950s | Quality Control | Shewhart | -Process Improvement -Statistical Methods |
| 1960s-1970s | Quality Assurance | -Deming, Juran, Crosby -ISO, JIT, Oil Crisis, Foreign Competition | -Mgmt Philosophy -Continual Improvement -Cost of Quality -Zero Defects |
| 1980s to Present | TQM | -Feigenbaum -Ishikawa | -Org. Focus -Customer Driven Quality -More Choices and Info -Higher Expectations -Affordable Quality |

- Quality Gurus

- Shewart

  - Promoted SPC

  - Shewhart Cycle

- Deming

  - Deming Wheel

  - 14 Point Quality Mgmt Philosophy

  - Promoted Quality Mgmt in Japan—Deming Prize

- Juran

  - Quality Control Handbook

  - Quality Trilogy

    - Planning

    - Control

    - Improvement

  - Cost of Quality

- Crosby

  - Cost of Poor Quality

  - Zero Defects

  - Quality is Free

- Feigenbaum, Ishikawa (Cause-and-Effect Diagram)

  - Total Quality Control

  - Company-Wide Commitment

- TQM vs. JIT

- Customer Focus

  - JIT: to achieve good forecast of customer demand

- Continuous Improvement (“kaizen”)

  - JIT: Require high quality products; under capacity scheduling

- Total Participation and Teamwork

  - JIT: Well-trained, multifunctional, empowered employees; group problem solving

- TQM vs. BPR

- Business Process Reengineering (BPR): the total redesign of a production process

  - Leading to organizational and cultural changes, empowered and multi-functional employees, management leadership

- Aim for rapid, dramatic improvement in process performance (BPR) vs. continuous improvement (TQM)

- Common BPR Characteristics:

  - Combining several jobs into one

  - Process have multiple versions

  - Work is done where it makes the most sense

- TQM Implementation Issues

- Lack of a Genuine Quality Culture

  - Forgetting customer-focus

  - Conflict with existing systems

    - E.g. Compensation and promotion systems, policies, and procedures

- Lack of Top Management Support and Commitment

  - Forgetting long-term benefits

    - “Bottom line” approach

    - Inadequate Training

- Over—or Under—Reliance on Quality Tools

  - Over-emphasis on teams, under-emphasis on individual efforts

- Six Sigma Quality Program

- Six Sigma: A high level of product quality with 3.4 defective parts per million

  - Coined by Motorola in 1986

  - Motorola won Baldridge Award in 1988

- Six Sigma Program: A quality management program with clear business goals, which are achieved by:

  - Cutting poor quality costs

  - Rewarding employees for quality improvement

  - Training employees--> Six sigma black belt certification

    - Black Belt: Employee trained and experienced in application of statistical techniques, problem solving, project management, team leadership skills

  - Apply a scientific method--> DMAIC, statistics, computers

- Methodology

- DMAIC Model

  - Define problem/opportunity

  - Measure characteristics that are critical-to-quality

  - Analyze the problem using benchmarking and gap studies

  - Improve by reducing variation and reducing defects

  - Control Performance

- Other Tools

  - Computer Simulation

  - Part Standardization

  - Supplier Qualification, SPC

  - Design of Experiments

  - Measurement System Analysis

- Quality Awards

- Baldridge Award (USA): Created by law in 1987

  - Manufacturing, service, small business, education, health care

  - Customer focus, process management, leadership, HR, strategic planning, information and analysis, business results

- Deming Prize (Japan): Created in 1957

  - Company, Business Unit, Individual

  - Allowed foreign companies to apply in 1984

  - Minimum application standards

- Common Themes

  - Privilege worthy experience, expensive

- Quality Standards

- ISO (International Organization for Standardization)

  - Headquarter in Geneva, established officially in 1947

  - Network of national standards institutes of 146 countries

  - Developed over 18,000 international standards on a variety of subjects

- Quality Management--> ISO 9000 Series

  - First published in 1987

  - To promote a uniform quality standard for cross-border transactions

  - A guideline for developing an organizational system for documenting quality management-related processes and procedures

  - Certification of suppliers, not products

  - Common themes:

    - Marketing Strategy

    - Customer Requirement

    - International Trade Requirement

- ISO 9000 Series Include:

  - ISO 9001 (production, procurement, installation, design, servicing)

  - ISO 9002 (production, procurement, installation)

  - ISO 9003 (inspection and test)

  - ISO 9004 (quality management)

- ISO 14000 series for environmental management (1996)

  - Environmental management systems, technical standards

  - Applicable in all nations, promote the interests of the public and user, cost effective and flexible

- Other non-ISO industry standards

  - QS 9000: For Chrysler, Ford and GM Suppliers

  - AS 9000: Aerospace industry

- ISO 9000

  - Registration Process:

- Application

- Documentation Review

- Pre-Assessment

- Registration Audit

- Registration Decision

- Surveillance Audit

  - Cost

    - Internal:

      - Analysis

      - Project planning

      - System development

      - Documentation

      - System implementation

      - Training

      - Internal audit

      - System modifications

    - External

      - ISO publication and software

      - Registrars (Fees & Travel)

      - Consultants (Fees & Travel)

  - Time: “3 Months to 3 Years”