Landslides

Learning Objectives (Lecture 1)

Describe how the impact of landslides depends on: population density, economic infrastructure and population preparedness

Human and economic harm are broadly governed by these three factors;

We care because:

  - We want to live (deaths & injury)

  - We want to own property that wont be destroyed ($$$)

Population density, economic infrastructure and population preparedness all play a key role in our prevention of landslides and safety precautions taken once they occur

Explain why British Columbia has the highest frequency of landslides in Canada and what we should expect as our population expands into the mountains.

Landslides are greatly affected by factors such as rainfall, material, undercutting, overloading etc. British Columbia has ALL of these factors, especially on the Sea to Sky Highway (which should have never been built).

As our population expands into the mountains, we can expect an increased amount of landslides to human interference and lack of knowledge regarding this topic.

- Distinguish between 3 major failure modes and how they are influenced by geology.

- i. Fall:

Occur on steep slopes

Material(s): usually rock

Material detaches because of weakness

Falls due to gravity

VERY FAST!!!

ii. Slide:

Material(s): usually soil, rock or debris

Material moves as a coherent mass along the slope of failure

Vary from SLOW to FAST

Two types of Slides: Rotational Slide & Translational Slide

Rotational Slide (Slump)

Rotation of material on a curved failure plane  characterized by curved scarp above slide

Weak materials (sediment)

Intermediate speed

Translational Slide

If surface is flat

Usually strong material moving along planes of weakness  cohesive motion of material

Slow to fast

iii. Flow:

Material(s): soil, mud, wet debris, (rock)

Water is usually very important

Fluid or plastic flow of material

Very slow to very fast

Learning Objectives (Lecture 2)

Define Angle of Repose.

At the angle of repose shear stress is perfectly balanced by shear strength.

(Factor of Safety is equal to or just above 1)

- Assess the balance between the strength of the slope and the destabilizing forces acting on it (Factor of Safety – FoS ).

Stability of slope involves resting forces (cohesion and friction) and driving forces (gravity)

If driving forces > resisting forces, motion can occur

Factor of Safety = ratio of shear strength (T1 – resisting forces) to shear stress (T2 – driving forces)

FS = T1 / T2     FS > 1  STABLE SLOPE, FS < 1  FAILURE OF SLOPE

- Compare and contrast landslide causes and landslide triggers.

- Causes are factors (often long term) that lead to the instability of a slope. They increase shear stress of a slope, but do not initiate movement.

- Triggers are factors (usually short events) that translate instability into motion.

- ** There can be MANY causes, but only ONE trigger **

- List and describe several external causes of landslides.

- External Causes (factors outside of the slope that contribute to mass movement):

  - Slope Angle

You must have slope for there to be mass movement

Steeper slopes = more movement

  - Undercutting

The lower part of the slope is removed  material supporting the slope

Caused by roads, rivers, buildings, etc.

  - Overloading

Adding weight

Caused by buildings, roads, landslides, trees, me, etc.

  - Vegetation

Roots bind material  removal of roots can make slopes unstable

Heavy trees can increase instability (example of overloading, c)

  - Climate

If average temperature and rainfall is high:

More water

Increased weathering of rocks (more fractures, soil, etc)

If average temperature is around 0 degrees, see internal causes

Learning Objectives (Lecture 3)

List and describe three internal causes of landslides.

Internal Causes (factors inside the slope that affect stability)

  - Water

Adds weight

Decreases normal force / normal stress

Increases weathering & acts as a medium for flow

Water Content:

In sediment:

Water can help / hinder cohesion, depending on the amount

No water = low angle of repose

A little water = high angle of repose

Too much water = very low angle of repose

In rock:

Water reduces shear strength along planes of weakness (fractures)

Water can cause frost wedging / freeze thaw

In colder climates  water gets between cracks in the rock, it freezes and expands which forces the rock apart

  - Inherently Weak Materials

Some materials are very weak  fail at relatively / very low angles of repose

CLAY IS ESPECIALLY ODD

When in salt water, clay is attracted to other clay particles

Quick clay is problematic

  - Adverse (Bad) Geologic Structures

Unfortunate bedding or fracture orientation

Structures angled in an instable direction or layered precariously

List several landslide triggers.

Earthquakes

Snow melts

Heavy rainfall

Loud noises

Vehicles

Volcanic Eruptions

Compare and contrast key triggers and causes of landslides and how they affect the force balance equation (i.e. Factor of Safety).

For example, inherently weak materials will decrease the force of cohesion and shear resistance, creating a low factor or safety. We will see mass movement in this case.

In adverse geological structures there is a decrease in friction  decreases the force of resistance  increases factor of safety.

Explain how liquefaction landslides develop in sensitive marine clays (quick clay slides).

- When in salt water, clay is attracted to other clay particles. Percolating ground water can remove salt from the clay, which in turn makes the substance collapse.

- List and describe the site conditions (causes and triggers) that lead to the development of the Oso Landslide in Washington State.

- Causes:

45 days of heavy rainfall  increase in water content

Collapse of glacial sediment (an inherently weak material)

Triggers:

Seismic activity

Learning Objectives (Lecture 4)

Relate the type of expected landslide damage as a function of its velocity.

Landslides are rated on a scale from 1 – 7, 1 being the slowest and 7 being the fastest. Landslides below the 4 mark are not a concern…

5 - 7 = danger zone

Identify tell tale signs of an unstable slope.

This can be done using GeoStatistics, Geologic Mapping and Hazard Mapping.

Look for:

Debris flow, scarp from pre-existing landslide, undercutting, steep slope, deforestation

- Compare and contrast avoidance, prevention and protection strategies for dealing with landslides.

- Avoidance:

Move to a different area and avoid the problem

Prevention:

Do something to make sure landslides don’t occur / don’t occur when people are there

Protection:

Strengthen the area that might be affected if a landslide happens

List the mitigation techniques commonly used for avoidance, prevention and protection strategies.

Avoidance:

In almost all cases avoidance is too expensive and includes:

Buying all of a town’s property, moving a completed highway, asking people to leave their family homes

Instead we use:

Hazard mapping

Computer modeling

Geological mapping

Determining frequency and magnitude of past landslides

Prevention:

Removal of Material

Very simple

Too expensive for many situations, but cheaper than avoidance

Effective for some rock falls

Stabilizing Slopes

Undercut slope?  Apply a resisting force at the bottom, such as retaining walls or gabions

Sliding rock?  Apply rock bolts

Drainage

Too much water?  Remove with drainage

Protection: let the landslide happen, but control where it goes

Rock fences

Catch falling and rolling rocks, but dissipate kinetic energy

Debris Flow Retention Structures

Debris flow = water + debris  remove debris from water = no more flow

Rock Barrier, Rock Net, Rock fall Shed