University of Ottawa

Professor: Dr. Voicu Groza

TAs: Heli Amarasinghe, Maryam Hezaveh, Opeyemi Adesina, Hamdam Nikkhah, Simran Deep Singh

Fall 2016

Lab 2- Design and Simulation of Sequential Logic Circuits

Experiment Date: September 28st 2016

Submission Date: October 9th 2016

LAB Section B

Introduction

This Lab serves as practice in the conversion of functional requirements into the logic circuits and implementation using Altera. This lab will also introduce students to the design of sequential circuits based on Altera’s Quartus development environment and their implementation and testing with an FPGA

These different parts of implementations include:

Entering the design of synchronous counters using Quartus II graphics editor

Assigning the input-output pins and preparing the design for downloading and testing on the Altera DE2-115 board

Testing the counter by: Displaing the counter outputs as binary values on LEDs and by using an oscilloscope and tracing and recording the waveforms at various flip-flops

Pre Lab Discussion

Part A

Part B

These are the counting sequences that are implemented with JK flip flops. We must derive and simplify the Boolean expressions of every flip-flop input using Karnaugh maps.

Pre Lab Solution

These are the two solutions of A and B taken from karnaugh maps of the JK flip-flops

- J0 = Q2Q1 K0=Q1’    J1=Q2’ K1=Q0    J2=Q1 K2=Q0’

- J0=1 K0=1    J1=Q3’Q0 K1=Q1Q0   J2=Q1Q0 K2=Q1Q0    J3=Q2Q1Q0 K3=Q1Q0

Design Section

Truth Table of part A

| Present State |  |  | Next State |  |  | Synchronous Inputs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q2 | Q1 | Q0 | Q2 | Q1 | Q0 | J2K2 | J1K1 | J0K0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 x | 1 x | 0 x |
| 0 | 0 | 1 | x | x | x | x x | x x | x x |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 x | x 0 | 0 x |
| 0 | 1 | 1 | 1 | 0 | 1 | 1 x | x 1 | x 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | x 1 | 0 x | 0 x |
| 1 | 0 | 1 | 1 | 0 | 0 | x 0 | 0 x | x 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | x 1 | x 0 | 1 x |
| 1 | 1 | 1 | x | x | x | x x | x x | x x |

Truth Table of part B

| Present State |  | Next State |  | Synchronous Inputs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Q3Q2 | Q1Q0 | Q3Q2 | Q1Q0 | J3K3 | J2K2 | J1K1 | J0K0 |
| 0 0 | 0 0 | 0 0 | 0 1 | 0 x | 0 x | 0 x | 1 x |
| 0 0 | 0 1 | 0 0 | 1 0 | 0 x | 0 x | 1 x | x 1 |
| 0 0 | 1 0 | 0 0 | 1 1 | 0 x | 0 x | x 0 | 1 x |
| 0 0 | 1 1 | 0 1 | 0 0 | 0 x | 1 x | x 1 | x 1 |
| 0 1 | 0 0 | 0 1 | 0 1 | 0 x | x 0 | 0 x | 1 x |
| 0 1 | 0 1 | 0 1 | 1 0 | 0 x | x 0 | 1 x | x 1 |
| 0 1 | 1 0 | 0 1 | 1 1 | 0 x | x 0 | x 0 | 1 x |
| 0 1 | 1 1 | 1 0 | 0 0 | 1 x | x 1 | x 1 | x 1 |
| 1 0 | 0 0 | 1 0 | 0 1 | x 0 | 0 x | x 0 | 1 x |
| 1 0 | 0 1 | 0 0 | 0 0 | x x | 0 x | x 0 | x 1 |

Part A Circuit Diagram:

This is the circuit of the logic equation mapped out on the Quartus II  Software.

These are the outputs of Part A and the synchronous inputs.

Part B Circuit Diagram:

Real Implementation

Part A

This is the completed waveform diagram. The simulation ran as expected and we ended up with a working design.

This is the complete waveform which shows the values of the outputs.

This is the waveform shown on the oscilloscope tracing the waveforms at different flip-flops.

Part B

This is the completed waveform of part B

After the waveform we also assigned the inputs and outputs to these pins

After the waveform we also assigned the inputs and outputs to these pins

A picture of the first output in the sequence to part B

Discussion

Through this experiment, we created two sequential circuits using JK flip-flops and implementing them through AND gates. Through the preparation we obtained the state tables and Karnaugh maps of the sequences, which were used as theoretical results. The truth tables obtained were then compared with the actual results, which was conducted during the lab with the Altera card and Quartus Software. Here we had the basic idea of how the output would be. And we successfully did after compiling and programming the circuit to the Altera DE2-115 card get the same results as expected. For Part A, we used an oscilloscope which shows how the flip-flops change over time, this allowed us to see a different way to implement our logic diagrams. For Part B, we used the switches and LED lights. Through this, we were able to check if we simulated the circuit correctly. Comparing it with the tables obtained from the prelab verified the results outputted by the Altera DE2-115 card and the oscilloscope. An error we had in the lab was setting up the oscilloscope to display the waveform correctly. In the end the experiment was a success as it helped our group design and implement sequential circuits through the Altera card and an oscilloscope.