[image: image13.png]UNIVERSITE D'OTTAW) INIVERSITY OF OTTAWA

of 9
Ecole d'ingénierie et de technologle de Iinformation
‘School of Information Technology and Engineering





CEG2131
Computer Architecture I
Section A: Prof. A. AlDaher

Section B: Prof. Misbah Islam

FINAL Exam, Dec 16, 2005
Duration:  3 Hours
Total marks: 100, Weighted Marks: 40
CLOSED BOOK:  No notes or texts permitted

PLEASE PROVIDE THE FOLLOWI_______________ First Name: ___________________

Signature:  
______________________
Instructions:

	For use of grader

	A
	20
	

	B
	30
	

	C
	30
	

	D
	20
	

	Total
	100
	


1. Please write down your SECTION Number.

2. No notes or texts are permitted. You have to abide by the University’s regulations regarding the conduct of exams.
3. You may use the back of the pages for any side work that will not be marked. 

4. Calculators are not allowed.
5. You can refer to the Appendix, ARC Reference page for Assembly language instruction, instruction and ALU codes and Microinstruction word formats.
6. Write down any assumptions that you may make.
Part A: Arithmetic (20)
Assuming a system with a 6-bit biased exponent (i.e. excess 32) and a 10-bit two’s complement mantissa. Perform the following operation. 

157.3

[image: image14.png]313029282726 25 24 23 222120 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

SETHI Format 0 0 rd Dp2 imm22
Branch Format [0 0|0 cond opz disp22
L Pidirian Pk P S S S T S Rl R S R S M R
313029282726 2524 232221201918 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00
CALL format 01 dls|p30
i
313029 26 2726 25 24 23 22 21 20 19 16 1716 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00
T T T T T T
s N 10 rd Op3 rsl 0 0 O 0 0 O 0 0 0 rs2
Arithmetic L L L L PR P
Formats 10 rd op3 rsl 1 simml3
31302928 2726 2524 23 22 21 20 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00
11 rd op3 rsl ojloooo00000O0 rs2
Memory Formats — ——— ——— ——— —— T
11 rd op3 rsl 1 simml3
op Format op2| Imnst. op3 (op=10) op3 (op=11) cond | branch
00| SETHI/Branch 010| branch 010000 addcc 000000 1d 0001 | be
01| CALL 100( sethi 010001 andcc 000100 st 0101 | bes
10| Arithmetic 010010 orcc 0110 | bneg
11| Memory 010110 orncc 0111 | bvs
100110 srl 1000| ba
111000 jmpl —
313029 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00
PSR n|z|v|c




- 12.6

[image: image15.bmp]
By completing the following steps (give your answer for each step, each step has 2 marks)

(i) convert 157.3 to binary



100 111 01 . 010 01

(ii) Normalize the fixed-point binary number (place the radix point immediately to the left of the leftmost, nonzero digit in the fraction) 
0.10 011 101 010 01 X 28
(iii) Round the mantissa to 10 bits.



0.100 111 011 

(iv) Write how the floating point number would be stored



101 000 010 011 101 1

(v) Convert 12.6 to normalized floating point number and rounded to 10 bits. 



0.110 010 011 X 24
(vi) Form the two’s complement of the mantissa for the 12.6



1.001 101 101

(vii) Equalize exponents



1.111 100 110 X 28 
(ix) Perform addition and give your final restult



0.100 100 001 X 28 = 100 100 00 . 1 

(x) Convert your result into decimal and compare to exact answer. 


Your result = …144.5………………………………..


Exact result = …144.7……………………………..

Part B: ARC Instruction Set Architecture and Assembly Language (30)
Q2. (20) Subroutine and Linkages
The code in ARC assembler language of a main program Myprg is given below.  It loads two numbers num1 and num2 and passes these as arguments to a subroutine Mysub that subtracts these numbers, stores the result onto the stack and returns to Myprog.

.begin

.org
  0x1000


! Start loading program at 4096
%sp

equ
  %r14


! Let %sp represent %r14
Myprg:
ld
  [num1], %r1
! %r1 ( 0xDEF


ld
  [num2], %r2
! %r2 ( 0xABC


addcc  %sp, - 4, %sp
! create space on the stack


st
  %r1, %sp

! push 0xDEF onto the stack


addcc  %sp, - 4, %sp
! create space on the stack


st
  %r2, %sp

! push 0xABC onto the stack


call
  MySub


! Jump to Mysub


ld
  %sp, %r3

! Store in r3 contents pointed to by sp
num1:
0xDEF

num2:
0xABC



.end



.begin



.org   0x1100

! Start loading subroutine at 4096+100
Mysub:
ld
  %r14, %r8

! Pop 0xABC from stack


addcc  %r14, 4, %r14
! Go to down the stack


ld
  %r14, %r7

! Pop 0xDEF from stack


subcc  %r8, %r7, %r9
! %r9 ( %r8 - %r7 0xABC – 0xDEF = -0x222


st
  %r9, %r14

! store -0x222 in %r14 


jmpl
  %r15+4, %r0
! Return to main at ld %sp, %r3 instruction


.end
a. (8) Provide meaningful comments wherever indicated by !

b. (8) Assuming that at the start of Myprog the stack pointer register (%r14 ) is pointing to location 0x4000 and the stack base is 0x5000, show the contents of the stack 

i. Before the call is made from Myprog to Mysub.

[image: image16.png]A B C
M M M
u u URW
A X B X C XDR ALU COND JUMP ADDR

NI T T




[image: image17.png]03 O, 01 O





[image: image18.png]op3

| I |

IR bits—> 31 30|24 23 22|21 20 19

op2




[image: image19.png]Fy Fy F| F, Operation Changes Condition Codes
00 00 ANDCC (A, B) yes
00 0 1 ORCC (A, B) yes
00 1 0 NORCC (A, B) yes
00 1 1 ADDCC (A, B) yes
01 0 0 SRL (A, B) no
01 0 1 AND (A, B) no
01 1 0 OR (A, B) no
01 1 1 NOR (A, B) no
1 000 ADD (A, B) no
1 0 01 LSHIFT2 (A) no
1 010 LSHIFT10 (A) no
1 0 1 1 SIMM13 (A) no
1 100 SEXT13 (A) no
11 01 INC (A) no
1 110 INCPC (A) no
1 1 1 1 RSHIFT5 (A) no





[image: image20.png]3 address

SUBY.W.X
DIVZ . X.X
SUBX,Z, X

2 address.

MOV'Y, X
SUBW, X

DIVZ. X
SUBZ. X

1 address

LOADY
SUB W
STO TMP
LOAD Z
DIV TMP
SUBZ
STOX

Size: 14 x 3 =42 bytes
Traffic: 3x4+3x3=21
words

words

X 10 =40 bytes

4x3+4x2=20

words

Note: I word = 4 bytes





[image: image21.png]G, C G Operation

Use NEXT ADDR

Use JUMP ADDR ifn =1

Use JUMP ADDR if z = 1

Use JUMP ADDR if v =1

Use JUMP ADDR ifc =1

Use JUMP ADDRif IR [13] =1
Use JUMP ADDR

DECODE

——_—_—occ oo
—— oo~ —oo














ii. Upon exit from Mysub.













c. (4) What is the contents of register %r3 upon return from Mysub?  - 0x222
Q3: (10) Multi Address Instructions
The following table gives the instructions available for Three-Address, Two-Address and the One-Address architecture machine:

	Available Instructions

	Three-Address
	Two-Address
	One-Address

	SUB a, b, c  ! c ( a - b
DIV   a, b, c  ! c ( a/b
	MOV a, b   ! b ( a
SUB  a, b   ! b ( b –a 
DIV   a, b    ! b ( b/a

	LD    a     ! Acc ( a
ST    a     ! a ( Acc

SUB a     ! Acc ( Acc - a

DIV   a     ! Acc ( Acc / a



a.  Using the above instructions write down three programs to compute the function 
          X = (Z / (Y-W)) - Z.

b. Compute the size of your program in bytes. Assume 16-bit opcodes, 32-bit operands and addresses, and that data is moved to and from memory in 32-bit chunks.

	Program to compute X = (Z / (Y-W)) - Z

	Three-Address
	Two-Address
	One-Address

	52
	54
	40


 PART C: Data Path and Control (30)
Q4: (10) Micro-programmed Control Unit
Refer to the following microcode segment for the ARC computer:
Address           

Micro Code Operation

0:

R[ir] ( AND(R[pc], R[pc]); READ;


1:

DECODE;






/ addcc

 1600:

IF T[IR[13]] THEN GOTO 1602;

 1601: 

R[Rd] ( ADDCC (R[rs1], R[rs2]));




GOTO 2047;

 1602:

R[temp0] ( SEXT13 (R[ir]);

 1603:
    R[rd] ( ADDCC(R[rs1], R[temp0]);




GOTO 2047;

a. (2) What is the purpose of the AND operation at Line 0?
To put the PC onto the A bus that is connected to the Address 
Register of the main Memory, in enable fetching of the 
next instruction into the IR

b. (8) What will be the assembled microcode in Microcode memory corresponding to lines 1600 to 1603. (Look up the Reference section on the last page)
1600: 000000  0  000000  0  000000  0  00  0101  101  11001000010
1601: 000000  1  000000  1  000000  1  00  0011  110  11111111111

1602: 100101  0  000000  0  100001  0  00  1100  000  00000000000

1603: 000000  0  000000  0  000000  0  00  0101  101  11001000110

[image: image1]
1600:


[image: image2]
1601:


[image: image3]
1602:


[image: image4]
1603

Q5. (20) Hard-wired implementation of Control Signals
The following is the HDL description of the control unit of a small-scale ARC computer. It has four 32-bit general purpose registers and one 32-bit instruction register.
Module: Small_ARC_Control_Unit

INPUTS:

OUTPUTS:

MEMORY: Rs1 [32], Rs2 [32], Rd[32], ir[32], temp0[32].
! Fetch Instruction from Program Memory at addr = PC

0: ir ( AND (PC, PC); READ ( 1;

! If bit 13 of Ir is set then transfer ir 0:12 bit values

! to register temp0 else transfer the contents of Rs2 to temp0

1: temp0 ( { SIMM13 (ir) CONDITIONED ON ir [13:],



  Rs2 CONDITIONED ON ir[13:]’ };

! If bit31, bit30 = ‘10’ , then it is an Arithmetic Operation


! If bit31, bit30 = ‘11’ then it is memory Read Operation.

2: GOTO { 
3 CONDITIONED ON ir[31:] x ir[30:]’,



4 CONDITIONED ON ir[31:] x ir[30:]  }.


! Perform the Arithmetic Operation ADD or AND after decoding
      ! bits 19:24 of ir.

3: Rd ( {ADDCC (Rs1, temp0) CONDITIONED ON XNOR ( ir[19:24], 111000)

    ANDCC (Rs1, temp0) CONDITIONED ON XNOR ( ir[19:24], 010001)};

   
GOTO 6.


! Calculate address in the Data memory
4: temp0 ( ADDCC (Rs1, temp0); 
GOTO 5.


! Set up address on the ABus, issue READ command 

! and load Register Rd with the contents at the memory address

5: Rd ( ANDCC (temp0, temp0); READ ( 1; 
GOTO 6.


! Increment PC by 4 to get to the next instruction 

    
! in the Program Memory

6: PC ( INCPC (PC); 
GOTO 0.

END SEQUENCE.

END Small_ARC_Control_Unit.

a. (2) How many flip flops will be needed for the state machine for this control unit, if ONE-HOT encoding approach is employed.
______7_______
b. (10) Draw the control unit flip flops and the interconnectivity between them


[image: image5]
c. (8) How will you generate the hardwired control signal for the 
1. (2) Read operation

    = CS0 + CS5
2. (3) Rs2 Selector signal
     = CS1. IR[13:]’
3. (3) Rd Selector signal
                     = CS3 + CS5

PART D: Memory Systems (20)

Q6: Memory System Design





[image: image6]
Given that RAM chips of type (4 word x 4-bit) shown above are available. 
Using these chips draw the diagrams for 
a. (7) A 4 word by 12-bit memory system

Answer: identical to 7.3
b. (7) A 16 word by  4-bit memory system

Identical to 7.4
c. (6) A Read Only Memory ROM should contain the following words.
	Location
	Value

	00
	1001

	01
	1110

	10
	1010

	11
	0101


Using OR gates and one 2-to-4 Decoder design the circuit for this ROM.

[image: image7]

Appendix: ARC Reference Page:

a. ARC Instruction Formats:

[image: image8]
b. ALU Function Codes:

	ALU Function
	ALU

Operation
	Condition

Code change

	A3  A2  A1  A0
	
	

	1     0     0      0 
	ADDCC (A, B)
	yes

	0     1     0      1
	ANDCC (A, B)
	yes

	1     0     1      1
	SIMM13 (A)
	no


c,ARC Memory Instruction Register Format (41 bits)


[image: image9]
d,Microprogram Memory Address:


[image: image10]
e: ALU Function Codes:


[image: image11]
f: COND field for the Microword

[image: image12]



D    Q





FF3








D    Q





FF2






























































D    Q





FF5





stack base (





0 (





%sp (





232 – 4 (





Free Area








Stack








D    Q





FF6








stack base (











0 (





%sp (





232 – 4 (





Free Area














Stack








D    Q





FF4








D    Q





FF0





-0x222





%r15





0xABC





0xDEF












































D    Q





FF1





UXin the book book�������������������������������������������������������������������������������������������������������������


 OR





 OR





 OR





 OR





 00


01


10


11


MUX














PAGE  
page 9 of 12

