Student ID: 8688440

Student Name: Himansu Sehgal

Date: Tuesday, September 27th, 2016

Problem 2.5

A stake is being pulled out of the ground by means of two ropes as shown. Knowing that α= 30º, determine by trigonometry;

- The magnitude of the force P so that the resultant force exerted on the stake is vertical.

- The corresponding magnitude of the resultant.

Since the forces acting on this object are in the same direction, they can be added using the “tip to tail” method, then the sine law can be used to determine the magnitude of the resultant and the vector P.

ASin(∝)=PSin(25)=RSin(r)

P=ASin 25Sin ∝

P=(120)Sin 25Sin 30

P=101.43 N

R=ASin rSin ∝

R=(120)Sin 125Sin 30

R=196.60 N

A = 120 N

α = 30 o

r = 180 o - 25 o – α

r = 125 o

25o

r

α

A

R

P

Therefore the magnitude of force P  is 101.43 N and the magnitude of the resultant is 196.60 N.

Problem 2.36

Knowing that the tension in rope AC is 365 N, determine the resultant of the three forces at Point C of the post BC.

Cable Force AC =

To solve this problem, the force components must first be solved.

Fx= -365N9601460=-240N

Fy= -365N11001460=-275N

500 N Force:Fx=500 N2425=480 N

Fy=500 N725=140 N

200 N Force:Fx=200 N45=160 N

Fy=-200 N35=-120 N

Therefore concluding that the x-component of the resultant force is the sum of all the forces in the x-direction.

Rx= ΣFx

ΣFx= -240N+480N+160N

ΣFx= 400N

Ry= ΣFy

ΣFy=-275 N+140N-120N= -225N

ΣFy= -225N

This leads to using the Pythagorean Theorem, stating that the square of the x and y components will result in the magnitude of the result vector.

R=Rx2+Ry2

R=[(400 N)2+-255 N)2

R=473.40 N

The angle that the resultant makes with the horizontal can be represented by the tan ratio.

tanα=OppositeAdjacent

tanα=225400

α=tan-1225400

α=32.5°

The resultant has a magnitude of 474 N at an angle of 32.4° below the horizontal.

Problem 2.48

Two cables are tied together at C and are loaded as shown. Knowing that P = 500 N and α = 60°, determine the tension in

(a) In cable AC

(b) In cable BC

To analyse the equation, a free body diagram must be drawn.

60°

45°

25°

P = 500N

TAC

TBC

Now use the sine law to solve for each tension.

TACSin 35°=TBCSin 75°=500 NSin 70°

TAC=500 NSin 35°Sin 70°

TAC=305 N

TBCSin 75°=500 NSin 70°

TBC=500 NSin 75°Sin 70°

TBC=514 N

Therefore the tension in wire AC is 305 N, whereas the Tension in wire BC is 514 N.

Problem 2.61

For the cables of Problem 2.48, it is known that the maximum allowable tension is 600 N in cable AC and 750 N in cable BC. Determine

(a) The maximum force P that can be applied at C

TAC = 600 N

P

25°

45°

α

TBC = 750N

(b) The corresponding value of α.

C

After constructing the free body diagram, use the properties of the cosine law and isolate for P.

P2=6002+7502-2600750cos70°

P=784.02 N

Now use the sine law to isolate for the corresponding value of α.

Note: (The sine law can be used both ways, the sine can be on the top or bottom, as long as it is kept consistent.)

Sinβ600 N=Sin(70°)784.02 N

β=Sin-1600 NSin70°784.02 N

β=46.0°

Proportionally since the beta angle is 46 degrees, then the 25 degrees would have to be added to that value to achieve the alpha value, resulting to be 71 degrees.