##### Lab 1 : The ON/OFF Controller

##### Systems and Control Theory-ENGG*3410

##### University of Guelph

##### Dr. Julie Vale & Dr. Simon Yang

##### Lab 1: On/Off Controller

##### Group #TBD, Friday 3:30-5:20 p.m.

##### Mohammad Zoubi - 0859269

##### Jesse Marcil - 0829197

# Introduction:

The purpose of this lab is to achieve a better understanding of ON/OFF controller systems, these are the simplest form of control system and are used in a variety of ways. These controllers are used when a system cannot produce the desired input. The controller limits the output to two different states, on and off. When the systems temperature crosses a given set point it causes a cycle to begin because the condition is continuously being adjusted. Throughout the lab, a variety of parameters are altered, which include hysteresis; set point and whether a disturbance is on or off. This lab helps engineering students to understand how these conditions affect the controller allowing them to create a controller which suits their needs.

# Experimental Apparatus & Procedure:

Refer to the ENGG*3410 Lab Manual: Lab 1: The ON/OFF Controller

- Section 2: Equipment and Setup

- Section 3: Procedure.

# Results :

Graphs 1-A through 3-H will be found in the Appendix at the end of this report.

# Discussion:

When a disturbance was introduced to the circuit, it increased temperature output of the system while the heater output showed a shorter positive voltage output and a longer negative voltage output. The positive and negative simply represent when the system was on and off respectively. The disturbance is an effect on the system that effectively interrupts the signal, in this experiment it created a delay in the heater output meaning the "on" period was shorter than the "off" period. The shortened "on" period lead to a higher temperature output to account for the induced disturbance and maintain the value of the set point.

Overall there was not much effect to the heater output when the set point values were altered although it increased the upper and lower trip points of the temperature graph. Changing the set point of the system should shift the curve up or down the y- axis on the temperature graph, because the graph must fluctuate around the new temperature point that was set. However the frequency and amplitude did not change. Simply, increasing the set point increases the ideal temperature.  In addition, the heat output graph was not altered because the amount of times the system would turn "on" and "off" is not affected by changing set temperatures.

Changing the value of hysteresis changed the length of time the controller was "on" and "off" as expected, but also increases the values of upper and lower trip points of the temperature graph. Hysteresis effectively increased the time intervals for the system, explaining the length of the positive and negative heater outputs. This increase in time interval forced the temperature to reach increased upper and decreased lower trip points because the heater itself has more time to reduce its temperature between intervals. The increased time the heater output is negative causes it to generate more heat therefore a higher temperature to maintain the set point.

If a very small hysteresis value is applied, the frequency of the system would be increased drastically. This essentially means that the control would be turned "on" and "off" more frequently, as a result decreasing the plants temperature fluctuation because its temperature has less time to remain at the low and high ends of temperature. This is the best approach to the application of this controller where a more consistent temperature is desired. Although a low hysteresis value results in a more stable temperature, it can have a severe amount of strain on the system due to an increased number of cycles the controller must go through. As a result, the life expectancy of the controller is reduced. This is the reason engineers must find ideal hysteresis values.

# Conclusion:

This laboratory experiment successfully demonstrates the operation of  a simple feedback controller. This On/OFF controller specifically operates to maintain a set point although it may require adjustments if disturbances or hysteresis are introduced causing a lag in response time. The first conclusion drawn from this lab was that for various applications there is a unique set point but it does not affect the systems overall performance. Secondly, a high hysteresis value is not ideal because it causes longer "on" and "off" intervals which wastes energy. Judging from the graphical results, an ideal controller would have minimal hysteresis, a determined ideal set point and designed to reduce unanticipated disturbances caused by influences in the environment or other system errors.  Understanding how to design and operate the ideal controller for a variety of applications is critical to the field of engineering.

# Appendix:

## Set Point 4:

Figure 1-A: Heater output versus time given a set point of 4, a minimum hysteresis and no disturbance.

Figure 1-B: Temperature sensor output versus time given a set point of 4, a minimum hysteresis and no disturbances.

Figure 1-C: Heater output versus time given a set point of 4, a hysteresis of 5 and no disturbance.

Figure 1-D: Temperature sensor output versus time given a set point of 4, a hysteresis of 5 and no disturbances.

Figure 1-E: Heater output versus time given a set point of 4, a maximum hysteresis and no disturbance.

Figure 1-F: Temperature sensor output versus time given a set point of 4, a maximum hysteresis and no disturbances.

Figure 1-G: Heater output versus time given a set point of 4, a hysteresis of 5 and disturbance on.

Figure 1-H: Temperature sensor output versus time given a set point of 4, a hysteresis of 5 and disturbance on.

## Set Point 6:

Figure 2-A: Heater output versus time given a set point of 6, a hysteresis of 5 and no disturbance.

Figure 2-B: Temperature sensor output versus time given a set point of 6, a hysteresis of 5 and no disturbances.

Figure 2-C: Heater output versus time given a set point of 6, a maximum hysteresis and no disturbance.

Figure 2-D: Temperature sensor output versus time given a set point of 6, a maximum hysteresis and no disturbances.

Figure 2-E: Heater output versus time given a set point of 6, a minimum hysteresis and no disturbance.

Figure 2-F: Temperature sensor output versus time given a set point of 6, a minimum hysteresis and no disturbances.

Figure 2-G: Heater output versus time given a set point of 6, a hysteresis of 5 and disturbance on.

Figure 2-H: Temperature sensor output versus time given a set point of 6, a hysteresis of 5 and disturbances on.

## Set Point 8

Figure 3-A: Heater output versus time given a set point of 8, a maximum hysteresis and no disturbance.

Figure 3-B: Temperature sensor output versus time given a set point of 8, a maximum hysteresis and no disturbances.

Figure 3-C: Heater output versus time given a set point of 8, a hysteresis of 5 and no disturbance.

Figure 3-D: Temperature sensor output versus time given a set point of 8, a hysteresis of 5 and no disturbances.

Figure 3-E: Heater output versus time given a set point of 8, a minimum hysteresis and no disturbance.

Figure 3-F: Temperature sensor output versus time given a set point of 8, a minimum hysteresis and no disturbances.

Figure 3-G: Heater output versus time given a set point of 8, a hysteresis of 5 and disturbance on.

Figure 3-H: Temperature sensor output versus time given a set point of 8, a hysteresis of 5 and disturbances on.