Chapter 2 – Understanding your computer

Computer: gathers data, processes data into information, outputs and stores data and information

Data: a representation of a fact, a figure, or an idea (number, word, picture, sound)

Information: data that has been organized or presented in a meaningful fashion

Processing: manipulating, calculating or organizing data into information

Binary language: language of computers, consists of only 0 and 1

Binary digit: each 1 and 0

Byte: 8 binary digits

What the computer uses to represent the data and information that it inputs and outputs

Kilobyte (KB): 1000 bytes

Megabyte (MB): 1 million bytes

Gigabyte (GB): 1 billion bytes

Terabyte: 1 trillion bytes

Petabyte, Exabyte, zettabyte

Hardware: any part of the computer you can touch

Software: set of computer programs that enables the hardware to perform different tasks

Application software: set of programs used to write a paper for example

System software: set of programs that enables your computer’s hardware devices and application software to work together

Operating system (OS): the program that controls the way in which your computer system functions – manages the hardware of the computer system

Notebook computer or laptop: portable computer that is powered by batteries – keyboard, monitor and other devices integrated into a single compact case

Netbook – small notebook computer 7 to 10 inches wide with a longer battery life

Tablet PC: similar to notebook but has a touch sensitive screen that can swivel and fold flat

Desktop computer: single use location – main components plus peripheral devices

Peripheral device: component such as a monitor or keyboard connected to the computer

All in one computer such as the apple iMac

Mainframe: large, expensive computer that supports hundreds of users simultaneously  - good at executing different computer programs at the same time

Supercomputer: specially designed computer that can perform complex calculations extremely rapidly – execute few programs as quickly as possible

Embedded computer: specially designed computer chip that resides in another device such as a thermostat – don’t receive input or interact with other systems

INPUT DEVICES

Input device: enables you to enter data and instructions into the computer (keyboard, mouse) – stylus, pen with no ink

QWERTY keyboard: standard English-language keyboard layout – slows typists down

Dvorak keyboard – alternative keyboard – puts most commonly used letters in the English language on “home keys” – reduces travel distance, increasing typing speed

Alternatives: flexible keyboards, virtual laser keyboard, DX1

Wireless keyboards use a form of wireless technology that uses radio frequency – 6 to 30 feet away some as far as 100

Optical mouse, trackball mouse (ball on top or on the side), touch pad (laptop), trackpoint device (laptop little joystick), wireless mice (use batteries and send data via radio frequency or bluetooth), Apple Magic mouse (like macbook trackpad), MoGo mouse

Microsoft and Logitech offer features such as magnifier (magnification box that you can drag around the screen) and customizable buttons on mice, web search (highlight then click on search button), file storage (wireless USB receiver to store files)

Other input devices, joysticks, game pads, steering wheels (most are wireless)

Touch screens: display screen that responds to commands initiated by a touch with a finger or a stylus (Apple ipod ipad, Nintendo DS)

Alternative to tablet PCs  digital pen like the Dane-Elec Digital Pen which captures your writing and then wirelessly transits and stores in in the flash drive, when the flash drive is connected to a computer, translates your writing into digital text

Image input: digital cameras, camcorders, cell phones  used in remote settings and later download to computer through cable or wirelessly, scanners, webcam

Sound input: microphone captures sound waves and transfers them to digital format on your computer, built-in mics in notebooks

Desktop microphones (attached base), unidirectional microphones pick up sound from only one direction (best for podcasts with one voice), omnidirectional microphones (all directions at once, good to pick up more than one voice, good when you need to keep your hands free), clip-on-microphones also called lavalier microphones, close-talk microphones (usually attached to a headset

Input devices for people with disabilities: voice recognition for the visually impaired, keyboards with larger keys, keyboards on the monitor that you click, Maltron keyboard when you can only use one hand, special trackballs only needing one finger for people with motor issues, head-mounted devices (uses a camera, mouse clicks controlled by a switch manipulated by hands or feet or mouth)

OUTPUT DEVICES

Output device: enables you to send processed data out of your computer in the form of text, pictures, sounds or video

Monitor, printer, speakers and earphones

Monitors

Liquid crystal display (LCD): also called flat-panel monitor, light, energy efficient, some use light-emitting diode technology – more energy efficient and better color accuracy and thinner

Cathode ray tube monitor (CRT) have been replaced by LCD because they are legacy technology (methods from an earlier time)

Grids made up of millions of dots called pixel (each pixel is made up of 3 subpixels of red, blue and green). Some new TVs also have yellow.

LCD monitors have 2 or more sheets of material filled with a liquid crystal solution and a panel at the back of the monitor generates light waves – electric current passes through liquid solution – crystals move around and block or let shine the fluo light which causes image to form on the screen

Aspect ratio is the width to height proportion of a monitor, resolution is the clearness or sharpness of the image (the higher the better – usually set by the screen size and aspect ratio) choose the monitor with the highest resolution available for the screen size, contrast ratio is the measure of difference in light intensity between the brightest white and the darkest black (400:1 – 1,000:1 is preferable), viewing angle tells how far you can move to the side before the image quality degrades to unacceptable levels (150 degrees is recommended for 17+ inches), brightness (300cd/ m2 or > is recommended), response time is the time it takes for a pixel to change color (lower the better)

Look for monitors with built-in features such as speakers, webcams and microphones, built in USB port

Output to a large group of people with a projector, some are small like the 3M MPro 150 fits in a hand

Printers: inkjet, laser are nonimpact printer (sprays ink or transfers laser beam marks to paper) have replaced impact printer that strike the paper through an inked ribbon (dot-matrix-printer legacy technology)

Inkjet printer: standard in homes, affordable, quick, quiet, consider the type and cost of the ink cartridges when buying

Laser printer: more often found in offices and classrooms, faster, higher quality, no need for ink laser beams, long run cost may be lower

Other types: portable printers run on battery power, wireless printers (Wi-Fi and Bluetooth up too 300 ft.), all-in-one printer (printer, scanner, copier, fax), plotter (good to print precise and continuous lines – maps, architectural plans), thermal printer either melt wax based ink on ordinary paper (thermal wax transfer printing) or by burning dots onto specially coated paper (direct thermal printing) used in airports

What to consider when buying: speed, resolution, color output, use and cost of the printer, cost of consumables

Sound output: speaker, surround-sound speaker (includes subwoofers – full 360 degree sound), headphones, earbuds

PROCESSING ON THE MOTHERBOARD

Motherboard: main circuit board that contains the central electronic components of the computer, processor, memory and many circuits

Located inside the system unit: metal or plastic case that houses power unit

Sets of chips that powers the system, ROM, RAM, cache (short-term memory), slots for expansion cards or adapter cards (sound and video cards), sound card (connection for the speakers and microphone), video card (connection for the monitor) - high end models vs. low-end where sound and video is integrated to the mother board, modem card (connection to the Internet via phone line), network interface card (connect with other computers)

Memory

RAM: random access memory – where programs and data are stored – delivers information to the CPU for processing. Series of small cards plugged into slots on the motherboard. Volatile storage, deletes when you turn off PC. Permanent storage on hard drive or storage device.

ROM: read-only memory – non volatile storage, instructions the computer needs to start up when powered on

Processing

CPU: central processing unit – controls all the functions performed by the computer’s other components and processes all the commands issued to it by software instructions (45 billion tasks/ second)

Speed measured in units of hertz (Hz)

Machine cycle – process of CPU getting data from RAM and decoding it, then executes them and stores result in system memory

Old machines ran speed in megahertz (MHz – millions of machine cycles/ s.) now its gigahertz (GHz – billions/ s.)

Other factors of performance: cores (processing paths – now 2,4, 8 paths), cache memory, front side bus (how fast data goes from CPU and RAM)

STORING DATA AND INFORMATION

Permanent storing is located in drive bay. There are 2 kinds of drive bay, internal and external.

Internal: internal hard drive – holds permanently stored programs and data

External: CD, DVD drives, you can attach some others

Hard drives

Hard drive: primary device for permanent storage – today memory of 3.5 terabyte around – located inside the computer, some external hard drive that you can fit in your pocket (1 or 2 TB of memory, used to back up data)

Optical storage

Optical drive can read from and maybe even write to CDS, DVDS, or blue-ray discs – tiny pits are burned into the disc by a high speed laser

Capacity: Blue-ray dual layer > Blue ray > DVD dual layer > DVD > CD

Flash storage (jump drive, USB drive, thumb drive)

Stores portable data (as much as 256 GB)

Many computers come with slots for flash memory card (let you transfer data between computer and other devices such as smartphones, cameras etc.)

CONNECTING PERIPHARALS TO THE COMPUTER

Port: place through which a peripheral device attaches to the computer so that data can be exchanged (many located at the back but commonly used ports can be located in the front)

USB is the most common port type – transfer data quickly (USB 2.0 transfer data at 480 megabits per second)

Serial ports send data one bit at a time – use to be used to connect modems. Parallel port much faster (500 Kbps – were used to connect printers) – USB much quicker therefore they have become legacy technology

Other types of ports: Firewire 400 (400 Mbps) and 800 (800 Mbps) – good for hard drives, video cameras, etc.

Connectivity port can give you access to networks and the internet or to function as a fax machine (looks like large phone jack) – Ethernet port (transfers data at 1 000 Mbps)

Modem port – 2nd connectivity port, standard phone line connector

Other ports: back of the computer audio and video ports (hook up monitors – video graphics array which CRT monitors connect and older LCD monitors). Newer ones along with TVs DVD players, projectors connect to digital video interface and S-video ports. Audio ports where you connect headphones, microphones and speakers

High-definition multimedia interface: compact audio-video interface carries high-def video and uncompressed digital audio in one cable (TV and gaming consoles)

Desktop should have at least 6 USB ports and notebook 3 – you can install expansion cards in your system to provide additional ports. If no expansion slats on the motherboard, you can add an expansion hub that connects to one port to add new ports. Can also add ports to an empty drive bay.

POWER CONTROLS

Power supply (inside system unit) transforms the wall voltage to voltage required by computer chips  - powering computer from turned off state is called a cold boot

Powering off helps save energy, keeps computer secure and ensures data is saved – use power off button or shut down button on start menu

Ppl argue to turn off computer every time: environment, money. Others argue to let it run: stress from heating and cooling down – modern operating systems include power-management settings that shut down power-hungry components such as monitor and hard-drive

Sleep mode: documents, applications and data remain in RAM

Hibernate: stores your data in memory and saves it to computer’s hard drive – info protected if power failure

Warm boot: restarting the system when its powered on

SETTING IT ALL UP

Ergonomics is important

Position your monitor correctly (25 inch. From eyes)

Purchase an adjustable chair

Assume a proper position while typing

Take breaks from computer tasks

Ensure the lighting is adequate

Chapter 3 – using the Internet

Internet was developed during the Cold War to establish a secure form of military communications and creating a means by which all computers could communicate

Internet evolved from Advanced Research Projects Agency Network (ARPANET) – first communication in 1969 – Vinton Cerf and Robert Kahn known as fathers of the internet

The web is only one component of the internet – other components include FTP, Gopher, Telnet, and BitTorrent

What differences the web from the rest of the internet is its use of common communication protocol and special links that enable users to navigate from one place to another on the web

First web browser (software to interact with text and other media on the web) developed in 1989

1993: National Center for Super-computing applications releases Mosaic browser for use on the Mac and Windows

Internet was global by 1997

COMMUNICATING THROUGH THE INTERNET

Email:  fast, convenient, reduces cost (postage, long-distance calls). Not private, can be viewed by others (printing or forwarding), employers have access to e-mail sent from the workplace

E-mail clients such as Microsoft Outlook are software programs running on your computer that act like a post office by accessing your internet service provider – can only be used on the computer where its installed. Can download email from many accounts, several features to manage and organize emails (task, sender, priority)

Web-based e-mail client – can look at e-mails directly from the web (Yahoo, Gmail)

Instant messaging can be used as communication between co-workers

AOL, FB chat, windows live messenger are proprietary, you can only chat with those who use the same IM service

Trillian, Pidgin, Digsby are universal, they allow users of all the popular IMs to chat together

Group communication: e-mail, IM (one or a few people that you know), Facebook (circle of friends and family), chatrooms, newsgroups (people you want to connect with but don’t know personally)

Chat room – real time conversation, newsgroup – people create threads, Internet relay chat (IRC) – means of synchronous group communication used in discussion forums

Netiquette: etiquette of behavior

THECHNOLOGIES: COLLABORATING AND COMMUNICATING THROUGH THE INTERNET

Social networking: how people use the Internet to communicate and share information among their immediate friends, meet and connect with others

Facilitates business networking (likedin)

Weblog or blog: personal log or journal posted on the Web – most recent post on the top, searchable, organized, user friendly

Video log or vlog: personal journal that uses video as the primary content

Problems: spam blogs – fake articles or stolen text, have links to other sites associated with the splog’s creator

Wiki: type of website that allows user to change its content, good for collaborative writing (classrooms, business collaboration)

Podcast: clip of audio or video content that is broadcast over the Internet using compressed audio and video files (radio shows, audiobooks, magazines). When you subscribe to podcasts, when the content changes, it is brought to you.

Podcasts possible because of RSS (really simple syndication): XML-based format that facilitates the delivery of frequent content updates on Web pages

Web content formatted in a way that aggregators (software programs that go out and get latest updates according to your specifications) can find it and download only the new content to your computer

You can create a podcast using basic equipment (microphone and computer, camera for video podcast, editing software)

Webcast: broadcast of audio or video content over the Internet (usually live or one-time events). Not updated automatically, uses media technology that continuously feeds the audio and video content (easier viewing of audio and video content). Can be used to broadcast meetings.

WEB ENTERTAINMENT: MULTIMEDIA AND BEYOND

Multimedia on the wed: graphics, audio files, video files

Streaming audio or video: continuously feeds a file to your browser so you avoid having to wait for the entire file to download completely

Multiplayer online games, massive multiplayer online role-playing games (MMORPGs)

Most web graphics appear on browser without any additional software

For some podcasts, videos or audio files you may need a special software called a plug-in – you may need to update them in order to minimize risk

Your browser keeps track of visited web sites (cache) – delete your temporary internet cache periodically, and click refresh if you revisit a site in the same browsing session

CONDUCTING BUSINESS OVER THE INTERNET

E-commerce or electronic commerce: process of conducting business online, such as through advertising and selling products

Click-and-brick business (stores with a physical building and online presence)

Business-to-consumer (B2C): transactions between businesses and consumers

Business-to-business: businesses buying and selling to other businesses

Consumer-to-consumer (C2C): consumers selling to each other through online auction

Businesses must have some form of security certification to give their customers a level of comfort

Make sure http:// changes to https:// for secure socket layer, appearance of a small icon of a closed padlock

Shop at well-known, reputable sites

Avoid making online transactions when using public computers

Pay by credit card, not debit card

Check the return policy

ACCESSING THE WEB: WEB BROWSERS

Web browser: software that allows you to locate, view and navigate the web – today we use graphical browsers (display pictures, text and other forms of multimedia)

Common browsers: internet explorer, firefox, safari, google chrome

Tabbed browsing, quick tabs for convenient navigation and web management tools, built-in search box, inprivate browsing

GETTING AROUND THE WEB: URLs, HYPERLINKS, AND OTHER TOOLS

You gain access to a website by typing its unique address or uniform resource locator (URL), move around the site through the hyperlinks

URL:

Protocol: first part of the URL – generally followed by ://www and the domain name or host name

Individual pages within a website are further identified after the domain name, following another /

Protocol: most URLs begin with http (hypertext transfer protocol) – allows files to be transferred from a web server on your computer

Another common protocol is file transfer protocol (FTP)- used to upload and download files from your computer to a web server

FIT files use an FTP file server, HTTP files use a web server

Domain name: identifies the site’s host

Top-level domain: the suffix (.com or .edu), which indicates the kind of organization to which the host belongs. Each country has his own.

After a /additional text, indicates a particular file or path (or subdirectory)

Hyperlinks: specially coded element that allows you to jump to another web page or web site, images can also be hyperlinks

Breadcrumb trail: a list of pages within a web site you’ve visited

Favorites or bookmarks feature: marker of the site’s URL for easy retrieval

Live bookmark feature of Firefox adds technology of RSS feeds to book marking – delivers updates to you as soon as they become available

Social bookmarking or tagging: lets you store, organize and manage bookmarks of web pages – keyword that is assigned to a piece of information so that it can be found again by browsing or searching

SEARCHING THE WEB

Search engine: set of programs that searches the web for keywords and returns a list of web sites on which those keywords are found

Subject directory: structured outline of web sites organized by topics and subtopics (ex: librarian’s internet index)

Metasearch engine: search other search engines rather than individual websites

Search engines have 3 parts

Spider: constantly collects data on the web, following links in web sited and reading web pages

Indexer program: organizes the data into a large database

The search engine software: searches the indexed data, pulling out relevant information according to your search

Different search engines give different results because: different algorithms, how they rank the search results, differences in proprietary formula, some search the entire web whereas specialty search engines search only sites relevant to the subject

Boolean operators: words such as AND, NOT, OR that describe the relationship between keywords in a search

Tips:

Search for a phrase – use quotations

Search within a specific web site

Use a wild card ( * ) ex: Congress voted * on the * bill (mention how Congress voted on various bills

Google scholar, Google custom search, Google News

Evaluating web sites:

Authority

Bias

Relevance – is the material current?

Audience – does it match you needs? (content, tone, style)

Links – are the links available and appropriate?

If the answer to all is yes, website is a good source

THE INTERNET AND HOW IT WORKS

A computer connected to the internet is either a client (asks for data) of it is a server (receives the request and returns the data to the client)

Internet is referred to as a client/ server network

How do computers “talk” to each other?

Type address  client asking for data from x website

Your request travels along pathways, the largest and fastest are called Internet backbones

Data flows along backbone and then to smaller pathways until it reaches the destination

Server returns data using the most expedient pathway system

Web browser interprets data and displays it on your monitor

Internet protocol address (IP address): set of 4 numbers separated by periods (dotted quad or dotted decimal) – means b which all computers connected to the Internet identify each other – websites are given text versions of their IP address

CONNECTING TO THE INTERNET

Dial-up connection: originally the only means to connect to the Internet (using a standard telephone line) – becoming legacy technology (max transmission of 56 Kbps)

Broadband (high speed Internet): type of connection that offers a faster means to connect to the internet (256 Kbps)

Digital subscriber line (DSL): uses a standard phone line to connect your computer to the internet

Cable: uses your television’s cable service provider to connect to the internet

Fiber-optic service: uses plastic or glass cables to transfer data at the speed of light

Satellite broadband: mostly used in rural or mountain areas that cannot get DSL, cable or fiber-optic

Cable:  uses the same coaxial cable used by cable TV (but need to upgrade their networks with two-way data transmission capabilities). Slower at peak times because it is shared between users.

DSL: no need for an additional phone line, DSL uses remaining 2 lanes in copper wires to receive and send data – much faster than dial-up. Local Phone Company must have DSL technology to offer the service. DSL is limited by distance (max of 3 miles from where signal originates)

Fiber optic: fiber-optic lines (strands of optically pure glass or plastic thin as a human hair), arranged in bundles (optical cables) and transmit data via light signals over long distances – huge amounts of data super fast – data converted to electrical pulses that computer reads.

A broadband internet connection requires a modem – either cable of DSL modem which translates the broadband signal into digital data and back again – connected to an expansion card called a network interface card (NIC) which is located inside your computer – need a router to connect with more than one computer

Satellite Internet: need a satellite dish – data is transferred between personal satellite dish and the satellite company’s receiving dish

Wireless

WiMAX: designed to extend local WiFi networks across greater distances

Mobile WiMAX: alternative to cellular transmission of voice and high-speed data

To access wifi you need wireless capability built in your device

Aircard: devices that fit either into a USB prot or a special slot on the side of a notebook (express card slot) – enables users to have wireless internet access with notebooks  (require a service plan)

Concerns: public hotspots are unsecured (don’t use credit card)

Dial-up connections: needs a standard phone line and a modem

Dial-up modem: device that converts the digital signals the computer understand into analog signals that can travel over phone lines – computer on other end must have a modem to demodulate signal (usually built into system unit)

Least costly, slower but fine for casual internet users, ties up your phone line if you don’t have a separate line

How to choose?

Depending on area, may not have a choice

Speed: data transfer rate (how fast data travels between computers i.e. connection speed) – fiber-optic > DSL and cable > satellite > dial-up

Which other services you want bundled into your payment?

THE FUTURE OF THE INTERNET

More bandwidth and offer increased services, more accessible

Large scale networking (LSN): aim to fund the research and development of cutting-edge networking and wireless technologies and to increase the speed of networks

Internet2: research and development consortium of more than 300 universities that seeks to expand the possibilities of the Internet by developing new Internet technologies and disseminating them as rapidly as possible to the rest of the internet community

Internet enabled devices – homes virtually running themselves

Chapter 4: application software

Software: set of instructions that tells the computer what to do – also called a program – provides a means for us to interact with and use the computer

Application software: software you use to do tasks at home, school and work

System software: help run the computer and coordinate instructions between application software and the computer’s hardware devices – includes the operating system and utility programs (help manage system resources)

PRODUCTIVITY SOFTWARE FOR HOME AND OFFICE

Productivity software: programs the enable you to perform various tasks (word processing, spreadsheet, presentation, database, personal information manager programs

Word processing software: most widely used in application – Miscrosoft Word and Corel WordPerfect

Open source software: program code that is publicly available and has few restrictions – code can be copied, distributed, or changed (ex: openoffice.org or abiword.com) vs. proprietary software which you cannot

Web based word processing programs: googledocs, Microsoft office web apps

Advantage of word processing software: produce professional result

Special tools: translate words or phrases, automatically correct your spelling as you type, automatically summarize key points, add bibliographical references, include illustrations with different picture styles

Spreadsheet software: enables you to do calculations and numerical analyses easily (ex: excel)

In the grid consisting of columns and rows you can enter: text, values, formulas, functions

Benefit: ability to recalculate all functions and formulas in the spreadsheet automatically when values for some of the inputs change – can quickly test what-if-analysis, create variety of charts, powerful mathematical functions, databases capabilities and can sort, filter and group data

Presentation software (Powerpoint, zoho show)

Database software (oracle, MySQL, Microsoft Access) are powerful applications that allow you to store and organize data

More robust fully featured database used to manage larger and more complicated groups of data than spreadsheets

Organized into fields, records, and tables

Note taking software help students take notes during lectures and organize and maintain their lecture notes and the recordings they create from lectures

Microsoft OneNote: move pieces of text around, tabbed sections, search for a term

Online note taking applications (evernote, stickynotes)

Personal information manager software: strive to replace the management tools found on a traditional desk (calendar, address book, notepad, to-do list) (ex: Microsoft outlook or lotus)

Some PIMs contain e-mail management features

Can check people’s availability is you share a network at home or work with them

Many web based e-mail clients such as Yahoo!, Google, AOL have coordinating calendar contacts programs

Productivity software features – tools to increase efficiency

Wizard: systematic guide that walks you through the steps necessary to complete a complicated task

Template

Macro: small program that groups a series of commands so they will run as a single command

Integrated software application: a single software program that incorporates the most commonly used tools of many productivity software programs into a single integrated program (ex: Microsoft Works – word processing, spreadsheet, database functionality, templates, calendar, dictionary, map features)

Not substitutes for full programs  - good if you don’t need the more advanced features, less expensive

Software suites: group of software programs that have been bundled as a package (software, productivity, graphics, virus protection)

3 primary developers: Microsoft, Corel, Apple

Most contain word processing, spreadsheet, presentation, PIM software can also include database programs and desktop publishing software

Cheaper than buying individually, work well together, share common features

Personal Financial Software

Tax preparation software: (ex: Turbotax) enables you to prepare your state and federal taxes on your own instead of hiring a professional – must obtain a new version every year

Financial planning software helps you manage your daily finances: electronic checkbook registers, automatic bill payment tools – print checks, pay monthly payments, analyze spending patterns

Web based programs as well (mint, yodlee moneycenter)

Coordinate with tax preparation software

MEDIA SOFTWARE FOR HOME

Multimedia software: image, video, and audio editing software, animation software, other specialty software to produce computer games, animations, and movies

Digital image editing software: edit photographs and other images – tools for basic modifications (removing red-eye, contrast, etc.), some include painting tools

Adobe Photoshop and Corel PaintShop Pro Photo: fully featured image editing applications (layering images, masking images) – used to create enhanced digital images

Adobe Photoshop elements and Roxio Photosuite: casual home user

Digital audio software: novels, newspapers, radio shows, podcasting (free), radio shows, music videos, compose you own music

MP3 (MPEG-1 Audio Layer 3): type of audio compression format that reduces the file size of traditional digital audio files so that they will take up less storage capacity – also allows quick and easy distribution over the Internet

Most digital audio software programs support one of the following functions – others incorporate many (Itunes)

MP3 recording

CD ripping

CD burning

Encoding and decoding

Format conversion

Audio editing software (ex: audacity, sony sound forge pro): cutting dead air space, cutting a portion, add special sound effects, remove static or hiss

Digital video editing software

Adobe Premiere Pro and Final cut Pro (most expensive) widest range of special effects

Microsoft Live Movie Maker and Apple iMovie: simple to create professional-quality movies with little or no training

Many of the affordable video editing software packages support most types of video files

Portable media players: MP4

Media management software

(ex: Windows Media Player, iTunes) allow you to organize audio and video files so that you can sort, filter, and search your music collection by artist, album or category

Several online photo sharing and photo storing sites  (ex: Flickr – organize your images and share them with millions of users of just your close friends and family)

SOFTWARE FUN FOR HOME

Entertainment software: digital games and virtual reality programs

Gaming software: make sure computer has enough processing power or RAM and appropriate sound cards, video cards, speakers, monitor and CD or DVD drives, also some programs may require special controllers

Entertainment software rating board: rating symbols and content descriptors

Educational software

Teach users new skills, preparation software, programs to improve the health and function of our brains, programs that provide tutorials for popular computer applications, simulation programs (commercial and military flight training, machine operation)

Course management software (blackboard, moodle) provide traditional classroom tools (calendars, grade books)

Drawing software: lets you create or edit 2 dimensional, line-based drawings

Both creative and technical drawings

BUSINESS SOFTWARE FOR HOME AND OFFICE

Home business consider: accounting software, desktop publishing and web page creation tools, a number of software packages are designed to organize and help with the daily operations of a typical business

Home business software

Accounting software helps manage finances more efficiently by providing tools for tracking accounts receivable and accounts payable – also inventory management, payroll and billing tools

Desktop publishing software: incorporate and arrange graphics and text in your documents in creative ways – format text and graphics

Web page authoring software: allows even the novice to design interesting and interactive webpages without knowing HTML

Large business software

Project management software: helps project managers create and modify scheduling charts

Customer relationship management software: stores sales and client contact information in one central database

Enterprise resource planning system lets a business consolidate multiple systems into one and improve coordination of these business areas across multiple departments

Mapping programs are perfect for businesses that require employees to travel frequently – provide street maps and written directions to locations nationwide – online mapping services also available (Google maps, mapquest)

Many companies use a geographic information system to assist with managing, analyzing, and displaying data – used by power companies to manage electric grids, water distribution companies, shipping and transportation companies to determine most efficient routes

Programs for e-commerce (WebSphere, GoEmerchant) bundled web site creation and hosting services

Specialized business software

Vertical market software: software designed for a specific industry

Computer-aided design: 3D modeling to create automated designs, technical drawings, model visualizations (engineers)

GETTING HELP WITH SOFTWARE

FAQs, online help and support (user’s manual, chat with online support team)

Integrated help: documentation for the product is directly built into the software

Tutorials may be offered online by developer or you can search the internet

BUYING SOFTWARE

You purchase a license that gives you the right to use the software – not yours to lend or copy

Software license: an agreement between you, the user, and the software company – legal contract – some come with licenses for more than one computer

Multiuser license: per-seat (limit number of users overall), concurrent licenses (limit the number of users accessing the software at any given time)

Free software license: grants the recipient the right to modify and redistribute the software (copyleft) – without this you are in violation of copyright laws

Pre-installed software: every new computer comes with an operating system as well as some form of application software (productivity software, multimedia-enriched computers may offer graphics software or a productivity suite that includes page authoring software, many include image editing or financial planning software)

Having so many pre-installed programs on your system can degrade the system performance by allocating memory away from active applications and for notebooks it can reduce battery life (referred to as bloatware)

Web-based applications

Software as a service (SaaS): the application is hosted online by the vendor and made available to the customer over the Internet

Microsoft Office Web Apps, Google Docs

Advantages: you can access your files as long as you have a web browser, great for collaborations (work with others in real time)

Most are free, but some aren’t

Discounted software

Students or educators can sometimes purchase software at lower prices

Can buy software through online auction – make sure it’s a licensed copy

You can buy and download software directly from many companies and retail web sites  - you should request to be sent the CD or DVD

Freeware and shareware

Freeware: any copyrighted software that you can use for free

Postcardware and e-mailware: request that you mail them a postcard or send an e-mail to thank them and with your opinion

Open source programs: free to use but if you make any changes to improve the source code you must also distribute it for free

Be cautious, people use freeware to distribute viruses

Beta version: application that is still under development – many available for a limited trial period free of charge

Shareware: distributed for free but with certain conditions – must be registered after a certain period of time, no support available unless its registered – not freeware

No expense of marketing and advertising

Many have codes in the program to stop it from working completely or alter the output after the trial period

Freeware and shareware can possibly crash your computer if not made for your system

Beta version is rarely bug free

Make sure your virus protection is up to date

Software versions and system requirements

Numbers after software names indicate new versions

No need to always replace software – upgrades may not be sufficiently different – do you really need it?

Vendors make new versions of software backward compatible (older versions can recognize it) but some are not forward compatible

System requirements: minimum recommended standards for the operating system, processor, primary memory (RAM), and hard drive capacity to support a software (all software have them)

INSTALLING, UNINSTALLING, AND STARTING SOFTWARE

Most programs for PC are installed with an installation wizard (CD)

If you download from the Internet: everything you need to install and run the program is in one zipped file – will be saved on your computer’s hard drive

Sometimes files don’t install automatically – you need to locate the files on the hard drive (usually named setup.exe - .exe are executable files or applications)

Full installation will copy all the files and programs from the distribution disc to the computer’s hard drive

Custom installation: decide which features you want installed on the hard drive

Deleting the main file is not ridding your system of all the pieces of the program

Some programs place uninstall icon in the main program folder – or click start menu, control panel, programs and features, chose to uninstall

To start application click all programs list on start menu

Chapter 5: using system software

System software: set of programs that helps run the computer and coordinates instructions between application software and the computer’s hardware devices –consists of 2 primary types of programs:

Operating system

Utility programs

Operating system (OS): group of programs that controls how your computer system functions. Manages the hardware (processor, memory, storage devices, peripheral devices, monitor, and printer)

Coordinates the hardware, software, user interface, processor, and system memory

Many types exist: some don’t require any user intervention (appliances, car engines), proprietary systems (specifically for devices they manage)

First widely installed OS: Microsoft Disk Operating System (unfriendly), was replaced by Apple’s Mac OS and Microsoft’s Windows (allowed a single user to multitask)

Networking capabilities later added (multiuser, multitask)

Utility program: small program that performs many of the general house keeping tasks for the computer (maintenance and file compression)

TYPES OF OPERATING SYSTEMS

Real-time operating system (RTOS): programs written specifically for the needs of the devices and their functions, minimal user interaction (measurement instruments in scientific, defense and aerospace industries, car engines, printers, dryers, furnaces)

Multiuser operating system (network operating system): enables more than one user to access the computer system at one time by efficiently handling and prioritizing requests from multiple users

Users access the server at the same time (the computer on a network that manages network resources)

UNIX: multiuser multitask operating system used as a network operating system (mainly found on main-frames, can also be found on a PC)

Main-frame: responsible for storing, managing, and simultaneously processing data from all users – multiuser

Supercomputer also use multiuser OS

Most modern smartphones have modest multitasking capabilities such as checking e-mails while on a phone call

Common OS: Symbian (Nokia), Blackberry (RIM), Windows Mobile (Microsoft), iPhone OS X (Apple)

Gaming systems all require some sort of customized system software specifically developed for the particular device – includes system programs a.k.a. firmware as well as other programs that come with the personal media player or gaming device

Windows began with MS-DOS and added a user-friendly interface – Windows 95 adds multitasking capabilities – Windows XP provides networking capabilities in its consumer editions – Widows 7 & Windows Vista builds on security  - over time has increased user functionality and friendliness, improving Internet capabilities and enhancing file privacy and security

Different editions of Windows 7 for different users (home users, business, combination and 32-bit & 64-bit)

MAC OS: first OS to incorporate a graphical user interface (198)

OS depends on type of processor a computer has – combination of OS and processor is referred to as a computer’s platform

PCs and Macs can’t run the same software applications but they can be networked (share files and peripherals)

You can run more than one OS on a single computer due to the large hard drives – need to separate the hard drive into 2 different sections called partitions

Linux: open source operating system designed for use on personal computers and as a network operating system – open source software can be modified by anyone

Can be tweaked quickly to meet any new need

Reputation as a stable OS not subject to crashes or failures

Android is Linux based

Can download open source versions for free from the Internet – several versions must be purchased

WHAT THE OPERATING SYSTEM DOES

Provides a way for the user to interact with the computer

Command-driven interface: you enter commands to communicate with the computer system (too complicated for average user)

Menu-driven interface: choose commands from menus displayed on the screen (still not easy enough)

Graphical user interface (GUI) is what we use today – point and click technology, more user friendly

Linux has many available GUI interface vs. Windows or Mac OS with only one

Manages the processor

Processor needs OS to arrange the execution of all activities in a systematic way so that it appears simultaneous - OS switch among different processor millions of times/ second

OS responds to events (ex: a mouse click) and switches back and forth between them

Interrupt: signal that tells the OS that something needs immediate attention

Every device has its own type of interrupt, associated with an interrupt handler (special numerical code that prioritizes the requests)

Preemptive multitasking: OS processes tasks with higher priority before others

Spooler: program that helps coordinate all print jobs

Manages the memory and storage

Ensure that there is enough space in RAM for all the pending instructions and data – cleans the items when the processor no longer needs them

4 GB or more of RAM  can get 64-bit version of Windows

Computers now have between 2-12 GB of RAM

Virtual memory: storage by borrowing hard drive space when computer runs out of RAM – swaps out data in RAM that hasn’t been used for a while and moves them to a temporary storage area on the hard drive called the swap file and if its needed later, swaps them back into active RAM (process known as paging)

Trashing: condition of excessive paging (only a portion of the hard drive is allocated to virtual memory) – solution: increase RAM

OS has a file-management system

Manages the computer system’s hardware and peripheral devices

Device driver: special program that comes with each device attached to computer to facilitate communication between the hardware device and the OS – translates the device’s commands into commands that the OS understands

Today most devices come with the driver already installed in Windows  - called Plug and Play (PnP) created with Windows 95

If a device is not PnP you insert the driver provided or download the driver from the manufacturer’s web site

New drivers can crash a computer – Roll Back Driver from Windows removes newly installed drivers and replaces them with the last one that worked

Provides a consistent means for software applications to work with the CPU

The OS includes the blocks of code – each called an application programming interface (API) – that application software needs in order to interact with the OS

Software programmers only refer to the API code blocks when they write an application

All applications that have incorporated these APIs have similar interface features, many features of the application have the same look – applications sharing these formats can exchange data

THE BOOT PROCESS: STARTING YOUR COMPUTER

Boot process: start-up process (4 steps):

Basic input/ output system (BIOS) is activated by powering on the CPU

BIOS is a program that manages the exchange of data between OS and all the input and output devices attached to the system

BIOS stored on ROM which is not erased when powered off

The BIOS checks that all attached devices are in place (power-on-self-test or POST)

POST: test on the video card and video memory, a BIOS identification process, and a memory test to ensure that memory chips are working properly

BIOS compares results with hardware configurations stored in CMOS – if results are favorable, boot process continues, if new hardware you will be alerted

The operating system is loaded into RAM

BIOS searches for the drives that contains the system files (main files of the OS) and loads it into RAM from its permanent location on the hard drive

Then the kernel or supervisor program (essential component of the OS – manages the processor and all other components) is loaded into RAM – said to be memory resident (stays in RAM the entire time)

Parts of the OS that are less critical and are copied over to RAM on an as-needed basis are called nonresident

Configuration and customization settings are checked

OS checks the registry (contains all the settings used by the OS and other applications) for the configuration of other system component

Authentication: verification of your login name and password (ex: network environment such as a college)

Boot is complete when the system is ready to accept your first command

Handling errors

If you have a new software or hardware, try desinstalling it

Safe mode: special diagnostic mode designed for troubleshooting errors – only essential devices (mouse, keyboard, and monitor function)

Can use device manager: feature in OS that lets you view and change the properties of all devices attached to your computer

If windows detects a problem in the boot process it will add “Last known good configuration” to the windows advanced options menu – if you choose to boot with the last known good configuration the OS starts your computer by using the registry info that was saved during the last shutdown

Safe mode and last known good configuration: most widely used methods of booting when you cannot with the current configuration

If all fails, try a system restore

Sometimes BIOS skips a device (ex: keyboard) and it won’t respond after the booting, if so resolve the problem by rebooting

THE DESKTOP AND WINDOWS FEATURES

Desktop: first interaction you have with the OS and the first image you see on your monitor – you’ll find:

Recycle bin: location for deleted files from the C: drive only

Gadget: easy-to-use mini program that gives you information at a glace or quick access to frequently used tools

Taskbar: displays open and favorite applications for easy access

In Windows 7 start menu you’ll find:

Documents: keep all your documents in one place

Computer: easy access to disk drives

All programs

Both windows and mac use windows, menus and icons, both have streamlines mechanisms to access commonly used applications

Macs have a Dock and a dashboard with widgets (mini-application that enables quick access to frequently used tools and activities)

Latest version of Windows has gadgets that provide similar functionality

Windows (rectangular panes that display applications)

Toolbars: shortcuts to frequently used tabs

Scrollbars

Ribbon: further organized into task-specific tabs with relevant commands

ORGANIZING YOUR COMPUTER: FILE MANAGEMENT

File management: provides organizational structure to the computer’s content

Organizes contents in a hierarchical directory structure that includes files, folders, libraries and drives

Organizing your files:

File: collection of related pieces of info stored together for easy reference

Folder: collection of files

Library: collection that gathers files from different locations and displays them as if they were all saved in a single folder, regardless of where they are actually physically stored

Windows organizes computer content in a hierarchical structure comprising drives, folders, subfolders, and files

C drive (hard drive) where you permanently store most files

A drive: reserved for a floppy drive

Additional files represented by D, E, F and so on

C drive referred to as the root directory – areas that the OS has filled with files and folders holding special OS files (shouldn’t be accessed)

Windows explorer: main tool for finding, viewing and managing the contents of your computer

Viewing and sorting files and folders

Title views: displays files and folders as icons in list form

Details view: list form, additional file info displayed in columns alongside the name of the file

List view: smaller than in titles view

Small and medium icons views: icons in list form but icons are either small or medium

Large and extra large icons views

Windows instant search searches through your hard drive or other storage device to locate files that match criteria you provide

Naming files

File name

Extension or file type: what comes after the dot (ex: .doc, .docx, .wpd, .pptx) – added automatically for you

Files must be uniquely identified – no 2 files should have the same file name and extension

File path: start with drive and includes all folders, subfolders, the file name, and the extension

Path separator: backlash (\) between the drive, primary folder, subfolders, and file name – Mac files use a colon ( : ) – UNIX and Linux use a forward slash(/)

Working with files

Can move, delete files – to permanently delete, empty the recycle bin

UTILITY PROGRAMS

Utility programs: small applications that perform special functions – some are incorporated into the OS, some are so large and require frequent updating they are offered as Web-based services or stand-alone programs, software suites, some are freeware or shareware

Display utilities – change the appearance or your desktop

Programs and features utility

When you add a new program, wizard runs. If it doesn’t go to the programs and features utility found in the programs folder in the control panel

To delete program use the uninstall option, if not you might miss a lot of other files

File compression utilities: program that takes out redundancies in a file to reduce the file size

Windows built-in compression (zip)

Most compression programs look for repeated patterns of letters and replace them with a shorter placeholder

Placeholders are cataloged and stored temp. in a file called the dictionary

Can reduce by 50% + depending on the file – PDF already compressed, some graphics and audio formats have been reduced b permanently discarding data

Decompress the file to restore it to its original state

System maintenance utilities

Disk cleanup: windows utility that cleans, removes unnecessary files from your hard drive  (accumulated files in bin, temporary files, temporary internet file – you check off which type of files you would like to delete

Disk defragmenter regroups related pieces of files on the hard drive, allowing the OS to work more efficiently – Macs don’t have this option, but you can use iDefrag which is an external program

Error-checking: windows utility that checks for lost files and fragments as well as physical errors on your hard drive (files are lost when you save, resave, move, delete, and copy files) – can recover data but usually deletes the files that are unnecessarily taking up space

Sometimes system gets confused leaving references on the file allocation table to files that no longer exist or have been moved

Physical errors: mechanism that reads the hard drives data can no longer determine whether the area holds a 1 or 0 (called bad sectors)

Use task manager utility if a program has stopped working to check the program or to exit it

System restore and backup utilities

System restore: utility that lets you roll your system settings back to a specific date when everything was working properly (latest versions of windows)

System restore point: snapshot of your entire system’s setting (made every week), you can also create it manually

Every time you start your comp. or install a new application or driver, windows creates a system restore point – good idea to create before making changes to your computer

Windows backup and restore utility: create a duplicate copy of all the data on your hard drive and copy it to another storage device – Mac OS X Snow leopard (time machine automatically backs up your files, time capsules)

The task scheduler utility

Important to run some of these utilities on a routine basis – many ppl forget

Windows task scheduler utility: schedules task to run automatically at pre-determined times with no additional action necessary on your part

Accessibility utilities: windows includes an Ease of Access center (centralized location for assistive technology and tools to adjust accessibility settings)

High contrast

Magnifier

Narrator

On-screen keyboard

Windows speech recognition

Chapter 6 -  understanding and assessing hardware: evaluating your system

Moore’s law: the pace at which CPUs (central processing units – brain of the computer) improve. Predicts that the number of transistors inside a CPU will increase so fast that CPU capacity will double every 18 months (# of transistors helps determine how fast it can process data)

Other system components also improve – ex: the capacity of the dynamic random access memory increases abt 60%/ year, hard drives by 50%/year

CHOOSING EITHER A DESKTOP OR NOTEBOOK

If main need is Internet connectivity not processing power, don’t mind small screen and small keyboard, netbook is an option, portability

Notebook you pay more for each component (made smaller), shorter life span (dust, water, temperature fluctuations)

If you have a fast transfer port (ex: external SATA), can easily add an external hard drive

Expresscard slot: can add a solid state drive

Desktop offers more expandability options (easier to add ports), large monitors, more reliable

ASSESSING YOUR HARDWARE EVALUATING YOUR SYSTEM

System evaluation to determine if your computer has the right hardware for what you want to do: look at your computer’s subsystems, see what they do

CPU subsystem

Memory subsystem (RAM)

Storage subsystem (hard drive)

Video subsystem

Audio subsystem

Ports

EVALUATING THE CPU SUBSYSTEM

Consider the type of processor in your system

Comprised in 2 units: the control unit (coordinates the activities of all the other computer components) and the arithmetic logic unit (responsible for performing all the arithmetic calculations, makes logic and comparison decisions)

Every time it performs a program instructions it uses the same steps: fetches data from RAM, decodes it, executes it and stores the result into RAM – process is called a machine cycle

Main distinction between CPU is processing power which is determined by a number of factors

Core: complete processing section from a CPU embedded into one physical chip

Clock speed: how quickly the processor can work

Cache memory: amount of immediate access memory the CPU has

Hyperthreading: quicker processing of info by enabling a new set of instructions to start executing before the previous set has finished

2 different programs to be processed at one time, but sharing the computing resources of the chip

Multiple cores on one CPU chip – 2 or more processors on the same chip (2 sets of instructions at the same time)

Each program has the full attention of its own processing core

Faster in processing, smoother multitasking

Possible to design a CPU with multiple cores and hyperthreading

Cache memory

Level 1 cache: block of memory built into the CPU chip for the storage of data or commands that have just been used

Level 2: slightly farther from the CPU or on a separate chip (longer to access, more storage area)

Level 3: slower to reach, larger in size

Front side bus also impacts overall speed (highway between CPU and RAM)

Faster the FSB, faster you get data to the processor – important to determine CPU performance

Benchmarks: measurements used to compare CPU performance between processors – generated by running programs that push the limits of CPU performance

CPU for notebooks

Intel and AMD make these processors

Need better power savings to improve battery life

Check your CPU by accessing System Properties, find more detailed info on the manufacturer’s web site

CPU usage: percentage of time that your CPU is working

Measuring CPU and RAM usage is very useful

Whether you should upgrade

Investigating if your computer’s performance seems to drop off for no apparent reason

Upgrading CPU only affects the processing portion of the system performance, not how quickly data moves to and from the CPU, overall performance depends on many factors such as amount of RAM and hard drive speed

EVALUATING RAM: THE MEMORY SUBSYSTEM

RAM is volatile storage – remembers everything that the computer needs to process data into info when computer is on

Hard drive provides the greatest non volatile storage – permanent storage when computer is off

Way faster for CPU to retrieve data from RAM than from hard drive

Now we use double data rate 2 (DDR2) memory modules, high-performance systems double data rate 3 memory (DDR3 – operate at lower temp. more reliable)

Older models dynamic RAM (DRAM), static RAM (SRAM), synchronous DRAM (SDRAM)

RAM present on memory modules (memory cards) which are small circuit boards that hold a series of RAM chips and fit into slots on the motherboard – memory modules today called dual inline memory modules (DIMMs)

Amount of RAM on a computer is called physical memory (shown in system properties window, Apple about this mac)

RAM capacity measured in gigabytes – machines with Windows have at least 2 GB of RAM today

Windows 7 uses a memory management technique called SuperFetch: monitors the app. You use the most and loads them in RAM so its faster

Kernel memory: memory that OS uses

The more programs you use at once, the more RAM you need – buy as much as you can afford but no more than your system will handle (maximum limit of amount of RAM a computer can handle)

Adding RAM is fairly easy and inexpensive

EVALUATING THE STORAGE SUBSYSTEM

Storage options for a typical personal computer: hard drive, USB flash drives, optical drives, and external drives – nonvolatile

Hard drive: largest capacity storage device (2 terabytes), most economical (GB/ $), access time (time for stored data to be available for processing) faster than other permanent storage (milliseconds)

Solid state drive (SSD): same memory as flash drive but faster than fast drives (0.1 ms vs. 1 ms), no noise, little heat, little power, storage capacity keeps increasing, start seeing more

Data transfer rate: speed of transfer of data to other computer components (such as RAM) – performance specification for a hard drive

Hard drive composed of platters (small thin plates of metal stacked on a spindle), magnetized spots created on them when you save data (when aligned in a direction (1), in the other (0)), computer translates patterns into data when you retrieve it

You need space for all your files + storage space for OS + software applications

Types of hard drives: integrated drive electronics (IDE) is an older style that uses wide cables to connect h.d. to motherboard – Serial advanced technology attachment (Serial ATA) h.d. are thinner cables and transfer data quicker (god for software developers and graphic designers)

Latest and fastest hard drive is the solid state drive (SSD) – popular in netbooks, little power, cool and quiet, access time 0.1 ms, can be as large as 1 TB, but much more expensive than mechanical hard drives, some use SSD just to hold the OS

Adding external hard drive – use a USB 2.0 port (400 Mbps) or some offer an eSATA port (transfer rate of 3 Gbps)

One big drive or many small drives?

2 smaller drives connected using RAID technology (data is spread across 2 physical drive – RAID 0), twice as fast as using one drive, if one drive fails you lose all your data (good for performance)

RAID 1 (data on one drive is mirrored on 2nd drive), instant backup

OPTICAL STORAGE

Optical drive: disc drive that uses a laser to store and read data saved on a CD, DVD, or blue-ray disc – store data as tiny pits that are burned into the disc by a high-speed laser

Pits on BDs (0.1 microns in diameter) < CDs and DVDs (less than 1 micron)

Pits and nonpits (lands) translate into 1s and 0s

Laser used to read and write data

CD-ROM, DVD-ROM, BD-ROM are prerecorded discs, cant save data on them CD drive cant read DVD but DVD can read CD, Blu-ray players can play DVDs and CDs same thing for recording

DVD formats: DVD-R/RW, DVD+R/RW, DVD-RAM

Purchase blank DVD that matches the type of drive you have

Most new DVD drives support the +/- RW

CD and DVD drive speed:

Speed is listed recording speed, rewrite speed (except CD-R and DVD-R can’t rewrite), playback speed (ex: 52X32X52X)

For CDs, the X represents the transfer of 150 KB of data/ sec.

DV drives faster than CD drives, blue ray fastest (1X = 36 MB/ sec.)

EVALUATING THE VIDEO SUBSYSTEM

How video is displayed depends on your video card and your monitor

Video card: expansion card that is installed inside your system unit to translate binary data into the images you view – include own RAM called video memory

Standards of video memory: graphics double data rate 3 (GDDR3), newer GGDR5

Video cards also have their own graphics processing units (GPUs)

GPU: same kind of work as CPU, specialized in 3D graphics and image and video processing at high efficiency and speed

Basic video processing on motherboard

Sophisticated video cards connect through the PCI express bus

Info on video card: advanced settings of screen resolution dialog box

How much memory: basic word and web (128 MB – 1 video card), serious gamer (512 MB – 2 or 3 video cards)

Controls # of colors monitor can display

Bit depth: # of bits the video card uses to represent each pixel on monitor (defines color quality, the more the better)

Most today are 24 bit cards – 16 million colors

Upgrade if (add a new video card to the slot on motherboard):

Monitor takes a while to refresh

Added features are important to you

Want to use multiple monitors

EVALUATING THE AUDIO SUBSTYSTEM

Sound card: expansion card that attaches to the motherboard inside your system unit – most systems have separate sound cards, some low end computers have integrated sound cards

3D sound card: basic sound card, more omnipresent than stereo sound (can tell if coming from left or right ear) but its not surround sound

Surround sound: type of audio processing that makes the listener experience sound as if it were coming from all directions

Standard from Dolby (Dolby Digital Ex, Dolby Digital Plus), newest standard Dolby TrueHD

Dolby takes digital sound from a medium, reproduces it in eight channels, 7 cover the listening field and 1 holds low-frequency sound data

Need surround sound speakers, and a sound card that is Dolby digital compatible

If you want to add a right and left speaker individually or any other audio devices you need more ports provided on an upgraded sound card

Then you can connect portable media players, jukeboxes, headphones, CD players

EVALUATING SYSTEM RELIABILITY

Even normal use of a computer can lead to deteriorating performance and even system failure

Ensure your system is reliable:

Clean out your startup folder: delete only unnecessary programs

Clear out unnecessary files

Run spyware and adware removal programs

Run the disk defragmenter utility on your hard drive

Disk cleanup, disk defragmenter and the antivirus and spyware can run automatically – you can set up a sequence to run for ex. When you sleep

If system crashes and rebooting doesn’t help, you need to troubleshoot:

Check if you have enough RAM

Make sure you have properly installed new software or hardware

If you see an error code in windows visit the Microsoft knowledge base

Should upgrade or update your OS, browser software, and application software – makes your system much more reliable

You can configure windows so it automatically checks for downloads and installs updates,

If nothing helps? Options:

Upgrade your operating system to the latest version – But may need hardware updates (more RAM), updated graphics processor, larger hard drive

Reinstall the OS – back up all files, need all of original discs for software

MAKING THE FINAL DECISION

Does your system meet your needs?

What would you like your computer to do?

Deciding between upgrading or buying a new system is better, price both scenarios

SEVEN: NETWORKING, CONNECTING COMPUTING DEVICES

Network: 2 or more computers that are connected via software and hardware so that they can communicate with each other

Node: each device connected to a network

Advantages: resource sharing, share peripheral devices, share files – can access files on any other computer

Disadvantages: need additional equipment, need to be administered

Network administration involves: installing new computers and devices, monitoring the network, updating and installing new software, configuring security

NETWORK ARCHITECTURES

Network architectures: design of a network – classified by the way they are controlled and distance between their nodes

Describing networks based on administration: administered or managed locally or centrally

Local administration: configuration and maintenance of the network must be done on each computer attached to the network (ex: peer-to-peer network)

Central administration: tasks can be performed from one computer and affect the other computers on the network (ex:  client/ server network)

Peer-to-peer network: each node can communicate directly with every other node on the network (up to 10 nodes)

Client/ server network: contains 2 different types of computers; clients and servers

Client: computer on which users accomplish specific tasks and make specific requests

Server: computer that provides information or resources to the client computers on the network and central administration

P2P networks are more often used in home vs. client/server

Home network server: designed to store media, share media across the network, and back up files on computers connected to the network

All computers can access

Doesn’t make it a client/server network – all network administration must still be performed locally

Describing networks based on distance:

Local area network (LAN): network in which the nodes are located within a small geographic area (ex: school)

Home area network (HAN): network located in a home

Wide area network (WAN): made up of LANs connected over long distances (ex: 2 campuses of a school)

Metropolitan area network (MAN): network that provides access to a specific geographical area

NETWORK COMPONENTS

To function networks must include

Means to connect the nodes on the network (cables, wireless)

Devices that allow the nodes to communicate with each other and send data

Software that allows the network to run

Transmission media: establishes a communications channel between the nodes on a network and can either be wireless (radio waves) or wired (cables)

Twisted-pair cable: copper wires that are twisted and surrounded by a plastic jacket

Coaxial cable: single copper wire surrounded by layers of plastic

Fiber-optic cable: plastic or glass fibers that transmit data at extremely fast speeds

Different types of transmission media transmit data at different speeds

Data transfer rate (bandwidth): maximum speed at which data can be transmitted between 2 nodes on a network (Mbps)

Twisted-pair cable, coaxial cable, and wireless media usually provide enough bandwidth for home networks

Throughput: actual speed of data transfer that is achieved  (Mbps)

Network adapters: devices connected to or installed in network nodes that enable the nodes to communicate with each other and to access the network (today – inside the device - called network interface card)

Most NICs built to use wireless media but many also wired media

Most notebooks have a port that accommodates cable for a wired connection – why would you want that?

Wireless signals more prone to interference from magnetic and electrical source

Interference from other wireless networks

Building materials that decrease throughput

Throughput varies on distance from your network equipment

When you want highest throughput you may want to connect using a wire

Network navigation devices: facilitate and control the flow of data through a network

Packet: bundle of data

Must be able to flow between network nodes for computers to communicate

Router: transfers packets of data between 2 or more networks (ex: router sends data between home network and internet)

Switch: receives data packets and sends them to their intended nodes on the same network

Home network need OS that supports P2P networking (windows, Os X, Linux)

C/S networks are controlled by centralized servers that have network operating system (NOS) software (handles request for info, internet access, use of peripherals for the rest of the network nodes)

C/S nodes communicate through server – more efficient but requires more complex NOS software than P2P networks

HOME ETHERNET NETWORKS

Ethernet network: uses the Ethernet protocol as the means by which the nodes on the network communicate (dev. By institute of electrical and electronic engineers)

Extremely efficient at moving data

Use wireless and wired transmission media

IEEE develops standard specifications for electronic data transmission (802.11 for wireless, 802.3 for wired)

Current standard for wireless is 802.11n

802.11 = WiFi – different standard: 802.11a, b, g, n

Backward compatibility to networks for devices using older standards

Transceiver: device that translates the electronic data that needs to be sent along the network into radio waves and then broadcasts these radio waves to other network nodes, also receive them

Multiple input multiple output (MIMO): devices that use multiple antennas for transmitting and receiving data

Multiple data streams, faster transmission

Throughput of 802.11n devices is around 300 Mbps

Net Meter shows you your throughput: 50-200 Mbps on wireless network is sufficient, if not consider wired Ethernet connection

Gigabit Ethernet: most commonly used wired Ethernet standard – 1,000 Mbps throughput is possible

Wired Ethernet uses cables vs. radio waves

Unshielded twisted-pair (UTP) cable – most popular media option, need to use RJ-45 connectors

3 main types: Cat 5E – cheapest 100 Mbps, Cat6 is the best choice, Cat 6a ultrafast 10 gigabit Ethernet but unnecessary

Can’t exceed 100 meters

One Ethernet network can support nodes with both wireless (notebooks, smartphones) and wired connections (TV, blueray)

Ethernet networks are:

Based on a well-established standard

Easy set-up

Good throughput for home networking needs

Are cost effective

HOME ETHERNET EQUIPMENT

Most manufacturers make devices that are a combination of routers and switches called routers or broadband routers

People buy routers with wireless capacities

Usually have 3 or 4 Ethernet ports, need more buy a switch plug it into the port it will give you additional ports

A switch in an Ethernet network acts like a traffic signal (helps data packets find their destinations without running into each other)

The more devices transmitting data on a router, the smaller portion of the router’s bandwidth each device receives

ISPs offer devices that combine a broadband modem and a wireless router (one monthly fee)

You want a router that supports 802.11n

Should be connected to your broadband modem by an Ethernet cable

Work with windows or OS X – some routers optimized for apple (Apple Airport)

Set up:

Access router from web browser by entering router’s IP address

You’ll need username and password (in documentation)

Run the wizard

Connecting network nodes: computers need wireless network interface cards to communicate with wireless media on 802.11n network

Check what network adapter your windows computer has and to see if its working use device manager

CONNECTING OTHER DEVICES TO NETWORKS

Many peripheral devices (scanners, printers) come with built-in Ethernet adapters, many home entertainment devices as well (network-ready)

Network ready device: device that can be connected directly to a router instead of to a computer on the network

Network attached storage devices (NAS): computing devices designed to store and manage your data – centralized data storage and access

Software on computers to ensure that data saved on computer is also stored on NAS

Time Capsule (Apple) wireless router combined with a hard drive for facilitating backups of all computers connected to the network

More sophisticated NAS device is a home network server: provide a specific set of services to computers on a home network (ex: Acer Aspire easyStore server)

Automatically back up all computers connected to the network

Act as a repository for files to be shared across the network

Access gateway to allow any computer on the network to be accessed from a remote location via the internet

Digital entertainment devices on a network to access and share digital content

Purchase more content (movies, videos, music)

Play multiplayer games

Content downloaded or streamed (ex: Netflix)

Network ready TVs and home theaters (wired or not)

Internet appliance: easy access to the Internet, social networking sites, e-mail, video, news and entertainment – between smart-phones and computers (ex: iPad)

Digital picture frames, monitoring cameras (indoor and out) connect to network (ex: Logitech Digital video security system)

SECURING WIRELESS NETWORKS

Firewall: hardware or software solution that helps shield network from intruders

Piggybacking: connecting to a wireless network without the permission of the owner which can slow you down

Precautions to secure a wireless network:

Change your network name (service set identifier) – hackers know default network names and codes

Disable SSID broadcast – more difficult for a hacker to detect your network and hard for neighbors to connect to it

Change the default password on your router – hackers know the default passwords and if they access your router they can break into your network

Turn on security protocols such as wired equivalent privacy (WEP) or WiFi protected access (WPA), you create a security encryption key

Implement media access control – network adapters have a unique # called a media access control address – you restrict access to the network to only certain MAC addresses

Limit your signal range – cut your power to low or medium

Apply firmware upgrades

CONFIGURING SOFTWARE FOR YOUR HOME NETWORK

Before running any wizards you should:

Make sure there are network adapters on each node

For any wired connections plug all the cables into the router, nodes, and so on

Make sure your broadband modem is connected to your router and that the modem is connected to the internet

Turn on your equipment in the following order

Your broadband modem

Your router

All computers and peripherals

Ensure that network discovery is shown as on (allows computer to locate other computers and peripherals

Verify options for file and printer sharing and public folder sharing are shown as “on”

Network and sharing center- set up a new connection or network

Select connect to the internet wizard to use broadband modem to connect to the internet

Enter a memorable name and check box to allow others to use internet connection

Run the set up a wireless router wizard

Set up computer with newest version of windows first

Each computer on a network needs a name so network can identify it and know which computer is requesting services and data can be delivered to correct computer

Macs: much easier, need to know the SSID and passphrase

Networks with SSID broadcast turned off won’t appear when booting the Mac, click other button, enter the SSID name and security passphrase, click join, click the remember this network

Wireless node configuration

Wired: plug a cable into the device and router

Wireless: usually set up steps in setup menu of device

Websites to test speed of internet connection

Troubleshooting network problems

Max range of wireless devices under the 802.11n standard: 350 ft.

Walls, floors, metal objects can cause interference with wireless signals

Move the node to get adequate throughput, if not move it closer to router, if not add:

Wireless range extender: device that amplifies your wireless signal to get it out of parts of your home that are experiencing poor connectivity

CHAPTER 9 – SECURING YOUR SYSTEM

KEEPING YOUR DATA SAFE

Cybercrime: any criminal action perpetrated primarily through the use of a computer (cybercriminals)

Fraud, computer intrusion, unsolicited e-mail, child pornography

Virus: computer program that attaches itself to another computer program (host program) and attempts to spread to other computers when files are exchanged

Many designed to gather sensitive information such as credit card numbers

Theft of computer equipment on the rise (larceny)

COMPUTER THREATS: COMPUTER VIRUSES

Engineered to evade detection – hide within the host program

Any device can be affected (phone, iPad)

Main purpose: replicate itself and copy its code into as many other files as possible – objective ranges from playing annoying messages to destroying files or the contents of the hard drive

Exposition with a file infected with the virus – if you never expose your computer to new files it won’t be infected (impossible). Common infection sites:

Downloading peer to peer file sharing (music)

Shared flash drives

E-mail

Passed from one computer to the next:

An individual writes a virus program disguised as a music file of a popular music group’s hit song and posts it to a file sharing site

Someone downloads the music file – infects his computer

Sends someone else the file via e-mail – infects 2nd computer

Person B saves MP3 file to flash drive and then copies it to his computer and infects that machine

Everyone who copies files from person B’s computer or is networked to that computer is at risk

Types of viruses

Boot-sector viruses: replicates itself into a hard drive’s master boot record (program that executes whenever a computer boots up, ensuring that the virus will be loaded into memory immediately, even before some virus protection programs can load)

Often transmitted by a flash drive left in a USB port

Comp. boots up with flash drive connected, tries to launch master boot record from flash drive (trigger for virus to infect)

Logic bombs: virus that is triggered when certain logical conditions are met (opening a file or starting a program a certain number of times) & Time bombs: virus triggered by the passage of time or on a certain date

Range from annoying messages to complete data loss

Worm: attempts to travel between systems through network connections to spread an infection – waits until file is executed on another computer to replicate

Much more active in spreading itself

Script: miniprogram that is executed without your knowledge (some are legit) malicious ones infect your computer & Macro virus: attaches itself to a document that uses macros (Word, Excel)

E-mail viruses: use the address book in the victim’s e-mail to distribute the virus

Encryption viruses: when they infect your computer, they run a program that searches for common types of data files and compresses them using a complex encryption key that renders your files unusable. Then you receive a message that asks you to send money to an account if you want to receive the program to decrypt you files.

Easier to catch perpetrators – trace bank account

Virus classifications (methods they take to avoid detection)

Polymorphic virus: changes its code to avoid detection

Multipartite virus: infects multiple file types in an effort to fool the antivirus

Stealth viruses: temp. erase their code from the files where they reside and then hide in the active memory of the computer

Now antivirus scans also memory

COMPUTER SAFEGUARD: ANTIVIRUS SOFTWARE AND SOFTWARE UPDATES

Best defense against viruses

Symantec, Kaspersky, AVG, McAfee – software packages available to protect you from other threats as well

Run a scan at least once a week

Virus signature: portion of the virus code that is unique to a particular computer virus (antivirus looks for virus signature in files)

When detected, virus is placed in secure part of hard drive so it won’t spread (quarantining), then choice of deleting or repairing file (can’t always – keep backup copies)

Inoculation: antivirus records key attributes about files on you computer and keeps these statistics in a safe place on your hard drive – compares files while scanning to attributes to detect attempts by virus to modify files

Catch known antivirus and scan for virus-like activities and virus signatures

Can be attacked and antivirus doesn’t recognize it

Keep software up to date

If you have a virus: boot up comp. using antivirus installation disc

Attacks can happen from IM. To keep IM safe:

Allow contact only from users on your friend list

Never automatically accept transfers of data

Avoid using instant messaging programs on public computers

Drive-by-download: malicious web sites set up to attack your computer by downloading harmful software onto your computer

Option for updates

Install updates automatically

Download updates but let me choose whether to install them

Check for updates but let me choose whether to download and install them

Give me recommended updates

Microsoft update – ensures you receive updates for other Microsoft products

COMPUTER THREATS: HACKERS

Hacker: anyone who unlawfully breaks into a computer system (individual or network)

White-hat hacker: someone who breaks into systems just for the challenge of it

Black-hat hacker: use knowledge to destroy information or for illegal gain

Script kiddies: use tools created by skilled hackers to wreak the same havoc as professional hackers

Credit card information can reside on your hard drive, hacker can access information online by using a packet sniffer or a keylogger (program that captures all keystrokes made on a computer)

Packet: small pieces of data that travel together the Internet – identified with an IP address

Packet sniffer: computer program deployed by hackers that looks at each packet as it travels

Some capture all packets, others just some with particular info (cc #s)

Wireless networks vulnerable

Install a firewall and use data encryption to protect yourself

With cc # hacker can purchase items or sell # to someone who will

Identity theft: someone using personal information about you to assume your identity for the purpose of defrauding others

Trojan horse: program that appears to be something useful or desirable (ex: game) but while it runs does something malicious in the background

Often installs backdoor program: allows hackers to take almost total control of your computer without your knowledge – computer is referred to as a zombie

Denial-of-service (DoS) attack: legit user is denied access to a computer system because hacker is repeatedly making requests of that computer system through a zombie

Distributed denial-of-service attack: launches DoS attacks from more than one zombie at a time (so you can’t trace it)

Botnet: large group of software programs that runs autonomously on zombie computers

DDoS attacks can be financially distressing for owners of web sites

How hackers gain access: direct or indirect

Direct: sitting down at a computer and installing hacking software

Indirect: through internet connection

Logical ports: virtual communications gateways or paths that allow a computer to organize requests for information from other networks or computers

Open logical ports invite intruders

Take precautions to restrict access to your logical port

RESTRICTING ACCESS TO YOUR DIGITAL ASSETS

Firewall: program or hardware device designed to protect computers from hackers – personal firewall (home network)

Close open logical ports

Types of firewalls:

For maximum protection use software and hardware firewall

Many security suites (Norton internet security, McAfee Internet security) also include firewall software

If you are using a security suite, you should disable the firewall that came with your OS

Hardware firewall devices – many routers include firewall protection (good for novices)

Websites exist to test your computer’s vulnerability

Bluetooth vulnerability (2):

Bluesnarfing: exploiting a flaw in the Bluetooth access software for the purpose of accessing a bluetooth device and stealing the information contained on it

Bluebugging (more difficult, more dangerous): hacker takes control of a Bluetooth-enabled device. Can make phone calls, establish internet connection, read messages

Protecting from Bluetooth attacks:

Make your device invisible to unauthorized devices

Antivirus available for mobile devices

Password protection and password management

Password + ID: major way to restrict access to computers, networks and online accounts

Password cracking programs more sophisticated than ever, important to create a secure password

Strong password

At least 14 characters (numbers, symbols, upper- and lowercase letters)

Should not be a single word or any word found in the dictionary

Ideally it should be a combination of several words with strategically placed uppercase characters

Password should not be easily associated with you

Use a different password for each system

Never tell anyone your password or write it down

Change your password on a regular basis (monthly)

Online password strength testers exist, so do password generators

Managing your passwords: remembers passwords for you

Most security suites provide password-management tools – master password that fills in remembered passwords for different websites

Anonymous web surfing

Tools set up on public computers, you leave traces

Google chrome, firefox, and internet explorer include privacy tools to surf the internet anonymously (no records of web sites you visit, files you download, and temporary files delete right after)

Portable privacy devices (ex: Ironkey): plug device into USB port, all sensitive internet files will be stored there which is encrypted

Take Linux OS with you on a flash drive and avoid using public computer’s OS – reduces risk of infection by malware, attacks on Linux far less likely

Biometric authentication device: reads a unique personal characteristic such as a fingerprint or the iris pattern in your eye and converts its pattern to a digital code

MANAGING ONLINE ANNOYANCES

Malware: software that has a malicious intent (3 forms – adware, spyware, viruses)

Adware: software that displays sponsored advertisements in a section of your browser window or as a pop-up

Pop-up blockers

Spyware: unwanted piggy back program that usually downloads with other software you want to install from the internet – transmits info about you to the owner of the program so that the info can be used for marketing purposes

Keystroke logger: monitors keystrokes with intent of stealing passwords, login IDs, CC info

Many security suites include antispyware software

Stand alone programs also

Update and run your spyware frequently

Spam: unwanted or junk email

Find your email from a list they purchase or from software that looks for e-mail addresses on the internet

Avoid spam:

Create a free web based email for purchases and stuff, easier to get rid of

Spam filter: check incoming email subject headers and sender’s addresses against databases of known spam

Before registering to a site, read its privacy policy to see how it uses your email address

Don’t reply to spam to remove yourself from the spam list

Subscribe to an email forwarding service such as emailias

Cookies: small text files that some web sites automatically store on your computer’s hard drive when you visit them – assigns ID to your computer, when you visit website again more efficient and better geared to your interest

Information: your browsing habits, product’s you’ve looked at – identify user preference

Some sites sell the personal info

PROTECTING YOURSELF FROM YOURSELF

Info to never reveal online: social security number, phone number, street address – key to stealing identity

Change privacy settings on FB, set options to only friends

Backing up data – 3 threats to your data: unauthorized access, tampering, destruction

Backups: copies of files that you can use to replace the originals if they are lost or damaged (store away from computer)

2 types of files need to be backed up:

Program file: used to install software and usually comes on CD or DVD or downloaded from web

Data file: file you have created or purchased

Incremental backup (partial backup): backing up only files that have changed or been created since the last backup was performed

Image backup (system backup): all system, application, and data files are backed up

Back up data frequently – you can perform image backups on a less frequent basis (weekly) but data files on a daily basis

Store all data files in one folder on hard drive

Where do backups reside?

Online sites: data available everywhere (windows – skydrive, non-windows – Adrive). Storage backup probably wont fit. Fees can be expensive.

Local drives: popular for backups of data files and for complete image backups. Best in conjunction with an online backup strategy

Network-attached storage devices and home servers: large hard drives that are connected to a network of computers instad of one computer, and they can be used to back up multiple computers simultaneously

Windows 7 backup and restore utility: schedule file backups, restore files from backups, perform image backups

Mac OS X: time machine feature detects when an external hard drive is connected to the computer or a NAS device is connected to your network

Social engineering: any technique that uses social skills to generate human interaction that entices individuals to reveal sensitive information

Pretexting: creating a scenario that sounds legit enough that someone will trust you

Phishing: lures internet users to reveal personal info that could lead to identity theft

Pharming: malicious code is planted on your computer that alters your browser’s ability to find web addresses (ex: enter banks web site but fall on website that looks like your banks and takes all your info)

Never directly reply to email asking for info

Never click on a link in email to go to a web site

Never give info over internet unless you know site is secure

Have internet security software installed on your computer, make sure its constantly being updated

Never use your cc # when you shop online: use virtual account numbers

Hoax: attempt to make someone believe something that is untrue

PROTECTING YOUR PHYSICAL COMPUTING ASSETS

Power surge: excess of normal voltage

Surge protector: device that protects your computer against power surges

Metal-oxide varistors bleed off excess current during minor surges a

Major surge: fuse infide the protector blows, stops

Whole-house surge protector: usually need electrician

Uninterruptible power supply: device that contains surge protection equipment and a large battery – keeps sending power even during blackout

Deterring theft

Motion alarm on notebook

Chaining computer

Tracking devices  - not visible to thieves

Bomb software: destroys data on both internal memory and external data if repeated attempts are made to crack passwords