Bearing

Bearings are built so as to support different loadings cases:

- Radial loads (竖向负载)

- Axial loads (轴向负载)

- Both radial and axial load

Ball bearings: Used for low & medium loads

Roller bearings: Used for high loads, large diameter shafts and reducing bearing volume

Ball Bearing

- Single row deep groove(凹槽) ball bearing

- Groove depth and conformity to ball diameter enables carrying of some axial loading in both axial directions.

- But not for very large amplitude along with the radial loads

- Double row ball bearing

- Much higher loads can be supported, for a relatively low increase in length

- Better packaging, but whole bearing to replace if fails

- single row Slot-fill(夹槽) ball bearing

Can support large radial loads, but cannot support axial loads a all

- Angular contact ball bearing

- Can support both radial and axial loads.

- Some cases axial loading may be two times the radial load

- Thrust(推力) ball bearing

- Can support large axial loads, but support no radial loads

- Self-aligning ball bearings

The rotation axis of the assembly made of the inner ring, balls and retainer can itself rotate relatively to the rotation axis of the outer ring; lending alignment freedom to rotation axis.

Roller bearing

- Single row cylindrical roller bearings

- Double and multiple row cylindrical roller bearings

- Needle bearings

- Conical(圆锥) roller bearing

- Capable of carrying combined load if rollers are inclined

- Spherical(球面) roller bearings

- Conical bearing with spherical rollers

- Rod ends: Enables use of bearings in linkages