Shaft

- Driven shaft: transmits torque from a motor to a machine or component of a machine

- Countershaft: supports components (gears, pulleys, sprockets) and transmits torque between these components.

- Axle(轮轴): stationary or rotating shaft that does not transmit torque; serves only to support/position components.

Mount gears on shafts

Transmit low torques:

1. Press fit   2.Thermal expansion  3.Set screw

Transmit medium torques:

- Keys – parallel, square or rectangle

- Gib-head(勾头) keys and other tapered keys

- Key is better secured

- Machining of hub is more difficult

- Vibrations at higher ration speed

- Woodruff keys(半圆)

- Used for improving concentricity

- Best keys for high rotation speeds

- Cotter pins, shear pins, conical pins

Best way of transmitting power

Splines: Suitable for transmitting high to very high torque, high to very high speeds, and anything below; but more expensive to make.

Disassemble gears: Gear extractors

Axially(横向) mount gears on shafts:

- Circlips internal and external

- Nuts also need along with changes in diameter

Support shafts:

- Bushings

- No mobile parts

- Used for large load, low speed

- Economical assemblies

- Bearing

Join two shafts – Coupling

- Chain coupling

- Double slider coupling

- Flexible coupling

- Universal joint