Screw

Power transmission screws are used for transforming a rotation motion into translation motion(平移).

- Transfer power

- Effect a linear positioning

Regular contact

Ball screws

ACME regular 29° & trapezoidal ISO:

General purpose

ACME 29° wide:

High loads, or limited space justifying a less deep thread

ACME 60° wide:

Some special machine tool applications

Square and modified square:

Some precision applications

Circular thread, simple or complex:

For use with balls

The loading to the screw should be axial co-linear, no lateral

Assembly screw:

How to ensure that the nut(螺帽) is held in place on a bolt(螺栓), i.e. the assembly resists vibrations?

- Use a second nut

- Nut with deformable ring, most of the time made of polymer: Lock nut

- Locker washer (垫锁圈)

- Cotter pin

- Belleville(碟形) springs

- Adhesiv(粘合剂)

When a bolted assembly (bolt + nut) is loaded in tension

- Bolt(螺栓) can fail in tension, if engaged length of thread > radius at root(螺栓半径)

- Threads can fail in shear, if radius at root > engaged length of thread

Failure in bolt have maximum load that can be withstood