Assignment #2: Feasibility Report

Submission Date: 2/16/2017

Statement of originality:

I certify that this submission is my original work and meets the Faculty’s Expectations of Originality.

Date: 16/2/2017

CMAPP ANALYSIS:

| Context | Technical explanation. |
| --- | --- |
| Message | This report is prepared to recommend using a better alternative material for clay bricks so that the company benefits from less time and money spent on wall construction. |
| Audience | Consultant Engineers Company. |
| Purpose | I Request the consultant company to consider results achieved by this report, in designing the project and make alterations needed in the design. |
| Product | Replacing clay bricks with Autoclaved Aerated Blocks (AAC) for wall construction in Rosemont Library project. |

Grand Construction Company

MEMORANDUM

FROM: 	Seyedhossein Noorbakhsh, Construction Manager

TO: 		Ray Harkala, Senior Consultant Engineer

DATE:		 February 14, 2017

SUBJECT: 	Feasibility of Using Autoclaved Aerated Concrete (AAC) Blocks for Wall Construction

Introduction:

The purpose of this document is to compare and contrast two construction materials being considered for wall construction of Rosemont Library project. The report draws conclusions about choosing the best material suited for the company’s budget. I have investigated the feasibility of using Autoclaved Aerated Concrete (AAC) blocks to replace traditional red clay bricks.

Regarding the project characteristics, as the company aims to construct a library, red clay brick is not a proper material as it is not a good sound insulator nor a good fire resistance material. Moreover, the process of constructing a brick wall is time-consuming which prolongs the total duration of the project.

Some companies are now replacing red clay bricks with AAC blocks. These blocks are not only an excellent sound and fire insulator but also because of their bigger size (about 17 times larger effective surface area than red clay bricks), they require less time of construction which helps the company saving time. Besides, due to less density (about one-third of clay bricks), AAC blocks reduce the dead weight loads on the structure, leading to savings in steel and concrete, so they significantly reduce the cost of the project.

The table below shows a detailed comparison of AAC Blocks vs. Clay bricks [1]:

Evaluation of advantages of using AAC blocks:

The three benefits of using AAC blocks I evaluated are insulation properties, time savings, and cost benefits.

Insulation properties:

One of the most important traits of a good library is to be well-insulated from outside environment, so a reader enjoys a noise free place. AAC blocks have excellent Sound reduction index (i.e. a very low index of A0), while clay brick is known as a moderate sound insulator material [2].

Moreover, AAC provides a high fire resistance meets the most stringent fire safety criteria. This kind of material can withstand fire up to 1200°C. Cellular air pockets existing in AAC block make it an excellent thermal insulator. Giving a comparison, the thermal transmittance of AAC is 60% of that of clay brick [2].

Time Savings:

AAC block is very easy to use and manipulate because it can be easily cut by wood-cutting saws while cutting a clay brick requires a cutting machine [2].

In comparison to clay bricks, AAC is also quicker-to-lay because of two factors: Firs, because it comes in larger sizes, fewer blocks are needed per installation, reducing the installation time significantly. Second, fewer joints, lessen the amount of masonry required and consequently it cause lowering time-to-finish [2].

Cost benefits:

Since AAC blocks weigh almost 80% less than clay bricks, they significantly reduce the dead weight of the structure which results in a costs savings up to 30% attributed to the reduction of steel and concrete use. Significant cost savings are also achieved due to a thinner structure of AAC blocks that results in increasing the overall carpet area of the structure [2].

Another aspect of using AAC block with regards to cost benefit is its less total labor hours needed to finish the job. As described before, due to its ease of use, AAC blocks require less time to complete the wall construction; consequently, less money would be spent on labor expenditures [2].

Conclusion:

All in all, using AAC blocks as an alternative for clay bricks appears to be a right choice for our company. Using these blocks can save us time and money and on the other hand, they seem to be better insulators which make them perfect for using in a library project.

I recommend that before starting the execution process we consider replacing clay bricks with AAC blocks and redesign the structure.

References:

[1] https://www.quora.com/Our-architect-has-suggested-we-use-AAC-blocks-autoclaved-aerated-concrete-instead-of-traditional-red-clay-bricks-He-has-cited-a-lot-of-benefits-of-AAC-blocks-Is-the-architect-correct-Should-we-use-AAC-blocks-or-should-we-stick-to-red-bricks/

[2]            https://autoclaveblocks.wordpress.com/advantages-of-aac/