Question 1

1 out of 1 points

A 2.2 kΩ resistor has 4 mA of current passing through it. The resistor dissipates

Selected Answer:

Correct 35.2 mW

Answers:

4 mW

35.2 uW

Correct 35.2 mW

35.2 W

Question 2

1 out of 1 points

A series circuit consists of three resistors with values of 1.2 kΩ, 2.2 kΩ, and 3.3 kΩ across a 15-V source. The current through the 2.2 kΩ resistor is

Selected Answer:

Correct 2.24 mA

Answers:

10 mA

Correct 2.24 mA

2.24 uA

2.24 A

Question 3

1 out of 1 points

A series circuit consists of three resistors with values of 200 Ω, 470 Ω, and 330 Ω. The total resistance is

Selected Answer:

Correct 1,000 Ω

Answers:

10 kΩ

Correct 1,000 Ω

less than 200 Ω

200 Ω

Question 4

1 out of 1 points

How much voltage is dropped across R2 and R3 in Figure 4-1 if R1 = 4.7 kΩ, VR1 = 10 V, R2 = 4.7 kΩ and R3 = 4.7 kΩ?

Selected Answer:

Correct VR2 = 10 V, VR3 = 10 V

Answers:

Correct VR2 = 10 V, VR3 = 10 V

VR2 = 4.7 V, VR3 = 10 V

VR2 = 14.7 V, VR3 = 14.7 V

VR2 = 10 V, VR3 = 4.7 V

Question 5

1 out of 1 points

The formula for total resistance (RT) in a series circuit is which of these?

Selected Answer:

Correct RT = R1 + R2 + R3

Answers:

RT = ( R1 X R2) / ( R1 + R2)

Correct RT = R1 + R2 + R3

RT = R1 X R2 X R3

RT = R1 - R2

Question 6

1 out of 1 points

The sum of the voltage drops in a series circuit equals the:

Selected Answer:

Correct Source voltage

Answers:

Current in circuit

Total resistance

Correct Source voltage

Resistance in branch

Question 7

1 out of 1 points

What is the total resistance in Figure 4-1 if R1 = 10 kΩ, R2 = 10 kΩ and R3 = 15 kΩ?

Selected Answer:

Correct 35 kΩ

Answers:

Correct 35 kΩ

0 kΩ

25 kΩ

infinite Ω

Question 8

1 out of 1 points

When a 12-volt battery is connected to a series circuit with three resistors of 1 ohm, 3 ohms, and 2 ohms, how many amperes of current will flow through the circuit?

Selected Answer:

Correct 2

Answers:

Correct 2

1

3

4

Question 9

1 out of 1 points

When the resistance and voltage are known, what is the formula for finding the current (amperes)?

Selected Answer:

Correct I = V/R

Answers:

I = R/V

V = I x R

Correct I = V/R

I = V x R

Question 10

1 out of 1 points

kilo (k) means which of these?

Selected Answer:

Correct

1000

Answers:

100

1

10

Correct

1000

Sunday, September 24, 2017 6:15:29 PM EDT