[image: image1.png]N

[

uOttawa

L’Université canadienne
Canada’s university




Université d’Ottawa









University of Ottawa

Faculté de genie









Faculty of Engineering

Département de











Department of 

génie chimique et biologique 






   Chemical and Biological Engineering
CHG 2317 (Sep. – Dec. 2017)

Introduction to Chemical Process Analysis 

Tutorial 9

Two liquid phase mixtures of water and ammonia are combined, as shown in the attached diagram.  Stream #1 contains 10 wt% NH3 and has a temperature of 155(F and a flow rate of 100 kg/h.  Stream #2 contains 52 wt% NH3 and has a temperature of 20(F and a flow rate of 100 kg/h.  The combined stream #3 flows through a heat exchanger.  The heat exchanger increases the enthalpy of Stream #3 by 500 Btu / (lbM of mixture).  Stream #3 enters a separator to form a vapour (Stream #4) and a liquid (Stream #5).  

(a)What is the mass fraction of ammonia in Stream #3 immediately after the two streams are combined?

(b)What is the temperature of Stream #3 immediately after the two streams are combined?

(c)What is the temperature of Stream #3 at the heat exchanger exit?

(d)What is the composition of Stream #4?

[image: image2.wmf]NH3 - H2O  Mass and enthalpy balances  Fig 8.5-2 Felder and Rousseau 3rd edition

Mass balance - feed

100

kg/h Stream1

100

kg/h Stream 2

200

kg/h total

Stream 1

0.1

mass fraction NH3

0.9

mass fraction H2O

Stream 2

0.52

mass fraction NH3

0.48

mass fraction H2O

Stream 3 - mass fraction NH3

This is the mid-point of a line between the point for Stream 1 and

0.31

mass fraction NH3

Stream 2 on fig 8.5-2.  Both streams have the same flow rate

0.69

mass fraction H2O

therefore it is the mid-point.

Stream 3 - Temperature after initial mixing

80

deg F - intersection of (a) line from 0.1 mass fraction NH3 to 0.52 mass fraction NH3 and 

(b) 0.31 mass fraction NH3 on Fig 8.5-2

Stream 3 - Enthalpy after initial mixing

-25

Btu/lb(m) - intersection of (a) line from 0.1 mass fraction NH3 to 0.52 mass fraction NH3 and 

(b) 0.31 mass fraction NH3

Add 500 Btu/lb(m).  New point 475 Btu/(lb)m and 0.31 mass fraction NH3 on Fig 8.5-2

Stream 3 - Enthalpy at exchanger exit

500

Btu/lb(m) - amount of enthalpy added by exchanger

475

Btu/lb(m) - enthalpy at exit = amount of enthalpy added by exchanger - enthalpy entering exchanger

Stream 4 - Vapour composition

0.6

mass fraction NH3 - from Fig 8.5-2 Felder and Rousseau

0.4

mass fraction H2O

Stream 5 - Liquid composition

0.08

mass fraction NH3 - from Fig 8.5-2 Felder and Rousseau

0.92

mass fraction H2O

(e)What is the composition of Stream #5?












#4





 #1





#3





 #2





#5








