[image: image1.png]N

[

uOttawa

L’Université canadienne
Canada’s university




Université d’Ottawa









University of Ottawa

Faculté de genie









Faculty of Engineering

Département de











Department of 

génie chimique et biologique 






   Chemical and Biological Engineering
CHG 2317 (Sep. – Dec. 2017)

Introduction to Chemical Process Analysis 

Tutorial 10

Calculate the adiabatic flame temperature [K] of propane, C3H8 ( 0% excess air, both air and propane in the vapour phase at 1 atm and 25(C).    The experimental value is 2003 K  .  The calculated value should be greater than the experimental one.

The heat of vapourization of water is 44.03 kJ/gmol.  Use the heat capacity values given in the table below.

               CP [ kJ/(gmol K) ]

C3H8 (g)
0.0680

N2 (g)

0.0291

O2 (g)

0.0293

CO2 (g)
0.0371

H2O (g)
0.0336

H2O (L)
0.0754

[image: image2.wmf]Tutorial 10  Adiabatic Flame Temperature of Propane

deg C = (deg F - 32)*(5/9)

Experimental value =

1930

deg C   =

+ 273  =

2203

 K

Basis of Calculation = 1 gmol C3H8(g)

Heat of formation

Component

In

Change

Out

delta-Hf [kJ/gmol]

C3H8

1

-1

0

-103.8

O2

5

-5

0

N2

18.80952

0

18.80952

CO2

0

3

3

-393.5

H2O

0

4

4

-285.84

in liquid phase

Heat of reaction

Use Heats of formation

delta-H-Rx = 3 * Hf(CO2)  +  4 * Hf(H2O-L) - 1 * Hf(C3H8) - 5 * Hf(O2)

3*(393.5 + 4*(-285.84) - 1-*(103.8) - 5*(0)

-2220.06

kJ/gmol-C3H8

Heat of vapourization

4*44.01 =

176.04

kJ

Liquid phase water

Heat from 25 to 100 C

4*0.0754*(100-75)  =

22.62

J

Heat of vapourization

4*44.01 =

176.04

kJ

Vapour phase water

Heat from 100 top T(adiab)

4*0.0336*(T-100) =

0.1344

(T-100)

Nitrogen plus vapour phase combustion products (n2 and CO2)

Heat from 100 top T(adiab)

( 3*0.0371 + 18.81*0.0291) *

( T - 25)

0.658671

*(T-25)

Heat of reaction  =  Heat to increase temperature

2220.6

 =

0.6587 * 25  =

16.4675

kJ

T (deg C) =

(2220.6 +13.44 + 16.47 -22.62 - 176.04) / (0.1344 + 0.6587 ) =

2587.126

deg C

2860.126

K

Heat 

of 

reaction

at 

25 deg C

Propane Vapour

at 25 deg C

Air  at 

25 deg C

Heat liquid

H2O from

25 to 100 C

Heat gas phase

combustion

products from

25 to "T-adiabatic"

Add heat of

vapourizaion

at 100 C

Heat water vapour

from 100 C to 

"T-adiabatic"

C3H8(g) + 5 O2(g) = 3 CO2(g) +4 H2O(L)


