- Moving to another question will save this response.

### Question 1

What is the network address for the following host address: 172.16.25.130 and mask: 255.255.255.128 ?

|  |  | 172.16.25.128 |
| --- | --- | --- |
|  |  | 172.16.25.192 |
|  |  | 172.16.25.0 |
|  |  | 172.16.25.130 |

- Moving to another question will save this response.

### Question 2

How many host addresses are contained in this network address: 192.168.1.0/30 ?

|  |  | 2 |
| --- | --- | --- |
|  |  | 1 |
|  |  | 3 |
|  |  | 4 |

- Moving to another question will save this response.

### Question 3

Convert the following binary number to decimal:  0101001

|  |  | 36 |
| --- | --- | --- |
|  |  | 41 |
|  |  | 25 |
|  |  | 18 |

- Moving to another question will save this response.

### Question 4

What is the first host address for the following network address: 10.205.53.32/27 ?

|  |  | 10.205.53.32 |
| --- | --- | --- |
|  |  | 10.205.53.31 |
|  |  | 10.205.53.33 |
|  |  | 10.205.53.34 |

- Moving to another question will save this response.

### Question 5

Convert this Subnet Mask to Dotted Decimal Notation: /26

|  |  | 255.255.255.0 |
| --- | --- | --- |
|  |  | 255.255.255.128 |
|  |  | 255.255.255.192 |
|  |  | 255.255.255.224 |

- Moving to another question will save this response.

### Question 6

Convert the host portion of this IP Address to binary: 25.125.32.235/24

|  |  | :eb |
| --- | --- | --- |
|  |  | 11101011 |
|  |  | 01111101 |
|  |  | 00011001 |

7, What is the last host address for the following network address: 10.205.53.32/27 ?

|  |  | 10.205.53.63 |
| --- | --- | --- |
|  |  | 10.205.53.61 |
|  |  | 10.205.53.62 |
|  |  | 10.205.53.64 |

### Question 8

What is the purpose of the subnet mask in conjunction with an IP address?

|  |  | to mask the IP address to outsiders |
| --- | --- | --- |
|  |  | to identify whether the address is public or private |
|  |  | to uniquely identify a host on a network |
|  |  | to determine the subnet to which the host belongs |